xx-x-xx

Toinen versio(2_0) ZIPattuna jossa puhevarasto poimitaan RAMilta

levykerilyn sijasta. Nopeampi vaste DXinfoon mutta hitaassa

koneessa nykivmpi puhe.

Puheclusterin toinen (2.0) versio:

+ koko ALL.VOC ladataan kerralla muistiin ja skannataan siell

tarpeiden mukaan. Saavutetaan lhes vlitn vaste DXinfolle.

+ levykeasemia kytettess niiden "hylminen" vhenee.

- vaatii 50k enemmn rammia ja ALL.VOC ei saa koskaan olla 350k

isompi.

- Hitailla koneilla puhe hidastuu (sanavlit kasvavat) ja sana-

vleihin muodostuu "plop":eja jotka kyll peittyvt jotenkin

filtterointiin (varsinkin DAC) mutta ovat sit isompia mit

hitaampi kone.

------------

Other version(2_0) in ZIP file. All words are stored & picked

up drom RAM instead of disk. Faster response to DXinformation

but some "hickups" in speech with slow PCs.

Another version (2.0) of audiocluster:

+ Whole ALL.VOC file is loaded into memory and scanned there as

needed. That way we get almost immediate response to DXinfo.

+ If floppies are used they will work lot more seldom.

- Version needs 50k more free ram and ALL.VOC can not be over 350k

in any case.

- With slow machines speech is slower (gaps between words are longer)

and there will be audible "pop" between words. Filtering (specially

with DAC) will remove lot of them, but as slower the PC is that

bigger are "pop"s.

14-5-94

Molempien versioden(1 & 2) hardware kttelyit TNC:lle korjattu

---------

Hardware handshake (to TNC) fixed in both versions (1 & 2)

30-7-94

Puheclusterin CWidentifiointia muutettu. Antaa IDn joka 5.

minuutti. Muuten antaa infon alkuun CWll vain "i" (info).

Helpottaa tilannetta jos useita infoja saadaan perkkin.

---------

CWIdentification of Puhecluster has been changed. Gives now

ID within every 5 minutes. Otherwise gives just CW "i" from

beginning of info. Helps spooling long queues of DXinformations.

4-8-94

Infomarkkeri listty. Ohjelma lhett puheen loppuun CWll

kirjaimen "T" jos DXinfossa esiintyy merkkijono joka on

asetettu CFG tiedostoon (param. #19).

Nin voidaan merkata vaikkapa eurovihjeet asettamalla <oh2ew>

markkerijonoksi.

Markkeri on iso/pikkukirjain selektiivinen.

-------------

Added Info marker. If string given in CFG (param #19) is included

in DXinfo one CW "T" is placed at the end of DX info speech.

Marker is case sensitive.

10-9-94

Zipattuna onkin nyt vanha ykksversio.(Sanat kerilln levylt

joka DXn.) Ykksversion pivitys on lopetettu ja versio 2.1

on se johon tulevaisuudessa keskitytn (hyvksi todettu OH1RCF:ll).

Versiossa 2.1 on mahdollisuus erillisen puhetiedoston puhumiseen

kun ulkoinen kosketin sulkeutuu. Nin voidaan saada puheclusteri

tiedottamaan vaikka murtohlytyksest tai muusta "trkest"

ulkoisesta tapahtumasta.

Kaikissa tiloissa (LPT, HamCom, TestTerminal) ulkoinen puhetiedosto

voidaan aktivoida (kerran) painamalla F5 nppint.

LPT-D/A muuntimen ollessa valittu CFG tiedostossa ulkoinen puhe

kynnistyy kun LPT-portin nasta 12 kytketn maahan. Puhe toistuu

niin kauan kuin kytkent on voimassa (10sek odotusvlein).

Jotta harhakynnistyksi ei sattuisi, kytke nastan 16 ja 12 vliin

4.7k vastus. (Nasta 16 on +5V:ssa ohjelman toimiessa).

HamComm modemin ollessa valittu CFG-tiedostossa ulkoinen puhe

kynnistyy kun COM-portin nasta 8 (25nap) kytketn positiiviseen (+12V)

jnnitteeseen. Puhe toistuu niin kauan kuin kytkent on voimassa

(10sek odotusvlein).

Jotta harhakynnistyksi ei sattuisi, kytke nastan 2 ja 8 (25nap) vliin

10k vastus. (Nasta 2 on -12V:ssa ohjelman toimiessa).

Kun ulkoinen puhe loppuu ladataan taas normaalit puhetiedostot muistiin

ja toiminta jatkuu normaalisti.

Ulkoisen puhetiedoston voi tehd digitoimalla 10kHz nytteenotto-

taajuudella 8bitin nyttein halutun puheen (nen). Tiedosto pit

ksitell niin ett siin ei esiinny tavua x00h paitsi VIIMEISEN

tavuna tiedoston lopussa (trke!).

Tiedoston pituus ei saa ylitt 350ktavua.

----------------

For now on you will find older (V1.5) program from ZIP file.(the

one that collects every word of info from disk). Updates for that

program has been finished. Version 2.1 has proved to be usefull

(at OH1RCF) and future updates will point only to that one.

Version 2.1 supports now external voice file. This file is loaded

and speech from it is outputted when external switch is closed.

With that system audiocluster can tell about something important

that is happening in outside world instead of DX infos.(use your

imagination).

In all configs (LPT, HamComm, TestTerminal) external voice file

can be activated (once) by pressing key F5.

When LPT D/A is selected in CFG, external file will be activated

when pin 12 of LPT is grounded. File will be activated as long

as this contact stays on (with 10sec waiting time).

For avoid false triggers connect 4.7k resistor between pin 12

and pin 16 of LPT. (Pin 16 will have +5V during program run).

When HamComm is selected in CFG, external file will be activated

when pin 8 of COM (25pins) is connected to +12V. File will be

activated again and again as long as this contact stays on

(with 10sec waiting time between starts).

For avoid false triggers connect 10k resistor between pin 8

and pin 2 of COM (25pins). (Pin 2 will have -12V during program run).

External voice file can be made by taking 8bit samples at 10kHz

rate from voice source. File must be edited so that it does NOT

contain byte x00h in data, EXEPT at the END of file as last byte

(important!).

Length of file may not exeed 350kbytes.

15-9-94

Jos DXinfo tulee clusterilta (reaaliaikainen DX) ei puhuta alkuun

kellonaikaa +Z. Jos taas viimeisint infoa pyydetn piipill

tulee infon alkuun kellonaika +Z inforivist. (Lyhent infon

puheaikaa reaalitapauksissa).

------------

If DXinfo is received from cluster (real time) there will be no

time +Z talked out, just the DXcall and QRG.

If info is requested with 1750Hz tone the time+Z is talked as it

is in DXinfo line received previously from cluster.

18-9-94

Aloitusta muutettu niin ett ohjelma tulostaa kaikki PUHECLUS.CFGss

annetut parametrit ruutuun ja pyshtyy sekunniksi. Jos parametrit

kaipaavat lhemp tarkastelua voi pyshdyksen aikana painaa PAUSE

nppint ja saada lisaikaa tarkasteluun. ENTER jatkaa suoritusta

PAUSEN jlkeen.

Toiminnan tarkoituksena on helpottaa virheiden lytmist CFGst.

HUOM ! Jos parametriksi on annettu NONE se nkyy tyhjn rivin

tss tulostuksessa.

------------

At the start of program it will show all parameters from PUHECLUS.CFG

(while reading it). Program will wait one second after the last

parameter so that you have time to press PAUSE key if further

examination of parameters is needed. ENTER will continue after PAUSE,

as usual.

Purpose of this is to help find bossible errors from CFG.

NOTE ! If parameter is given as NONE it will show up as empty line here.

10.10.94

Tykaluohjelmiin listty SPLITVOC.EXE. Sill voi hajoittaa ALL.VOC

tiedoston osiin editointia ja uudelleenkoostamista varten. (silt

varalta ett pidt puheesta, mutta haluaisit muotoilla siin jotain

yksittist sanaa (sanoja)).

-------

New program SPLITVOC.EXE added into TOOLS section. With it you can

split ALL.VOC into partial files, each containing one word.

If you like ALL.VOC, but would like to fix one (or several) words

it is now bossible by  SPLITVOC -> AUDIC -> VOCIDX chain to split,

"edit", and recombine ALL.VOC.