LAN-LINK Version 2.34 Page 1

_______

____|__     |                  (tm)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

Before you print the file, you can get a copy in Wordperfect 5.1

format with tables and figures. See Appendix 12.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 2

Table of Contents

Introducing LAN-LINK ...................................1

Appendix 1 Features of LAN-LINK ........................2

LAN-LINK PROGRAM HIGHLIGHTS ............................3

Getting Started ........................................6

Bringing LAN-LINK Up the First Time ....................7

Using the Quick Menu ...................................8

Connecting to Other Stations ...........................8

Using Different TNCs ...................................8

LAN-LINK Memory Requirements ...........................9

PC to TNC Communications ...............................9

LAN-LINK Runs Open Loop ................................9

In Case of Problems ....................................9

Logbooks ...............................................9

How To Get an Update ...................................10

Starting LAN-LINK in a Non Packet Mode ................10

Using LAN-LINK with a PacketCluster ...................10

Simulating a PacketCluster with LAN-LINK ..............12

Using ANSI Color for Transmitting Text Color Files ....12

Appendix 2 LAN-LINK Features, Menus and Commands ......14

1.0 LAN-LINK MESSAGE HANDLING FEATURES ................14

1.1 Answering Machine ................................14

1.2 Mail Beacon (Annunciator) ........................14

1.3 How to Leave a Message ...........................15

2.0 LAN-LINK WINDOWS ..................................16

3.0 HOT KEYS ..........................................20

3.1 Alt-3 Change Foreground Color ....................20

3.2 Alt-4 Change Background Color ....................21

3.3 Alt-A Alert Call (Packet) and AMTOR ARQ Call/Talk.21

3.4 Alt-B Send Break/AMTOR Changeover ................21

3.5 Alt-C Call/Connect to Someone ....................21

3.6 Alt-D Disconnect or Receive ......................22

3.7 Alt-E Enter Call .................................22

3.8 Alt-F Flush TNC Buffer ...........................22

3.9 Alt-G Grab Call and Frequency from

DX Alert Message ................................23

3.10 Alt-H Help Information for Function Keys ........23

3.11 Alt-J Jump to DOS ...............................23

3.12 Alt-K PacketCluster DX Alert ....................23

3.13 Alt-L Log Contact ...............................23

3.14 Alt-M Message Scan ..............................23

3.15 Alt-N Next Stream ...............................23

3.16 Alt-O Turn Robot ON .............................24

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 3

3.17 Alt-P Turn Printer ON/OFF .......................24

3.18 Alt-Q Call QRZ ..................................24

3.19 Alt-R Retry Call/Connect ........................24

3.20 Alt-S Scan Log ..................................24

3.21 Alt-W :QRA: .....................................24

3.22 Alt-X Exit to DOS ...............................24

3.23 Alt-Y Zap PacketCluster/RTTY Transmit "RY"s .....25

3.24 Alt-Z Zap PBBS ..................................25

3.25 Alt-= Toggle Miscellaneous Flag .................27

3.26 End "> +?" [Over] ...............................27

3.27 PgUp Scrollback (Scrolls back incoming text) ....27

3.28 The Home Key ....................................28

3.29 The Down Arrow Key ..............................28

3.30 Shift/Baudrate Control Keys .....................28

3.31 Control T (^T) ..................................29

3.32 Control-Z (^Z) ..................................29

3.33 Alt-T Talk to another station connected to

the PacketCluster ................29

4.0 FUNCTION KEYS .....................................30

4.1 Capture to Disk Toggle (F1) ......................30

4.2 Send Brag Tape and Talk (F2) .....................30

4.3 Transmit Callsign Sequence (F3) ..................31

4.4 Send Brag Tape (F4) ..............................31

4.5 Show Packet "MH" list and Point and

Shoot Connect (F5) ..........31

4.6 Auto CQ Call (F6) ................................31

4.7 Activate Type Ahead Buffer/AMTOR Relink (F7) .....32

4.8 Disconnect, Disengage or Receive (F8) ............33

4.9 Put TNC in CMD Mode (F9) .........................33

4.10 Put TNC in CONVERSE/Transmit/AMTOR FEC (F10) ....34

4.11 Alternate Function Keys .........................34

4.12 Shift Function Keys .............................34

4.13 Control Function Keys ...........................35

5.0 THE MAIN MENU .....................................36

5.1 The Quick Menu ...................................36

5.2 Auto CQ ..........................................37

5.3 Send Brag Tape ...................................37

5.4 Call Someone .....................................37

5.5 initialize TNC ...................................37

5.6 Log Menu .........................................37

5.7 Change Communications Mode .......................37

5.8 Call CQ ..........................................37

5.9 Send/Talk File ...................................37

5.10 QRZ .............................................37

5.11 Send/Talk LAN-LINK files 001..010 ...............38

6.0 THE PBBS/PacketCluster MENU .......................39

6.1 Download Text Files ..............................39

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 4

6.2 Download Binary File (Xmodem) ....................39

6.3 Download Binary File (Ymodem) ....................40

6.4 Zap PacketCluster ................................40

6.5 Zap PBBS .........................................40

6.6 Turn PacketCluster Mode ON/OFF ...................40

6.7 Send DX Alert Message ............................40

6.8 Clear All Search Flags ...........................41

6.9 Set All Search Flags .............................41

6.10 Set All CW Search Flags .........................41

6.11 Set All SSB Search Flags ........................41

6.12 Set/Clear Individual Band Search Flags ..........41

6.13 Set/Clear Current Band Search Flags .............41

6.14 Turn Auto Cluster DX Alert ON/OFF ...............41

6.15 Send DX Alert ...................................42

6.16 Send Announcement

(connected to a PacketCluster and using a

dual port TNC) ........................42

6.17 Talk to Station

(connected to a PacketCluster and using a

dual port TNC) ........................42

6.18 Set Current Band Search Flags ONLY ..............42

7.0 THE CALL MENU .....................................43

7.1 Alert Call .......................................43

7.2 ReMember Path ....................................44

7.3 Call CQ ..........................................44

7.4 Show Contents of Call Directory ..................44

7.5 Target Call ......................................44

7.6 Auto CQ ..........................................45

7.7 Toggle DX Flag ...................................45

7.8 Zap PBBS on Next Mail Beacon .....................45

7.9 Turn CQ File ON/OFF ..............................45

8.0 EDIT MENU .........................................46

8.1 Edit Brag Tape ...................................48

8.2 Edit Call Directory File .........................48

8.3 Edit Any Other File ..............................48

8.4 Answer Capture-to-disk (Ctdsk) File

(Split Screen) .........48

8.5 Answer Incoming Messages (Split Screen) ..........49

8.6 Leave Note for Someone ...........................49

8.7 Edit Capture-to-Disk file ........................49

8.8 Edit Two Files (Split Screen) ....................49

8.9 Edit Configuration file ..........................49

8.10 Edit LAN-LINK.001 through LAN-LINK.010 files ....49

8.11 Pick Capture-to-disk File to Edit ...............50

8.12 Pick Message file to Edit .......................50

8.13 Leave Messages on your local PBBS ...............50

8.14 Edit Incoming Message ...........................50

8.15 Edit LAN-LINK.OUT File ..........................50

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 5

8.16 Edit LAN-LINK.DMO File ..........................50

8.17 Pick dEmo File to Edit ..........................50

8.18 Edit CQ File ....................................50

9.0 EVENT/CLOCK MENU ..................................51

9.1 Set Alarm ........................................51

9.2 Connect ..........................................51

9.3 Display Event ....................................51

9.4 Periodic Event ...................................51

9.5 Turn Robot ON/OFF ................................51

9.6 Turn Automatic Contester ON/OFF ..................52

9.7 Turn Time Display ON/OFF .........................52

9.8 Cancel Event .....................................52

9.9 Zap PBBS .........................................52

10.0 FILES MENU .......................................53

10.1 Send Brag Tape ..................................53

10.2 File Directory ..................................54

10.3 Erase File ......................................54

10.4 Pick Capture-to-disk (Ctdsk) File to View .......54

10.5 Rename File .....................................54

10.6 Send File .......................................54

10.7 Send/Talk File ..................................54

10.8 Send Uppercase File .............................54

10.9 View file .......................................54

10.10 View Capture-to-disk (Ctdsk) File ..............54

10.11 View Incoming Message File .....................55

10.12 Print File .....................................55

10.13 View Color File ................................55

10.14 Turn Mode Files ON/OFF .........................55

10.15 Turn Expand Files ON/OFF .......................55

11.0 HELP WITH FUNCTION KEYS ..........................56

12.0 JUMP TO DOS SHELL ................................56

13.0 LAN-LINK MENU ....................................57

13.1 :QBU: External Protocol Command .................57

13.2 :QBM: ASCII File ................................58

13.3 :QSM: Message ...................................58

13.4 :QSP: Message ...................................58

13.5 :QDB: ASCII File ................................58

13.6 :QRU: Messages ..................................59

13.7 :QTA: Messages ..................................59

13.8 :QZD: Binary Files ..............................59

13.9 :QZU: Binary Files ..............................60

13.10 :QIC: Page Operator ............................60

13.11 :QMH: Monitor Heard ............................60

13.12 :QTR: Ask for Time .............................60

13.13 :QRA: Who is "ON-LINE" .........................60

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 6

13.14 Demo Mode ......................................60

13.15 Set Single/Dual PK-900/DSP 2232 In Windows .....61

13.16 Switch IN Windows (Dual port TNC only) .........61

14.0 LOG MENU .........................................62

14.1 Append From Another LAN-LINK Log ................64

14.2 Scan Log by Band ................................64

14.3 Change Logbook File .............................64

14.4 Delete Logbook File .............................64

14.5 Find Date .......................................64

14.6 Purge Log .......................................64

14.7 Help Function Keys ..............................64

14.8 Import From G3ZCZ's Contest Log .................64

14.9 Jump to an Entry ................................65

14.10 Pick a Log file ................................65

14.11 Pack a Logbook .................................65

14.12 Scan Log by Mode ...............................65

14.13 Create a New Logbook ...........................65

14.14 Print Log ......................................65

14.15 Re-index a Logbook .............................65

14.16 Sort on Date + Time ............................65

14.17 Turn Update Disk ON/OFF ........................65

14.18 Fuzzy Scan .....................................66

14.19 Turn Secondary Search ON/OFF ...................66

14.20 Log Menu Function Keys .........................66

14.21 QSL Menu .......................................69

15.0 MESSAGE MENU .....................................71

15.1 PBBS Command Files ..............................71

15.2 Current Messages ................................71

15.3 View Message ....................................71

15.4 Leave Note ......................................71

15.5 Old Messages ....................................71

15.6 Time Tag ........................................72

16.0 TNC MENU .........................................73

16.1 Set TNC Date/Time ...............................73

16.2 Flush TNC Buffer ................................74

16.3 Initialize TNC ..................................74

16.4 Show Calls Monitored List .......................74

16.5 Show Connect Status .............................74

16.6 Change CTEXT ....................................75

16.7 Activate TNC Mailbox ............................75

16.8 Deactivate TNC Mailbox ..........................75

17.0 AMSAT-OSCAR MENU .................................76

17.1 Full Duplex ON/OFF ..............................76

17.2 SAREX MENU ......................................76

17.3 SAREX Orbiter Features ..........................78

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 7

18.0 PARAMETER MENU ...................................81

18.1 Turn Beacon Mode ON/OFF .........................81

18.2 Clear Connect Counter ...........................82

18.3 TNC Custom Line .................................82

18.4 Turn LAN-LINK Beacon Control ON/OFF .............84

18.5 Turn Command Echo ON/OFF ........................85

18.6 Change Keyboard Beep Count ......................85

18.7 Set Message Count ...............................85

18.8 Turn Node Drop Link ON/OFF ......................85

18.9 Turn Printer ON/OFF .............................85

18.10 Turn QTC Snatch ON/OFF .........................86

18.11 Turn :QSP: Relay Flag ON/OFF ...................87

18.12 Turn Sound ON/OFF ..............................87

18.13 Turn Contest Mode ON/OFF .......................87

18.14 Turn DCD Flag ON/OFF ...........................88

18.15 Turn PBBS Snatch ON/OFF ........................89

18.16 Turn Time Id. ON/OFF ...........................89

18.17 Turn Automatic Capture-to-Disk Flag ON/OFF .....89

18.18 Change PBBS Parameters .........................89

18.19 Change Callsigns ...............................91

18.20 Change Directories .............................92

18.21 Enter Common Calls .............................92

18.22 Change File Names ..............................92

18.23 Change LAN-LINK Parameters .....................93

18.24 Change PC Communications Parameters ............96

18.25 Change Colors ..................................98

18.26 Change SAREX Parameters .......................101

18.27 Update Configuration File .....................101

18.28 Verify Changes ................................101

18.29 Change Scan Words .............................101

18.30 Turn Miscellaneous Flag ON/OFF ................102

18.31 Change Band/Mode ..............................103

18.32 Zap PBBS on Next Mail Beacon ..................103

18.33 PBBS Zap, Don't Disconnect After Sequence .....103

18.34 Turn Auto Answer ON/OFF .......................104

19.0 Q Codes [NC/L] ..................................104

20.0 COMMUNICATIONS MODES ............................105

20.1 VHF PACKET .....................................105

20.2 HF PACKET ......................................105

20.3 ASCII (RTTY) ...................................105

20.4 BAUDOT (RTTY) ..................................105

20.5 MORSE (CW) .....................................105

20.6 AMTOR (STANDBY) ................................106

20.7 AMTOR (MONITOR) ................................106

20.8 PACTOR (STANDBY) ...............................106

20.9 PACTOR (MONITOR) ...............................106

20.10 SIGNAL ........................................106

20.11 NAVTEX ........................................107

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 8

20.12 G-TOR .........................................107

20.13 Manual ........................................107

21.0 TERMINAL MODE MENU ..............................108

21.1 Packet Terminal Menu ...........................108

21.2 AMTOR Terminal Menu ............................112

21.3 Baudot Terminal Menu ...........................113

21.4 ASCII Terminal Menu ............................115

21.5 CW Menu ........................................116

21.6 Pactor Terminal Menu ...........................117

22.0 EXIT TO DOS .....................................119

23.0 THE ELMER MENU ..................................120

23.1 Change ELMER File ..............................120

23.2 Turn ELMER ON/OFF ..............................120

23.3 Load State Table (ELMER.QSO) ...................120

23.4 Pick File to Edit ..............................120

23.5 Edit State Table (ELMER.QSO) ...................120

23.6 Show State Table ...............................120

24.0 RADIO MENU ......................................122

24.1 Select VFO A/B .................................122

24.2 Set Radio Mode .................................122

24.3 Turn Flag QSY ON/OFF or N/A ....................122

24.4 Read VFO Frequency .............................122

24.5 Set VFO Frequency ..............................122

24.6 Speak VFO Frequency ............................122

Appendix 3 LAN-LINK Disk Files .......................124

1.0 LAN-LINK.EXE .....................................124

2.0 LAN-LINK.SYS .....................................124

2.1 CTEXT Line ......................................127

2.2 ELMER Flag ......................................127

2.3 PC Port 4 Custom Address ........................127

2.4 PC Port 4 Custom IRQ ............................127

2.5 VHF Stream Switch Character .....................127

2.6 HF Stream Switch Character ......................128

2.7 TNC Error Message ...............................128

2.8 Connect Timeout Delay ...........................128

2.9 ELMER Configuration File ........................128

2.10 Expand Transmitted Files Flag ..................128

2.11 First PacketCluster Command ....................128

2.12 Second PacketCluster Command ...................128

2.13 TNC Mailbox ON Command .........................128

2.14 TNC Mailbox OFF Command ........................129

2.15 VHF Maxframe Command ...........................129

2.16 HF Maxframe Command ............................129

2.17 Inhibit VHF Logging Flag .......................129

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 9

2.18 Inhibit HF Logging Flag ........................129

2.19 Printer Permanent Flag .........................129

2.20 Log Disk Update Flag ...........................129

2.21 Secondary Log Flag .............................129

2.22 Auto Answer Flag ...............................129

2.23 Ctdsk File Disk Update Flag ....................130

2.24 Default CW Speed ...............................130

2.25 Secondary Logbook ..............................130

2.26 BBS List Command ...............................130

2.27 BBS List Mine Command ..........................130

2.28 BBS Custom Command .............................130

2.29 BBS Bye Command ................................130

2.30 Start Up Packet Terminal Mode ..................130

2.31 Page Time (:QIC: Seconds) ......................131

2.32 PacketCluster DX Alert Tone CW Dit time ........131

2.33 Packet Cluster DX Alert Tone CW Note ...........131

2.34 Mode File Flag .................................131

2.35 BAUDOT Color Change Text String ................131

2.36 Dual Window Flag ...............................131

2.37 TNC String for Modem 9600 baud .................131

2.38 TNC String for Modem 1200 baud .................131

2.39 TNC String for Modem RTTY ......................131

2.40 TNC String for Modem 300 baud ..................132

2.41 TNC String for Modem RTTY and 1200 baud ........132

2.42 TNC String for Modem 300 baud and 1200 baud ....132

2.43 TNC String for Modem Fuji ......................132

2.44 Delay after a command line .....................132

2.45 Radio Name .....................................132

2.46 Radio Port .....................................132

2.47 Radio Port baud rate ...........................132

2.48 Radio Port Data Bits ...........................132

2.49 Radio Port Stop Bits ...........................133

2.50 Radio Port Parity ..............................133

2.51 Radio Address ..................................133

2.52 Flag QSY .......................................133

2.53 Flag DX Alert Prefix Check .....................133

2.54 Band parameters for DX Alert message log search 133

2.55 Modes for DX Alert message log search ..........134

2.56 TNC Parameters .................................134

3.0 *.RUN ............................................134

4.0 *.DBF (HF.DBF and VHF.DBF) .......................134

5.0 LAN-LINK.DIR .....................................134

6.0 CONFIG.SYS .......................................135

7.0 *.MBX ............................................135

8.0 *.BBS ............................................136

9.0 LAN-LINK.001 - LAN-LINK.010 ......................137

10.0 QBU-RX.BAT ......................................137

11.0 QBU-TX.BAT ......................................137

12.0 LAN-LINK.QTA ....................................138

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 10

13.0 QZ*.BAT .........................................138

14.0 ELMER.QSO .......................................139

15.0 LAN-LINK.OUT ....................................139

16.0 *.DMO ...........................................139

17.0 *.CQ files ......................................139

Appendix 4 Local Area Network Capability .............140

1.0 LAN Protocol (G3ZCZ Version) .....................140

2.0 Message Format ...................................141

3.0 ASCII File Uploading/Downloading .................141

4.0 Path Determination to a DX station ...............142

5.0 Binary File Transfers ............................143

6.0 Requirements for Transfer Protocols ..............143

7.0 The Zmodem Protocol ..............................144

8.0 The PCZ Zmodem Package ...........................144

9.0 Basic NC/L Dictionary ............................145

9.1 :QBM: ...........................................145

9.2 :QDB: ...........................................145

9.3 :QMH: ...........................................146

9.4 :QSM: ...........................................146

9.5 :QSP: ...........................................146

9.6 :QRA: ...........................................146

9.7 :QRT: ...........................................146

9.8 :QRU: ...........................................146

9.9 :QNO: ...........................................147

9.10 :QJG: ..........................................147

9.11 :QRV: ..........................................147

9.12 :QSL: ..........................................147

9.13 :QTA: ..........................................147

9.14 :QTC: ..........................................147

9.15 :QBU: ..........................................147

9.16 :QZU: ..........................................148

9.17 :QZD: ..........................................148

9.18 :QIC: ..........................................148

9.19 :QTR: ..........................................148

Appendix 5 Using Different TNCs ......................149

1.0 TNC2 (MFJ 1270) ..................................149

2.0 TNC1 (HK-4040) ...................................149

3.0 Kantronics KPC-2 .................................149

4.0 Kantronics KAM ...................................150

5.0 PK-232 ...........................................150

6.0 MFJ 1278 .........................................150

7.0 Heath HK-21 ......................................151

8.0 PK-88 ............................................151

9.0 PK-900 and DSP-2232 ..............................151

Appendix 6 Anomalies and Bugs ........................152

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 11

1.0 Command/Converse Mode Anomalies ..................152

2.0 Buffer Messages in AMTOR Beacon CQ Mode ..........152

3.0 FEC +? Responses in AMTOR Beacon CQ Mode .........152

4.0 Using a Monochrome Monitor .......................152

5.0 The Packet Conference Mode .......................152

6.0 Packet Conference Mode Disconnects ...............153

7.0 Strange Events ...................................153

8.0 *** DISCONNECTED .................................153

9.0 *** CONNECTED ....................................153

10.0 Greek Characters and Other Garbage in the

Incoming Screen .................154

11.0 Dumb TNC ........................................154

12.0 Intermittent Lockup .............................154

13.0 Automatic Functions Don't Work ..................154

14.0 BBS Zap Failure .................................154

15.0 LAN-LINK Forgets the Band/Power Information .....155

16.0 Zmodem Binary Transfer Problems .................155

17.0 LAN-LINK 2.32 Snatch problem ....................155

18.0 Double colon after :QTC: ........................155

19.0 Logbook 601 errors ..............................155

20.0 KAM HF Packet Problem ...........................156

21.0 ASCII ANSI color transfers ......................156

Appendix 7 Updates, Revision History .................157

Appendix 8 ELMER: An Expert System Based on

a Finite State Machine ..........158

1.0 INTRODUCTION .....................................158

1.1 Expert Systems ..................................159

1.2 The User Interface ..............................159

1.3 The Knowledge Base ..............................159

1.4 The Inference Engine ............................159

1.5 Features of an Expert System ....................159

1.6 Using an Expert System ..........................160

1.7 The Semantic Network ............................160

1.8 A State Machine .................................160

1.9 The User Perception .............................160

1.10 ELMER ..........................................161

1.11 Responses to an Input ..........................161

1.12 Syntactic Analysis .............................162

1.13 Programming ELMER ..............................162

2.0 The State Table (ELMER.QSO) File .................162

2.1 The Directory Path to the ELMER Text Files ......162

2.2 The Start Up File ...............................162

2.3 Window Colors ...................................163

2.4 The ELMER Initial State .........................163

2.5 State Machine Parameters ........................163

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 12

2.6 The State Machine File ..........................164

2.7 Example of Programming the State Table ..........165

3.0 The Stand Alone Version Status Window ............166

4.0 The Stand Alone Menu .............................166

4.1 Edit Any File ...................................166

4.2 Load State Table (ELMER.QSO) ....................166

4.3 Pick File to Edit ...............................166

4.4 Edit State Table (ELMER.QSO) ....................166

4.5 eXit to DOS .....................................166

4.6 Show State Table  ...............................167

Appendix 9 PCZ Binary file Transfers .................168

LEGEND ...............................................168

ZMODEM RECEIVE .......................................169

ZMODEM SEND ..........................................169

XMODEM VARIANTS ......................................170

ENVIRONMENT VARIABLES ................................170

"PORT & IRQ" .........................................172

"Super_Z" (tm) .......................................172

NETWORK SUPPORT ......................................173

MENU FRONTEND ........................................173

PHONE SUPPORT ........................................173

The PCZ Menu Transfer System .........................174

Appendix 10 Other PC Software

by Joe Kasser, W3/G3ZCZ .........176

1.0 PC-HAM 3.52 .....................................176

1.1 LOGBOOK .........................................176

1.2 CONTEST .........................................176

1.3 CQSS ............................................176

1.4 WHATSON .........................................176

2.0 STARTREK The Computer Program ....................176

3.0 WHATS-UP 1.40 ....................................177

4.0 ELMER 1.00 .......................................178

Appendix 11 How Shareware Works ......................179

Appendix 12 LAN-LINK 2.34 REGISTRATION FORM ..........181

Index ................................................182

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 1

Introducing LAN-LINK

LAN-LINK is distributed as a Shareware product. You may freely copy

and share the product for non commercial use, with your friends,

associates and other radio amateurs. If you decide to use the

product, you are asked to become a registered user by completing the

registration form in Appendix 12, and sending it (or a copy), and

$45.00 or equivalent in foreign currency to the author (Mastercard

and Visa accepted). All checks should be drawn on US banks.

Canadians, please use postal money order. Upon receipt of your

registration, you will receive one free update disk, telephone and

mail (electronic and regular) support. If you are not satisfied with

the program after registering it, your money will be refunded no

questions asked, for up to 30 days from the date your registration

was acknowledged.

European Radio Amateur users may register (30.00 Pounds Sterling,

includes V.A.T.) and obtain support from Terry Dansey at ReadyCrest

Ltd., PO Box 75, Chatham, Kent, ME5 9DL, England. ReadyCrest Ltd.

accepts credit cards (Access, Visa, MasterCard and Eurocard).

Telephones: Voice 44 (0) 634-687168, FAX 44 (0)634-687178, Data

(BBS) 44 (0)634-200931.

LAN-LINK may not be sold or distributed with another product without

the express written permission of Joe Kasser, G3ZCZ. Joe Kasser,

G3ZCZ will only support unmodified copies of this software.

Potential Commercial Users please contact Joe Kasser directly for

modifications and/or details of Site licensing.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 2

Appendix 1 Features of LAN-LINK

LAN-LINK is a Personal Packet Terminal Program for the TNC1, TNC2,

KPC-2, PK-88 and, most of all, a smart multi-mode digital

communications controller for the KAM, MFJ1278, PK-900, DSP-2232 and

the PK-232. LAN-LINK is designed to optimize the configuration of

the TNC in each communications mode and to provide some smart

terminal features. It takes advantage of the extra features of the

PK-232 which is made by Advanced Electronics Applications Inc. It is

designed to allow anyone to use and get the most out of their packet

TNC as well as from the PK-232, the MFJ1278, and the KAM for Morse,

ASCII, BAUDOT, AMTOR as well as Packet Radio communications without

having to keep the manual handy.

LAN-LINK is a sophisticated program. In its basic state it allows

you to use the TNC in an optimal manner. It configures the TNC (it

types the commands) for you to maximize the communications

efficiency in the communications mode of your choice. That means,

for example, when working Packet on HF you need to program the TNC

parameters to different values than you would use on VHF to make

maximum use of the mode. One significant difference is the length of

the packet itself, for the longer it is, the greater the probability

of QRM destroying it. This program will adjust the packet parameters

for you.

Since the computer is now involved, other features have been added

to simplify operation, and several features have been automated. All

these operations are performed using menus and function keys as

documented below. It will take a while to learn how to use this

program in a manner which suits you. Read this document and have

fun. After all, isn't that one of the purposes of Amateur Radio.

The LAN-LINK manual is organized by menu. It describes the

operation of each menu option and function key in each

Communications Mode.

The manual tells you what LAN-LINK can do. It does not tell you how

to use LAN-LINK.

Any Terminal Node Controller (TNC) is a complex piece of equipment.

It can operate in a Command Mode, in which you tell it to do

something, or in a Converse Mode in which you are using it to talk

to other stations. Many people confuse the two when first getting on

Packet. If you monitor the channels you will recognize Command Mode

TNC instructions on the air, and when you use the TNC you will

receive the "error" reply when you type something thinking you are

in the Converse Mode but are really in the Command Mode. LAN-LINK is

designed to make Packet operation simple using Menus and high level

(Function and Hot key combinations) commands.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 3

Morse, ASCII, BAUDOT communications modes are HALF DUPLEX. One

station is transmitting at any time. You thus have to use the

Transmit-Converse and Receive-Disconnect function keys to turn the

transmitter on and off respectively.

AMTOR is also HALF DUPLEX but has some of the attributes of full

duplex. In this Mode you control who is talking through software.

Only one station can send data at a time, but you can type

characters in to the buffer and they will be transmitted

automatically when the other station turns the QSO over to you. Once

you have linked with someone, you do not use the Receive/Disconnect

function until the QSO is over.

Both stations are communicating automatically thanks to the TNC. If

you want the other operator to respond type the "+?" key sequence

(or use the "End" Key).

Packet Radio communications on the other hand is a full duplex mode

of communications. Once you have connected with someone, you do not

use the Receive-Disconnect function until the QSO is over.

Both stations are communicating automatically thanks to the TNC. If

you want the other operator to respond type the ">" key followed by

the "Enter" Key. Better yet, use the "End" Key.

Morse, AMTOR, ASCII and BAUDOT are character mode communications in

that each character you type is sent as you type them. In Packet

Communications, the TNC stores them up and sends out a burst or

packet when you type the "Enter" Key or you type enough characters

to fill up a packet. Studies have shown that the human attention

span when sitting at the keyboard is of the order of 2 seconds. If

you are in a keyboard to keyboard packet radio connect, remember

this and don't forget to use the "Enter" Key frequently, especially

before you stop to think about what to send next.

AMTOR and PACKET are full duplex modes of communications. When you

connect with someone, there is no need to use the transmit and

receive function keys. In fact you will have problems if you do. Do

not disconnect or break the link until the QSO is over.

LAN-LINK PROGRAM HIGHLIGHTS

First packet radio software to be flown in Space.

Function key and Menu driven.

Smart PacketCluster features, i.e. automatic QSY to the

frequency of a DX Alert (if in PacketCluster mode).

Automatic search of HF logbook in response to a PacketCluster DX

Alert Message. Customizable levels of notification. Audio

Notification of Prefix status.

Start Up Communications mode is customizable.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 4

Start Up Packet terminal mode is customizable.

Allows you to search through 2 logbooks for previous contacts.

60 Different Bragtapes/files, 10 in each communications mode.

Automatic logbook entries for Pactor, Packet and Mailbox Beacon

Mode AMTOR Connects, semiautomatic logbook entries for

other modes.

Provides artificial intelligence in the shape of a second

operator (ELMER).

Logbook file is dBASE compatible and can be processed by the

LOGBOOK Package of PC-HAM by G3ZCZ for indexed listings,

tracking of DXCC and other AWARDS, etc.

Contest operation, sends standard message and automatically

increments QSO count.

Automatic optimized configuration of the TNC for each commu-

nications mode.

All mode Function key "OVER" feature (End key).

There are 10 files with fixed names (LAN-LINK.001 through LAN-

LINK.010) which may be viewed and transmitted by means of

function keys. They may also be edited from the Edit Menu.

Set up of TNC for AMSAT-OSCAR Telemetry reception.

Time display and event scheduler.

ASCII Text Editor.

Customizable Colors.

Access to the TNC Command Mode is provided in case the user

wishes to override any defaults.

CQ file.

QSL database and label printing.

In PACKET RADIO

LAN message store and forward capability.

Automatic capture to disk of all packet radio connects.

Automatic indication of the number of Packet connects.

Capable of automatic connect attempts to download a QTC from

another station in the LAN.

Capable of automatic connect attempts to a packet BBS to

download your incoming messages, when your callsign appears

on the BBS mail beacon annunciator.

Capable of automatically requesting Bulletins on subjects that

interest you from your local PBBS.

Digipeat monitoring and capture.

Alert signal to let you know when a predetermined call shows up

in a packet header on frequency.

Conference and Bridge modes in multi-connect situations.

Path determination to DX station via :QMH:.

Indicator that a specific station designated as the "target"

call connected in Packet Mode, or linked to AMTOR Beacon or

Mailbox while you were away.

Automatic NET/ROM and KA-Node path set up from directory file.

Selective answering machine and MAILBOX using NC/L command

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 5

dialogue.

Screen indication of connect by desired station (target call).

Automatic Beacon Mode CQ caller.

Automatic contest (DX-pedition) mode.

Will call CQ repetitively and either work the connect and keep

going after disconnect or signal you when a reply is

received.

Zmodem binary file transfer capability

Customizable BBS Zap Commands.

Automatic search of HF logbook in response to a PacketCluster DX

Alert Message. Customizable levels of notification. Audio

Notification of Prefix status.

CTEXT File (LAN-LINK.OUT) which can be sent to all incoming

connects.

ANSI Colors for Color "Brag Tapes".

SAREX Attack mode lets you digipeat/CQ via the SAREX station as

well as try a connect. Use the CQ feature to digipeat via

MIR, and the connect for the Space Shuttle.

Trigger to determine who else is on channel.

Single keystroke PacketCluster connect.

#�#PacketCluster DX Alert simulation in unconnected mode.

In Pactor

Smart Answering Machine.

CTEXT File (LAN-LINK.OUT) which can be sent to all incoming

connects.

ANSI Colors for Color "Brag Tapes".

Automatic logging of connects.

In AMTOR

Automatic SELCAL determination.

Automatic CQ caller. Will call CQ repetitively and signal you

when a reply is received.

LAN message store and forward capability.

Selective answering machine and MAILBOX using NC/L command

dialogue.

Function key change from monitoring FEC CQ's to QSO's in

progress (chirpcopy).

Automatic Beacon Mode CQ caller.

Automatic contest (DX-pedition) mode.

Color changes between incoming and outgoing (echoed as sent)

text.

In MORSE/BAUDOT/ASCII

Automatic CQ caller. Will call CQ repetitively and signal you

when a reply is received.

RTTY SELCAL.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 6

NAVY MARS RTTY file transmitting protocols.

Getting Started

MAKE A BACK UP COPY OF THE DISK, BEFORE YOU REMOVE THE WRITE

PROTECT TAB. If you are upgrading from a previous version of LAN-

LINK, the 2.00 SYS files are NOT compatible with earlier ones. Copy

all your current LAN-LINK files to a floppy disk and run the

SETUPLL.BAT file on this disk as instructed below.

Place the floppy disk in Drive A: Type "A:" (without the "")

followed by the "Enter" key so that you see the A:> prompt. The

floppy disk contains a batch file which will set up the correct

subdirectories on your hard disk. Type "SETUPLL", touch the "Enter"

key, and the batch file will go to work.

The Install program customizes the most often used parameters for

you. Section 18 of Appendix 2 describes each of the parameters. Read

it as you install LAN-LINK.

Install first asks you for the TNC you are using, reply with the

number associated with your TNC. If you don't find your TNC listed,

try the TNC2 option.

The main things you need to change are:

Your Callsign

Your SELCAL         (if you are using a Multi-mode TNC).

PacketCluster Call  the callsign of your local PacketCluster.

BBS Call            the callsign of your local BBS.

If you have not already connected to your local BBS, skip the next

few parameters, and re-run install at some later time when you know

which answers to reply with, or change them from the LAN-LINK

Parameter menu. These parameters are:

BBS Mail Trigger    text in the mail beacon from your local PBBS.

Read Request        the command you send to the PBBS to read a

message.

Bulletin Read       the command you send to the PBBS to read a

bulletin).

BBS Subject         the PBBS prompt for the subject of a message you

are sending.

BBS Message         the PBBS prompt for the text of a message you

are sending.

The next few parameters set up the serial port between the PC and

the TNC. Read the TNC Manual to see what the manufacturer's defaults

are. These parameters are

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 7

PC Com Port         A 0 disables serial I/O. Set to the Port number

you wish to use.

PC Baud Rate        Enter the value you want to use.

Data Bits           7 or 8. Note: 7 will inhibit binary transfers.

Parity Bit          Enter the correct letter.

Stop bits           Use 1 unless the baud rate is 300 or less.

Read Section 18 of Appendix 2 to learn what the remaining

parameters mean. You can change most of them, once you have loaded

LAN-LINK, by bringing up the Parameters Menu, and following the

prompts.

Bringing LAN-LINK Up the First Time

1    If you haven't run the INSTALL program yet, do it now.

2    Turn on the TNC.

3    Load the program by typing LAN-LINK in the way you normally

load a program.

4    WAIT until the program says it has initialized the TNC and

shows the band and power in the status window at the top of the

screen.

5    If there is no battery in your TNC do what the TNC manual tells

you to do to synchronize the TERMINAL baud rate to 1200 baud.

6    After LAN-LINK has loaded, use the Parameters Menu to change

anything you still need to customize. Use the "U" option to

save the updated file. After reading the rest of this manual,

you may want to change other defaults to set up LAN-LINK the

way you like it.

7    Use the "N" and "I" options in the TNC Menu to configure the

TNC. Then select the desired mode of operation from the

displayed menu by pressing the appropriate key.

8    Exit from LAN-LINK by using the Alt-X Key or the Esc Key

followed by the X Key.

9    Reload LAN-LINK the same way you did before. This will load the

edited LAN-LINK.SYS file with the new settings. You are now

ready to use LAN-LINK. If this is your first time, try the

Quick Menu.

10   If you are updating from previous versions the following

applies. The LAN-LINK.SYS configuration files are incompatible

with any earlier version.

If you have batteries in the TNC, make sure the TNC is set for a

terminal baud rate of 1200, and try to run the program. If you see

Greek characters or it doesn't work, remove the batteries and try

again. You can then replace the batteries and from then on the

program will be correct. Make sure you leave the TNC on when you

replace the batteries or your initialization will revert to the TNC

default settings - the batteries are there to power the TNC RAM

while your primary power is off.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 8

If you have trouble with a PK232MBX, then, with LAN-LINK running,

turn the PK232MBX off, count to ten and turn it on again. Type one *

character. If that fails, repeat it a few times. If it still fails,

open the PK232 and remove the battery jumper. Count to 10 slowly

before replacing it and try again.

Using the Quick Menu

The Quick Menu is designed for new users and provides the most

commonly used functions in a single Menu. You bring up the Quick

Menu by touching the "Escape" key to access the Main Menu and then

selecting the "A" option.

The Quick Menu is described in Section 5.1 of Appendix 2.

Connecting to Other Stations

LAN-LINK treats incoming connects and outgoing connects

differently. To connect to another station, Use the Alt-C hot key,

Function key 5, or bring up the Quick Menu using the "Esc A"

character sequence as described in Section 5.1, or by bringing up

the Call Menu via the "Esc C" character sequence.

DO NOT TYPE "C Callsign" in the command mode. If you do, the connect

will still take place (the TNC won't know if you or LAN-LINK issued

the command) but LAN-LINK will treat it as an incoming connect and

issue the "[ZCZ] LAN-LINK 2.32>" handshaking signal (if the LAN-LINK

flag is in its default, or enabled state). If you connect to a PBBS

or a Node this way, you will get an error message back from the PBBS

or node, since it does not recognize LAN-LINK's handshake.

When you use the menu, Alt-C or F5 approach you can also make use

of the "path memory" feature in the LAN-LINK.DIR file.

Using Different TNCs

There are differences between the command dialogue and the features

offered by the PK-232, DSP-2232, PK-900, PK-88, TNC2, KAM, and

MFJ1278. LAN-LINK makes use of many of them. For example, the KAM

shows the HF and VHF packets as if they were separate streams. LAN-

LINK detects them and displays the different headers in different

colors. As this feature is not present in the PK-232, LAN-LINK does

not offer it for PK-232 users. From time to time, as each

manufacturer updates the firmware (EPROM) in the TNC, LAN-LINK may

be updated and new features added.

LAN-LINK is normally loaded by typing LAN-LINK followed by the

"Enter" key. In this instance, LAN-LINK gets its configuration

information from the LAN-LINK.SYS file. LAN-LINK.SYS is the default

SYSTEM or configuration file. If you have more than one TNC you may

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 9

want to set up different configuration files for each TNC.

To load LAN-LINK with the optional configuration file, type "LAN-

LINK" followed by the name of the file (without the filetype) on the

command line then press the "Enter" Key. For example, if you have a

PK-232 and a TNC2 and occasionally use the TNC2, set up the LAN-

LINK.SYS file for the PK-232. Then type "LAN-LINK" to run the

program normally. On the rare occasions when you want to use the

TNC2, you may type "LAN-LINK TNC2" to run LAN-LINK with the TNC2.SYS

configuration file. Make sure you configure that TNC2.SYS file

before you put any signals on the air.

LAN-LINK Memory Requirements

LAN-LINK 2.32 requires a minimum of 450k of RAM. You will need more

RAM if you wish to use the Editor. The Editor files will each take a

maximum of 64K additional RAM. If your memory is marginal, you may

get an error when you try to log a station or edit a file/message.

PC to TNC Communications

All communications between the computer and the TNC normally use

the 8 BIT NO PARITY and 1 stop bit RS-232 Serial Port configuration.

If you want to limit the character set to the 7 bit ASCII set, set

the TNC parameters accordingly. The baud rate defaults to 1200, but

you may set it to 2400, 4800 9600 or even 19200 if your CPU clock is

fast enough and the TNC will respond correctly.

LAN-LINK Runs Open Loop

IF YOU USE LAN-LINK, YOU STILL MUST READ THE TNC MANUAL. WARNING

THIS PROGRAM OPERATES IN AN OPEN LOOP MODE. IT REMEMBERS WHAT

INSTRUCTIONS OR COMMANDS IT GAVE THE TNC. IF FOR SOME REASON, THE

TNC IS NOT SYNCHRONIZED TO THE PROGRAM YOU WILL NOT GET CORRECT

DISPLAYS ON THE SCREEN.

In Case of Problems

In case you have problems, always check the default settings using

the "Verify Changes" option of the Parameters Menu. If you work CW,

read about the XMITOK flag, and set it for the way you have wired

your TNC to the radio. Problems can be caused by incorrect settings

of the TNC parameters. If all else fails, (hard) reset the TNC. Make

sure XFLOW and FLOW are OFF.

Logbooks

LAN-LINK uses three dBASE 3 compatible logbook files. One is used

for VHF packet logging (VHF.DBF), the second for all HF modes

(HF.DBF) and an optional third logbook designated as a secondary HF

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 10

logbook. See Appendix 2, Section 14 for more information.

How To Get an Update

There are two ways to get an updated version, as follows. When you

register your copy of LAN-LINK, if this version is still current,

your registration will be acknowledged, and the following upgrade

will be sent to you automatically at the time it is released (in

this case, if you move, make sure you notify me of your new

address). If a later version exists at the time that you register,

it will be sent to you when your registration is acknowledged and

you will not be put on the list for an automatic upgrade.

If you are not on the list for an automatic upgrade (i.e., you

received a disk in the mail as a result of your registration), then

when you hear that a new release is out, collect together at least

300 kBytes of traffic on your local VHF/UHF LAN by running LAN-LINK

with the capture-to-disk activated when you are not connected. If

you can add to that, connect to your local PBBS and dump all current

bulletins with full headers showing date/time into your capture-to-

disk file. If necessary compress the file (ZIP preferred), and send

in the disk in a mailer together with a label addressed to you, and

sufficient return postage, and the upgrade will be yours.

Alternatively, you may either download the new version from a land-

line BBS, get it from a friend, or send in $5.00 and remind me what

size disk to send you.

Starting LAN-LINK in a Non Packet Mode

You may start LAN-LINK in a non packet mode, such as Pactor or

Baudot from the DOS command Line. If you wish to do so, the format

of the command is 'LAN-LINK SYSFILE MODE', where SYSFILE is the name

of the LAN-LINK.SYS file or its equivalent, and MODE is the letter

used in the Communications Menu to put the TNC in the particular

mode (See Appendix 2 Section 20). For example, to start LAN-LINK in

AMTOR from DOS, type 'LAN-LINK LAN-LINK A'. To start up in Pactor,

type 'LAN-LINK LAN-LINK P'.If you are using a different

configuration file, such as HF.SYS, type 'LAN-LINK HF A' or 'LAN-

LINK HF P'.

Using LAN-LINK with a PacketCluster

Connect to the PacketCluster using the Alt-Y key. When the

connection is achieved, and the PacketCluster sends you a ">"

character at the end of a line of text, LAN-LINK will issue the

first cluster command, followed by the second one, to the

PacketCluster and return to manual operation. The default commands

are "SHOW/WWV/1" command followed by a "SHOW/DX". Note:

PacketCluster is a specialized PBBS and conference node by Pavillion

Software for DX chasers.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 11

LAN-LINK also sets an internal PacketCluster Mode Flag. When this

flag is set, LAN-LINK will interpret DX announcements, check your hf

logbook and alert you to the status of that prefix in both sound and

color. If you want LAN-LINK to react to the DX Alerts and search

your HF logbook, to see if you have QSLd or Worked or Need the

prefix, customize the Packet Cluster DX Alert Mode in line 112 of

the LAN-LINK.SYS file to your liking. The parts of the DX Alert

message will be shown in different colors.

Options in the PBBS/PacketCluster Menu allow you to customize the

check to set up which bands and which modes are checked. For example

if you never work the 160 Meter band, you may customize LAN-LINK to

ignore 160 Meter DX Announcements.

The results of a logcheck show up at the end of the DX Alert

message. LAN-LINK also changes the current band (see status window)

to the band of the call shown in the DX alert.

The displays and log check function are slightly different in the

Contest and Non contest modes of operation.

In the contest Mode, LAN-LINK checks the logbook for the complete

callsign. The meaning of the character shown at the end of the DX

Alert line are as shown below.

Letter    Meaning

--------------------

B    worked on the same band

W    worked on another band

Q    the callsign has been QSLd

-    the prefix was not found (check mark)

In the non contest Mode, LAN-LINK checks the logbook for the prefix

(2 or 3 letters) of the callsign. The meaning of the character shown

at the end of the DX Alert line are as shown below.

Letter         Meaning

-------------------------------------

W    worked on the same band

w    worked on another band

Q    the prefix has been QSLd on the same band

q    the prefix has been QSLd on another band

-    the prefix was not found (checkmark)

In both contest and non contest modes, a blank space at the end of

the line means that the prefix was not checked.

You may use the Alt-G key to grab a DX Alert from the incoming or

scrollback windows. LAN-LINK will set the Radio to frequency and

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 12

post the call. This is similar to the action of F10.

You can also enable an automatic QSY to the frequency of a DX Alert

via the Radio Menu.

Simulating a PacketCluster with LAN-LINK

The PacketCluster is a very useful tool for those of us who are

interested in working DX. However, many radio amateurs live in areas

which are not served by PacketClusters. LAN-LINK 2.34+ allows these

"deprived" hams to enjoy the DX-alert feature of the PacketCluster.

If you send a DX-alert when connected to another station, LAN-LINK

assumes you are connected to a PacketCluster and transmits the DX-

alert message in the format expected by the PacketCluster. If you

are non connected LAN-LINK transmits the DX-alert as an unconnected

packet in the same format as transmitted by the PacketCluster. The

message is repeated a number of times depending on the value stored

in the CQ Lines parameter (Section 18.23.5). To make sure everyone

in the group sees the message, set LAN-LINK to digipeat through a

well sighted station (e.g. DIGICALL). For example, in the command

mode set the TNC 'UNP DX via DIGICALL'. The turn on the

PacketCluster mode (BBS Menu option 'K') and enjoy the DX alerts.

Using ANSI Color for Transmitting Text Color Files

ANSI control codes are designed to allow you to control the display

at the other end of the link. LAN-LINK 2.10+ reacts to some ANSI

control codes including the color codes. This feature allows you to

send colorful bragtapes and other files using the text mode graphics

of the PC character set when operating packet or Pactor. The feature

was updated in LAN-LINK 2.30 to also react to cursor move commands

to allow transmission of dynamic pictures.

ANSI color codes are a sequence of characters beginning with the

'Escape' Character (represented as "Esc" or "^[") followed by a [

character, one or two numbers follow and the sequence terminates

with the letter "m". Typical examples are 'Esc [37m' and 'Esc [40m'.

To save time, several codes may be combined using the semi-colon

character as in 'Esc [37;40m'.

The ANSI color control code sequences are shown in the following

table. If you are using the LAN-LINK editor, you must use the '^P^['

pair of keystrokes to enter the 'Escape' character into the file,

which will then show up in white instead of the customary yellow.

First Digit    Second Digit           Function

1             None                Sets Lighter/bolder foreground

color

3             0-7                 Changes foreground color see

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 13

below

4             0-7                 Changes background color see

below

5             None                Start Blinking

7             None                Start Reverse Video

0             None                Changes to Terminal's low text

color, clears blinking, reverse

video or bold colors

The color values for the secondary digits are as follows.

Digit     Foreground Color    Background Color

0             Black               Black

1             Red                 Red

2             Green               Green

3             Yellow              Brown

4             Blue                Blue

5             Magenta             Magenta

6             Cyan                Cyan

7             White               Light Grey

When transmitting lines of color, the ANSI control codes add

characters to the line, so your line will probably contain more than

80 characters. Make sure you set the correct TNC parameter (i.e.

SCREENLN) to 255 to ensure that the TNC doesn't add extra 'CR/LF'

codes to your picture and mess it up.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 14

Appendix 2 LAN-LINK Features, Menus and Commands

LAN-LINK provides an interface between you and the TNC. It works in

a client-server mode. It provides you with menus, and then sends the

TNC the commands that you would have had to type to exercise the

particular option. LAN-LINK also provides smart features to smarten

up your packet radio operations.

This appendix contains a summary of the commands and menus

available to you in LAN-LINK. The commands are organized by Menu.

Some function keys provide you with short cuts so you don't have to

go through the entire menus. These function keys are described

below.

1.0 LAN-LINK MESSAGE HANDLING FEATURES

This section describes the mail box and message handling features

built into LAN-LINK.

1.1 Answering Machine

LAN-LINK contains an AMTOR (Robot beacon CQ mode only) and PACKET

Mode SMART "answering machine" facility. You can leave messages on

your disk (in the same directory as the LAN-LINK.* files, if you are

using a hard disk), for different stations. When someone connects to

you, if you left a message for him, he (or she or even it as the

case may be) and only that station will receive it automatically. No

one else will normally be able to download that message.

1.2 Mail Beacon (Annunciator)

To ensure that people know that you have left a message for them a

"MAIL for" list is loaded into your Packet Beacon and transmitted

every 30 minutes (Refer to the BTEXT command in the TNC manual) as

":QTC:" followed by a list of calls. If no mail is pending, or the

only message in your system is one addressed to you, the beacon

transmissions are inhibited. This conforms to good operating

practice on crowded channels (at least inhibiting the beacon does).

The mail beacon in the AMTOR Mode is transmitted as part of the

beacon AUTOCQ message. It is automatically updated when a message is

transmitted. If you don't set the beacon (see below) you will not

transmit a mail beacon text in your AMTOR CQ message.

The Mail beacon text is set up either when you tell the computer to

set it up or when you disconnect a packet QSO from another station.

In this manner you may update the list while the program is running,

for example, by using Borland's Sidekick or the included Editor, to

write a message, or preferably the NOTE feature.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 15

Any station using LAN-LINK can be configured to automatically

attempt a connect (QTC-Snatch) when seeing their callsign in someone

else's LAN-LINK :QTC: beacon mail message list.

If for some reason the message got lost (e.g. because the link was

marginal), :QSM: will get you a repeat of that message.

1.3 How to Leave a Message

You can use the "note" feature or type the message as a non-

document file on the included Editor, or if you use another word

processor, you must leave the message as an ASCII text file. In that

case, you just name the message file as the callsign of the station

to whom you wish the message to be sent at connect time. You must

also give it the filetype of "OUT".

For example, a message for G3ZCZ would be stored on the disk as a

file named "G3ZCZ.OUT", and similarly a message to 4X6AA would be

stored on disk as "4X6AA.OUT".

Once the message has been sent, an entry to that effect will be

made in the LAN-LINK.RUN data capture file. The message file will

then be renamed and given the file extension ".OLD". At that time

any previously existing mail file with the file extension ".OLD"

will be deleted. This stops the addressee receiving the message on

successive connects if you can't delete it for some reason (you

forget) while at the same time, the message is still available to

you in case you need it.

You should use a separate utility program if you want the calls in

the mail beacon sorted. You may, for example, wish to sort them in

alphanumeric order, or in the order of the date that they were left

in your answering machine.

Incoming messages triggered by the QTC-Snatch will be left in a

message file named as YOURCALLSIGN.OUT, which can be scanned by

using the Alt-M Key combination. If you have non LAN-LINK friends,

train them to trigger the QTC-Snatch by simulating a beacon, and

prefixing and ending their text with lines containing the ">" as the

only character. Better yet, give them a copy of LAN-LINK and tell

them to use it and register it.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 16

2.0 LAN-LINK WINDOWS

LAN-LINK displays information in different windows as shown in

Figure 2. The window at the top of the screen is the status window.

The big blank window in the center is the incoming text window. The

lower third of the screen is the outgoing text window. All outgoing

text are echoed in this window. The bottom line is a prompt window.

Figure 2 Typical LAN-LINK Windows

#�##�##�##�##�##�##�##�##�##�##�#G8BTB's LAN-LINK 2.32 (c) G3ZCZ 1988-1994#�##�##�##�##�##�##�##�##�##�##�##�##�##�#

#�# PACKET     1200W  TRFC                              #�#  03:19:11 #�#

#�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�#

#�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�#OUTGOING TEXT#�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�#

#�#                                                                 #�#

#�#                                                                 #�#

#�#                                                                 #�#

#�#                                                                 #�#

#�#                                                                 #�#

#�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�##�#

:Esc=Menu FK:1=Ctdsk 3=Calls 5=MHlist 6=aUtoCQ 8=Disc 9=Cmd 10=Conv

The indicators in the status line depend on the state of the LAN

Link and are described in the sections that deal with the functions

they are associated with. Some typical status window displays are

shown below.

2 Meters  2 Watts PACKET 1200 W [R] TRFC ->N4QQ

20 Meters 100 Watts MORSE TEST[4]

20 Meters 100 Watts PACKET 300 N TRFC

CAPTURE TO DISK   PACKET 300 N [C1][Z] TRFC ->N4QQ

If the automatic Capture-to-disk is turned off, a yellow bug will

be flashing on the left side of the line.

If the Printer Flag is set, a yellow [P] will be flashing on the

left side of the line.

If the miscellaneous flag is set, an asterisk "*" followed by two

numbers and a "]" will be displayed. The first number is the PBBS

Sequence State, the second is the Connect Sequence State.

These numbers reflect the internal state of LAN-LINK and should be

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 17

quoted when describing problems.

If a message transfer is taking place, the callsign of the station

involved will be displayed.

When the Capture-to-Disk is turned on, the words "CAPTURE TO DISK"

will be displayed. If it is turned off, the operating band (Meters)

and power (Watts) will be shown. If a Zap PBBS is in operation, the

Bulletin Request Count will then be shown in square brackets ("[]").

The communications mode will be shown next, it may be one of the

following;

PACKET AMTOR AMTOR-(MAIL) AMTOR-FEC

BAUDOT ASCII MORSE NAVTEX

SIGNAL ???? DEFAULT {boot up}

A flashing yellow musical note following the Communications Mode

indicates that the sound has been inhibited.

The next number to be displayed is the RF communications baudrate.

The letter "W" or "N" indicates that the Wide shift or the Narrow

shift has been selected. A "down arrow" will be shown if the shift

is normal. If the shift is reversed a flashing yellow "up arrow"

will be displayed.

In the BAUDOT mode, "USOS" will be displayed if unshift- on-space

is selected.

If the Contest Mode is selected, the word "TEST" will then be

displayed followed by the Contest QSO number in "[]".

In certain Robot and AutoCQ states, namely while the computer is

waiting for a reply, a flashing cumulative CQ count will be

displayed next.

If the RTTY SELCAL is active, the word "SELCAL" will be displayed

next.

If the Navy MARS BAUDOT RTTY protocol is enabled, a "[n]" will be

displayed.

In the packet and AMTOR Robot modes, if at least one station has

connected or linked, a flashing yellow "[C]" will be displayed next.

The number in the square brackets with the "C" is the number of

stations that have connected/linked. If the target call station was

one of those station, a happy face will also be displayed in the

square brackets.

If at least one QTC snatch has taken place a flashing yellow "[Q]"

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 18

will be displayed next. The number in the square brackets is the

number of QTC Snatches that have taken place.

In the packet communications mode, if a Zap PBBS, a QTC Snatch or a

PacketCluster Zap is in progress a flashing yellow "[Z]""will be

displayed next. However, if LAN-LINK is set to stay connected when

the sequence ends, the flashing indicator will be a "{Z}" instead of

"[Z]".

If the PacketCluster DX Alert Flag is enabled a flashing yellow

"[K]""will be displayed.

In the packet or AMTOR communications mode, a "[R]" will be

displayed next if LAN-LINK is configured for store and forward

(:QSP:).

In the packet and AMTOR communications modes, if the beacon mode is

enabled a "[B]" will be displayed next. If someone has connected and

sent a :QRT: sequence, the "[B]" will be flashing. The "[B]" will be

followed by a "[M]" if there are messages pending. The number in the

square brackets with the "[M]" is the number of messages (*.OUT

files) pending.

In the packet communications mode, if the SAREX mode is enabled, a

"[S]" will be shown next. The "[S]" will be flashing while capture

to disk is in progress. If SAREX packets have been copied a happy

face will be displayed in the square brackets together with the "S".

If the SAREX Orbiter robot is active a flashing ">>" will be

displayed.

In the packet communications mode, if the digipeat Detect mode is

enabled, a "[D]" will be displayed next.

In the packet communications mode, if connected and ELMER is

active, an "[E]" will be displayed. The number with the "E" shows

ELMER's state.

In the packet communications mode, the terminal sub-modes are then

displayed as follows;

SOLO This lets you only see messages addressed to you.

CQ/B This lets you see beacons and CQ calls as well.

TRFC This lets you see packets containing messages as well.

EVRY This lets you see everything on channel, including the link

control packets.

MAIL This lets you monitor packets from a specific station. In the

PK-232, you can do it without displaying the headers.

The next item to be displayed is a callsign, under the following

conditions: the call of the station connected with, will be prefixed

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 19

by a "*>" when connected, and by a "->" when a connect request is in

progress. If a disconnect is in progress, the callsign will be

replaced by a "$$$$". The callsign will be seen as flashing when a

connect is in progress.

In the MAIL mode, the callsign of the station that you are "reading

the mail" on will be displayed. Otherwise, the callsign displayed

will be that of the one you are connected with.

If an Alert Call has been entered, it will be displayed next.

Similarly, if a Target Call has been designated, the target call

will be displayed next. The display will be slightly different of

the calls have been heard/worked. A "!!>" before the flashing call

indicates that the Alert function is active. A "<->" before the

Alert Call signifies that packets to or from that call have been

heard on frequency. A smiling face in front of the call, signifies

that it has been worked. A "->" is displayed before the Target Call.

The next items to be displayed are the packet multi user mode

displays. If the conference bridge is up, a flashing yellow bridge

will be shown. If only the conference mode is set, the letter "C"

will be flashing, if the basic multi user mode is set, the word "M"

will be seen. The callsign will be prefixed by the I/O channel

number/letter, and will be in the color allocated to the traffic on

that channel. The number shown after the flashing letter is the

number of streams that LAN-LINK thinks that you are connected on.

In the non packet modes, the callsign displayed will be that of the

station being worked (entered by the AMTOR robot, the automatic CQ

call recognition, or by the operator using the Alt-C or Alt-E keys).

In AMTOR, the SELCAL of the other station will also be displayed

sometimes, usually if you entered the call.

An "[L]" will be displayed in the non packet modes when a callsign

is entered. The "[L]" will flash to remind you (to log it) until the

QSO is logged.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 20

3.0 HOT KEYS

LAN-LINK contains a number of "Hot" keys to allow you to perform

common operations with a minimum of key strokes. The keys perform

related but slightly different tasks in each communications mode. A

list of those available is shown in Table 3-1. Should you require

any more hot keys, I suggest you use an external program with a

Keyboard Macro capability such as Borland's Superkey. Consider the

action of each key in detail.

Table 3-1 LAN-LINK Hot Keys

HOT  PACKET         AMTOR          BAUDOT & ASCII CW

KEY

3              Change foreground color

4              Change background color

A    Set ALERT Call ARQ Call/Talk       -         -

B    Send "Break"  Force Changeover    -         -

to TNC

C    Connect        ARQ Call       Call           Call

D    Disconnect     Toggle         Receive      Disengage

Receive Modes                Speed Lock

E    Enter Call     Enter Call/Selcal  Enter Call Enter Call

F                   Flush TNC Buffer

G    Grab call and frequency from DX Alert

H          Display Function key Information Screen

J                   Jump to DOS

K              PacketCluster DX-Alert

L              Log Entry

M              Scan Message

N   Next I/O Stream -                   -         -

O   Turn Robot ON   Turn Robot ON       -         -

P                        Turn Printer ON/OFF

Q                             Call QRZ

R                        Retry Last Connect/Call

S                        Scan Logbook

T    -         Talk in PacketCluster (PK-900/DSP-2232 only)

W    :QRA:     -         -              -         -

X                        Exit to DOS

Y    Zap PacketCluster             Transmit RY's

Z    Zap PBBS  Drop Link              Receive     -

after sending            mode after

contents of buffer       sending contents

of buffer

3.1 Alt-3 Change Foreground Color

This key combination allows you to change the foreground color of

the outgoing text. It will transmit the ANSI color sequence to

change color.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 21

3.2 Alt-4 Change Background Color

This key combination allows you to change the background color of

the outgoing text. It will transmit the ANSI color sequence to

change color.

3.3 Alt-A Alert Call (Packet) and AMTOR ARQ Call/Talk

In the Packet Communications Mode, this key combination performs

the same task as the Alert Call option in the Call Menu.

In AMTOR, this key combination will attempt to send an ARQ call to

the desired station, transmit the callsign sequence when linked and

allow you to continue typing text (i.e. it will not transmit the

"+?" characters). Use this command option to log on to an APLINK

PBBS.

3.4 Alt-B Send Break/AMTOR Changeover

In the Packet Communications Mode, activating this key combination

sends a "BREAK" signal to the TNC. Use this key combination to get

out of the TNC's transparent mode.

In the AMTOR Communications Mode, use this key combination to force

a changeover. You can type text into the outgoing buffer and then

force a changeover without losing that text.

3.5 Alt-C Call/Connect to Someone

This key combination works in almost the same manner in both the

packet and non packet communications modes as described in the

following sections.

3.5.1 Non Packet Modes

You use this key combination to call a station heard on the band.

In AMTOR the link request will be sent in the ARQ Mode to the SELCAL

of the desired station. (Note that this option sends the "+?" after

the callsign sequence).

3.5.2 Packet Mode

You use this key combination to attempt a connect with another

packet station. In a multiconnect situation the LAN-LINK will

automatically select a free I/O channel to try the connect.

3.5.2.1 Names, Handles and Paths: If you so desire, you can create a

directory file (default name is LAN-LINK.DIR) and keep a list of

names and calls. The computer will look up a name and call the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 22

station by its callsign. For example if you put "Joe G3ZCZ" as a

line in the file, when you tell the computer to connect to Joe, it

will try to connect to G3ZCZ. If you enter "4X4HF 4X4HF V 4X1AA,

4Z4ZB, 4X4IL" on a line in the file, you only need to type "4X4HF"

to set up the correct connect path.

You can also use this feature to automate a NET/ROM and/or a KA-

Node path connection.

3.5.2.2 Loop Backs: In the Packet Communications Mode, if you want

to loop back through someone else you would have to type "C YOURCALL

VIA OTHERCALL" to the TNC. For example if I wanted to loop back

through 4X6AA, I would have to type the command "C G3ZCZ VIA 4X6AA"

to the TNC.

In LAN-LINK you just have to enter the "/" character followed by

the call of the station you want to loop back through. For example

if I wanted to loop back through 4X6AA, I just need to enter

"/4X6AA" as the call to connect to at the correct prompt, and LAN-

LINK will tell the TNC to try to connect to G3ZCZ via 4X6AA, namely

issue the command "C G3ZCZ VIA 4X6AA" to the TNC.

3.6 Alt-D Disconnect or Receive

This key combination performs the same task as Function Key 8 (F8)

EXCEPT in MORSE and AMTOR.

In the Morse communications mode, this key disengages the receive

speed lock in the PK-232 to unlock the PK-232 from the last signal

copied. It lets you lock onto another signal faster.

In AMTOR this key toggles the receive mode between ARQ and FEC

signals (QSO's in progress and CQ calls).

3.7 Alt-E Enter Call

This key combination lets you enter the call of any station calling

you into the program. If you type the Enter Key in response to the

prompt, you can later use Function Key (F3) to transmit the sequence

"DE YOURCALL". You may also use the Retry feature to call that

station.

In AMTOR, you use the feature to enter the callsign and SELCAL of

the station you are listening to and may want to call later.

3.8 Alt-F Flush TNC Buffer

This key combination performs the same task as the Flush TNC

Outgoing buffer option in the TNC Menu.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 23

3.9 Alt-G Grab Call and Frequency from DX Alert Message

Activate this key combination, then move the cursor to the line

containing the DX Alert message and touch the 'Enter' key. LAN-LINK

will then parse the line and extract the callsign and frequency.

3.10 Alt-H Help Information for Function Keys

This key combination lets you display a brief reminder of the

action of each hot key. It performs the same function as the "H"

option in the Main Menu.

3.11 Alt-J Jump to DOS

This key combination lets you "Jump to a DOS Shell". It performs

the same function as the "J" option in the Main Menu.

3.12 Alt-K PacketCluster DX Alert

This key combination initiates the PacketCluster DX Alert sequence

described in the BBS Menu.

3.13 Alt-L Log Contact

This key combination appends the callsign for the current QSO to

the Log.

3.14 Alt-M Message Scan

This key combination performs the same task as the Message Scan

choice in the Message Menu.

3.15 Alt-N Next Stream

In the Packet Communications Mode, if you are using the multiple

connect capability of LAN-LINK, this key combination prompts you to

change I/O streams, and display the state of the I/O streams,

showing which ones were last used to send or receive something. To

change I/O streams, move the cursor to the desired stream then push

the "Enter" Key. The calls in the Next Stream window update when

stations connect and disconnect. In the event of the window not

being correct, you may manually enter or delete calls using the

"Insert" Key to enter a call, and the "Delete" Key to remove a call.

The data about the streams will be shown in the same colors as the

multi-stream text colors.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 24

3.16 Alt-O Turn Robot ON

This key combination performs the same function as the Set ROBOT

option in the AMTOR Terminal Menu. The hot key combination however,

is active in the Packet, Pactor, G-TOR and AMTOR communications

modes.

This function key also puts the Packet and AMTOR Contest operation

into automatic.

3.17 Alt-P Turn Printer ON/OFF

This key combination performs the same function as the toggle

printer on/off option in the Parameter Menu.

3.18 Alt-Q Call QRZ

This key combination transmits the sequence "QRZ QRZ QRZ DE

YOURCALL" and turns the transceiver back to receive. In AMTOR, it

sends the sequence in FEC and adds the SELCAL.

3.19 Alt-R Retry Call/Connect

This key combination allows you to retry a connect that for some

reason did not go through the first time. It saves wear and tear on

your fingers particularly if the desired connect path is via a

number of digipeat relays or through a number of Nodes.

In the non packet modes, it allows you to retransmit a call to

another station without having to re-enter the call.

If you have previously used the "Enter Call" feature to put a

callsign in the program when you hear that station, you can use the

Retry command to call him later when the QSO in progress terminates.

3.20 Alt-S Scan Log

This key combination performs the same function as the Scan option

in the LOG Menu.

3.21 Alt-W :QRA:

In the Packet Communications Mode, this key combination performs

the same task as the "W" option in the LAN-LINK menu. It puts the

TNC in the converse mode, sends a ":QRA:" and returns the TNC to the

command mode.

3.22 Alt-X Exit to DOS

This key combination performs the same task as the Exit to Dos

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 25

choice in the Main Menu. It exits the program and returns you to

DOS.

3.23 Alt-Y Zap PacketCluster/RTTY Transmit "RY"s

In the Packet Communications Mode, this key combination first

prompts you with the callsign of the PacketCluster. If you enter a

callsign, the entered callsign will become the new PacketCluster

call. LAN-LINK then issues a connect request to the PacketCluster.

When the connection is achieved, and the PacketCluster sends you a

">" character at the end of a line of text, LAN-LINK will issue the

first cluster command, followed by the second one, to the Packet-

Cluster and return to manual operation. The default commands are

"SHOW/WWV/1" command followed by a "SHOW/DX". Note: PacketCluster is

a specialized PBBS/conference by Pavillion Software for DX chasers.

In the RTTY communications mode, this key combination causes a

sequence of "RY"s to be transmitted.

3.24 Alt-Z Zap PBBS

In the Packet Communications Mode, the Alt-Z key initiates a PBBS

Mail Snatch. You use this key if you connect to a PBBS before you

hear its beacon, or if you have disabled the PBBS Mail Snatch

feature. If you are connected to a PBBS, the key must be used BEFORE

the first ">" is received. The first ">" triggers the sequence. If

you have already received that first ">", send the PBBS any command,

and the sequence should begin when the PBBS signals that it has

completed processing that command. If you are not connected, LAN--

LINK will first try to connect to the PBBS, then perform the

sequence.

When you use this command you will be asked if you want to upload

or download. If you want to download, just push the "Enter" Key. A

Download (Mail from the PBBS to you) will automatically be followed

by an Upload if the *.BBS file exists.

The download default sequence performed by the PBBS Mail Snatch is

as follows:

RM   To read your messages, (Configurable)

L    To let you know what is new on the PBBS,

LM   To list ALL your messages on the PBBS,

B    To log you off the PBBS.

LAN-LINK will wait for the ">" as the last character of the line of

text being received from the PBBS, before issuing the next command

in the sequence. Your messages will be placed in a message file

YOURCALL.OUT as if someone else had left a message for you.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 26

The "RM" may be customized in the LAN-LINK.SYS file to "RN" or "VM"

depending on the software in your local PBBS, and, on your

preference.

During the time that the PBBS is responding to the "L" command with

a list of new messages, LAN-LINK is scanning the lines containing

the messages to see if it finds one containing a Scan Word. If it

does find a Scan Word, LAN-LINK will change the color of the line,

and queue a request to download that message.

The PBBS Mail Snatch can be initiated in two ways. It can be

initiated when your incoming screen displays a "Mail" beacon

(received from your local PBBS) with your call in the lines

following the header, or by LAN-LINK's Zap PBBS function (this key

combination). The automatic PBBS Zap will not take place if LAN-LINK

is in the multi-user mode.

In the event of a PBBS Mail-Snatch, if the *.BBS file exists for

that PBBS, the file will be processed and the contents sent to the

PBBS. Thus if you don't get a chance to send the mail before LAN-

LINK downloads the mail from the PBBS, it will send it for you.

When the PBBS has accepted the commands the name of the file is

changed from "*.BBS" to "*.n", where "n" is a number increasing

sequentially each time the function is performed from a base value

set up in the LAN-LINK.SYS file. For example, the file name KE8X.BBS

will be changed to KE8X.001 the first time it happens. In this way,

you will have a copy of the outgoing file on your disk until you

choose to delete it.

In the real world however, message formats are nonstandard, and can

contain any set of characters. The algorithm developed here

transmits the next line in the *.BBS file when LAN-LINK sees a

"trigger" character string unless one of the "inhibitor" character

strings is also present on the line of the message text received by

LAN-LINK.

These "trigger" character strings are ">" (as the last character of

the line of text), and the customizable ones such as "Enter",

"Subject", "Sj:" and "Msg:". The "inhibitor" character strings are

"R:", "<" and "Message" ("R:" as in routing headers and "<" and

"Message" as in Message-Id: <message number> in message headers).

This should cover most simulated manual PBBS message reading

(WA7MBL and W0RLI/VE3GYQ) and avoid false triggering due to ">"s

showing up in message and routing headers.

When LAN-LINK finishes processing the *.BBS file, it will send a

"B" command to the PBBS to terminate the session.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 27

This PBBS Zap feature is designed to minimize the time you spend

connected to the PBBS by allowing you to scan the message list when

automatically receiving your mail, prepare your replies, and

download requests off-line and then later read the incoming messages

or files without tying up the PBBS. For this reason, it thus

disconnects you from the PBBS. You can, however, override the

automatic disconnect command for the duration of the connection

using the "F" option in the Parameter Menu.

3.25 Alt-= Toggle Miscellaneous Flag

This key combination performs the same function as the "+" option

in the parameters menu.

3.26 End "> +?" [Over]

In the Packet Communications Mode this command sends your callsign

followed by the ">" and "Carriage Return, Line Feed" character

combination. You may use this to signal "OVER" in a packet contact.

In the AMTOR Communications Mode this command sends the "+?"

sequence of characters which is the software "OVER" signal. LAN-LINK

will change colors when it recognizes the '+?' in the incoming

window.

In the Pactor Communications Mode this command sends the "=>"

sequence of characters and switches the link to allow the other

station to send. LAN-LINK will change colors when it recognizes the

'=>' in the incoming window.

In the MORSE and RTTY (BAUDOT and ASCII) communications modes, this

command lets you transmit the "KN" sequence and returns you to the

Receive Mode. LAN-LINK will change colors when it recognizes the

'KN' in the incoming window. In these modes there is no standard for

signalling 'over'. The 'KN' sequence may show up in text (i.e. in

the word KNOW) or in a callsign (i.e. G9KNA) and cause a color

change. If this "random" color change bothers you in BAUDOT, change

line 114 in the LAN-LINK.SYS file from KN to some other pair of

characters that should not show up in normal text.

If you have typed something into the "type ahead buffer" and then

press this key, the contents of the buffer will be transmitted

followed by the turnover command.

For example, if you are 4X6AA, and you are working G3ZCZ, then you

would use Function Key 3 (F3) to transmit the sequence "G3ZCZ de

4X6AA" and this key to send "over" and revert to receive.

3.27 PgUp Scrollback (Scrolls back incoming text)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 28

This key lets you repeat the display of up to 512 lines of incoming

text which has scrolled out of the incoming window. You may print it

or capture it to disk during the repeat.

The bottom line menu displays your options when showing the scroll

back information. Use the "Escape" key to terminate the Scrollback

Mode.

You can save the contents scrollback buffer displayed on the screen

by turning the "Capture-to-Disk" on, and then writing text to the

window. Anything written will be captured. If the Capture-to-disk is

ON before or during scrollback, you will get two copies of the text

in the file.

3.28 The Home Key

The Home Key will clear the window screen that the cursor happens

to be in at the time that the key was depressed, and will then

position the cursor at the top left hand corner of that window.

3.29 The Down Arrow Key

The Down Arrow Key toggles the Normal/Reverse Sideband command in

all communications modes except MORSE. The sideband is depicted in

the status window by the "Up" and "Down" arrows. If the "Down" Arrow

is showing, you are set for Lower Sideband (LSB) which is the one

normally used.

If you set your transceiver to LSB, you will be set correctly for

all modes. If you have reversed the sidebands, a flashing "Up" Arrow

will be displayed. For CW operation you may set your rig to the CW

position and ignore the sideband setting. If you use FSK, just set

it so that it works for you.

3.30 Shift/Baudrate Control Keys

The following keys are used to control the Shift (Wide or narrow)

and the Radio link baudrate in the BAUDOT and ASCII communications

modes. In the BAUDOT and ASCII communications modes all baud rates

supported by the PK-232 may be selected.

3.30.1 RIGHT ARROW

This key speeds up the baud rate. In the CW communications mode,

the right arrow increases the CW speed by 1 wpm.

3.30.2 LEFT ARROW

This key slows down the baud rate. In the CW communications mode,

the left arrow decreases the CW speed by 1 wpm.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 29

3.30.3 UP ARROW

This key toggles the shift from Wide shift (850 Hz) to Narrow shift

(170 Hz) and back. If you are using a KAM, it will also pass through

the 425 Hz shift position.

3.30.4 DELETE

This key toggles the USOS command (RTTY only).

The state of each of the above commands are displayed in the status

window at the top of the screen. If USOS is active, "USOS" will be

shown. The shift is shown as "W" or "N" for wide or narrow

respectively.

3.31 Control T (^T)

This key combination inserts the date-time-text string into the

outgoing buffer. A typical example is "11-Mar-93 01:39".

3.32 Control-Z (^Z)

This key combination is used in the Packet Communication Mode to

signal the end of a file or message, when loading messages into

another LAN-LINK system or into a PBBS. In the AMTOR Communications

Mode, it is automatically translated into the sequence " :EOF: +? "

which can be used to terminate a message being stored in another

LAN-LINK AMTOR mailbox. In the other communications modes, it also

causes a changeover sequence to take place.

3.33 Alt-T Talk to another station connected to the PacketCluster

This function key performs same function as the 'T' option in the

BBS/PacketCluster Menu.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 30

4.0 FUNCTION KEYS

Function keys are used to perform operations common to all

communications modes, or to allow fast performance of a function

without having to go through several layers of menus. The action of

each of the function keys is described in the following sections.

The function key number corresponds to the paragraph number, namely

4.1 refers to Function Key 1 (F1), etc.

4.1 Capture to Disk Toggle (F1)

This key toggles the "capture-to-disk" command. When the "capture-

to-disk" is active, all incoming text (including "Echo as sent"

outgoing text) is stored on a disk (floppy or hard) in a file called

"YYMMDD.RUN" where YY is the last two digits of the year, MM the

month and DD the day (E.G. 900824.RUN). If this file does not exist

the first time that LAN-LINK is loaded, it is created. From that

time on, for the rest of that day, successive activations append new

data to the contents of the file.

Each time the command is activated a datestamp (or timetag) will be

written to the file.

When a Packet station connects "capture-to-disk" will be activated

automatically unless you instruct LAN-LINK not to do so with the

Capture-to-disk flag. Packet connects will thus be recorded in their

entirety automatically unless you turn off the "Capture-to-disk"

during the course of the QSO. "Capture-to-disk" will be stopped at

disconnect time.

You may later edit the file at your convenience with your favorite

word processor or the included Editor. However, if your word

processor can only handle files smaller than 64k, make sure that the

"capture-to-disk" file remains smaller. You can do this by renaming

the file to something like "LAN-RUN.001". LAN-LINK will

automatically close the "capture-to-disk" file when you terminate

the program and return to DOS.

You can make Capture-to-disk active when you scroll back. You can

use this feature to save data that has already scrolled by the

window.

4.2 Send Brag Tape and Talk (F2)

This command lets you transmit the Brag Tape (Default LAN-

LINK.TXT). In the non packet modes, it will leave the transmitter on

after sending the file. In the packet mode it will send a ^Z after

sending the file to notify the recipient that the file is complete.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 31

4.3 Transmit Callsign Sequence (F3)

This command lets you transmit the following sequence OTHERCALL DE

YOURCALL. For example, if you are 4X6AA, and you are working G3ZCZ,

then this command would transmit the sequence "G3ZCZ de 4X6AA". You

may use this key at the start and end of a transmission in any mode

except Packet.

4.4 Send Brag Tape (F4)

This command lets you transmit the Brag Tape (Default LAN-

LINK.TXT). In the non packet modes, it will turn the transmitter off

after sending the file.

4.5 Show Packet "MH" list and Point and Shoot Connect (F5)

This command lets you see what calls the TNC has heard recently on

the Packet channels. You can see up to 18 callsigns, with the date

and time they were last logged by the TNC. Refer to the "MH" command

in the TNC manual for the particulars of the display.

The MH list shows up in a special window and freezes until you

depress any key. While the list is frozen, LAN-LINK is not

monitoring received data, so don't freeze it too long.

When you activate F5, LAN-LINK redirects all data coming from the

TNC into the MHwindow. If the MH list is blank, or information comes

in from the TNC between the time you entered the command and the

time the list is sent back, subsequent text from the TNC will show

up in the MH window until the next "Cmd:" appears or 18 lines of

text have been received.

You can also use this command in a point and shoot method for

attempting a connect with a station you have heard. Bring up the MH

Window and move the cursor to the desired callsign. When the cursor

is positioned on any character in the call, press the "Enter" Key

and that call will be recognized as the callsign. If the call is

joined to the word "Cmd:" or the cursor is positioned on a space or

blank, the connect attempt will be inhibited.

LAN-LINK has no way of knowing when the MH list is complete.

Consequently, it sends two commands to the TNC to implement this

feature. The "MH" command instructs the TNC to start sending the MH

List, the "ZCZ" command forces an error response from the TNC. LAN-

LINK uses the error response to close the MH Window.

This command does not work on the TNC1.

4.6 Auto CQ Call (F6)

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 32

This command will send a CQ sequence, wait a presettable delay and

then repeat the CQ sequence until it receives your call back from

someone else. When it does so, it will alert you with a Connect

Alarm signal, reset the delay timer, and revert to the normal or

listening mode. In AMTOR it will revert to the mode in which it is

waiting for an ARQ call and is able to copy FEC signals on

frequency.

When the Automatic CQ sequence is in operation, an indication will

be shown in the status window. The indicator will be steady while

the transmitter is sending the CQ text, and will blink while the

receiver is listening for a reply.

The time delay between calls is set using the "Q" command in the

Parameters Menu (Set CQ delay time). It is also set as a defined

default by you when you setup your LAN-LINK.SYS file.

The automatic CQ command will only work in the TNC non packet modes

if the "Echo as Sent" parameter is set to "ON" per the LAN-LINK.SYS

file. In other words, you must be able to see the CQ text slowly

echo in the incoming window for the command to work. This is because

the "K K K" (or "+?" in AMTOR) sequence is used to arm the software

to look for a reply.

In AMTOR you will get the connect alarm only when the replying

station transmits the "+?" character combination.

The check for the detection of your callsign and the connect alarm

sequence only occur after a line feed character has been received.

Under most conditions in AMTOR, many in RTTY, and some in MORSE,

LAN-LINK will recognize the callsign of the station answering your

CQ and display it in the status window. In the event that it fails

completely, the sequence "--?--" will show up in the status window.

Since the callsign recognition algorithm used just looks for the

first word following the "DE space" sequence and assumes that it has

the callsign, be careful to check it when operating under QRM

conditions. You can override the automatic call selection with the

Enter Callsign command in the Call Menu.

The automatic CQ sequence is canceled when an incoming call is

detected (only if the Beacon or Robot-Mailbox Mode is not selected),

if you push the disconnect command key, the "End" key, F3, or if you

change communications modes.

4.7 Activate Type Ahead Buffer/AMTOR Relink (F7)

This key is not available in the Packet Communications Mode. It

also functions somewhat differently in the AMTOR and other communi-

cations modes as described in the following sections.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 33

4.7.1 Activate Type Ahead Buffer

The use of this key sets LAN-LINK to the "Type Ahead" Mode. This is

necessary for the PK-232 only, the KAM has the feature built in.

(The MFJ1278 does not have it, so the function key is inactive). Any

characters then typed at the keyboard will be placed into the TNC

transmit buffer. The contents of the buffer will be transmitted over

the air when the transmit command is invoked with Function Key 10

[F10].

This feature allows you to "type ahead" as you can enter text into

the transmit buffer while viewing incoming words in the INPUT window

in the screen. Any characters typed from the keyboard will be echoed

in the OUTGOING window on the screen.

4.7.2 AMTOR Relink

If you return to the Command Mode when linked to another station in

AMTOR, use this key to get back to the Converse Mode. If the link

times out, you can try to relink with this key if you have entered

the SELCAL of the other station.

4.8 Disconnect, Disengage or Receive (F8)

This function key returns the station to the receive communications

(TNC command) mode. It may also be used to abort the automatic CQ

sequence. If you use it to abort a CQ sequence while the

transmission is in progress, you may also want to flush the PK-232

transmit buffer by using the Flush command in the TNC Menu or the

Alt-F hot key combination.

In a multi-connect situation the program will prompt you to tell it

which I/O channel to disconnect.

In AMTOR this command key toggles LAN-LINK between the Chirpcopy

Mode (listening to a QSO in progress) and monitoring FEC

transmissions (CQ calls).

4.9 Put TNC in CMD Mode (F9)

This command puts the TNC into the Command Mode. At this time, the

program is transparent and acts as a dumb terminal (except for

placing incoming and outgoing text in the relevant windows). You use

this Mode for overriding any of the TNC default conditions.

(See I told you that you had to read the TNC manual).

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 34

4.10 Put TNC in CONVERSE/Transmit/AMTOR FEC (F10)

This command places the TNC into the transmit or Converse Mode. The

transmitter is turned on and the contents of the transmit buffer are

transmitted. In the non packet communications modes, the outgoing

characters will be echoed back from the TNC and displayed in the

INCOMING window on the CRT as they are transmitted on the air. In

the AMTOR communications modes, this key puts the TNC in the FEC

Mode and turns the transmitter on.

4.11 Alternate Function Keys

These keys transmit the files with the fixed names LAN-LINK.001

through LAN-LINK.010 (the suffix number corresponds to the function

key). These files must be located in the same sub-directory as the

LAN-LINK.EXE program. The transmitter is left on when the file has

been sent.

4.11.1 Send/Talk file LAN-LINK.001

4.11.2 Send/Talk file LAN-LINK.002

4.11.3 Send/Talk file LAN-LINK.003

4.11.4 Send/Talk file LAN-LINK.004

4.11.5 Send/Talk file LAN-LINK.005

4.11.6 Send/Talk file LAN-LINK.006

4.11.7 Send/Talk file LAN-LINK.007

4.11.8 Send/Talk file LAN-LINK.008

4.11.9 Send/Talk file LAN-LINK.009

4.11.10 Send/Talk file LAN-LINK.010

If the Mode File Flag is set (see 10.14), the file names will

change according to the modes.

In the non packet communications modes, the transmitter is left on

when the file has been sent. In AMTOR, the changeover character pair

(+?) is not sent.

4.12 Shift Function Keys

These function keys show the contents of the files with the fixed

names LAN-LINK.001 through LAN-LINK.010 (the suffix number

corresponds to the function key). These files must be located in the

same sub-directory as the LAN-LINK.EXE program.

4.12.1 Show contents of file LAN-LINK.001

4.12.2 Show contents of file LAN-LINK.002

4.12.3 Show contents of file LAN-LINK.003

4.12.4 Show contents of file LAN-LINK.004

4.12.5 Show contents of file LAN-LINK.005

4.12.6 Show contents of file LAN-LINK.006

4.12.7 Show contents of file LAN-LINK.007

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 35

4.12.8 Show contents of file LAN-LINK.008

4.12.9 Show contents of file LAN-LINK.009

4.12.10 Show contents of file LAN-LINK.010

If the Mode File Flag is set (see 10.14), the file names will

change according to the communications modes.

4.13 Control Function Keys

These function keys show the first lines of the files with the

fixed names LAN-LINK.001 through LAN-LINK.010 (the suffix number

corresponds to the function key). These files must be located in the

same sub-directory as the LAN-LINK.EXE program.

If the Mode File Flag is set (see 10.14), the file names will

change according to the modes.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 36

5.0 THE MAIN MENU

When you press the "Escape" Key, the Main Menu shown in Figure 5

appears on your screen and provides you with a number of choices as

described in the following sections. Type in the single letter to

make your choice.

Figure 5 Main Menu

A Quick Menu

B PBBS/PacketCluster Menu

C Call Menu

D eDit Menu

E Event/Clock Menu

F Files Menu

H Help with Function keys

J Jump to DOS Shell

K Lan-linK Menu

L Log Menu

M Message Menu

N TNC Menu

O AMSAT-OSCAR Menu

P Parameter Menu

Q Q Codes [NC/L]

R Kenwood Radio Menu

S communicationS Mode Menu

T Terminal Mode Menu

X eXit LAN-LINK

Z Elmer Menu

Different options will show up in each menu depending on the

connect state (in packet) or the type of TNC that you are using. For

example, if you are using a TNC that only supports packet, the non

packet menus will not be seen. The PBBS Menu is only active when you

are connected to another station.

5.1 The Quick Menu

The Quick Menu shown in Figure 5.1 is designed for new users and

provides the most commonly used functions in a single Menu. Type in

the single letter to make your choice. Each option is described

below.

Figure 5.1 The Quick Menu

A Auto CQ

B Send Brag Tape

C Connect to Someone

I Initialize PK232

L Log

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 37

M Change Mode

Q Call CQ

S Send File

T Send File/Talk

Z QRZ

1..0 Send/Talk 1..10

5.2 Auto CQ

This option initiates the automatic CQ sequence in which LAN-LINK

sends out a CQ call, listens for a reply, and if none is received,

sends out another CQ call, and so on, ad infinitum. It is the same

as using Function Key 6 (F6).

5.3 Send Brag Tape

This option transmits the Brag Tape.

5.4 Call Someone

This option is used when you want to call or connect to another

station.

5.5 initialize TNC

This option is used when you want to initialize the parameters in

your TNC.

5.6 Log Menu

This option invokes the logbook.

5.7 Change Communications Mode

This option invokes the Communications Mode Menu. You use it to

change TNC communications modes.

5.8 Call CQ

This option is used to initiate a single CQ call.

5.9 Send/Talk File

This option is used to send a file. When the file has been sent,

the transmitter will stay on.

5.10 QRZ

This option is the same as using Alt-Q Function Key (Alt-Q).

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 38

5.11 Send/Talk LAN-LINK files 001..010

This option is used to transmit the contents one of the LAN-LINK

buffer files. You select the number, LAN-LINK does the rest. The

option is the same as that provided by the Alternate Function Keys.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 39

6.0 THE PBBS/PacketCluster MENU

The PBBS menu shown in Figure 6 gives you smart file transfer

capabilities with a PBBS and log searching with a PacketCluster. The

menu is only active when you are connected to another packet

station. The options shown/active at any time depend on whether you

are connected to a BBS or a PacketCluster. Type in the single letter

to make your choice.

Figure 6 PBBS/PacketCluster Menu

A Send Announcement

C Zap PacketCluster

D Send DX Alert

D Download Text File

K Turn PacketCluster Mode OFF

L Turn Auto Cluster DX Alert ON/FF

P Zap PBBS

T Talk to Station

X Download Binary File (Xmodem)

Y Download Binary File (Ymodem)

1 Clear All Search Flags

2 Set All Search Flags

3 Set All CW Search Flags

4 Set All SSB Search Flags

5 Set/Clear Individual Search Flags

6 Set/Clear Current Band Search Flags

7 Set Current Band Search Flags ONLY

6.1 Download Text Files

This option lets you download a text file from the PBBS into a

separate text file on your disk. Enter the full PBBS download

command at the prompt line.

To download a file from a W0RLI PBBS such as FILENAME.TYP in the

"A" sub-directory you'd enter "DA FILENAME.TYP". To download the

same file from the PACKET sub-directory of a WA7MBL PBBS you'd enter

the command "D \PACKET\FILENAME.TYP".

If you use the "D" command to download a text file, it will end up

in a separate file on your disk ONLY IF a file of the same name does

not exist on your drive. If a file with the selected name already

exists on your disk drive you will get an error message.

6.2 Download Binary File (Xmodem)

This option lets you download a binary file from the PBBS or other

packet station using the Xmodem protocol into a separate text file

on your disk.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 40

You have to configure the Xmodem Receive Batch file before you use

this command.

When you use this command, you first have to tell the PBBS the name

of the file you are going to download, and then bring up this option

to tell LAN-LINK that a binary file is on its way. LAN-LINK uses an

external binary file transfer protocol to do the actual file.

6.3 Download Binary File (Ymodem)

This option lets you download a binary file from the PBBS or other

packet station using the Ymodem protocol into a separate text file

on your disk.

You have to configure the Ymodem Receive Batch file, before you use

this command.

When you use this command, you first have to tell the PBBS the name

of the file you are going to download, and then bring up this option

to tell LAN-LINK that a binary file is on its way. LAN-LINK uses an

external binary file transfer protocol to do the actual file.

6.4 Zap PacketCluster

This option is the same as the Alt-K function key.

6.5 Zap PBBS

This option is the same as the Alt-Z function key.

6.6 Turn PacketCluster Mode ON/OFF

This option turns the PacketCluster Mode Flag ON and OFF. If the

flag is on LAN-LINK will scan the incoming lines for a DX-Alert

message.

You may monitor a PacketCluster in the TRAFFIC mode without

connecting to it. When a DX Alert shows up, it is sent to each

connected station in turn. LAN-LINK will only interpret the first

one.

LAN-LINK displays the parts of the DX Alert message in different

colors. The colors correspond to the Multi String Colors, shown in

the color option in the Parameter Menu.

6.7 Send DX Alert Message

This option is the same as the Alt-K function key. Use it AFTER you

have made the contact.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 41

If you have a valid Radio, LAN-LINK will read the frequency from

the Radio. If not, LAN-LINK will prompt you to enter the frequency.

LAN-LINK will then prompt for the call of the DX Station, and will

assume that you want to send the call of the one you have just

logged.

6.8 Clear All Search Flags

This option clears all the band search flags. LAN-LINK will not

check any bands. These options are provided for fast temporary

changes when working a contest.

6.9 Set All Search Flags

This option sets all the band search flags. LAN-LINK will check

every band.

6.10 Set All CW Search Flags

This option sets all the CW band search flags. LAN-LINK will check

every CW band.

6.11 Set All SSB Search Flags

This option sets all the SSB band search flags. LAN-LINK will check

every SSB band.

6.12 Set/Clear Individual Band Search Flags

This option prompts you to set or clear each flag for each band.

6.13 Set/Clear Current Band Search Flags

This option allows you to set or clear the search flags for the

band you are operating on. Use this option and the "clear all"

option in a contest to change settings when changing bands.

Notes:

1:   Use the Update option in the Parameter Menu to save the state

of the individual flags permanently.

2:   Set the debug flag on, to verify the prefix, if you are think

LAN-LINK is incorrect in its searches.

6.14 Turn Auto Cluster DX Alert ON/OFF

If this flag is set, LAN-LINK will prompt you to send a

PacketCluster DX alert AFTER you log a contact.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 42

6.15 Send DX Alert

This option is same as the function performed by the Alt-K

function key. The sequence is as follows.

If the radio interface is active, LAN-LINK will read the radio vfo

frequency, other wise lan-link will prompt you to enter the

frequency. LAN-LINK will then prompt you for the callsign of the

other station, default being the one you logged, then ask you for a

comment to add. LAN-LINK will then transmit the DX alert.

If you are connected, the DX alert will be transmitted once in the

format expected by the PacketCluster. If you are not connected, it

will be broadcast as an unconnected packet in the same format as the

packetcluster message. The number of times that the unconnected

packet will be transmitted is determined by the CQ line counter

parameter (LAN-LINK.SYS line 48).

6.16 Send Announcement (connected to a PacketCluster and using a

dual port TNC)

This option prompts you for the text of the announcement then

transmits it to the PacketCluster with an 'A ' prefix.

6.17 Talk to Station (connected to a PacketCluster and using a dual

port TNC)

This option prompts you for the callsign and the text then

transmits them to the PacketCluster with a 'T ' prefix. Leave a

space character between the callsign and the text. This option is

for a one line message only.

6.18 Set Current Band Search Flags ONLY

This option sets the current CW and SSB band search flags, and

clears all others. Use it when changing hf radio bands, if you are

not interested in hearing about activity on other bands.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 43

7.0 THE CALL MENU

The call menu is used to enter calls, either to call/connect/link a

station or to remember a call or path. Each time you are prompted

for a call, LAN-LINK will remember the previous 15 calls and you may

use the Up and Down arrows to move the cursor to a remembered call.

If you wish to capture a call from the incoming text window, push

Function Key 10 (F10) to enter the incoming text window and move the

cursor to the desired callsign. When the cursor is positioned on any

character in the callsign, push the "Enter" Key and that word will

be recognized as the callsign. Any characters joined to the call

(such as a ".") will be picked up as part of the call. If the cursor

is positioned on a space or blank, the connect attempt will be

inhibited.

In AMTOR, whenever you are asked to enter a callsign, LAN-LINK will

prompt you with its guess at the SELCAL. If you agree with it, just

press the "Enter" Key, if you disagree, enter your choice over the

prompt version.

The Call Menu is communications mode sensitive. It gives you

different choices in each communications mode. In the packet

communications mode it provides the choices shown in Figure 7.1. In

AMTOR, it provides the choices shown in Figure 7.2. Type in the

single letter to make your choice.

Figure 7.1 The Packet Mode Call Menu

A Alert Call

D Show Directory

M ReMember Path

P Packet Cluster

T Target Call

Q Call CQ

U aUto CQ

! Zap PBBS [on Beacon]

Figure 7.2 AMTOR Mode Call Menu

D Show Directory

F Turn CQ File ON/OFF

Q Call CQ

U aUto CQ

X Turn DX ON

7.1 Alert Call

This option toggles the Alert call feature ON and OFF. If it is

off, you are prompted for the call to be monitored, and, if it is

on, it is turned off. The toggle is set when you enter a callsign,

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 44

and cleared when you depress the "Enter" Key instead of a callsign.

The Alert call will be shown flashing in the status window when in

the disconnected state.

The Alert call feature is used in the Packet Communications Mode

when disconnected and the terminal set for "TRFC" or "CQ/BEACON".

LAN-LINK will scan the packet headers received from the TNC, and,

when it sees a packet originated (or digipeated if the MRPT

parameter in the TNC is set to "ON"), by the station whose call you

have entered as the "Alert" call, it will sound an alarm at the

console. The line containing the packet header will also be

displayed in the Alert call color. If the terminal is set for

"CQ/BEACON" the Alert will only sound if the specified station

transmits CQ or BEACON packets.

7.2 ReMember Path

You may use this option to put an entry in the LAN-LINK.DIR file.

7.3 Call CQ

You use this command key to call CQ. In the Packet Communications

Mode, the TNC will transmit a line of text along with the CQ packet.

The line of text is in line 3 of the LAN-LINK.SYS file.

In the AMTOR Communications Mode, your SELCAL will be transmitted

along with the CQ call in the FEC Mode.

7.4 Show Contents of Call Directory

This option displays the contents of the call directory file (LAN-

LINK.DIR). Use this option to verify the contents of the file.

7.5 Target Call

This option toggles the Target call feature ON and OFF. If it is

off, you are prompted for the callsign to be detected. If it is on,

it is turned off. The toggle is set when you enter a callsign, and

cleared when you press the "Enter" Key instead of a callsign.

The Target call feature is used in the Packet Communications Mode

when disconnected. LAN-LINK will monitor the stations connecting to

you. When it sees a connect by the station which you have entered as

the "Target" call, it will set the flashing Connect Count display to

show a "happy face". The Target call will be shown flashing in the

status window when in the disconnected state.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 45

7.6 Auto CQ

This option is the same as Function Key 6 (F6).

7.7 Toggle DX Flag

LAN-LINK allows you to call either "CQ" or "CQ DX". If this flag is

set, all CQ calls will be to "CQ DX". If it is cleared, all CQ calls

will be plain CQ's.

7.8 Zap PBBS on Next Mail Beacon

This option causes LAN-LINK to Zap your local PBBS next time it

beacons with a list of calls for which it has Mail. You set this

option active when you want to upload a set of messages, or are not

sure if there are any messages waiting for you on the PBBS. A

typical situation is where there are a lot of NTS messages stacked

up in the PBBS.BBS file waiting to go. This parameter will be reset

when the connect goes through, so if the Zap attempt failed, it will

try again on the following Mail beacon. This will not work in the

Multi-user mode.

7.9 Turn CQ File ON/OFF

This option is active in the non-packet communications modes. In

the packet mode use the Metabeacon (see Appendix 2 Section 17.3.3).

This option allows you to turn the non packet mode CQ file

transmit flag ON and OFF. If the flag is set, the CQ file (See

Appendix 3 Section 17.0) will be transmitted, if the flag is

cleared, the file will not be transmitted.

Each time LAN-LINK sends out a non packet CQ call it tests for the

presence of the CQ file. If the CQ file exists, LAN-LINK will

transmit the CQ file when you call 'CQ' after transmitting built in

the CQ lines. This feature allows you to customize CQ calls. There

is one CQ file for each mode. The CQ files are named as MODE.CQ,

e.g. BAUDOT.CQ, PACTOR.CQ, etc. You can temporarily override the

default name.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 46

8.0 EDIT MENU

LAN-LINK contains an ASCII text editor suitable for small files. It

is based on Borland's Turbo Pascal Editor Toolbox and the commands

are compatible with Sidekick and Wordstar. Each time you invoke the

editor, LAN-LINK checks that enough RAM exists to contain the file

to be edited. If you do not have enough RAM, LAN-LINK will give you

an Error Message.

The Edit Menu is shown in Figure 8-1. Type in the single letter to

make your choice.

Figure 8-1 The Typical Edit Menu

A LAN-LINK.OUT

B LAN-LINK.TXT

D LAN-LINK.DIR

E Pick dEmo File to Edit

F Any other File

G Incoming Message

K Answer Capture-to-disk file

L Leave Messages >> N4QQ

M Answer Incoming Mail

N Leave Note

O Pick OUT File to Edit

P Pick Ctdsk File to Edit

Q Edit CQ File

R Capture-to-disk file

S LAN-LINK.SYS

T Two Files

Z LAN-LINK.DMO

1,2 - 0 LAN-LINK.001 - 010

A summary of the commands used in the Editor is presented in Figure

8-2. The ^ sign in front of a character identifies that character as

a "Control" character. To activate it, hold down the Control key AND

the character key. Some of the commands require two keystrokes. Use

^P to embed a control character in the text. For example, if you are

creating a file containing commands to be sent to the TNC, to enter

a Control-C character into the file, use the ^P^C sequence of

keystrokes.

Figure 8-2 Summary of Editor Commands

F10           Switch Windows (if more than one window open)

Backspace     Delete left char

Del           Delete Char

Enter         New line

Ins           Toggle Insert mode

PgUp          Move cursor up one page

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 47

PgDn          Move cursor down one page

^A            Move cursor left one word

^C            Move cursor down one page

^D            Move cursor right one character

^E            Move cursor up one line

^F            Move cursor right one word

^G            Delete one character

^H            Delete left one character

^I            Tab

^L            Find/replace next occurrence

^P            Insert a control character into the text

^M            New line

^N            Insert line

^R            Move cursor up one page

^S            Move cursor left one character

^T            Delete one word after cursor

^V            Toggle insert mode

^W            Scroll up

^X            Move cursor down one line

^Y            Delete line at cursor

^Z            Scroll down

^K^B          Mark beginning of block

^K^C          Copy block to position of cursor

^K^D          Save file and exit edit

^K^H          Hide block marker

^K^K          Mark end of block

^K^Q          Abandon file and exit edit

^K^R          Read file into screen

^K^T          Mark single word as block

^K^V          Move block to position of cursor

^K^W          Write block to disk file

^K^Y          Delete block

^K 1..9       Set marker 1 .. 9

^Q^A          Find text and replace

^Q^B          Move to beginning of block

^Q^C          Move to end of file

^Q^D          Move to right of line

^Q^E          Move to top of window

^Q^F          Find text

^Q^I          Toggle autoindent mode

^Q^K          Move to end of block

^Q^R          Move to top of file

^Q^S          Move to left of line

^Q^X          Move to bottom of window

^Q^Y          Delete to end of line

^Q 1..9       Jump to marker 1..9

The following options can be set in the Find/Find and Replace (^Qf

and ^Qa) operations.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 48

#          locates #th occurrence

G          global replace

N          replace without Y/N question

U          ignore upper case/lower case

W          match whole words only

While you are using the editor, LAN-LINK can't receive and process

characters from the TNC. All other features are inhibited or locked

out. Thus if someone connects to you, the TNC will know but LAN-LINK

will not. Be warned!!!

The Edit menu allows you to call up the various files for editing as

follows.

8.1 Edit Brag Tape

This option allows you to create and edit the Brag Tape File (LAN-

LINK.TXT).

8.2 Edit Call Directory File

This option allows you to create and edit the Call Directory File

(LAN-LINK.DIR).

8.3 Edit Any Other File

This option allows you to create and edit any other file.

8.4 Answer Capture-to-disk (Ctdsk) File (Split Screen)

This option allows you to edit the capture-to-disk file and reply

to incoming bulletins/messages in a split screen mode. The contents

of the incoming bulletins and messages will be displayed in the top

window, and the reply file in the lower window.

The reply file is assumed to be named according to the Zap

convention (*.BBS file). You will have to set up the replies/-

outgoing messages as if you were typing them by hand.

You put "SP Call @ PBBS" in the first line, then the "Subject of

the message" in the next. Enter the message, and end it with ":EOF:"

, "/ex" or "/EX" as the first word on a separate line. For example,

a message to me could take the form shown below.

SP G3ZCZ@N4QQ

Test

This is a test

/EX

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 49

8.5 Answer Incoming Messages (Split Screen)

This option allows you to answer incoming messages in a split

screen mode. The contents of the incoming messages will be displayed

in the top window, and the reply file in the lower window. The reply

file is assumed to be named according to the Zap convention (LOCAL-

BBS.BBS file). Reply messages are set up as described in Section

8.4.

Use Function key 10 (F10) to switch windows. At this time you

cannot transfer text between windows.

8.6 Leave Note for Someone

In all modes you can leave a brief note (message) for another

station. When you invoke this option, you will be prompted for the

call of the station that you want to leave a note for. As you enter

the text it is stored on the disk in the usual format.

After you have left the note, if you are in the Packet Communi-

cations Mode, press Function Key 8 (F8) or Alt-D to load the

callsign of the recipient into the beacon.

When you leave a note, leave it for the root call of the connectee,

for example to leave a note for G3ZCZ/W3 leave it for G3ZCZ,

similarly to leave a note for WB8WKA-1 leave it for WB8WKA. LAN-LINK

is smart enough to extract the root call from any callsign it

links/connects with.

8.7 Edit Capture-to-Disk file

This option allows you to edit the Capture-to-Disk file as long as

it is smaller than 64k.

8.8 Edit Two Files (Split Screen)

This option allows you to create and edit any two files. Use

Function Key 10 (F10) to switch windows. At this time you cannot

transfer text between windows.

8.9 Edit Configuration file

This option allows you to edit the Configuration File (LAN-

LINK.SYS).

8.10 Edit LAN-LINK.001 through LAN-LINK.010 files

This option allows you to create and edit the 10 files (LAN-

LINK.001 to LAN-LINK.010). Use the suffix number to access the

respective LAN-LINK.* file, use the number 0 to access the LAN-

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 50

LINK.010 file. If the Mode File Flag is set (see 10.14), the file

names will change according to the modes.

8.11 Pick Capture-to-disk File to Edit

This option presents you with a directory window containing a list

of capture-to-disk files (YYMMDD.RUN). Move the cursor down to the

desired file and touch the "Enter" key. If you don't want to edit

any of them, then move the cursor down to a blank line. If you have

more files than one screen full, use the Pgdn (Page Down) key to

show the next screen full.

8.12 Pick Message file to Edit

This option presents you with a directory window containing a list

of message files (*.OUT). Move the cursor down to the desired

message file and touch the "Enter" key. If you don't want to edit

any of them, then move the cursor down to a blank line. If you have

more files than one screen full, use the Pgdn (Page Down) key to

show the next screen full.

8.13 Leave Messages on your local PBBS

This option allows you to edit your LOCAL-BBS.BBS file in a single

window.

8.14 Edit Incoming Message

This option allows you to edit your incoming message file in a

single window.

8.15 Edit LAN-LINK.OUT File

This option allows you to edit the LAN-LINK.OUT file.

8.16 Edit LAN-LINK.DMO File

This option allows you to edit the Demo or Test file. This is a

file you can read into LAN-LINK and LAN-LINK treats it as if the

data came from the TNC. Use this option to customize a file to test

features and custom settings.

8.17 Pick dEmo File to Edit

This option allows you to pick a Demo file to edit.

8.18 Edit CQ File

This option allows you to edit the CQ file (Section 7.9)

associated with the communications mode in which you are operating.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 51

9.0 EVENT/CLOCK MENU

LAN-LINK contains some time dependent features which allow you to

schedule events. These features are controlled from the  Event/Clock

Menu shown in Figure 9. Type in the single letter to make your

choice.

Figure 9 Event/Clock Menu

A Set Alarm

C Connect ?

D Display Event

P Periodic Connect ?

R Turn Robot ON/OFF

S Turn ConteSt ON/OFF

T Turn Time Display ON/OFF

X Cancel Event

Z Zap PBBS

To set a scheduled event, first select the event, then enter the

time. You may set the event to occur once a day, or once an hour. If

you wish the event to occur once an hour, enter an hour value

greater than 23 when asked to enter the hour. Each time the event

occurs, the feature will be cleared.

9.1 Set Alarm

This option allows you to set an alarm clock.

9.2 Connect

This option allows you to set LAN-LINK to issue a call or connect

request at some later time.

9.3 Display Event

This option allows you to display the scheduled event.

9.4 Periodic Event

This option allows you to set a periodic connect request or call

event. You can set the start time, the end time and the time

interval between the connect/call attempts. The first connect that

goes through will clear the event scheduler.

9.5 Turn Robot ON/OFF

This option allows you to turn the AMTOR/Packet Robot on or off at

a fixed time.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 52

9.6 Turn Automatic Contester ON/OFF

This option allows you to turn the AMTOR/Packet Automatic Contester

on or off at a fixed time. You will be prompted for the time, a

starting QSO number (in case you a restarting the contest) and the

Contest Text (in case you wish to change it).

9.7 Turn Time Display ON/OFF

This option allows you to turn the clock display on or off.

9.8 Cancel Event

This option allows you to cancel a scheduled event.

9.9 Zap PBBS

This option allows you to schedule a PBBS Zap at a predetermined

time (like in the middle of the night). You may for example set it

to attempt to connect to a PBBS at a fixed time or hourly at any

desired minute after the hour. Should the connect be made, the event

scheduler will be cleared and no further connect attempts will be

made.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 53

10.0 FILES MENU

The Files Menu is shown in Figure 10. Type in the single letter to

make your choice.

Figure 10 The Files Menu

B Send Brag Tape

C View Color File

D File Directory

E Erase File

F priNt File

M Turn Mode Files ON/OFF

N priNt Ctdsk File

P Pick Ctdsk File to View

R Rename File

S Send File

T Send/Talk File

U send Uppercase File

V View File

Y View Capture-to-disk File

X Turn eXpand Files ON/OFF

Z View Messages

10.1 Send Brag Tape

This command allows you to transmit a standard text file

historically known as a "Brag Tape". The default name of the brag

tape file is LAN-LINK.TXT, and it should be created with a word

processor in the non-document mode, or with LAN-LINK's editor.

During the file transfer, the outgoing text will appear in the

Outwindow. If the text is long enough and fills the TNC buffer, you

will see it stop, and then continue when the TNC is ready to accept

more text. LAN-LINK uses hardware flow control not "^S/^Q."

When you transmit a file you are returned to the receive mode after

the file is sent except in the Packet Communications Mode. In the

Packet Communications mode, when the file has been loaded into the

TNC the computer will add the ^Z character to the outgoing text to

signal to the recipient that the file transmission is over.

The computer will signal you (using the bell character) when the

file has been loaded into the TNC. That is not the same as the time

that the TNC has transmitted the file, because the TNC contains a

character buffer. LAN-LINK dumps the file into the TNC at the baud

rate used between the PC and the TNC. The TNC, then transmits the

file at the radio link data rate. Flow control is used to regulate

the passage of data between the various buffers in the link.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 54

10.2 File Directory

This command shows you the directory of the disk drive.

10.3 Erase File

This command allows you to erase or delete a file.

10.4 Pick Capture-to-disk (Ctdsk) File to View

This command allows you to view a *.RUN file. When you exercise

this command you will be presented with a list of capture-to-disk

files (YYMMDD.RUN). Move the cursor down to the one you want to view

and touch the "Enter" key to select it. Use this option, if you

must, on long files. On files containing less than 64k characters,

use the Edit Menu.

10.5 Rename File

This command allows you to rename a file. You could for example

rename a document file to a "CALLSIGN.OUT" file so that someone will

receive it automatically next time they connect.

10.6 Send File

This option lets you transmit an ASCII file. In the non packet

modes, it will turn the transmitter off after sending the file. In

the packet mode it will send a ^Z character after sending the file.

10.7 Send/Talk File

This option lets you transmit an ASCII file. In the non packet

modes, it will leave the transmitter on after sending the file.

10.8 Send Uppercase File

This option lets you transmit an ASCII file and converts each line

of text in the file to UPPERCASE before transmitting it. The file

itself is not changed. In the non packet modes, it will turn the

transmitter off after sending the file.

10.9 View file

This option lets you view the contents of a file. Use it if you

must on long files. On files containing less than 64k characters,

use the Edit Menu.

10.10 View Capture-to-disk (Ctdsk) File

This option lets you scroll forward through, or view, the contents

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 55

of the capture-to-disk file (YYMMDD.RUN) which you specify by name.

Use it if you must, on long files. On files containing less than 64k

characters, use the Edit Menu.

10.11 View Incoming Message File

This option lets you view the contents the Incoming message file

(yourcall.OUT). Use it if you must, on long files. On files

containing less than 64k characters, use the Edit Menu.

10.12 Print File

This option lets you print the contents of a text file.

10.13 View Color File

This option allows you to view an ANSI color file in glorious

color. If you use the regular edit or file view options, you will

see the ANSI color control characters inside the file. Use this

option to check on your color editing.

10.14 Turn Mode Files ON/OFF

This option lets you change the state of the Mode File Flag. If

this flag is set, LAN-LINK will change the name of the LAN-LINK.001

to LAN-LINK.010 when you change modes. The name will change to the

mode followed by a number, i.e. BAUDOT.001 to BAUDOT.002, or

AMTOR.001 to AMTOR.002. This feature allows you to use different

files in different modes and still use the same Alt key combination

to transmit them. Thus, you can if you wish, set up a Pactor file

with all 128 characters, and an AMTOR file with the AMTOR character

set as MODE.002, (where MODE is the communications mode) and

transmit each using Alt-F2. If the Mode File Flag is not set, then

the file names never change.

10.15 Turn Expand Files ON/OFF

This option lets you change the state of the Expand File Flag. If

this flag is ON, LAN-LINK will replace the macro characters with

QSO specific values in the outgoing text, as described in Section

18.13 when you transmit a file.

The macro characters allow you to set up bragtapes and contest text

exchanges whose contents appear to change in each QSO.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 56

11.0 HELP WITH FUNCTION KEYS

This Main menu option will activate the HELP display which presents

you with a (very) brief reminder summary of the function and hot

keys. This window will remain on display until a character is typed

at the keyboard.

12.0 JUMP TO DOS SHELL

This Main menu option allows you to enter a DOS shell. If you do

not restore the system to the directory which contains LAN-LINK

before you EXIT from the shell, LAN-LINK will do so for you.

You may use the DOS shell while in QSO when you have loaded the

computer/TNC output buffers and output will continue to flow.

To exit from the DOS shell back to LAN-LINK, enter the command

"EXIT". LAN-LINK will automatically return you to the correct sub-

directory.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 57

13.0 LAN-LINK MENU

The LAN-LINK menu provides different options depending on whether

you are connected to another station or not. Figure 13.1 shows

options that are available when connected. Figure 13.2 shows the

menu when not connected. In each case, type in the single letter to

make your choice.

Figure 13.1 LAN-LINK PBBS Menu When Connected

B :QBU: Protocol

D :QZD: Binary File

G :QIC: Page Operator

H :QMH: Activity

M :QBM: ASCII File

P :QSP: Message

Q :QDB: ASCII File

R :QTR: Ask for Time

S :QSM: Message

T :QTA: Message

U :QRU: Messages

Z :QZU: Binary File

2 Select Dual Inwindows

9 Demo Mode

Figure 13.2 LAN-LINK Menu When Not Connected

9 Demo Mode

W Who is "ON-LINE"

2 Select Dual Inwindows

Each of the features (except the :QBU:) may also be performed

manually by typing the full sequence of Network Control Language

character strings.

13.1 :QBU: External Protocol Command

This is a Sequence for external operations. It requires that the

batch file "QBU-TX.BAT" be present in the transmitting (initiating)

station, and the batch file "QBU-RX.BAT" be present in the receiving

station. When the QBU sequence is executed the following occurs:-

The transmitting station sends a :QBU: The receiving station sends

a :QRV: and the amount of memory left on disk, enters the

transparent mode, jumps to dos and executes the batch file "QBU-

RX.BAT".

Upon receipt of the :QRV:, the transmitting station then puts the

TNC in the transparent mode and jumps to Dos and executes the batch

file "QBU-TX.BAT". The batch file should exit and return to LAN-

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 58

LINK. LAN-LINK then exits the transparent mode.

When the receiving station batch file finishes and returns to LAN-

LINK. LAN-LINK then exits the transparent mode and transmits a :QSL:

if the DOS Error level was 0 and a :QNO: error if it wasn't. At this

time both stations are back in the converse mode.

You may use this technique for transmitting pure binary files using

external (to LAN-LINK) protocols). You may change the default names

of the batch files in the LAN-LINK.SYS configuration file.

13.2 :QBM: ASCII File

You use this option to download an ASCII file from another LAN-LINK

station. It is the same as manually typing ":QBM: FILENAME".

13.3 :QSM: Message

You use this option to request a repeat of a message. It is the

same as manually typing ":QSM:".

13.4 :QSP: Message

You use this option to store a message on another LAN-LINK station.

It is the same as manually typing ":QSP: CALLSIGN".

LAN-LINK contains store and forward routines for messages in a

Local Area Network (LAN).

When connected to another LAN-LINK station, you may use the NC/L

:QSP: command to leave a message at that station. This QSP command

automates that feature for you. It lets you just enter the callsign

of the station you wish to leave a message for. It will then send

the text sequence :QSP: <callsign>. The other LAN-LINK HOST station

will either reply :QNO: followed by a number or :QRV: <callsign>.

The "QNO" message means that the other computer cannot accept the

message, the "QRV" message means that it is waiting for you to send

it.

If you get the :QRV: reply, go ahead and type the message, or send

the text file containing the message. If you type the message, use a

Control-Z (^Z) character and carriage return to terminate the

message. (In AMTOR, LAN-LINK will translate the ^Z to :EOF: +? to

terminate the message). You will then receive either a :QSL: from

the other station meaning that the message is accepted, or a :QNO:

followed by a message that something went wrong and the message was

not stored.

13.5 :QDB: ASCII File

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 59

You use this option to upload an ASCII file to another LAN- LINK

station. It is the same as manually typing ":QDB: FILENAME". Using

this option saves you typing the file name twice.

13.6 :QRU: Messages

The NC/L QRU command allows you to upload a number of messages to

the other station. You use it when you have messages pending on your

system and want to pass them to a third computer for storage in the

LAN, while you take your computer off-line.

You use this option to begin the upload sequence to transfer

messages for a number of stations to another LAN-LINK station. It is

the same as manually typing :QRU:.

You can only use this option with a station which you have

previously designated as a store and forward mail box, for specific

stations. You must also have set up a CALLSIGN.MBX file for that

station.

When the upload sequence is over, the computer sending the messages

will transmit the NC/L command word :QJG: to signal that the

sequence is complete. Should you receive it before you get any

messages, then there weren't any pending.

If you want to upload messages to another LAN-LINK system, connect

to it and type the sequence :QRU: followed by the "Enter" character.

That system will then respond to you as if the operator there had

initiated a QRU command.

13.7 :QTA: Messages

If you have a bad path and your messages are not automatically

deleted by the other station after they have been sent to you, you

use this option to delete your message. It is the same as manually

typing ":QTA:".

13.8 :QZD: Binary Files

This command uses the external batch files to download a Binary

file from another LAN-LINK Station using the Zmodem Protocol. If you

attempt to download a file which exists in your computer, LAN-LINK

will use ZModem's "restore after interrupt" capability to pick up

the transfer at the point when (it assumes) the transfer was

interrupted. You must configure all four QZ*.BAT files before trying

to :QZD: as described in Section 24 (Install takes care of the

configuration in most instances).

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 60

13.9 :QZU: Binary Files

This command uses the external batch files to Upload a Binary file

to another LAN-LINK Station, making use of the Zmodem Protocol. LAN-

LINK will not let you upload a file to the other system, if a file

with that name already exists on the system. This stops you

overwriting existing files. If the link drops out while you are

uplinking a file, and you wish to try again later, rename the file

you are sending, then reconnect and try again. Don't forget to :QSP:

the other station and tell the operator what happened. You must

configure all four QZ*.BAT files before trying to :QZU: as described

in Section 24.

13.10 :QIC: Page Operator

This command allows you to page the operator at the other LAN-LINK

2.10+ station. The sequence is:- you send :QIC: to another LAN-LINK

2.10 station. Use the LAN-LINK Menu or type it by hand. Upon

receipt, LAN-LINK begins to page the operator if the sound flag is

on. At that time it sends you a :QRX: followed by a number, i.e.

':QRX: 30'. The number is the number of seconds it will page the

operator. At the end of the paging sequence it will send the prompt

i.e. 'G3ZCZ>'. If the sound is inhibited it will send ':QRX: QRT' to

signal that it is in a silent configuration.

13.11 :QMH: Monitor Heard

This command causes your copy of LAN-LINK to issue the :QMH:

command. When the other station's copy of LAN-LINK receives a :QMH:

command, it drops back to the TNC Command Mode and issues a "MH"

command to the TNC (except to a TNC1 which does not support it). It

stores the list received in a temporary buffer, and then returns to

the Converse Mode and transmits the list back to you.

13.12 :QTR: Ask for Time

This command allows you to get the date-time-text string from the

other LAN-LINK 2.10+ station. Don't confuse it with :QRT:.

13.13 :QRA: Who is "ON-LINE"

You use this option (when unconnected and monitoring the traffic on

a channel) to see who is "ON-LINE" on the LAN. It is the same as

going manually into the Converse Mode and transmitting an

unconnected packet containing the character string :QRA:. You may

also use the Alt-W function key to transmit the same sequence.

13.14 Demo Mode

Use this option to enter the demo mode, and simulate incoming lines

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 61

of text from a disk file. LAN-LINK will treat any lines of text in

the Demo file as if they were received off-the-air. This feature

allows you to test many of the features without needing another

station to transmit the trigger signals (for that feature).

13.15 Set Single/Dual PK-900/DSP 2232 In Windows

This option is only active for the PK-900 and DSP-2232. It allows

you to separate the incoming text from the two radio ports into two

windows. When operating hf/vhf the hf window is usually the middle

incoming window.

13.16 Switch IN Windows (Dual port TNC only)

This option is active for the PK-900 and DSP-2232 and only when

the dual incoming windows are active (non-packet communications

modes). It allows you to change the windows in which the incoming

data is displayed.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 62

14.0 LOG MENU

LAN-LINK 2.10+ allows you to have up to 3 logbooks on-line. One is

designated as the HF logbook, the second is the VHF logbook, and the

last one is known as the Secondary Logbook. They can be different or

the same.

LAN-LINK performs automatic log book entries for Packet and AMTOR

Beacon mode Connects, and semi-automatic log book entries for other

modes. The Log files are in dBASE 3 format and can be processed by

the DBASE Log book Package in PC-HAM for indexed listings, tracking

of DXCC and other AWARDS, etc. This is ideal for special event

stations and DX-peditions, for the whole QSL process and further

statistical analyses of their operation. The structure of the

logbook files are as shown in Table 14.

Table 14 Structure of the Logbook Database File

Field     Field Name     Type      Length

1         DATE           Character 8

2         TIME           Character 4

3         BAND           Character 3

4         CALL           Character 10

5         RX             Character 3

6         TX             Character 3

7         MODE           Character 4

8         POWER          Character 4

9         QSLSENT        Character 1

10        QSLRX          Character 1

11        COMMENTS       Character 20

QSLSENT contains 1 character for information about any QSL sent

out. Use the following notation in this column:

B sent via the bureau

D sent direct.

M sent via the QSL manager

X someone you don't want to send a card to for some reason.

QSLRX contains 1 character for any QSL received information. Use

the following notation in this column:

R card received.

You will notice that a blank in the QSL status (sent or received)

is shown as a '-'. This character may be used in the future in

various modules within the PC-HAM logging package.

The size of the COMMENTS field is a compromise. Each field is a

fixed size so that each log entry takes up a minimum of 62

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 63

characters worth of space on the disk even if all the character

space is unused. You may change the size of the field if you wish,

but to do that you will have to use dBASE. Note also, that if you do

make the comments field longer, the display will be screwed up.

For the logging feature to work, the logbook files must be present

on the disk.

When an AMTOR beacon mode, Pactor, or packet connect occurs, the

connect is automatically logged (if the automatic logging feature is

enabled). In the Packet Mode, the connect path is placed in the

comments section of the logbook entry. In the non packet modes, the

log data entry is set up for you with default 599 values for the

reports exchanged.

When you bring up the Logbook you will be shown the last screen or

page of log entries. The last entry will be highlighted. At this

time a number of function keys can be used, or you may depress the

"Escape" key to bring up the Main Log Menu. When you bring up the

Main Log Menu, it provides the choices shown in Figure 14.1. Type in

the single letter to make your choice.

Figure 14.1 The Log Menu

A Append from Another LAN-LINK Log

B Scan Log by Band

C Change Log

D Delete Logbook

F Find date

G Purge Log

H Help Function Keys

I Import From G3ZCZ's Contest Log

J Jump to an Entry

K PicK Log

L pack Logbook

M Scan Log by Mode

N Create New Blank Log

P Print Log

Q QSL Menu

R Re-index Logbook

T Sort on Date + Time

U Turn Update Disk OFF

Z fuZzy scan

2 Turn Secondary Search OFF

Note: If you see error 601 or similar messages in the logbook, or

LAN-LINK tells you a callsign is not in the logbook when you

know it is, re-index the logbook.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 64

14.1 Append From Another LAN-LINK Log

This option lets you append entries from another LAN-LINK logbook.

Use it after a contest, to merge the contest log into the main

logbook.

14.2 Scan Log by Band

This option lets you scan the logbook file and display QSOs made on

a particular band.

14.3 Change Logbook File

This option lets you temporarily change the logbook file. If you

want to change it permanently, update the LAN-LINK.SYS file via the

Parameters Menu. When you enter the new name, LAN-LINK will assume

that it is a filetype DBF. For example, if you have different

logbooks for different operations on-line, such as 4X.DBF or W8.DBF,

you only need answer "4X" in response to the question, to change to

the 4X.DBF logbook. In accordance with the dBASE convention, you

cannot use any filetype other than DBF.

14.4 Delete Logbook File

This option lets you delete both the logbook (.DBF) and its

associated index file (.NDX).

14.5 Find Date

This option lets you locate and move the highlight to entries for a

particular date. If no entries are found for the desired date, the

first entry after that date will be highlighted. You can also scan

for parts of a date, such as "90/01", for the first entry in January

1990.

14.6 Purge Log

This command allows you to purge one callsign from the log. Use it

to delete the callsign of your local PBBS if the logbook file gets

too big. Enter an '*' to purge all calls from the logbook.

14.7 Help Function Keys

This option brings up a menu listing the log mode function keys.

14.8 Import From G3ZCZ's Contest Log

This option lets you import logs from earlier releases of LAN-LINK

(up to and including Version 1.56). It can also be used to import a

log generated by Contest Program in PC-HAM.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 65

14.9 Jump to an Entry

This option lets you jump to an entry by its record number. This is

the number displayed in the first column.

14.10 Pick a Log file

This command allows you to pick a *.DBF file. When you exercise

this command you will be presented with a list of log files (*.DBF).

Move the cursor down to the one you want to use and touch the

"Enter" key to select it.

14.11 Pack a Logbook

This function key allows you to pack the logbook files. All entries

marked for deletion will really be deleted at this time.

14.12 Scan Log by Mode

This option lets you scan the logbook file and display QSOs made

using a particular mode.

14.13 Create a New Logbook

This command allows you to create a blank logbook (*.DBF) file.

14.14 Print Log

This command prints the contents of the logbook file in a formatted

manner to the printer.

14.15 Re-index a Logbook

This command allows you to index logbooks that you have ported over

from other DBASE formats, or to re-index logbooks that have had

their index files damaged in some manner. You damage the index if

you edit the key (i.e. the callsign) in an entry.

14.16 Sort on Date + Time

This command allows to you create a new logbook from the current

one with the entries is sequential order sorted on date and time.

You use this option after entering old QSOs manually. Make sure

there is space on the disk for a temporary logbook before evoking

this operation.

14.17 Turn Update Disk ON/OFF

If this parameter is set ON, LAN-LINK will close the logbook and

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 66

re-open it after every entry. If the parameter is set OFF, LAN-LINK

saves a few entries in memory and only writes them to disk when the

buffer fills, or the logbook is closed. Setting the parameter

guarantees you will not lose any entries if the power fails, at the

expense of a slightly slower logging operation.

14.18 Fuzzy Scan

This option allows you to scan the callsign or the comment field

(your choice) for a character string. For example you could scan the

comments field for all entries with the word "Joe". You could scan

the callsigns for all calls with the suffix of "ZCZ".

14.19 Turn Secondary Search ON/OFF

This option allows you to turn the secondary search mode on and

off. If it is on, and if the secondary logbook exists, every time

you command a search of the logbook to find a prefix or callsign,

LAN-LINK will also search the secondary log. Use this feature if you

have more than one log, as a result of operating from more than one

location or for any other reason.

14.20 Log Menu Function Keys

The Function Keys shown in Figure 14.2 are active in the Log book.

The operation of each function key is described below.

Figure 14.2 Function Keys Active in Log book

A   Append Entry

E   Edit Log Entry

H   Scan for Log Entry

P   Change Print Lines

Q   QSL entry

S   Scan for Call

U   Toggle Delete Mark

X   eXit Log Menu

Ins  Toggle Insert Mode

End  Show Last Page

Home Show First Page

PgUp Page Up

PgDn Page Down

Up   One Entry

Down One Entry

14.20.1 Alt-A Append Entry

This function key allows you to manually append an entry to the

log. You can also use this command to enter the odd SSB contact into

the logbook file. If you do, use the characters "SSB" or "FM" for

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 67

those voice modes to remain compatible with the rest of the DBASE

logbook package.

When appending or editing an entry, LAN-LINK will automatically

enter the date, time, callsign of the other station, band, mode and

QSL information in upper case.

The comments field in the LAN-LINK logbook file is restricted to a

maximum of 20 characters. If you try and enter more than 20, the

surplus will be ignored.

14.20.2 Alt-E Edit Log Entry

This function key allows you to edit the highlighted entry. Move

the cursor with the arrow keys, and end the edit process by

depressing the "Escape" key.

You may use the "QSL" entry to tag the fact that you have written

out or received a QSL card.

14.20.3 Alt-H Scan Log by Call

This function key allows you to scan the logbook by callsign prefix

for the callsign in the highlighted entry in the logbook. This

command displays the contents of the logbook file in a formatted

manner on the screen. It requests the callsign of the log entry to

be displayed. If you want to see entries for particular callsigns or

parts of a callsign, enter those callsigns or the front parts of the

callsign. Valid entries are G, G3, G3Z etc. If you want to see every

entry in the log, don't use this command, use the Page Up and Page

Down keys from the Main Log menu.

14.20.4 Alt-S Scan Log by Call

This function key allows you to scan the logbook by callsign prefix

for the callsign entered with the Alt-C or Alt-E keys. This command

displays the contents of the logbook file in a formatted manner on

the screen. It requests the callsign of the log entry to be

displayed. If you want to see entries for particular callsigns or

parts of a callsign, enter those callsigns or the front parts of the

callsign. Valid entries are G, G3, G3Z etc. If you want to see every

entry in the log, don't use this command, use the Page Up and Page

Down keys from the Main Log menu.

14.20.5 Alt-U Toggle Delete Mark

This function key allows you to mark an entry for deletion, or

unmark it in case you change your mind. Entries marked for deletion

will be flagged by an "*" character next to the record number.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 68

14.20.6 Alt-X eXit Log

This function key lets you exit from the Logbook.

14.20.7 Ins Toggle Insert Mode

This function key allows you to turn the insert mode ON and OFF for

the edit/append functions.

14.20.8 End Show Last Page

This function key moves you to the last callsign on the last page

of the logbook.

14.20.9 Home Show First Page

This function key moves you to the first callsign on the first page

of the logbook.

14.20.10 PgUp Move Up One Page

This function key moves the display up one page of the logbook.

14.20.11 PgDn Move Down One Page

This function key moves the display down one page of the logbook.

14.20.12 Up Arrow Move Up One Entry

This function key moves the highlight up one entry in the logbook.

14.20.13 Down Arrow Move Down One Entry

This function key moves the highlight down one entry in the

logbook.

14.20.14 Alt-P Change printer line default lines per page

This key allows you to change the default number of lines on a

page when a log is printed. The current default is 55 lines before

the page feed character is transmitted. Previous versions of LAN-

LINK had a default value of 60 (fixed).

14.20.15 Alt-Q QSL current entry

This key allows you to update the QSL information on a log entry.

When you activate the function key, the selected entry will be

edited. See section 14.21 for details of how the LAN-LINK QSL

function works.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 69

14.21 QSL Menu

The QSL menu provides QSL database features. The following options

are available.

C Change QSL State

F Print from QSL File

Q QSL Contacts

LAN-LINK allows you to print QSL labels to be stuck onto postcards,

QSL cards or any type of card. since QSL bureaus want the cards

sorted in alpha-numerical order, LAN-LINK also allows you to copy

the entry from the log book into a QSL file (QSL.DBF) and print it

out in a sorted manner when you are ready to send a batch of cards

to the bureau.

Labels are only printed in the Epson/Standard DOS printer format,

one-up labels.

14.21.1 Change QSL State

The allowable states are : Printer, Disk File, or Nothing [P, D or

N], where:

Printer - prints to a basic printer.

Disk file - appends the entry to another disk file (QSL.DBF)

Nothing - does nothing.

The active state is shown at the base of the QSL menu.

14.21.2 Print from QSL File

The QSL file (database) must be indexed before you print the

labels. to make sure there isn't a problem change to the QSL file

(logbook), and use the re-index option before you print labels. You

will also have to set up the label in the printer before running a

batch. Use the 'printer' state to print one or two labels directly

to set up the printer before running the batch.

14.21.3 QSL Contacts

If the QSL state is "disk", then you will receive the following

prompt:

QSL Exists, Append entries [Y or N] ?

It is to remind you the QSL file already exists. This is in case

you printed a batch of labels and forgot to purge the QSL file.

The next prompt is 'Prefix'. Enter the prefix of the call you wish

to QSL. If you enter 'G3' for example, you will get a display of all

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 70

contacts with stations having the "G3" prefix. If you enter 'G3ZCZ'

for example, you will get a display of all the contacts with

"G3ZCZ". By entering the prefix for a particular country, all

contacts with that country may be seen. Enter the number associated

with the log entry, change the qsl information, then push 'Esc' to

terminate the sequence. The log entry will be updated only when the

sequence for all QSO's for the prefix is changed.

This feature allows you to update your log books based on an

incoming batch of cards from the bureau, without having to sort

them. The labels will always be printed in a sorted manner.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 71

15.0 MESSAGE MENU

The Message Menu is shown in Figure 15. Type the single letter to

make your choice.

Figure 15 The Message Menu

B PBBS Command Files

C Current Messages

L Leave Note

N Print Message

O Old Messages

T Time Tag

V View Messages

15.1 PBBS Command Files

This option shows you a list of the PBBS command files on the

default directory. In other words, all files with the file extension

of ".BBS".

15.2 Current Messages

This option shows you a list of the outgoing messages that have not

been sent. In other words, all files with the file extension of

".OUT".

15.3 View Message

This feature allows you to view messages in the computer. After

depressing the key you will be prompted for INcoming or OUTgoing

(I/O) messages. If you request "I" (Incoming) messages you will see

the messages addressed to your callsign. If you request "O" (for

OUTgoing) you will be presented with a directory window containing a

list of messages. Move the cursor down to the desired message, and

touch the "Enter" key. If you don't want to view any of them, then

move the cursor down to a blank line. If you have more messages than

one screen full, use the Pgdn (Page Down) key to show the next

screen full.

15.4 Leave Note

This feature allows you to leave a note for another station on the

LAN in your system. It works using the editor in the manner

described in Section 8.

15.5 Old Messages

This feature shows you a list of the messages that have been sent.

In other words, all files with the file extension of ".OLD".

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 72

15.6 Time Tag

This option is used to place a date time header on a message file

previously prepared by a word processor. The name of the file must

be in the format "CALLSIGN.OUT" (i.e. G3ZCZ.OUT). It requests the

callsign of the message file. It then opens a new file, writes the

header to the new file and copies the contents of the original

"*.OUT" file to the new one. When all is done, the original filename

is changed to "*.BAK" and the new one becomes "*.OUT".

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 73

16.0 TNC MENU

The TNC Menu depends on the TNC. For example the MFJ1278 provides

the choices are shown in Figure 16-1. The PK-232 TNC menu is similar

as shown in Figure 16-2. The actual menu you see also depends on the

mode. In the non packet modes, the non packet options will be locked

out. For example, in the PK-232 Baudot mode, the menu will appear as

shown in Figure 16-3.

Figure 16-1 MFJ1278 TNC Menu

D Set MFJ1278 Date/Time

E Disable MFJ1278 Port

F Flush MFJ1278 Buffer

I Initialize MFJ1278

M calls Monitored List

S Show Connect Status

T Change cText

X Activate MFJ1278 Mailbox

Y Deactivate MFJ1278 Mailbox

Figure 16-2 The PK-232 TNC Menu

D Set PK232 Date/Time

E Disable PK232 Port

F Flush PK232 Buffer

I Initialize PK232

M calls Monitored List

S Show Connect Status

T Change cText

X Activate PK232 Mailbox

Y Deactivate PK232 Mailbox

Figure 16-3 The PK-232 TNC Menu in BAUDOT

D Set PK232 Date/Time

E Disable PK232 Port

F Flush PK232 Buffer

I Initialize PK232

Whichever menu is displayed, type in the single letter to make your

choice.

16.1 Set TNC Date/Time

This feature puts the TNC into the Command Mode and loads the date

into it from the computer time of day clock. This command does not

work on the TNC1.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 74

16.2 Flush TNC Buffer

This option is used to flush LAN-LINK's incoming or outgoing text

buffer and that of the PK-232. When used with other TNCs it will

only flush contents (if any) of the buffers in the PC. If you flush

the incoming buffer, you will only flush the buffer in the PC. Any

characters still in the TNC buffer will still come through. If that

happens, bring up the function but wait a while before activating

it.

16.3 Initialize TNC

This feature sets up the defaults for the TNC from the file LAN-

LINK.SYS. If you have put batteries in the TNC, you should normally

only have to run this command when you first power up the TNC or

change the batteries. However, the batteries must be removed

initially since they are there to retain the default settings of the

TNC (particularly the PK-232). Once the initialization is complete,

you can replace the batteries but leave the TNC on since the battery

provides power to the RAMs after the TNC power is turned off.

The function will also set the date and time within the TNC from

the PC computer clock, and, when it has loaded the configuration

file, may prompt you to set the TNC Terminal mode. If you are using

a TNC1 or a TNC2, it will not prompt you because it assumes that

they only operate on VHF packet. If you are using a KPC-2, you may

select "H" or "V", and of course, if you have a PK-232, you may

choose any of the options.

16.4 Show Calls Monitored List

This option performs the same operation as Function Key 5.

16.5 Show Connect Status

When you are in the Packet Communications Mode, this command lets

you see the state of the connection. It will show you if there are

any unacknowledged packets en route to the other station. If you

have been sending text, it is advisable to use this command before

you disconnect, to ensure that all the data has been received and

acknowledged by the other party. You may use this command in either

the TNC Command or the TNC Converse Mode.

In a multi-connect situation, it will show you the status on all

the I/O channels.

You may also use this command to reset the status window display

after a relink or any other event which was not reflected in the

status window.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 75

16.6 Change CTEXT

This feature allows you to change the message that will be received

by another packet station when it connects to you. Refer to the

CTEXT command in the TNC manual.

16.7 Activate TNC Mailbox

The mailbox in your TNC provides a convenient way for people to

leave you a message when your PC is switched off or is in use for

something other than packet radio. If you do choose to make use of

the personal PBBS in the mailbox, make sure that you have a battery

in the TNC. You will have to interrogate the mailbox in the command

mode to view any messages. Type the commands listed in your TNC

manual.

This menu option allows you to activate the personal mailbox built

into your TNC to read any messages stored in it, or, before shutting

down, if you have not set the shut down mode to 5.

To save your local full service PBBS SYSOP some hassles, it is

strongly recommended that you let LAN-LINK do your outgoing and

incoming message and bulletin store and forwarding. Use the "L"

option in the Edit menu to enter any messages off-line, the "M" and

"K" options in the Edit menu to answer any incoming messages or

comment on any bulletins. Once the messages are written into the

LOCALPBBS.BBS file, let the PBBS Zap take care of transferring the

messages to the PBBS.

16.8 Deactivate TNC Mailbox

This option allows you to deactivate the personal mailbox built

into your TNC.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 76

17.0 AMSAT-OSCAR MENU

This menu allows you to configure the KAM, MFJ1278, or the PK-232

to copy signals from AMSAT-OSCAR Spacecraft telemetry beacons

depending on the TNC. The following options are available.

D Full Duplex ON/OFF

S SAREX Menu

Type in the single letter to make your choice.

17.1 Full Duplex ON/OFF

This option allows you to turn the FULLDUPLEX parameter in the TNC

ON and OFF. Set it to OFF (default) when working a station simplex

(same uplink and downlink frequencies) i.e. MIR. Set FULLDUPLEX to

ON when working a station duplex (different uplink and downlink

frequencies) i.e. SAREX and Fuji-OSCAR 20.

17.2 SAREX MENU

This menu shown in Figure 17-4, is only active in the Packet

Communications Mode. It is designed for use with the Shuttle Amateur

Radio Experiment (SAREX), MIR and the packet radio Microsats, or any

terrestrial station you want to connect with as soon as they appear

on frequency. Apart from the Attack Mode, it can also be used as a

diagnostic LAN tool for monitoring LAN traffic.

Figure 17-4 The SAREX Menu

A Set Attack Mode ON

C Set SAREX Call

D Edit SAREX Capture File

L Turn SAREX Capture ON/OFF

M set Attack Mode Connect/CQ

Z Turn Zap R0MIR-1 ON/OFF

0 Multiple User Connects

17.2.1 Attack Mode ON/OFF

If the Attack Mode is set, this option will cause LAN-LINK to issue

a connect request to the SAREX Call whenever a packet sent to or

from it is heard. The mode, is cleared when the connect is made (and

does not retry out) or when the "A" option is selected a second

time, or when the Alert Call is cleared. If this mode is enabled,

the Alert/SAREX Call prefix shown in the Status Window will indicate

accordingly.

A happy face will be displayed next to the ">" before the call once

the connect has been achieved.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 77

Be careful using this feature, as it has the potential to cause a

great deal of QRM. It can also be cleared by another station

connecting to you and telling you to ":QRT:".

In the SAREX configuration you are listening on one channel while

transmitting on an other. If this feature is used on a simplex

channel (everyone transmits and listens on the same channel) aimed

at a DX-pedition or the MIR space station, there is a potential

lockout mode, in which any one station sending a connect request to

the desired/SAREX call will trigger the other ones on frequency who

will in turn trigger the first. If this pile up situation occurs, I

hope the DX-station will QSY and leave the automatic stations to do

their thing. IN any event, if you use this feature and cause QRM,

anyone can connect to you with the callsign of the DX station (as a

pirate) and shut you down. You will then not get the coveted DX QSL.

17.2.2 Change SAREX Call

This option lets you temporarily change the SAREX call.

17.2.3 SAREX Packets Capture to Disk ON/OFF

This option allows you to configure LAN-LINK to capture to disk any

packets addressed to or from the SAREX callsign. A "[S]" will be

displayed in the status window when this option is active. The

capture to disk file is opened by a packet header containing the

SAREX call, and closed by another packet header not containing the

call. When the file is open, the "[S]" will blink. Packet headers

are considered to be lines with a ">" character in them. LAN-LINK

thus considers both of the lines below as packet headers.

N4QQ*>G3ZCZ

N4QQ PBBS>

If you use this feature for monitoring PBBS traffic, the PBBS

prompt lines ("E.G. N4QQ PBBS>") will not be captured to disk. Do

not get your SYSOP to remove the ">" from the prompt line, or the

ZAP feature will then not work.

A happy face will be displayed in the "[S]" after the SAREX call

has been heard.

If you want to see an ORBITER menu, temporarily change the SAREX

call to be the same as your call and bring up the AMSAT-OSCAR Menu.

17.2.4 Edit SAREX Capture-to-disk File

This option lets you edit the SAREX Capture-to-disk file as long as

it is less than 64 kBytes.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 78

17.2.5 Zap SAREX ON/OFF

This option combines the "Set SAREX Attack Mode" and the "SAREX

Packets Capture to Disk ON/OFF" options in one fast operation.

17.2.6 Set Attack Mode to Connect/CQ

This option allows you to configure the SAREX Attack mode to

digipeat or CQ via the SAREX station as well as to try for connect.

Use the CQ feature to digipeat via MIR, and the connect for the

Space Shuttle. If the Attack Mode is set, LAN-LINK will either try

for a connect or send an unproto packet containing the CQ text. If

you want it to go through the SAREX Callsign you must configure the

UNPROTO parameter in the TNC. For example, to attempt a digipeat via

R2MIR, set 'UNPROTO CQ via R2MIR' from the command mode.

17.3 SAREX Orbiter Features

This was the menu as seen by WA4SIR in orbit on STS-35. You need

the SAREX ROBOT EPROM for this menu to actually do anything to the

TNC. The EPROM may be available from The Radio Amateur Satellite

Corporation (AMSAT) for DX-pedition use. Other options which control

LAN-LINK parameters may be used without the SAREX ROBOT EPROM.

In the Orbiter mode, LAN-LINK does not scan incoming text for the

"*** CONNECTED"

and

"*** DISCONNECTED"

messages when the SAREX Robot is active. This inhibits the bells

and the capture-to-disk. It also means that the Robot must be OFF

for any :Q: code uploads or downloads to occur.

17.3.1 Status Window Display

A flashing ">>" appears in the status window when the SAREX robot

is active. The SAREX Configuration boots up assuming that the SAREX

Robot is active (Default).

17.3.2 Alt-O Function key

The Alt-O Function Key brings up the Orbiter menu. This is a faster

route than the "Esc O" sequence.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 79

17.3.3 The Metabeacon

The Metabeacon is loaded from the contents of one of the META.00n

files. These files may be created by the LAN-LINK editor. When

loading, LAN-LINK will insert a pass character (^V) into the text

before a carriage return, so as not to end the packet before at

least 240 characters, and not more than 255 characters have been

output to the TNC. The maximum number of characters that can be

loaded into the Metabeacon is 1776. The maximum number of packets in

a Metabeacon is 7. You can also force a hard carriage return or "end

of packet" in the Metabeacon by inserting the "\" (backslash)

character, which will not be transmitted, into the text.

17.3.4 SAREX Orbiter Menu

The following commands are active in the Orbiter Menu.

17.3.4.1 Activates Beacons/SAREX Robot: This option activates the

SAREX Robot and loads the Meta Beacon. This command is designed for

use when the SAREX is returned to the regular frequency.

17.3.4.2 Set QRZ/QSL Beacon Interval: This command allows you to

change the beacon timer.

17.3.4.3 Set SAREX Call: This command allows you to change the SAREX

call. Don't use this command on orbit (it's for ground use only).

17.3.4.4 Turn SAREX ROBOT OFF: The Robot must be turned ON and OFF

this way to synchronize LAN-LINK to the TNC. When the Robot is

turned OFF the following commands are sent to the TNC by LAN-LINK.

USERS 1

|A

RO OFF

CMSG OFF

NO OFF

17.3.4.5 Set SAREX ROBOT Heard Count: This command lets you set the

Heard Count number.

17.3.4.6 Show I/O Status: Shows the callsign of the station connect-

ed on each stream, the SAREX Robot state, the number of USERS, the

beacon timer value, the metabeacon timer value and the CTEXT.

17.3.4.7 ChecK Count: Shows both heard and worked counts.

17.3.4.8 List Heard Calls: Lists the callsigns of the last few

stations heard.

17.3.4.9 Load Metabeacon: This command loads the Metabeacon. You

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 80

will be prompted for a text file number in the META.00n series.

Enter a number 1-9 corresponding to META.001-META.009. The 0 value

is for file META.010. Note the delays after loading the file to

allow the TNC to stabilize.

17.3.4.10 Show Worked Calls: Shows the last 35 or so callsigns

worked.

17.3.4.11 Turn SAREX ROBOT ON: The SAREX Robot must be turned ON and

OFF via the menu. When the Robot is turned ON the following commands

are sent to the TNC by LAN-LINK.

USERS 9

CONOK ON

CMSG ON

RO ON

NO ON

17.3.4.12 Stop Metabeacon: This command stops the metabeacon

transmission. When activated, the contents of the beacon are dumped

to the UNPROTO address (default SAREX) by the TNC, rather like a

regular PBBS dumping packets when a user disconnects abnormally.

17.3.4.13 Set Metabeacon Interval: This command allows you to set

the metabeacon timer. It will be loaded into the TNC the next time

that the metabeacon is loaded. Even though loaded into the TNC

immediately, it does not take effect till then.

17.3.4.14 Set SAREX ROBOT Worked Count: Lets you set the starting

number.

17.3.4.15 Set Connect TeXt: This is the text (CTEXT) that will be

transmitted to an incoming station when the ROBOT mode is off. This

option lets you set up a new text string to go with the QSO number.

17.3.4.16 Selects Silent Mode: This command turns off the beacons,

the SAREX Robot and also stops the metabeacon. This command is

designed to be used just before any prearranged packet activity.

17.3.4.17 Restart TNC: This command RESTARTs the TNC, loads the date

from the computer's clock and turns the "EC OFF". It seems to have

no effect on the SAREX ROBOT and counters, but does kill the

metabeacon.

17.3.4.18 Capture QSL/QRZ Calls: This command initiates the log-to-

disk operation. It turns the capture-to-disk "on" and sends the

following commands to the TNC:- "DA", "SERW", "SERH","MW", "MH".

When the calls have stopped scrolling, use Function key 1 to turn

the capture-to-disk off.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 81

18.0 PARAMETER MENU

A typical Parameter Menu is shown in Figure 18. The colors used for

the option letter differentiate between those options that are

"remembered" in the SYS file and those that are only set temporarily

for the session. Type in the single letter to make your choice.

Figure 18 A Typical Parameter Menu

A Send LAN-LINK.OUT is ON    0 Change Band & Power

B Beacon/Robot Mode is OFF   1 Change PBBS Parameters

C Clear Connect Count        2 Change Callsigns

D Digipeat Detect is OFF     3 Change Directories

E Command Echo is ON         4 Enter Common Calls

F PBBS Zap Drop Link is ON   5 Change File Names

I Packet Beacon Control is ON  6 Change LAN-LINK Parameters

L Automatic Ctdsk is ON      7 Change PC Communications Parameters

M Set Message Count          8 Change Colors

N NODE Drop Link is OFF      9 Change SAREX Parameters

P Printer is OFF

Q QTC Snatch is ON           U Update LAN-LINK.SYS

R :QSP: Relay Flag is ON     V Verify Changes

S Sound is ON                W Change Scan Words

T Contest Mode is OFF        Z Time Id. is OFF

X DCD Flag is ON             ! Zap PBBS On Next Beacon is OFF

Y PBBS Snatch is ON          + Misc. Flag is OFF

18.1 Turn Beacon Mode ON/OFF

This command toggles the beacon option for the automatic CQ caller

in the AMTOR and Packet Communications Modes. When it is active a

"[B]" will be displayed in the status window. In this mode, the

Automatic CQ sequence will resume after the AMTOR link is dropped or

a Packet Communications Mode disconnect.

In the Beacon Mode, the last few logbook entries will be displayed

in the logpage window which overlays the outgoing text window at the

bottom of the screen.

In the Packet Mode, the CTEXT message is the only transmission from

you that the other station will receive, so make it good and don't

forget to set "CMSG ON".

In the AMTOR Mailbox/Beacon Mode, the contents of the LAN-LINK.TXT

file will be transmitted to the station who called you. The capture-

to-disk will then be turned on to record one transmission from that

station. The QSO will then be logged to the logbook file. Don't

forget to check the entries in the logbook and text in the LAN-

LINK.RUN files in case the computer made a mistake in recognizing

the call sign of the other station.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 82

The Beacon mode is designed to encourage people to QSY off the

crowded HF packet channels. Do not use it on a crowded HF channel or

on an active VHF channel. If you do, you will soon find that other

stations have set their TNCs to ignore your packets, and you will

not work anyone. You will just be a source of QRM and will be known

as such.

The Beacon mode is also designed for robot beacons on the higher

frequencies. For example, if you leave it running on 6 or 10 meters,

you never know when a short opening will allow a QSO with some part

of the world, a DX contact you would otherwise have missed. Don't

leave it running on 20 Meters when the band is wide open. Further

applications are in meteor scatter and other "exotic" communication

modes.

If you are using the Beacon Mode in a special event station or DX-

pedition you may type characters into the transmit buffer, while the

other station is sending to you. When he, she, or even it, turns it

back to you, you will transmit those characters before the sign-off

sequence.

In both the Packet and AMTOR communications modes, the robot may be

shut down by stations connecting to you and telling you to QRT.

18.2 Clear Connect Counter

You use this option to reset the PACKET and AMTOR Beacon Mode

"connect has taken place indicator" and QTC Snatch counters. These

are the flashing "[C]" and "[Q]" displays in the status window.

Whenever someone else connects or links up to you the "[C]" display

will be incremented. The number shown with the "C" is the number of

connects that have taken place since the Indicator was last reset.

If a "happy face" is flashing along with the connect count, your

"target station" was amongst those connecting. The "[Q]" indicator

alerts you to the fact that LAN-LINK tried to connect to another

station after recognizing your call in a mail beacon. Both are reset

by this option. If you are monitoring Packet traffic on a channel,

you may not notice that a connect has previously taken place. These

indicators alert you to check the contents of the Incoming mail file

("yourcall.OUT") for any incoming messages.

If the "Q" count is much greater than the "C" count, you may be

copying a beacon from a station that you cannot connect with

directly.

18.3 TNC Custom Line

This line has different effects depending on which TNC you are

using. To set the line, add the numbers up for the parameters you

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 83

want, and put the number in line 32.

18.3.1 All TNCs

18.3.1.1 TNC Sync Shift (128): LAN-LINK normally ensures that the hf

SSB/CW Transmitter and Receiver data sense are in phase (i.e. both

set to same sideband). This presupposes AFSK operation. If you are

running FSK and need them out of phase, set this byte to 128. If

this line includes the number 128, LAN-LINK will NOT set the

receiver shift when you change modes.

18.3.1.2 Zap Stay Up (64) If you include the number 64 in this line,

LAN-LINK will default to staying connected to a PBBS when the Zap

sequence ends. Use this parameter setting if the messages contain a

lot of ">" characters at the end of lines and LAN-LINK is faked into

thinking the messages ended prematurely. You may change this

parameter via the 'F' option in the Parameter Menu.

18.3.1.3 TNC Initialize Date (32) LAN-LINK normally sets the time of

day clock in the TNC from the PC during initialization. If you don't

want LAN-LINK to set the TNC time of day clock, include the value 32

in this line.

18.3.2 PK-232, PK-900, PK-88 and DSP-2232

18.3.2.1 PK-232 Digipeat Display Flag (1) Set this line to 1 to

activate this feature, 0 to inhibit it. If the Digipeat Display is

active, any signals digipeated through you will be stored to the

LAN-LINK.RUN file and a "beep" will be sounded for each packet being

digipeated. The status window will also contain a "[D]".

18.3.2.2 PK-232 Converse Mode Default (2) LAN-LINK runs the PK 232

is command mode in the RTTY an CW modes. Function key (FK) 7 allows

you to set the PK 232 into the converse mode. The MFJ 1278 and KAM

operate in the Converse mode in all communications modes. If you set

this bit to 2, LAN-LINK will run the PK-232 in the converse mode in

all modes, and you will not need to use FK 7 to enter the converse

mode before transmitting.

18.3.2.3 PK-232 Serial Port Initialization (4) If this bit is set to

4, LAN-LINK will assume there are no batteries in the PK-232 and

issue the '* * *' sequence to the PK-232 on start up. If this bit is

not set, LAN-LINK will assume the PK-232 contains batteries and will

not try to synchronize the PC to PK-232 serial port.

This feature was removed from LAN-LINK in 2.00 in response to

numerous users who had batteries in their PK-232's and didn't want

to see the initialization sequence. Then came the complaints from

the users who didn't have batteries in their PK-232's.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 84

18.3.2.4 PK-232 XMITOK Flag (8) In the CW communications mode, you

may have to change the value of the PK-232 XMITOK Parameter

depending on how you connect the PK-232 to your transmitter. This

LAN-LINK parameter determines if LAN-LINK instructs the PK-232 to

turn the XMITOK PK-232 parameter on and off when entering and

leaving the CW communications Mode. You may temporarily change the

value of the XMITOK Flag in the CW menu to set things up.

As an example, to configure LAN-LINK to initialize the PK-232

serial port (4) and default to the Converse Mode (2), Set this

parameter to 4+2 or 6.

18.3.3 KAM

18.3.3.1 KAM Set LFADD (2) If this bit is set, LAN-LINK will send a

'LFADD ON' command to the KAM when you enter the BAUDOT

communications mode, and send a 'LFADD OFF' command when you exit

that mode to another one.

18.3.3.2 KAM Pre 3.0 EPROM (4) If your KAM contains an EPROM with a

Revision number of less than 3.0, then set this bit. LAN-LINK will

issue NODE commands when selecting SOLO mode. You must set this bit

if you plan to use the mailbox in the KAM, or the traffic on

frequency will fill the memory in the KAM, and people will not be

able to leave you any messages.

18.3.4 MFJ 1278

Each time MFJ release an update to the MFJ1278 something seems to

change. The setting of these parameters depends on which EPROM

version you own.

18.3.4.1 MFJ Radio Port (1) This bit allows MFJ users to set the

default radio port for VHF. Set this bit to 1 to assign Radio 1 to

VHF (Radio 2 to HF). Set this bit to 0 to assign Radio 2 to VHF

(Radio 1 to HF).

18.3.4.2 MFJ Echo ON/OFF Flag (2) Some versions of the MFJ firmware

require 'ECHO ON' in AMTOR. IF bit 2 is set, LAN-LINK will command

'ECHO ON' before entering AMTOR, and will command 'ECHO OFF' when

leaving AMTOR, if you switch modes via the Menu.

18.3.4.3 MFJ AMTOR Robot Converse Flag (4) It seemed to me that the

MFJ1278 went into the Converse mode automatically on an incoming

AMTOR link, while the MFJ1278B did not. Set this parameter if you

use the AMTOR Robot.

18.4 Turn LAN-LINK Beacon Control ON/OFF

This option toggles the LAN-LINK Handshake features. When they are

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 85

on (default), the Connect handshake message

[ZCZ] *** LAN-LINK 2.32>

will be transmitted on incoming connects. The beacon text and time

will be controlled by LAN-LINK as a function of the messages you

have posted on your system. When they are off, LAN-LINK does not do

anything to your beacon, nor does it transmit the handshake.

18.5 Turn Command Echo ON/OFF

This option toggles the LAN-LINK "Command ECHO flag" ON and OFF

(not the ECHO parameter in the TNC). If the Command Echo flag is

set, the commands and (contents of) files LAN-LINK sends to the TNC

will be displayed in the outgoing window. Set this flag to see how

LAN-LINK commands the TNC in the different modes.

18.6 Change Keyboard Beep Count

LAN-LINK counts the number of characters entered on a line at the

keyboard. When it gets to a preset value it will beep, signaling you

to start a new line. If you have the Word wrap option set, it will

also automatically insert a "carriage return/line feed" character

sequence into the outgoing text for you. This option changes the

value that the keyboard beep counter is compared with to sound the

audio signal.

18.7 Set Message Count

As an alternative to Function Key 8 (F8) or Alt-D, using this

option will also load a QTC "Mail" list into your Packet Communi-

cations Mode Beacon and initiate a LAN-LINK beacon transmission

every 30 minutes (Refer to the BTEXT command in the TNC manual). If

no messages are present, or the only message is to you (incoming or

snatched), beacon transmissions are inhibited. This conforms to good

operating practice on crowded Packet channels.

18.8 Turn Node Drop Link ON/OFF

When connecting via a Node, the normal way of operating is to

disconnect if the connect attempt fails. You may configure the Node

connect attempt flag to stay connected at the node where the failure

occurred. Use the normal mode when trying to get through on a known

path, and the stay alive mode when exploring Node circuits.

18.9 Turn Printer ON/OFF

This option toggles the printer ON and OFF. When the printer is

turned on, any incoming data will be printed in real time. If you

intend to print files received at 1200 baud in the Packet Communi-

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 86

cations Mode, use a print spooler or you may lose data. The printer

is automatically turned off in the Packet Mode when a disconnect

occurs.

18.10 Turn QTC Snatch ON/OFF

This option toggles the QTC Snatch flag ON and OFF. If the QTC

Snatch flag is enabled, LAN-LINK scans the contents of packets

appearing in the Inwindow on the screen while the terminal is

disconnected. Should it recognize the three text strings "LAN-LINK",

":QTC:", and "YOURCALLSIGN" in the contents of a packet, it will

then issue a connect request to the callsign of the LAN-LINK station

that originated the beacon transmission. If the connect is made, the

other station will send you the message automatically which LAN-LINK

will "capture to disk". The connect will time out and terminate a

few minutes later.

The normal connect request will be made DIRECT to the station. If

you expect messages from stations you can only connect with via a

digipeater, place the path in your LAN-LINK.DIR directory file. It

should be there anyhow if you connect to the station regularly. For

example, if you expect messages from 4Z4ZB whom you can only work if

digipeated via 4X1AA then place an entry like "4Z4ZB 4Z4ZB V 4X1AA"

in the LAN-LINK.DIR file.

The QTC Snatch leaves your messages in a message file YOURCALL.OUT.

It is triggered by the first ">" in the initial connect response of

"LAN-LINK 2.32R>" and terminated by the "over" at the end of the

message e.g. G3ZCZ/W8>. If you train stations not using LAN-LINK who

want to leave you messages to trigger your QTC-Snatch, by

transmitting the following un-addressed packet

LAN-LINK :QTC: YOURCALL

your system will connect to them. (For example to leave a message

for me, they would send

LAN-LINK :QTC: G3ZCZ

as an un-addressed packet [in Converse Mode when not connected, or

in their beacon text].)

If they then send you a ">" on a line by itself, followed by the

message itself, and terminated by another ">" as the last character

of the line, the message will end up in a message file rather than

in the LAN-LINK.RUN file and you can scan it fast using Alt-M

(Message Scan Menu Options). It goes without saying that the message

itself should not contain the ">" character at the end of a line. If

it does, by the way, the first part of the message will be in the

message file, the remainder will be in the LAN-LINK.RUN file.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 87

18.11 Turn :QSP: Relay Flag ON/OFF

This option toggles the Store and forward Message Relay Capability

of your system ON and OFF. If the Message Relay Capability is active

(toggled on), any one who connects can store a message in your

system using the ":QSP: CALLSIGN" NC/L command. If the Message Relay

Capability is not active anyone who attempts to ":QSP:" a message

will receive a ":QNO: error" message. The status display window will

show a "[R]" when the Message Relay Capability is active. The

default state for the Message Relay Capability is loaded from the

LAN-LINK.SYS file.

When the Store and forward capability is active, the suffix "R"

will be added to the "[ZCZ] *** LAN-LINK 2.32". i.e, "[ZCZ] *** LAN-

LINK 2.32R"  message received by anyone connecting to your system to

notify them that they can store messages. In this manner they don't

have to try to leave a message and then get ":QNO:" error message to

find out that the feature is toggled off.

Remember, if you configure your computer to store and forward

messages, you have a moral obligation to deliver them.

18.12 Turn Sound ON/OFF

This option toggles the sound ON and OFF. If the sound is off, a

flashing indicator will be displayed in the Status Window. When the

sound is toggled on again, a chime will sound to verify the audio.

18.13 Turn Contest Mode ON/OFF

This option first toggles the Contest Mode. If the Contest Mode is

active, the word "TEST" and the number of the next QSO will be

displayed in the status window. When the Contest Mode is toggled on,

this command prompts you to set the starting QSO number, and the

standard message that will be transmitted as part of the exchange of

messages during the contest. The QSO number will be incremented each

time you log a contact.

The contest feature is toggled off the next time you select this

feature.

While these features do give you some semiautomatic contest

operation by using command keys, there is no checking of callsigns

for duplicates built into the program. You may do that by using the

Alt-S hot key.

In a contest, you enter the callsigns and call the other station in

the normal manner, then use the "End" Key to transmit the exchange

(QSO number and standard text). LAN-LINK will only transmit the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 88

contest text in this manner. If you want to add a "K" or "KN", you

will need to put it in the contest text string. It is also advisable

to use a different log file for each contest to cut down the search

time. The contest QSO number increments each time you log the QSO

(Alt-L).

You may put the Contest Mode into automatic if you are operating

AMTOR or packet. To do so, use the Alt-O function key.

The following special character sequences can be entered in the

contest text string and are converted dynamically at time of

transmission.

18.13.1 ##:A "##" will be replaced by the QSO number when the text

is transmitted.

18.13.2 #1: A "#1" will be replaced by the callsign of the station

being worked in the contest text string.

18.13.3 #N: A "#N" will be replaced by the name of the TNC. Use this

if you have more than one TNC and want to use the same bragtape for

both of them.

18.13.4 #T: A "#T" will be replaced by the date and time as read

from the PC time-of-day clock and converted to Universal Coordinated

Time (UTC). A sample time text is "03-Mar-92 03:13".

For example, use the following entries for the following contests;

"##1 UR ##A" ... in the Sweepstakes,

"5905" ... in the WAZ,

"59 MD" ... in the ARRL DX, and

"59 ##" ... in the WPX.

As an example, if I (W3/G3ZCZ) am in the Sweepstakes contest, and

my Contest text line is,

"##1 UR ##A W3/G3ZCZ 68 MDC".

If the next QSO number is 345, and I am working KW3C, when I touch

the "End" key, LANLINK would transmit the following line

"KW3C UR 345A W3/G3ZCZ 68 MDC".

18.14 Turn DCD Flag ON/OFF

If you are using a PK-232 then this flag should be set to ON. If

you are using a KAM, this flag should be set to OFF. If you are

using another TNC, and if you can set the TNC so that pin 8 on the

RS-232 interface changes state when connected, do so, and set this

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 89

flag to ON. If you can't, then set this flag to OFF. If this flag is

OFF, the multiple-connect modes will not work so don't attempt to

invoke them. If this flag is OFF, the words "*** CONNECTED" and "***

DISCONNECTED" when seen in the LAN may cause spurious apparent

connects and disconnects in LAN-LINK.

18.15 Turn PBBS Snatch ON/OFF

When the PBBS Mail Snatch is enabled, and a PBBS sends a packet

announcing "Mail", if your callsign is in the first line of that

message, LAN-LINK will issue a connect request (again via the path

in LAN-LINK.DIR) to that PBBS. When connected, it will act as if you

have initiated a Zap PBBS command. The Zap counter will be

incremented each time a PBBS Mail Snatch takes place.

If your version of LAN-LINK is less than 2.20 and if the PBBS Mail

beacon has an extra line in it (between the header and the list of

calls) as follows, the PBBS Mail snatch will not work.

If N4FG is monitoring the LAN, and a message is posted for any

callsign containing the character string "N4FG" such as N4FGA, N4FG

will respond to the message. This gives rise to a lock out

situation, because N4FG will respond to the beacon, go through the

sequence described in the Zap PBBS section and disconnect. The

beacon message will not be cleared because it was not read, so the

next mail beacon from the PBBS will contain the call N4FGA, and N4FG

will once again respond to it.

Should you be unlucky and happen to be in this situation, disable

the PBBS Snatch flag and set a Periodic Event to Zap your PBBS

sometime in the middle of the night. Alternatively, when you come

into the shack and the PBBS is in use, set the "Zap PBBS on Next

Mail Beacon" parameter to ON.

18.16 Turn Time Id. ON/OFF

This option allows you to determine if the callsign sequence

transmitted using Function key 3 (F3) will contain a date/time

stamp. If the Id. is set ON a date/time stamp will be transmitted.

18.17 Turn Automatic Capture-to-Disk Flag ON/OFF

This option allows you to turn the automatic "capture-to-disk"

feature ON and OFF.

18.18 Change PBBS Parameters

This sub-menu allows you to change the PBBS Parameters, as

described in the following paragraphs.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 90

18.18.1 Local PBBS Callsign

This option lets you change the call of the local PBBS. This is the

call that LAN-LINK will show in the prompt for the "Zap" command.

18.18.2 Mail-Snatch Trigger Word

You can set the Mail-Snatch Trigger in the PBBS Mail beacon. Most

PBBS's use word "Mail" and a string of calls to indicate the

presence of messages. If your PBBS uses something else, you may

customize LAN-LINK on this line to tell it what word your PBBS uses.

This word is case sensitive.  If your PBBS uses "QTC" instead of

"Mail", you may end up calling yourself if you disconnect while

traffic is flowing on the LAN.

18.18.3 Read Request Command

This is the command LAN-LINK issues to the PBBS during a Zap

sequence to request your mail. The default value is "RM" for Read

Mine. On a WA7MBL type PBBS you may change it to "VM" for a Verbose

Read of the full packet headers. On a W0RLI PBBS you may change it

to RN to only request new messages. If you do change it, use the "V"

option in the Parameters Menu to make sure it is correct. If it

isn't correct, the Zap PBBS feature will not work.

18.18.4 Read Bulletin Request Command

This is the command LAN-LINK issues to the PBBS during a Zap

sequence to read bulletins identified by the scan words. The default

value is "R" for Read. For a Verbose Read of the full packet headers

on a WA7MBL type PBBS, change it to "V"; on a W0RLI type PBBS change

it to "RH".

18.18.5 PBBS Subject String

When you enter a message into a PBBS, you first use the "S"

command. The PBBS then sends you a reply asking you for some

information about the PBBS; typically the subject of the message.

When you have entered the subject, the PBBS then asks you for the

rest of the message. This line is used in the Zap sequence and is a

unique word seen in the text that the PBBS sends you asking for the

subject.

18.18.6 PBBS Message String

When you enter a message into a PBBS, you first use the "S"

command. The PBBS then sends you a reply asking you for some

information about the PBBS; typically the subject of the message.

When you have entered the subject, the PBBS then asks you for the

rest of the message. This line is used in the Zap sequence and is a

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 91

unique word seen in the text that the PBBS sends you asking for the

text of the message.

18.19 Change Callsigns

This sub-menu allows you to change the Callsign and AMTOR SELCAL,

as described in the following paragraphs.

18.19.1 Callsign

The default callsign is G8BTB. Please do not use G8BTB. Replace

G8BTB with yours. This is the callsign that will be used in the non

packet modes. You may use "portable designators" such as G3ZCZ/A,

G3ZCZ/W3, W3/G3ZCZ or G3ZCZ/4X, which you cannot do in Packet.

This is the call that will be monitored for in the Digipeat

Monitoring Mode. In this Mode, which is toggled on and off by the

selection of the "D" Option included in the Parameter Menu (PK-232

only), if someone uses you as a digipeater while you are not

connected to anyone else, the event will be logged to disk. This

Mode will only work if your callsign as stored in this line is a

valid (uppercase only) packet callsign.

For example, in the PK-232, if the callsign is G3ZCZ then the

sequence >G3ZCZ> will be recognized as a digipeat and the event

logged. Thus 4X6AA*>G3ZCZ>4Z4ZB would be logged as a digipeat event.

Store your callsign using UPPERCASE characters if you want the

"DIGIPEAT DETECT" option to work (i.e. G3ZCZ). If you use lower case

(i.e. g3zcz), recognition will never take place. The TNC displays

all callsigns in upper case in the non packet communications modes.

If you are using an SSID (e.g. G3ZCZ-8) in the Packet

Communications Mode, you should enter the whole call INCLUDING the

SSID.

18.19.2 The SELCAL

This is your AMTOR SELCAL. LAN-LINK will try to guess it and prompt

you with a SELCAL configured for your callsign. If you want a

different one, overwrite the prompted version.

If you are not using a PK-232 or KAM or the MFJ1278, you don't have

to change anything, but the SELCAL line must still be in the

Configuration file.

18.19.3 The PacketCluster Call

This option allows you to change the default PacketCluster Call

without connecting to it.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 92

18.20 Change Directories

This sub-menu allows you to change the Directories used by LAN-

LINK, as described in the following paragraph.

18.20.1 Capture-to-disk Files Directory

This is the drive and directory that LAN-LINK will put the *.RUN

capture-to-disk files. Note: the last "\" MUST be present if you are

using a different sub-directory.

18.20.2 QBM Download Directory

This is the directory the :QBM: command will search to find a file

to be downloaded. The default directory for the files is \LAN-

LINK\FILES\. Note: the last "\" MUST be present.

18.21 Enter Common Calls

Each time you call another station, LAN-LINK remembers it (up to

the last 16). When you update the SYS file, those calls will be

saved to disk. You can use this option to add calls, delete calls or

edit calls in the list. Use the "Delete" key to delete a call.

18.22 Change File Names

This sub-menu allows you to change the default file names used by

LAN-LINK, as described in the following paragraphs.

18.22.1 VHF Logbook File (VHF.DBF)

This is the name of the file to be used as your VHF Packet logbook.

18.22.2 HF Logbook File (HF.DBF)

This is the name of the file to be used as your HF logbook. It is

used for all HF modes, including HF packet.

18.22.3 Path Directory File (LAN-LINK.DIR)

This is the name of the file used as your call directory.

18.22.4 Brag Tape (LAN-LINK.TXT)

This is the name of the file used as your brag tape.

18.22.5 QBU Upload File Name (QBU-TX.BAT)

This is the name of the batch file used to initiate a QBU.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 93

18.22.6 QBU Download File Name (QBU-RX.BAT)

This is the name of the batch file used when a :QBU: command is

received from a remote station.

18.23 Change LAN-LINK Parameters

This sub-menu allows you to change the LAN-LINK Parameters, as

described in the following paragraphs.

18.23.1 Change CQ Text

This feature allows you to change the contents of the CQ text

message string. This is the message string that is transmitted along

with the CQ call. Think of this message as a CQTEXT line.

18.23.2 Set CQ Delay Time (Seconds)

This option allows you to change the time (seconds) that the LAN-

LINK waits for a response after a CQ call until the following one is

transmitted. The default value (in minutes and seconds) is loaded

from the LAN-LINK.SYS file.

18.23.3 Set CQ Delay Time (Minutes)

This option allows you to change the time (minutes) that the LAN-

LINK waits for a response after a CQ call until the following one is

transmitted. The default value (in minutes and seconds) is loaded

from the LAN-LINK.SYS file.

18.23.4 Set Connect Time Timeout (Minutes)

This option allows you to change the time (minutes) that the LAN-

LINK waits for a response in a packet connect before issuing a

Disconnect command. The default value is loaded from the LAN-

LINK.SYS file.

18.23.5 Set Number of Lines in a Non-Contest CQ Call

This option allows you to change the number of lines in a non-

contest CQ call from the default value of 3 to any number you wish.

The default value is loaded from the LAN-LINK.SYS file.

18.23.6 Keyboard Beep Character Count

When your typing at the keyboard gets to a point where you may

overrun the normal line length, LAN-LINK issues a beep to remind you

to touch the "Enter" key. This option allows you to change the

number of characters you can type in a line before the beep sounds.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 94

Note the beep will not sound if the sound flag is inhibited.

18.23.7 Word Wrap Mode

This option allows you to change the word wrap mode. It is really

more of a "force carriage return" parameter, rather than "word

wrap". There are four options as follows:

18.23.7.1 Off: Word wrap is off.

18.23.7.2 AutoCR: LAN-LINK will insert an "Enter" character for you

into the outgoing text when you enter the first space character

after the beep has sounded.

18.23.7.3 Word: LAN-LINK will hold all entered text until you either

type a space character or touch the "Enter" key. At that time LAN-

LINK will send the word to the TNC.

18.23.7.4 Line: LAN-LINK will hold all entered text until you either

touch the "Enter" key or the beep sounds. At that time LAN-LINK will

send the word to the TNC. Set this parameter value as follows,

0 = OFF,

1 = AutoCR,

2 = Word,

3 = Line.

18.23.8 Set Shut Down Mode (QRT Flag)

This option allows you to customize the shut down and start up

sequences of the TNC as described in the following sections. If the

Shut down mode is not set to 1, you also have the option to

customize the packet terminal mode.

18.23.8.1 State 0: On shut down, turns the TNC into a VHF Digipeater

and turns CONOK OFF (CFROM NONE for the PK-232). In this condition

the TNC will reject connect requests. On start up turns CONOK or

CFROM on.

18.23.8.2 State 1: Does not send any commands to the TNC on shut

down or start up.

18.23.8.3 State 2: On shut down, sends CMSG ON to the TNC. On start

up, sends CMSG OFF to the TNC.

18.23.8.4 State 3: On shut down, turns on the mailbox in the TNC. On

start up, turns off the mailbox.

18.23.8.5 State 4: On shut down, turns on the mailbox in the TNC and

puts the TNC into the SOLO Mode so that the internal buffer in the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 95

TNC does not fill with on-the-air traffic. On start up, turns off

the mailbox.

18.23.8.5 State 5: Does not send any commands to the TNC on shut

down or start up. This option allows you to leave the TNC in a non

packet communications mode. Since LAN-LINK runs open loop, you will

need to manually synchronize LAN-LINK to the TNC next time you power

up the TNC.

18.23.9 Set Default Users

This option allows you to set the maximum number of multiple users

that can be connected to your TNC at the same time, and to configure

the default start up mode of the program. If this parameter is set

to 1, LAN-LINK will configure itself to the single user mode, and

set the maximum number of users to 9. If this parameter is set to

any number other that 1, LAN-LINK will assume that value to be the

maximum number of allowed connects, and configure itself in the

Multiple user Mode (equivalent to the TNC Menu Option 0).

18.23.10 Set UTC Offset

This parameter contains the difference in hours between Universal

Coordinated Time (UTC) and your local time. For example, if you are

located on the east coast of the U.S.A., and it is winter, you are

in the Eastern Standard Time (EST) Zone, which is 5 hours behind

UTC. In this case, the value of the UTC Offset parameter is 5. Don't

forget to change this parameter when the clocks are adjusted each

spring and autumn.

18.23.11 VHF Packet Length

This option allows you to change the default VHF packet length. The

TNC has a maximum permissible value of 255. Set the parameter to 127

or 255.

18.23.12 HF Packet Length

This option allows you to change the default HF packet length. The

default value of about 30 seems to be optimal.

18.23.13 Beacon Time

This option allows you to change the default beacon time. The KAM

requires a value in minutes, all other TNCs in tens of seconds. Use

good operating practice and don't beacon too often. Every 30 minutes

on a well used channel is more than enough.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 96

18.23.14 Custom Prompt before "Over"

This option allows you to add a word or two (up to 20 characters)

before the ">" in packet or the "+?" in AMTOR. If you are working an

AMTOR APLINK PBBS, clear this feature by changing it to a space

character.

18.23.15 Packet Cluster DX Alert Mode

This parameter determines the action LAN-LINK takes when you are

connected to a PacketCluster, and a DX Alert message shows up. The

states are 0, 1, 2, or 3.

If it is 0, LAN-LINK takes no action at all. In all other states,

LAN-LINK splits the prefix from the DX callsign, searches the HF

logbook for the prefix and posts a message in the status window (in

the Alert Call color) as follows: Prefix, Logbook Status, DX

callsign. The logbook status shows up as 'Q' for QSL'd, 'W' for

worked and '?' for not in log. You can configure LAN-LINK to

generate an audible alarm for each condition to save you monitoring

the screen. If the Sound is inhibited, the audible alarm will also

be inhibited. The visual display and audio alarms are controlled by

this parameter as shown in Table 18.23.5.

Table 18.23.5 PacketCluster Logbook Search Parameters

State     QSL'd (Q)      Worked (W)     Not in Log (?)

0         No Action      No Action      No Action

1         Audio+Display  Audio+Display  Audio+Display

2         Display        Audio+Display  Audio+Display

3         Display        Display        Audio+Display

You can customize the CW tone and speed, by changing the states of

the "Packet Cluster DX Alert Tone CW Dit time" and "Packet Cluster

DX Alert Tone CW Note" parameters in the LAN-LINK.SYS File.

In this version of LAN-LINK, the prefix scan is simple minded, so

set this parameter to 3 to get used to it, then set it to 1.

18.24 Change PC Communications Parameters

This sub-menu allows you to change the Communications Parameters

between the PC and the TNC, as described in the following

paragraphs.

18.24.1 The Communications Port

This is the default Communications port. The default value is 1.

You may set it at any value between 1 and 4. If you only have one

serial port in your PC, set the parameter to "1" even if your card

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 97

is configured as Com2. LAN-LINK will ALWAYS assume Com1 for a single

card. If you use a different port instead of Com1, and plan to do

binary transfers, don't forget to customize the QZ*.BAT files.

The default Com3 and Com4 default PC port values are as follows.

Port      Address (Hex)  Address (decimal)   IRQ

Com 3     3E8            1000                4

Com 4     2E8            744                 3

If you set the port number to 0, LAN-LINK will not accept or

transmit any character to the serial port. All other features will

work.

18.24.2 The Terminal Baud Rate

This Parameter tells LAN-LINK what baud rate the TNC is set to.

This is the baud rate on the RS-232 line between the TNC and the PC.

Valid baud rates for LAN-LINK are 300, 1200, 2400, 4800, 9600 and

19,200. If you specify something else LAN-LINK will default to 1200.

You may experience problems when using baud rates greater than

1200. It depends on the CPU clock rate, any TSR's in the PC, and the

I/O card in the PC. LAN-LINK scans the incoming text constantly,

something which requires time. My PK-232, when used with my XT Clone

at 10MHz, does not like baud rates faster than 2400, and sometimes

not even that. The PK-232 seems to drop the first character of some

lines at 4800 baud. Use 1200 until you have tested, and are sure

that a faster one will work.

This program has been tested on a TNC2 and an MFJ1278 using a

terminal baud rate of 9600 baud and worked correctly. It also works

correctly with a KAM at 2400 Baud.

If you use a different rate instead of 1200, and plan to do binary

transfers, don't forget to customize the QZ*.BAT files.

18.24.3 TNC Data Bits

You can set the number of data bits on the RS-232 line between the

PC and TNC to 7 or 8. Use 7 with a TNC2 if you have problems with

Greek characters. Set this value to either 7 or 8.

18.24.4 TNC Parity

Use this parameter to set the Parity bit on the RS-232 line between

the PC and TNC. Enter the letter corresponding to the desired

parity. N = None, O = Odd, E = Even, M = Mark, S = Space.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 98

18.24.5 Number of TNC Stop Bits

You can set the number of stop bits on the RS-232 line between the

PC and the TNC to 1 or 2. Use 1 at all terminal (not packet) baud

rates except 300. Set this value to 1 or 2.

18.25 Change Colors

This option lets you change the default colors. A display of all of

the combinations is shown on the left side of the screen. You will

then be prompted to sequence through each window/message color that

can be customized. Enter the number shown in the combination window

followed by the "Enter" key to set a value. If you don't wish to

make a change, just press the "Enter" Key.

The following identifications are made to the color choices:

18.25.1 OutWindow Color

This is the color of the text (typed at the keyboard) in the

outgoing window.

18.25.2 StatusWindow Color

This is the color of the Status Window.

18.25.3 PromptWindow Color

This is the color of most of the menus.

18.25.4 LogWindow Color

This is the color of the text in the log window.

18.25.5 InWindow Color

This is the color of the text in the window showing the incoming

characters.

18.25.6 ScrollBack Window Color

This is the color of the text in the Scroll back window.

18.25.7 MHWindow Color

This is the color of the text in the monitor heard calls (MH)

window.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 99

18.25.8 Alarmwindow Color

This is the color of the text in the Error and Alarm window.

18.25.9 BottomWindow Color

This is the color of the text in the prompt line at the bottom of

the screen.

18.25.10 Emphasis Color

This is the color of the emphasized text in the Menu windows.

18.25.11 Prompt Color

This is the color of the prompt text in the Menu windows.

18.25.12 Option Color

This is the color of selected option text in the Menu windows.

18.25.13 Connected Text Color

This is the color of the text in the incoming window when connected

in the packet communications modes.

18.25.14 Computer Color

This is the color of the computer generated text in the outgoing

window when the Command Echo Flag option is enabled.

18.25.15 VHF Header Color

This is the color of the VHF Header lines in the Incoming window

when not connected.

18.25.16 HF Header Color

This is the color of the HF Header lines in the Incoming window

when not connected.

18.25.17 Alert Call Color

This is the color of the packet header lines containing the Alert

call in the Incoming window when not connected. It is also the color

of the non-permanent Parameter Menu prompts.

18.25.18 Outgoing Text Color

This is the color of the outgoing text echoed in the Incoming

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 100

window in the AMTOR Communications Mode.

18.25.19 Multi I/O Stream Color[1]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 1.

18.25.20 Multi I/O Stream Color[2]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 2.

18.25.21 Multi I/O Stream Color[3]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 3.

18.25.22 Multi I/O Stream Color[4]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 4.

18.25.23 Multi I/O Stream Color[5]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 5.

18.25.24 Multi I/O Stream Color[6]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 6.

18.25.25 Multi I/O Stream Color[7]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 7.

18.25.26 Multi I/O Stream Color[8]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 8.

18.25.27 Multi I/O Stream Color[9]

This is the color of the text lines in the Incoming window in the

Multiconnect Mode when connected to I/O stream 9.

18.25.28 Multi I/O Stream Color[10]

This is the color of the text lines in the Incoming window in the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 101

Multiconnect Mode when connected to I/O stream 10.

When you change the colors, LAN-LINK does not overwrite all

existing windows. If you want to be sure of seeing the changed

colors, save the color changes using the Update option, and then

exit from and reload LAN-LINK.

18.26 Change SAREX Parameters

This sub-menu allows you to change the SAREX Ground Station

parameters, as described in the following paragraphs.

18.26.1 SAREX CallSign

This is the callsign of the SAREX station.

18.26.2 Change SAREX Capture to Disk File

This option lets you change the name of the SAREX Capture- to-disk

file. Don't change file names while "capture-to-disk" is in progress

(when a flashing "[S]" in present in the status window), or the old

one will stay open and may not close properly when LAN-LINK is

terminated, resulting in data loss.

18.27 Update Configuration File

Use this option to save the changes (permanently) in the

configuration file (LAN-LINK.SYS). Activate it twice to see most of

the changes.

18.28 Verify Changes

This feature lets you display how LAN-LINK was configured by the

LAN-LINK.SYS file. It also displays the state of any flags you have

set/reset if they are not already shown in the Status window or

Parameter Menu.

18.29 Change Scan Words

In the ZAP PBBS feature you can ask the program to scan the lines

read from the PBBS and if it sees any of the scan words in the line,

it will queue a request for that bulletin by number, and as of LAN-

LINK 2.10, change the color of the line. You can only set 16 such

Scan words (should be enough).

Use the "Delete" key to delete a word from the list.

Words that show up in the PBBS prompt line cannot be used as scan

words. For example, if your PBBS prompts you with

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 102

N4QQ PBBS>

then "N4QQ" and "BBS" should not be used as Scan Words. Don't leave

a blank line in the Scan Word section of the configuration file.

18.30 Turn Miscellaneous Flag ON/OFF

The Miscellaneous Flag performs several different functions in the

different Communications modes as follows.

18.30.1 All Modes:- Debug or state-sequence numbers

Shows two debug or state-sequence numbers in left hand corner of

the status window in the format N-M]. N is the BBS State and M is

the Communications State. These are used to figure out what went

wrong in the automatic sequences. If an automatic sequence fails to

work, set the miscellaneous flag, and observe the state in which

LAN-LINK seems to hang. It will be waiting for something, and you

should get a clue as to what, from Table 18.30.1. BBS State (N)

numbers are shown in Table 18.30.1-A and the Communications states

are shown in Table 18.30.1-B.

Table 18.30.1-A BBS States

1    Connected to BBS waiting for first '>'

2    Receiving incoming messages

3    Receiving list of new messages

4    Receiving your messages (in case missed some)

5    Custom command (i.e. KM)

6    LAN-LINK QTC Snatch in progress

7    Receiving Mail going to YOURCALL.OUT

8    :QDB: waiting for QRV/QNO

9    :QDB: sending file waiting for QSL/QNO

10   AUto UPload Opening BBS.BBS command file

11   AUto UPload sending one line command

12   AUto UPload sending subject line of message

13   AUto UPload sending message

14   AUto UPload waiting for last '>' before disconnecting

22   Receiving file

25   Packet cluster connect waiting for first '>'

26   Packet cluster connect waiting for first '>'

18.30.2 BAUDOT Mode:- Navy MARS Protocol Verify

When the flag is set, and the BAUDOT Navy MARS option is enabled,

the character string ":CR:" will be displayed in the outgoing window

each time a carriage return character is received in the incoming

window. Similarly, every time a line feed character is received, a

":LF:" will be displayed in the incoming window.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 103

You may use this feature to verify that another station is sending

files using the correct protocol. If you do, make sure that you tell

the PK-232 not to add a line feed to incoming carriage returns by

setting the ALFD parameter to OFF in the Command Mode. You will have

to set "ALFD ON" again when you change to other communications

modes, or all incoming lines will overwrite each other.

Table 18.30.1-B Communications States

0    Disconnected packet mode or normal non packet mode.

1    Connected in packet mode

2    Trying to connect in packet mode

3    Trying to disconnect in packet mode

4    Sending mail file in packet mode

5    ARQ called someone, or in QSO

6    Connected in autocq mode

7    Calling autocq in packet/pactor

8    Sending file in autocq

9    Waiting for reply in autocq packet/pactor

10   Autocq non packet sending CQ message

11   Autocq non packet waiting for reply

12   SELCAL waiting for ZCZC or YOURCALL

13   SELCAL waiting for NNNN

14   Autocq AMTOR sending bragtape

15   Autocq AMTOR receiving message

16   Autocq AMTOR requesting repeat of his_call

17   Autocq MFJ1278/KAM waiting for 73 to complete and sign off

21   QRU mode packet

22   Calling someone in pactor

25   AMTOR sending :QQQ: response

26   QRU mode packet

18.31 Change Band/Mode

This option is used to set the band and radio mode without invoking

the Log Menu. You can use it when changing bands on HF before

logging a new QSO. The value is not remembered when you shut LAN-

LINK down, unless you log the QSO.

18.32 Zap PBBS on Next Mail Beacon

This parameter causes LAN-LINK to Zap your local PBBS next time it

beacons with a list of calls for which it has Mail.

18.33 PBBS Zap, Don't Disconnect After Sequence

This parameter stops LAN-LINK from issuing the automatic disconnect

when the Zap sequence is over. You can set it during the Zap

sequence. Use it to stop the disconnect if the sequence gets out of

sync with the messages, or if there is something interesting you

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 104

wish to do to the PBBS. This parameter resets when the Zap sequence

terminates.

18.34 Turn Auto Answer ON/OFF

The Auto Answer parameter determines if the LAN-LINK.OUT file is

transmitted to an incoming packet connection or AMTOR link. If the

parameter is on, the file (if present) is transmitted, if the

parameter is off, the file is not transmitted.

19.0 Q Codes [NC/L]

This option displays help information about the use of the Network

Communications Language (NC/L) "Q" codes.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 105

20.0 COMMUNICATIONS MODES

This menu allows you to change the communications mode of the TNC.

If your TNC hardware cannot support those modes, don't bother to try

them. This is software not hardware, and in spite of what people

tell you, software can't do everything. The communications modes

shown in Figure 20 are available. Type in the single letter to make

your choice. While the KAM ports are set, and the MFJ-1278 ports are

defaulted in LAN-LINK (1 = VHF, 2 = HF), don't forget that YOU HAVE

to select the correct radio port on the PK-232.

20.1 VHF PACKET

This is the 1200 baud VHF Packet Mode. LAN-LINK will optimize the

TNC link parameters for this mode.

Figure 20 PK-232 Communications Mode Menu

V VHF PACKET

H HF PACKET

A AMTOR (STANDBY)

B BAUDOT (RTTY)

C MORSE (CW)

M AMTOR (MONITOR)

P PACTOR (STANDBY)

Q PACTOR (MONITOR)

S ASCII (RTTY)

N NAVTEX

G SIGNAL

20.2 HF PACKET

This is the 300 baud HF Packet Mode. LAN-LINK will optimize the TNC

link parameters for this Mode.

20.3 ASCII (RTTY)

This is the ASCII RTTY Mode. LAN-LINK will configure the TNC

parameters for this Mode at 110 baud.

20.4 BAUDOT (RTTY)

This is the BAUDOT RTTY Mode. LAN-LINK will configure the TNC

parameters for this Mode at 45 baud.

20.5 MORSE (CW)

This is the CW Mode. LAN-LINK will configure the TNC parameters for

this Mode at the default CW speed (20 for the PK-232 and MFJ-1278,

10 wpm for the KAM).

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 106

In this Mode the ALT-D Key will disengage the receive speed lock

inside the PK-232 to facilitate locking onto another signal and

Function Key 8 (F8) will turn the transmitter off.

LAN-LINK has a flag in the configuration file that lets you

configure the XMITOK parameter in the TNC. This flag allows you to

set XMITOK to "OFF" when you enter the CW communications mode, and

turn it "ON" again when you exit from the CW communications mode.

20.6 AMTOR (STANDBY)

When you select the AMTOR Standby Mode you are placed into the

"Standby Receive" Mode. If you are in AMTOR Standby Mode, you will

not be able to display signals of the Chirpcopy Mode (which means

that you will not be able to display a QSO in progress).

The Alt-D Key and Function Key 8 (F8) both toggle between the AMTOR

STANDBY and MONITOR communications modes.

20.7 AMTOR (MONITOR)

If you want to read the mail on an existing AMTOR QSO in progress,

select the AMTOR (MONITOR) or Chirpcopy Mode.

The Alt-D key and Function key 8 (F8) both toggle between the AMTOR

STANDBY and MONITOR communications modes.

20.8 PACTOR (STANDBY)

When you select the Pactor Standby Mode you are placed into the

"Standby Receive" Mode. If you are in Pactor Standby Mode, you will

not be able to display signals from a QSO in progress.

The Alt-D Key and Function Key 8 (F8) both toggle between the

Pactor STANDBY and MONITOR communications modes.

20.9 PACTOR (MONITOR)

If you want to read the mail on an existing Pactor QSO in progress,

select the Pactor STANDBY or Chirpcopy Mode.

The Alt-D key and Function key 8 (F8) both toggle between the

Pactor MONITOR and STANDBY communications modes.

20.10 SIGNAL

Use this option to select the TNC Automatic Signal Identification

Mode. Read the TNC documentation carefully to understand how this

mode works. Once the TNC has identified the digital signal it is

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 107

tuned to, use the Communications Mode Select Menu a second time to

select the relevant mode and then set the baud rates and shift to

display the data if necessary.

20.11 NAVTEX

Use this option to select the NAVTEX Mode. Read the TNC

documentation carefully to understand how this Mode works.

20.12 G-TOR

If you are using a KAM, you may use this option to select the G-TOR

Mode. Read the KAM documentation carefully to understand how this

mode works. If you are not using a KAM, this option will not show up

on your menu.

20.13 Manual

This option selects the HF logbook and copies the previous mode

entry in subsequent logging. Use this communications mode to log SSB

or FM contacts when monitoring the PacketCluster and working

stations spotted therein.

If you are using a PK-900 or DSP-2232, exercising the option will

also select a single incoming window.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 108

21.0 TERMINAL MODE MENU

This menu allows you to set the Terminal Configuration Mode. The

Terminal Menu provides different choices depending on which

communications mode you have selected.

21.1 Packet Terminal Menu

The Packet Terminal Mode of operation will be shown in the Status

window at the top of the screen. If you are reading the mail on a

station, the callsign of that station will be shown next to the word

"MAIL". If you are connected to someone, the call (and path) will be

shown prefixed by the "*>" sequence.

In the Packet Communications Mode, you are presented with the

choices shown in Figure 21.1. Type in the single letter to make your

choice.

Figure 21.1 Packet Terminal Menu

B Change Packet Baud Rate

C CQ/Beacons

E Everything

I Information

L Turn MCON ON

M Read the Mail

O Turn Meta Beacon ON

S Just My Station (Solo)

T All Traffic

0 Multiple User Connects

F Turn Conference ON

21.1.1 Change Packet Baud Rate

In the PACKET Mode you may change the RF baud rate between 300,

600, 1200 and 2400 baud. While LAN-LINK allows these baud rates, the

TNC may not.

21.1.2 CQ/Beacons

In this Mode, you will see CQ and BEACON packets on the channel.

This Mode only works on the PK-232 and the KAM.

21.1.3 Everything

You can set the TNC Mode to "Everything", i.e. show all packets on

the LAN.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 109

21.1.4 Read the Mail

You can read the mail on a PBBS or other station and get bulletins

without connecting to that station yourself. You use this Mode to do

so. You will be prompted for the call whose packets you wish to

monitor. Remember to enter the call of the station sending the

packets, not the call of the one receiving them. This mode only

works without showing the packet headers on the PK-232. On the other

TNCs the headers will also be displayed.

You can also use this command to monitor both sides (read the mail)

on a QSO on a busy packet channel.

You may want to turn the Digipeat Data Display Mode off when

reading the mail.

21.1.5 Just My Station (Solo)

In this Mode, you will only see messages addressed to you. You will

only get messages from people who connect to you.

21.1.6 All Traffic

In this Mode you will see all of the data packet traffic on

channel. You can use this Mode to check that the TNC is working.

21.1.7 Information Packets

In this Mode you will see all of the packet traffic on channel that

contain information. You can use this Mode to check that the TNC is

working.

21.1.8 Single User Connects

This is the normal one-on-one Mode of operation (USERS 1).

21.1.9 Multiple User Connects

When you are in a Multiple Connect Mode, an asterisk will be shown

in the status window next to the word "PACKET". Each I/O stream will

show up in the Inwindow in a different color.

LAN-LINK supports two multiple connect modes, Individual and

Conference.

An extract of an actual "off the air" CRT display (showing received

text only) in a multi-user mode is shown below. Each line of text as

received by the TNC is prefixed (by the PK-232) with the I/O channel

number and callsign of the station its connected with.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 110

1:4X4IL: *** CONNECTED to 4X4IL via 4X4IC, 4X6JP

0:4X4IC: joe do u read?

2:4Z4ZB: *** CONNECTED to 4Z4ZB

1:4X4IL: Look for me on PR via 4X6JP 73s and Shalom.

2:4Z4ZB: GE JOE I WILL DIS THEN U CAN GO VIA THE REP>

1: *** DISCONNECTED: 4X4IL via 4X4IC, 4X6JP

0:4X4IC: UNPROTO CQ

2:4Z4ZB: GO AHEAD BOYS...DE ALON..>

2: *** DISCONNECTED: 4Z4ZB

This is the normal Multi-connect Mode as described in the TNC

manual. Here you are connected to up to 10 stations and will send

different traffic to each of them. Each time you wish to send

something to a particular station, you must select the I/O channel

the station is connected on before typing the text or sending the

file.

The default stream change character for all TNCs in the VHF

multiple connect modes is "|" ($7C). In the KAM, it is "\" on HF and

"|" on VHF. They are both on the same key but one is in the shift

case. Be careful on HF as the "\" shows up in file directories and

confuses the computer. These values can be customized. However to do

so, they must be customized both in LAN-LINK and in the TNC at the

same time.

You select the I/O channel using the Alt-N key and moving the

cursor to the desired stream, or by typing the "|" character

followed by (in the TNC) the channel letter/number. For example to

do it manually, if you are connected to G8BTB on I/O channel 0 and

to G3ZCZ on I/O channel 1, then to talk to G8BTB you would type "|0"

before any text, and similarly to send something to G3ZCZ you would

type a "|1" before the text you"re going to send me. If you are not

using a PK-232, read the TNC manual to determine if the I/O streams

are identified by numbers or by letters. The TNC2 standard

configuration uses letters ("|A" etc.).

Use the Alt-N key and move the cursor up and down to select the I/O

channel automatically.

21.1.10 Conference

In the Conference Mode on the other hand, everything that you type

at the keyboard is automatically transmitted to each station that

you are connected with. Thus if you are linked to two stations each

line will be packeted twice by the TNC. You don't have to worry

about sending the wrong thing to the wrong person, as they will all

get the stuff.

You enter this Mode by selecting the "F" option in the Terminal

Menu. You terminate this Mode by selecting the "1" option.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 111

You connect in the usual manner using Alt-C or the Connect Option

in the Call Menu. You must wait for a connection to be established,

or to time out, before attempting to connect to the next station. If

you do not, you will get an error message back from the TNC.

In the Conference Mode, as the same text is transmitted on all I/O

channels, the throughput will be reduced. Slow down your typing

speed or use short packets. Should you check the link status or call

monitored list, you may not be able to get back into the Converse

Mode if you have too many packets outstanding. If that happens, wait

a while and then try to set the Converse Mode with Function Key 10

(F10). You may have to try it a few times, depending on how good

your links are.

Don't try to send a file in the conference mode, it is not designed

for it. Keep the packets short (use short lines). Be also warned

that if you use the conference bridge on bad links (such as crowded

HF channels, or VHF/UHF channels with lots of retries, the keyboard

may seem to lock up. This is because LAN-LINK has to wait for space

to appear in the TNC buffer before it can output the text.

The Conference Mode will only work in the PK-232 if DCDCONN is set

to ON. So if you are using a PK-232, LAN-LINK will do it for you

when you enter the Conference Terminal Mode in case you forgot

or/and haven't set the LAN-LINK.SYS file to include the line "DCD

ON". At the same time set LAN-LINK.SYS default states to show the

calls of other stations on each incoming line in the Multiconnect

modes (read the TNC manual on how to do this).

The Conference Mode will only work in the TNC2 if DCDCONN is

jumpered so that Pin 8 of the RS-232 Interface changes state when

you connect to someone else. If you get error messages and see the

characters "|J" on your screen, then the jumper probably isn't set.

The Conference Mode does not work on the TNC1.

In the Conference Mode, if any one of the stations disconnects, the

TNC will return to the Command Mode, and you must use Function Key

10 (F10) to return to the Converse Mode before you type another

character of text.

21.1.10 Conference Bridge ON/OFF

This option toggles the Conference Bridge ON and OFF. In the Bridge

Multiconnect Mode you are acting as a central switch. You can link

together up to 10 stations in one multi-way connection. In this Mode

any packets arriving on any stream are echoed to all other streams

that you have "connects" on except the one that they came in on.

Anything you type will be sent to all connected streams.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 112

In this manner, all stations will not see their own text echoed

back. Each line of text output by LAN-LINK in this Mode will contain

the callsign of the station originating the packet. Your packets

will contain your call as entered in line 1 of the LAN- LINK.SYS

file. Thus for example, if your call was KB8RP/4X and normally shows

up in the packet header as KB8RP here it will show in the data as

KB8RP/4X.

Just remember that the Bridge puts out a lot of packets, so do

multi-way connects on a quiet frequency. You must be in the

Conference Mode to turn the bridge on, and any disconnect will take

the bridge down. Should that happen, toggle it on again. A bridge

will be shown in the status window whenever the Conference Bridge is

up.

21.1.11 Turn Metabeacon ON/OFF

This option lets you turn the LAN-LINK Metabeacon ON and OFF. LAN-

LINK provides you with a metabeacon, that gives you the capability

to transmit a long beacon message at periodic intervals. The

Metabeacon is loaded from the contents of one of the LAN-LINK.00n

files. These files may be created by the LAN-LINK editor. When

loading, LAN-LINK will insert a pass character (^V) into the text

before a carriage return, so as not to end the packet before at

least 240 characters, and not more than 255 characters have been

output to the TNC. The maximum number of characters that can be

loaded into the Metabeacon is 1776. The maximum number of packets in

a Metabeacon is 7. You can also force a hard carriage return or "end

of packet" in the Metabeacon by inserting the "\" (backslash)

character, which will not be transmitted, into the text.

The frequency of Metabeacon transmissions is the same as that of

the automatic CQ timer.

The MetaBeacon is designed for special event stations. PLEASE do

not abuse it.

21.2 AMTOR Terminal Menu

In the AMTOR Communications Mode, you are presented with the

choices shown in Figure 21.2. Type in the single letter to make your

choice.

Figure 21.2 AMTOR Menu

L Force LETTERS Case

R Set ROBOT/Mailbox

W CW Identification

Z Set ROBOT Infinite Delay

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 113

21.2.1 Force LETTERS Case

This option lets you force the PK-232 receiver to the letters case

if for some reason you appear to be in the numbers case. It will

return you to the Converse Mode.

If your TNC allows you to use a control character (^L) to force the

letters case, you don't need to go through all these menu layers to

do the job.

21.2.2 Set ROBOT/Mailbox

This option lets you turn on the Mailbox during a regular AMTOR QSO

so that the other station can leave a message or download a file

remotely. You must only use it when the other station is sending

text.

21.2.3 CW Identification

This option only works on the PK-232. You use it to send a CW

identification (CWID). The CWID is sent in the FEC Mode. The CWID

consists of the sequence "DE YOURCALLSIGN" where YOURCALLSIGN is

that entered into the PK-232 as "MYCALL".

You use this feature if your license requires you to identify in

Morse code as well as in the operating mode.

21.2.4 Set ROBOT Infinite Delay

You use this option to prevent the Robot from ever transmitting

when not linked. In the normal manner, the Robot counts down the CQ

delay parameter, and, when it gets down to 0, puts a CQ call on the

air. If the Infinite Delay is set, the CQ call will be inhibited.

This option thus puts the Robot into an indefinite wait mode where

it thinks it has sent out a CQ call and is waiting for a reply.

You would use this Mode if you want to set up a MailBox on a known

frequency for some group or organization.

21.3 Baudot Terminal Menu

In the Baudot Communications Mode, you are presented with the

choices shown in Figure 21.3. Type in the single letter to make your

choice.

Figure 21.3 BAUDOT Menu

B Set Baud Rate

F Select Shift

N Turn NAVY MARS Message Protocol ON/OFF

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 114

U USOS ON/OFF

K Turn SELCAL ON/OFF

W CW Identification

21.3.1 Set Baud Rate

This feature allows you to set a desired communications baud rate

directly. You will be prompted to enter the desired rate. Use this

command to avoid having to shift through too many intermediate

states, if for example, you want to make a direct change from 45 to

100 baud.

21.3.2 Select Shift

This feature lets you select Wide (850 Hz) or narrow (170 Hz) shift

directly. The PK-232 will copy 170 Hz signals using its internal 200

Hz circuitry. If you have a KAM, you may also select the Commercial

Shift (425 Hz).

The shift is displayed in the status window at the top of the

screen. It is shown as "W"", "N" or "C" for wide, narrow or commer-

cial respectively.

21.3.3 NAVY MARS Message Protocol

The Navy MARS option toggles the Navy MARS Mode ON and OFF. When

the Navy MARS option is active, an "[N]" will be displayed in the

status window, and the feature performs the protocol described

below.

21.3.3.1: Any Carriage Return or Enter (CR) from the keyboard is

sent as CR, CR. (two sequential CR characters.

21.3.3.2: In a file transmission lines are terminated by a CR, CR,

LF (Line Feed) sequence.

21.3.3.3: In a file transmission, blank lines are sent as a LF

(without a CR).

21.3.3.4: In a file transmission, a plus sign "+" in the file is

replaced by a sequence of 12 "letters" characters. There can be more

than one + sign on a line.

21.3.3.5: In a file transmission, the sequence "NNNN" is transmitted

followed by a sequence of 12 "letters" characters. There can only be

one "NNNN" sequence on a line.

21.3.4 USOS ON/OFF

This option toggles the Unshift on space (USOS) feature ON and OFF.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 115

When the USOS is on, the TNC reverts to the letters shift each time

a "space" character is received. It helps when displaying noisy

copy.

If USOS is active, "USOS" will be shown in the status window at the

top of the screen.

21.3.5 SELCAL ON/OFF

This command key toggles the SELCAL feature ON and OFF. When the

SELCAL is activated, as shown in the status window, the incoming

text from the TNC is scanned. When either the letters "ZCZC" or

"your callsign" (as entered in line 1 of the LAN- LINK.SYS file) is

received, the "capture-to-disk" is turned on. All subsequent text

will be stored in the LAN- LINK.RUN file until the letters "NNNN"

are received. At that time the "capture-to-disk" is turned off.

You may use this feature when monitoring commercial RTTY broadcasts

to record news and other information, or when calling CQ to start

recording all incoming text and keep the QSO on disk.

You may use Function Key 1 (F1) to turn the capture to disk

permanently on to override the automatic turnoff if you wish.

21.3.6 CW Identification

This option is used to send a CW identification. You use this if

your license requires you to identify in Morse Code as well as in

the operating mode.

The identification consists of the sequence "DE YOURCALLSIGN" where

your callsign is that entered into the TNC as "MYCALL".

21.4 ASCII Terminal Menu

In the ASCII Communications Mode, you are presented with the

choices shown in Figure 21.4. Type in the single letter to make your

choice.

Figure 21.4 ASCII Menu

B Set Baud Rate

F Select Shift

K Turn SELCAL ON/OFF

W CW Identification

21.4.1 Set Baud Rate

This feature allows you to set a desired communications baud rate

directly. You will be prompted to enter the desired rate. Use this

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 116

command to avoid having to shift through too many intermediate

states. For example, if you want to make a direct change from 110 to

1200 baud.

21.4.2 Select Shift

This feature lets you select Wide (850 Hz) or narrow (170 Hz) shift

directly. The PK-232 will copy 170 Hz signals using its internal 200

Hz circuitry. If you have a KAM, you may also select the Commercial

Shift (425 Hz).

The shift is displayed in the status window at the top of the

screen. It is shown as "W", "N" or "C" for wide, narrow or commer-

cial respectively.

21.4.3 SELCAL ON/OFF

This command key toggles the SELCAL feature ON and OFF. When the

SELCAL is activated, as shown in the status window, the incoming

text from the TNC is scanned. When either the letters "ZCZC" or

"YOURCALLSIGN" (as entered in line 1 of the LAN-LINK.SYS file) is

received, the "capture-to-disk" is turned on. All subsequent text

will be stored in the LAN-LINK.RUN disk file until the letters

"NNNN" are received. At that time the "capture-to-disk" is turned

off.

You may use Function Key 1 (F1) to turn the capture to disk

permanently on to override the automatic turnoff if you wish.

21.4.4 CW Identification

This option is used to send a CW identification. You use this if

your license requires you to identify in Morse code as well as in

the operating mode.

The identification consists of the sequence "DE YOURCALLSIGN" where

your callsign is that entered into the TNC as "MYCALL".

21.5 CW Menu

In the CW Communications Mode, you are presented with the Menu

shown in Figure 21.5. Type in the single letter to make your choice.

Figure 21.5 CW Menu

L Lock to Receive Signal

S Set Morse Speed

U Unlock Morse Speed

X Turn XMIT Flag ON/OFF

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 117

21.5.1 Lock to Receive Signal

This option locks the TNC to the speed of the incoming CW signal.

21.5.2 Set Morse Speed

This option lets you set the CW speed.

21.5.3 Unlock Morse Speed

This option unlocks the TNC from the incoming CW signal. It

performs the same task as the Alt-D option keys. It is used to speed

up capture of a new signal or to break the lock when the TNC appears

to have locked up at the wrong speed.

21.6 Pactor Terminal Menu

In the Pactor Communications Mode, you are presented with the

choices shown in Figure 21.6. Type in the single letter to make your

choice. Do not use any of these options when linked to another

station or you may drop the link.

Figure 21.6 Pactor Menu

0 AUTO BAUD

1 Force 100 BAUD

2 Force 200 BAUD

H Enable Huffman

K Disable Huffman

21.6.1 AUTO BAUD

This option configures the TNC to switch between 100 and 200 baud

on the HF Pactor link as determined by propagation Conditions.

21.6.2 Force 100 Baud

This option forces the TNC to use 100 baud on the HF Pactor link as

determined by propagation Conditions.

21.6.3 Force 200 Baud

This option forces the TNC to use 200 baud on the HF Pactor link as

determined by propagation Conditions. This option is only available

for the KAM.

21.6.4 Enable Huffman

This option enables Huffman compression of the transmitted text.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 118

21.6.5 Disable Huffman

This option disables Huffman compression of the transmitted text.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 119

22.0 EXIT TO DOS

When you exit the program and return to DOS, if the Shut down (QRT)

flag is set to 0, the TNC is put into the VHF default mode in which

it rejects calls from any other stations. (Read about the CFROM

and/or CONOK command in the TNC manual). You can, if you wish, leave

the TNC powered, and, if you also leave the VHF radio powered up,

you will be active as a digipeater, but at the same time anyone

trying to connect with you will receive a "busy signal". The MBX

command (Reading the Mail) is also cleared at this time.

If you want to leave the TNC set up to accept messages when LAN-

LINK is not active, customize the shut down state as described in

Section 18.23.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 120

23.0 THE ELMER MENU

ELMER is a smart server or second operator feature described in

Appendix 7. The ELMER Menu presents you with the choices shown in

Figure 23. Type in the single letter to make your choice.

Figure 23 The ELMER Menu

C Change ELMER File

E Turn ELMER ON/OFF

L Load ELMER.QSO

P Pick File to edit

S Edit ELMER.QSO

T show state Table

23.1 Change ELMER File

This option allows you to change the ELMER state table file.

23.2 Turn ELMER ON/OFF

This option allows you to turn ELMER ON and OFF. If the ELMER state

table file (ELMER.QSO) is not present, you will not be able to

activate ELMER.

23.3 Load State Table (ELMER.QSO)

This option allows you to reload the state table. The name of the

state table will appear in the menu.

23.4 Pick File to Edit

This option allows you to pick and edit, a file with the default

filetype of "TXT".

23.5 Edit State Table (ELMER.QSO)

This option allows you to edit the state table.

23.6 Show State Table

This option allows you to see the contents of the state machine. It

is used in debugging the logic to ensure that the contents of the

table match what you thought you put into it. A typical set of

contents appears as shown in Figure 23.1. The keywords have been

capitalized and set to 15 characters wide. None existent characters

have been represented as periods.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 121

Figure 23.1 Typical ELMER State Table

#�##�#L#�##�##�#C#�##�##�#N#�##�##�#R#�##�##�#E#�##�##�##�##�##�#KEYWORD#�##�##�##�##�##�#U#�##�##�##�##�##�##�#FILE#�##�##�##�##�##�##�##�#

#�#  0#�#  1#�#  2#�# 0 #�#  0#�#WHAT...........#�# 0 #�#*               #�#

#�#  1#�#  2#�#  3#�# 0 #�#  0#�#ENGLISHMAN.....#�# 0 #�#*               #�#

#�#  2#�#  2#�#100#�# 0 #�#  0#�#SYSTEMS........#�# 0 #�#*               #�#

#�#  3#�#  3#�#  4#�# 0 #�#  0#�#DOING..........#�# 0 #�#*               #�#

#�#  4#�#  4#�#  1#�# 0 #�#  0#�#COUNTRY........#�# 0 #�#ME-USA.TXT      #�#

#�#  5#�#  1#�#  1#�# 0 #�#  0#�#EQUIPMENT......#�# 0 #�#PK232COM.TXT    #�#

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 122

24.0 RADIO MENU

The Radio Menu provides you with the capability to exercise the

interface to a Kenwood Radio. The options available are shown in

Figure 24-1. Type the single letter to make your choice. All

commands will be ignored if the radio is not enabled. The radio

interface is automatically enabled at LAN-LINK initialization time.

Figure 24-1 The Kenwood Radio Menu

F Select VFO A/B

M Set Radio Mode

Q Turn Flag QSY ON/OFF or N/A

R Read VFO A Frequency

S Set Frequency

V Speak Frequency

24.1 Select VFO A/B

This option sets the VFO that will be interrogated or set by LAN-

LINK.

24.2 Set Radio Mode

This option sets the radio mode (USB, LSB, FM, CW etc.). You will

be prompted to supply the mode.

24.3 Turn Flag QSY ON/OFF or N/A

This option set the QSY Flag as follows:

N/A       Radio is not enabled

ON        QSY to the frequency of a DX Alert (if in PacketCluster

mode)

OFF       Don't QSY in response to a DX Alert

24.4 Read VFO Frequency

This option reads and displays the frequency of the selected VFO.

24.5 Set VFO Frequency

This option sets the frequency of the selected VFO. You will be

prompted to supply the frequency.

24.6 Speak VFO Frequency

This option sends a command to the radio to announce the frequency

of the selected VFO. If the radio is equipped with a voice

annunciator the radio will announce the frequency. If the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 123

radio can't speak, the command will be ignored.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 124

Appendix 3  LAN-LINK Disk Files

The following disk files are associated with the LAN-LINK program.

LAN-LINK.COM

LAN-LINK.SYS

*.RUN

LAN-LINK.LOG

LAN-LINK.DIR

CONFIG.SYS

*.MBX

*.BBS

LAN-LINK.001 .. LAN-LINK.010

QBU-RX.BAT

QBU-TX.BAT

LAN-LINK.QTA

QZ*.BAT

ELMER.QSO

LAN-LINK.OUT

*.DMO

*.CQ

The "*" represents a "wildcard" character in the manner of DOS.

Consider each file in turn.

1.0 LAN-LINK.EXE

This is the program. You activate it from DOS by typing the word

"LAN-LINK" followed by "ENTER" in the normal manner of invoking DOS

commands.

2.0 LAN-LINK.SYS

This file contains the configuration information for both the

program and the TNC. Use the customization options in the parameters

menu to make any changes not taken care of in the INSTALL program.

The contents of the default file together with an item number are

shown below.

1. Callsign

2. Selcal

3. CQ Text

4. CTEXT Line

5. ELMER Flag

6. CQ Delay (seconds)

7. CQ Delay (minutes)

8. :QSP: Message Relay Flag

9. PC Serial Communications Port Number

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 125

10. TNC Type

11. PC to TNC Baud Rate

12. VHF Log Book File

13. HF Log Book File

14. GMT Offset

15. Path Directory File name

16. Brag Tape File name

17. PBBS Snatch Mail Trigger Word

18. QTC Snatch Flag

19. PBBS Snatch Flag

20. Shut Down Mode

21. PC to TNC Data Bits

22. PC to TNC Parity Bit

23. PC to TNC Stop Bits

24. DCD Flag

25. QBM Download Sub-directory

26. Run File Directory Path

27. PBBS Read Request Command

28. Local PBBS Callsign

29. SAREX Callsign

30. SAREX Capture to Disk File name

31. PacketCluster Call

32. TNC Specific Parameters

33. Node Drop Link Flag

34. Sound Flag

35. Display Computer Generated Commands Flag

36. Date/timestamp in Call Sequence Flag

37. Keyboard Character Beep value.

38. :QBU: Receiving Station Batch file (QBU-RX.BAT)

39. :QBU: Originating Station Batch file (QBU-TX.BAT)

40. Capture-to-disk Flag

41. Beacon Control/LAN-LINK handshake Flag

42. Read Bulletin Character

43. PBBS Subject String

44. PBBS Message String

45. VHF Paclen

46. HF Paclen

47. Beacon Time

48. Number of CQ Lines

49. Word wrap

50. PC Port 4 Custom Address

51. PC Port 4 Custom IRQ

52. VHF Stream switch character

53. HF Stream switch character

54. TNC Error message

55. Connect Timeout Delay

56. Custom Prompt

57. ELMER Configuration File

58. Expand Transmitted File flag

59. Input Message Base Count

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 126

60. Default users

61-88. Custom Colors

89. First PacketCluster Command

90. Second PacketCluster Command

91. TNC Mailbox ON Command

92. TNC Mailbox OFF Command

93. VHF Maxframe Command

94. HF Maxframe Command

95. Inhibit VHF Logging Flag

96. Inhibit HF Logging Flag

97. Printer Permanent Flag

98. Log Disk Update Flag

99. Secondary Log Flag

100. Auto Answer Flag

101. RUN File Disk Update Flag

102. Default CW Speed

103. Secondary Logbook

104. BBS List Command

105. BBS List Mine Command

106. BBS Custom Command

107. BBS Bye Command

108. Start Up Packet Terminal Mode

109. Page Time (:QIC: Seconds)

110. Packet Cluster DX Alert Tone CW Dit time

111. Packet Cluster DX Alert Tone CW Note

112. Packet Cluster DX Alert Mode

113. Mode File Flag

114. BAUDOT Color Change Text String

115. Default RTTY Baud Rate

Lines 116 to 123 (inclusive) apply to the DSP-2232 and PK-900 only

but must be present in all SYS files.

116. Dual Window Flag

117. TNC String for Modem 9600 baud packet

118. TNC String for Modem 1200 baud packet

119. TNC String for Modem RTTY

120. TNC String for Modem 300 baud packet

121. TNC String for Modem RTTY and 1200 baud packet

122. TNC String for Modem 300 baud and 1200 baud packet

123. TNC String for Modem Fuji

124. Delay after a command line

125. Radio Name

126. Radio Port

127. Radio Port baud rate

128. Radio Port Data Bits

129. Radio Port Stop Bits

130. Radio Port Parity

131. Radio Address

132. Flag QSY

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 127

133. Flag DX Alert Prefix Check

134. Start of band parameters for DX Alert message log search

135. * end of band parameters; modes for DX Alert message log search

136. Start of Scan Words (MUST HAVE AT LEAST ONE)

137. * end of Scan Words

138. Start of Common Calls (MUST HAVE AT LEAST ONE)

139. * end of Common Calls

140. Remaining TNC start up Commands.

The contents of each line are described in the section of the

Parameters menu which deals with customizing them. The others,

described below, can only be changed by using the Editor. The

remaining lines contain commands to the TNC. For further information

about them refer to the TNC manual. LAN-LINK assumes different

defaults (in some instances) to those set in the TNC. The LAN-

LINK.SYS file contains only those that are different to the TNC

default reset state.

2.1 CTEXT Line

This parameter is the CTEXT line changed in the TNC Menu.

2.2 ELMER Flag

This parameter is the ELMER Flag. If it is a "1", ELMER is primed

for activity. ELMER is only active in the Packet communications

mode, when you are connected to another station. If the ELMER flag

is a "0", or the ELMER state table file is not present in the

default directory, LAN-LINK will never activate ELMER.

2.3 PC Port 4 Custom Address

This parameter is the decimal value of the address of your Port 4.

Use this parameter if you are using a non standard Serial I/O card.

Configure the PC Port to 4. The PC default address for Com4 is 2E8

(hex) or 744 (decimal).

2.4 PC Port 4 Custom IRQ

This parameter is the decimal value of the IRQ of your Port 4. Use

this parameter if you are using a non standard Serial I/O card.

Configure the PC Port to 4. The PC default IRQ for Com4 is 3.

2.5 VHF Stream Switch Character

This parameter is the VHF stream switch character. If you wish to

change it, you must enter the ASCII representation in this line, and

the equivalent stream switch command in the LAN-LINK.SYS file.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 128

2.6 HF Stream Switch Character

This parameter is the HF stream switch character. If you wish to

change it, you must enter the ASCII representation in this line, and

the equivalent stream switch command in the LAN-LINK.SYS file. The

KAM is currently the only TNC that has different HF and VHF stream

switch characters.

2.7 TNC Error Message

This parameter is the error message your TNC generates. The PK-232

generates "?What?". Others may generate "EV?" or "?EH".

2.8 Connect Timeout Delay

This parameter is the time, in minutes, LAN-LINK lets you stay

connected to another packet station, after the receipt of the last

incoming character.

2.9 ELMER Configuration File

This parameter is the name of the default ELMER Configuration file.

It is changed through the ELMER Menu.

2.10 Expand Transmitted Files Flag

The Flag tells LAN-LINK to scan the outgoing text when a file is

transmitted and perform the same conversions it does to the contest

text. You can insert the '##', '#1', '#N' and '#T' parameters in the

files to transmit, your call sign, the other station's call, and the

time of day. Set this parameter to 1 to enable the feature, 0 to

disable it.

2.11 First PacketCluster Command

This parameter is the first command (up to 20 characters) LAN-LINK

sends to the PacketCluster after receiving a ">" character at the

end of line from the PacketCluster.

2.12 Second PacketCluster Command

This parameter is the second command (up to 20 characters) LAN-LINK

sends to the PacketCluster after receiving a ">" character at the

end of line from the PacketCluster.

2.13 TNC Mailbox ON Command

This parameter is the command (up to 15 characters) your TNC

expects to turn its internal mailbox ON.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 129

2.14 TNC Mailbox OFF Command

This parameter is the command (up to 15 characters) your TNC

expects to turn its internal mailbox OFF.

2.15 VHF Maxframe Command

This parameter is the command (up to 15 characters) your TNC

expects to set the VHF MAXFRAME parameter.

2.16 HF Maxframe Command

This parameter is the command (up to 15 characters) your TNC

expects to set the HF MAXFRAME parameter.

2.17 Inhibit VHF Logging Flag

When set to 1, automatic VHF Logging is disabled.

2.18 Inhibit HF Logging Flag

When set to 1, automatic HF Logging is disabled.

2.19 Printer Permanent Flag

When clear (set to 0), if you leave the printer on during a packet

connect, it will stop printing when you disconnect. This stops the

printer generating lots of garbage. If you set this parameter to 1,

the printer will not stop printing when you disconnect.

2.20 Log Disk Update Flag

If this flag is set to 1, LAN-LINK writes each log entry to disk as

you enter it. If this flag is not set (=0), LAN-LINK writes the log

entry to a buffer and updates the disk when the program is

terminated. Set the flag ON to ensure that rare DX contact is

logged, Set it OFF for speed, or if power dropouts are not a problem

in your area.

2.21 Secondary Log Flag

If this Flag is set to 1, LAN-LINK allows you to search both the

current log book and a secondary logbook for the call of a station.

2.22 Auto Answer Flag

If this flag is set to 1, AND you have a LAN-LINK.OUT file on your

system, LAN-LINK will transmit the contents of the LAN-LINK.OUT file

to every incoming connection.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 130

2.23 Ctdsk File Disk Update Flag

If this flag is set to 1, LAN-LINK will close the capture-to-disk

file (YYMMDD.RUN) between received messages. If the flag is not set,

LAN-LINK will close the file when all messages have been received,

or a DISCONNECT takes place.

2.24 Default CW Speed

This is the default CW speed for the multi-mode TNC (KAM, PK-232 or

MFJ1278).

2.25 Secondary Logbook

This is the name of the secondary logbook.

2.26 BBS List Command

This is the BBS List Command. The default value is 'L'.

2.27 BBS List Mine Command

This is the BBS List Mine Command. The default value is 'LM'.

2.28 BBS Custom Command

This is a configurable BBS List Command. The default value is 'KM'.

2.29 BBS Bye Command

This is the BBS Bye Command. Change this parameter when the default

'B' command only drops you to the Node.

2.30 Start Up Packet Terminal Mode

This is the Packet Terminal Mode when LAN-LINK is first activated.

Set this to the letter corresponding to the terminal mode you like

being in. See Appendix A, Section 21.1. Use the corresponding

letter, as shown in Table 2.30.

Table 2.30 Start Up Terminal Mode

Letter    Mode                Actions

I         INFO lets you only see packets containing information.

S         SOLO lets you only see messages addressed to you.

C         CQ/B lets you see beacons and CQ calls as well.

T         TRFC lets you see packets containing messages as well.

E         EVRY lets you see everything on channel, including the

link control packets.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 131

2.31 Page Time (:QIC: Seconds)

This is the number of seconds the system pages the operator in

response to a QIC command, if the sound is turned on.

2.32 PacketCluster DX Alert Tone CW Dit time

This is the time period for 1 dit sound in the PacketCluster

audible DX alert message.

2.33 Packet Cluster DX Alert Tone CW Note

This is the tone for the PacketCluster audible DX alert message.

2.34 Mode File Flag

If this flag is set, LAN-LINK will change the name of the LAN-

LINK.001 to LAN-LINK.010 when you change communications modes. The

name will change to the mode followed by a number, i.e BAUDOT.001 to

BAUDOT.002, or AMTOR.001 to AMTOR.002.

2.35 BAUDOT Color Change Text String

This is a pair of characters that LAN-LINK will scan text appearing

in the incoming window for. When LAN-LINK recognizes the pair, it

will change colors. This parameter is customizable to allow you to

override apparent random changes of color, since there is no agreed

standard way for signalling 'over' in BAUDOT.

2.36 Dual Window Flag

This flag applies to the PK-900 and DSP 2232 only. If this flag is

set, LAN-LINK separates out the text streams coming from each port

and directs them to two separate windows. If the flag is cleared,

the text streams are combined into a single incoming window.

2.37 TNC String for Modem 9600 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working 9600 baud packet.

2.38 TNC String for Modem 1200 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working 1200 baud packet.

2.39 TNC String for Modem RTTY

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working HF RTTY.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 132

2.40 TNC String for Modem 300 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working 300 baud HF packet.

2.41 TNC String for Modem RTTY and 1200 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working simultaneous HF RTTY

and VHF 1200 baud packet.

2.42 TNC String for Modem 300 baud and 1200 baud

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for simultaneous HF 300 baud packet

and 1200 baud VHF packet.

2.43 TNC String for Modem Fuji

This item applies to the PK-900 and DSP 2232 only. This is the

command used to select the modem for working Fuji 1200 baud PSK

packet.

2.44 Delay after a command line

This item is the number of milliseconds to wait between sending

lines to the TNC output buffer.

2.45 Radio Name

This item is the name of the Radio. Kenwood has been tested, Icom

may work (coded but not tested). If this is not set to Kenwood or

Icom, the Radio functions are disabled.

2.46 Radio Port

This item is the serial port for the Radio (1 to 4). If set to 0,

the Radio functions are disabled.

2.47 Radio Port baud rate

This item is the serial port baud rate for the Radio. Check the

correct value in the Radio manual.

2.48 Radio Port Data Bits

This item is the number of data bits on the serial port for the

Radio. Check the correct value in the Radio manual.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 133

2.49 Radio Port Stop Bits

This item is the number of stop bits on the serial port for the

Radio. Check the correct value in the Radio manual.

2.50 Radio Port Parity

This item is the parity value on the serial port for the Radio.

Check the correct value in the Radio manual.

2.51 Radio Address

This item is ignored for Kenwood radios. For Icom, it is the

decimal value of the radio address. Check the correct value in the

Radio manual.

2.52 Flag QSY

If the flag is set, LAN-LINK tunes the radio to the frequency of

the PacketCluster DX Alert message automatically.

2.53 Flag DX Alert Prefix Check

This flag determines how LAN-LINK responds to a DX Alert message.

If the flag is set, LAN-LINK checks to see of the prefix is in the

log.

2.54 Band parameters for DX Alert message log search

These lines contain the band, reference frequency (Hz), low and

high band search flags. Each element is separated by a ","

character. The elements can be changed via the Radio Menu. Typical

values are:

10, 28300000.0,Y,Y

12, 24930000.0,Y,N

15, 21200000.0,Y,Y

17, 18110000.0,Y,Y

LAN-LINK checks the frequency of the DX Alert message to see if

there is a match on any of these lines. If there is, then LAN-LINK

compares the frequency with the value on the corresponding line to

determine if the DX Alert frequency is higher or lower than the

reference frequency.

If the frequency is lower than the reference, and the low band

search flag is "Y", LAN-LINK will check to logbook to see if the

prefix is in the log.

Similarly, if the frequency is higher than the reference, and the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 134

high band search flag is "Y", LAN-LINK will check to logbook to see

if the prefix is in the log.

If LAN-LINK does not find a match, the check mark is displayed.

2.55 Modes for DX Alert message log search

This line marks the end of the band parameters for DX Alert message

log search with the *. It then contains the communications modes

corresponding to the low and the high parts of the band. Typical

values are:

*,CW,SSB

2.56 TNC Parameters

The remaining lines contains parameters loaded into the TNC at

initialization.

If you are not using a TNC1 you MUST set the HEADERLN parameter to

ON. The packet headers and the text of the packets must show up on

your screen on separate lines. You must also have the "Echo as Sent"

feature set ON ("EAS ON" for PK-232 and "XM" ON " for the KAM).

3.0 *.RUN

These files contain incoming data from the TNC recorded while the

"capture-to-disk" function was active. LAN-LINK names them as

YYMMDD.RUN automatically (YY = Year, MM = Month, DD = Date, i.e.

920903.RUN is a file for 3 Sept 1992).

4.0 *.DBF (HF.DBF and VHF.DBF)

These files contain the log entries. They are Dbase 3 and 4

compatible. The format of the logbook file is the same as that used

by the LOGBOOK package of the PC-HAM software package, with a little

additional space for comments.

5.0 LAN-LINK.DIR

This is the directory file. You use it to store names, handles,

NET/ROM, TheNet, KA-Node, ROSE or digipeat paths as well as

name/callsign relationships. Sample lines are shown below.

Joe G3ZCZ

Alon 4Z4ZB V 4X6AA

Milt 4X6AA

LR 4X6LR

hf-il 4x4hf v 4z4zb 4x4il

hf-rj 4x4hf v 4z4zb 4z4rj

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 135

K1HTV ELK v WB4APR-5! SNJ3! NNJ! WMA! SCCT!K1HTV

w9tnn-2 jrw !% w3eax-7 !& w9tnn-2

You create this file with the Editor or by using your word

processor in its non-document mode. You must leave at least one

space character between the first (key) word and the connect path.

When you type in the key word, the program does not care about upper

or lower case.

You may also use the "reMember Path" option in the Call Menu to

make entries in this file.

If you already have a YAPP.DIR or a PK232COM.DIR file just rename

it to LAN-LINK.DIR and add any NET/ROM and/or KA-Node paths.

All path entries in LAN-LINK.DIR must be on a single 80 column

line.

6.0 CONFIG.SYS

If you are going to use the logging features, you must have a

CONFIG.SYS file on the boot disk. This file must contain the line

FILES = 20.

See the DOS manual for more information about the CONFIG.SYS file.

7.0 *.MBX

These are the files associated with the stations you have

designated as LAN store and forward mailboxes. Other stations can

only :QRU: with you if you have designated them as QRU stations and

have a "CALLSIGN.MBX" file on your disk. For example, if you want to

use 4Z4ZB as a store and forward mailbox, you will have to create an

ASCII file called 4Z4ZB.MBX using Editor, or the non-document mode

of your word processor. In this file you will put a list of the call

signs of the stations 4Z4ZB can connect with reliably. For example,

if you want 4Z4ZB to be able to store messages from you to 4X6AA,

4X6LR, G3ZCZ, ON8IK, F0WN,and G8BTB, you would put those call signs

in the file, one call on a line as follows.

4X6AA

4X6LR

G3ZCZ

ON8IK

F0WN

G8BTB

When the QRU function is invoked either by you manually or by 4Z4ZB

connecting to you and sending you the command :QRU:, any message

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 136

files present on your system addressed to any of those stations

(E.G. G3ZCZ.OUT and ON8IK.OUT) will be transferred from you to 4Z4ZB

just as if you had tried to :QSP: the messages manually or as if

G3ZCZ or ON8IK had connected directly to you. The file names on your

system will be changed to *.OLD.

You must create one file for each designated store and forward

station, i.e. 4Z4ZB.MBX, G3ZCZ.MBX etc.

8.0 *.BBS

These are LOCAL-BBS.BBS AUTO-Upload Command files. Each file is

named for the callsign of the BBS that you wish to access. It must

have exactly the name as the callsign of the BBS. If the BBS is

W9TNN-1 the name of the file must be W9TNN-1.BBS, or if the BBS is

4Z4ZB then the file must be called 4Z4ZB.BBS. The file must be an

ASCII file. that means the word processor must be used in the non-

document mode to create it.

This file contains the command sequence that you would have typed

at the keyboard if you were accessing the BBS manually. The ONE

difference is that the text of a message you are uploading with the

S command must be followed by the character strings ":EOF:" or "/EX"

(on a separate line) as illustrated by the example below.

If you use the "D" command to download a text file, the file will

end up in a separate file on your disk ONLY IF a file of the same

name does not exist on your drive.

Do delete "read" mail from your local PBBS. Don't clutter the PBBS's

disk with your read mail. The "LM" command is built into LAN-LINK to

remind you of what you have out there.

The PBBS "S" command (including its variations such as "SP") is the

only multiple line command allowed. You can put all the single line

commands such as "D", "H", "?", "L", and "R" in the PBBS.BBS file.

A "multiple command" is defined in this instance as a command which

allows you to enter multiple lines before the PBBS will send back a

">" character (i.e. one that the PBBS tells you to type in text and

terminate it with the Control-Z (^Z) character).

A typical sequence of messages could be as shown below.

SP G3ZCZ @ N4QQ

LAN-LINK

I think it is great. However, why don't you change the

following...... (add your comments here).

73 de your name

/EX

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 137

k 1234

d \net\lan-link.doc

s g8btb

Test Message

This is a test message.

\EX

lm

r 567

r 593

9.0 LAN-LINK.001 - LAN-LINK.010

These are 10 different text files with fixed names. They can be

transmitted using the Alternate function keys. Their contents can be

viewed using the Shift function keys. In all cases the function key

to use for a particular file corresponds to the number in the file

suffix. For example, use Alt/Shift Function Key 3 (Alt-F3) to

transmit/view file LAN-LINK.003. You may also use the Edit Menu to

edit and/or create the files.

In case you forget what you put in each of the files, LAN-LINK

allows you to take a peek into all of them at the same time. Use

Control Function key 1 (Control-F1) to look at the first line of all

the LAN-LINK.001 - LAN-LINK.010 files and the Brag Tape.

If the Mode File flag is set, LAN-LINK changes the files as you

change modes. You thus have 10 files in EACH mode, accessible via

the same set of control function keys. The file names are shown in

Table 9.

Table 9 LAN-LINK Mode File Names

COMMUNICATIONS MODE      Mode Flag           FILE NAMES

All                           OFF       LAN-LINK.001 to LAN-LINK.010

Packet                        ON        LAN-LINK.001 to LAN-LINK.010

AMTOR                         ON        AMTOR.001 to AMTOR.010

BAUDOT                        ON        BAUDOT.001 to BAUDOT.010

ASCII                         ON        ASCII.001 to ASCII.010

CW                            ON        MORSE.001 to MORSE.010

G-TOR                         ON        G-TOR.001 to G-TOR.010

Pactor                        ON        PACTOR.001 to PACTOR.010

10.0 QBU-RX.BAT

This is the command or batch file used in the :QBU: originating

station.

11.0 QBU-TX.BAT

This is the command or batch file used in the :QBU: receiving

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 138

station.

12.0 LAN-LINK.QTA

You can leave a message on your system for someone else in a file

named CALLSIGN.OUT. When the station with that callsign connects to

you, the message will be sent, and the filename will be changed from

CALLSIGN.OUT to CALLSIGN.OLD. This is fine as long as the path

between the stations good. If the path is marginal, it is possible

that your station will transmit the message and change the filename,

but the link will drop out before the message is received at the

other station. If the other station does not connect to you and

:QSM:, the message may get lost. If this situation occurs, you can

tell LAN-LINK that there is a bad path between you and that station,

and not to change the file names without an explicit :QTA:

instruction from that other station. You do this by putting the call

of the station(s) having a bad path in an ASCII file called LAN-

LINK.QTA. Put one call on each line in the form

N9BE

G8BTB

You must use the root call of the station without an SSID. that

means, if the station is N9BE-4, just put N9BE in the file.

LAN-LINK will automatically acknowledge incoming messages from

stations with callsigns in the LAN-LINK.QTA file with a ":QTA:".

13.0 QZ*.BAT

As any LAN-LINK station can either receive or originate a :QZD: and

a :QZU: command, LAN-LINK uses four batch files to control Zmodem

transfers. These files are as follows.

QZU-RX.BAT Batch file implemented after receipt of :QZU:

QZU-TX.BAT Batch file implemented after transmission of :QZU:

QZD-RX.BAT Batch file implemented after receipt of :QZD:

QZD-TX.BAT Batch file implemented after transmission of :QZD:

Each LAN-LINK user must configure the QZ*.BAT files for their

systems. The port number in the batch file should be set to the PC

I/O Port used. The data rate in the batch file should be set to the

data rate used between the PC and the TNC, NOT the data rate used on

the radio link.

For example, the default QZU-RX.BAT file of "pcz 1 1200 rz %1"

should be changed to "pcz 2 1200 rz %1" if you are have interfaced

your TNC to Port 2 on your PC, and if, the baud rate between the TNC

is 2400 baud, the file should be changed to "pcz 2 2400 rz %1".

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 139

Each of the four batch files have to be customized for each user

who is not using PC Port Com1 and 1200 baud between the PC and their

TNC. The INSTALL program takes care of that chore. The high level of

customization required here is because different Zmodem imple-

mentations require different sequences of instructions and LAN-LINK

does not want to limit you to a specific one.

14.0 ELMER.QSO

This is the state table (configuration file) for ELMER. The

contents of the file are as described in Appendix 8. If this file is

not present in the same sub-directory as LAN-LINK, you will not be

able to activate ELMER.

15.0 LAN-LINK.OUT

This file will be transmitted to all incoming connects (except when

in the multi user mode), if the file is present, and if the Auto

Answer flag is enabled. You can think of this file as a CTEXT File.

16.0 *.DMO

These files are demonstration or test files. When LAN-LINK is put

into the Demo Mode (LAN-LINK Menu), LAN-LINK treats the lines of

text read from these files as if they came in from the TNC via the

serial port. Use these files for demonstrations and/or tests of

different features.

17.0 *.CQ files

These files allow you to customize the CQ call in the non-packet

communications modes.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 140

Appendix 4 Local Area Network Capability

VHF Packet radio systems can be considered as part of a Local Area

Network (LAN) in which messages can be left by one station in the

computer belonging to a second station. At HF the same is true, but

the area becomes greater. The basic problem here is that people can

only send and receive messages to or from you when you are on-line.

To compensate for this, BBS stations were developed which allowed

both messages and bulletins to be stored by anyone for later

retrieval.

LAN-LINK allows you to store messages in your computer so as to use

it as an automatic answering machine. By storing the messages in a

file called by the name of callsign, with the file extension ".OUT",

anyone connecting to the computer will get their message without

having to request it. LAN-LINK also extends that concept to cover

the eventuality of you wanting to take your computer off-line for

some reason. You may now load the message into any other computer

you can connect to using elements of the Q code adapted to the High

Level Network Communications Language (NC/L) first proposed in my

book Software for Amateur Radio, (Book number 1560) published by TAB

Books, Blue Ridge Summit, Pa. 17214 U.S.A.

1.0 LAN Protocol (G3ZCZ Version)

The protocol is as follows. When connected to someone who has their

LAN-LINK configured as a host, if you want to store a message on

that system, you use "QSP". You send the other station ":QSP:

CALLSIGN" where "CALLSIGN" is the callsign of the station that the

message is for, not the callsign of the host station in whose

computer you are storing the message. [Note use only one space

character after the :QSP:].

For example if you want to store a message for 4Z4ZB in 4X6AA's

computer which is configured as a LAN-LINK Store and Forward system,

you would first connect to 4X6AA and then send the command as ":QSP:

4Z4ZB". Better still use :QSP: option in the LAN-LINK Menu to

automate the sequence.

The computer at 4X6AA will respond either with a statement saying

that it is ready for you to go ahead, or send a message saying that

it can't comply. If it is ready you get a positive reply which will

take the form :QRV: CALLSIGN which if you know the Q code, means "I

am ready to accept a message for CALLSIGN".

At this time you may go ahead and send the message. If you type the

message at the keyboard, you may use either a Control-Z (^Z)

character or the character sequence ":EOF:" followed by a carriage

return (the "Enter" key) to terminate the message. If you have first

written the message into a text file you may then send it using the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 141

"Send File" option of the Files Menu and the termination sequence is

automatic.

Once you have completed the message, the other (host) computer will

either reply that the message has been successfully stored or give

you an error message.

If the message is stored and ready to be sent next time the

addressee connects to that computer, you will see the message :QSL:

on your screen. If something went wrong, you will get back a

negative message taking the form :QNO: followed by a number. The

number tells you why the operation failed.

2.0 Message Format

The message is stored just as if you had left it in your system

(except that a header is added identifying the time of reception and

the call of the sending station). It will contain a header showing

the date and time it was received and your callsign. Should a

message for that station already be in the system, yours will be

appended to it. In the event your upload is aborted, the amount of

text received before the abort occurred will be stored as the

message.

When you disconnect from the other LAN-LINK host station, its mail

beacon will be updated.

Once the message is loaded in the host, it can only be deleted by

the operator of the host station. When the addressee connects to the

host and receives the message, the file type will change from ".OUT"

to ".OLD". Note QTA stations require positive confirmation of

receipt of message for the renaming to take place.

3.0 ASCII File Uploading/Downloading

There comes a time when you want to leave a file on your system for

someone to download later. You can do this by using NC/L to control

the transfer. You must leave the file in the file download sub-

directory defined in the LAN-LINK.SYS FILE (default value is C:\LAN-

LINK\FILES). You must first create the sub-directory, if it does not

exist. The station who wants the message then asks for it using

:QBM: FILENAME.TYP. They do not need to use the full sub-directory

path. They must, however, leave a single space character between

:QBM: and the filename. In AMTOR the Mailbox must be turned on for

remote functions.

For example, you have the latest ARRL DX bulletin, and you want to

pass it on. You could pass it to selected people by copying the file

to individual messages with the filetype ".OUT", e.g. 4X6AA.OUT,

4Z4ZB.OUT or even G3ZCZ.OUT if you want me to get it, which wastes a

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 142

lot of disk space.

On the other hand you could tell people that the file was available

for downloading, either in the CTEXT connect message line which

everyone gets when connecting to you by configuring LAN-LINK

accordingly, or in individual (".OUT") messages.

If you include the following line in your CTEXT message

"ARRLDX.015 now available"

people will know that to download it, they just have to type the

line ":QBM: ARRLDX.015" (one space character between :QBM: and the

file name) to get the file. LAN-LINK is not designed as a BBS,

however, if you copy the contents of the \LAN-LINK\FILES sub-

directory into a file called DIR (USE DIR > DIR in DOS), anyone can

get a list of the files you have stored in it for downloading by

typing :QBM: DIR. Since they do not get the real directory, just

what was in it at the time you copied it into the file "DIR", you

may keep "hidden" files in this sub-directory. You may thus leave a

file for someone to download, tell them that its there by leaving

them a message (which they will get automatically when they connect)

and no one else connecting will know that it is there.

If you want to upload a file, ":QDB:" allows you to upload text or

ASCII files.

If you use the "lan-linK" option of the Packet Terminal Menu, LAN-

LINK will automate the QDB and QBM features for you.

4.0 Path Determination to a DX station

If you want to establish a digipeat path to a station somewhat out

of your direct range, you need to know which of the stations that

you can connect to can hear that desired DX station. If you could

get a call monitored (MH list) from the stations that you connect

to, you would be able to see if the station you are connected to has

heard your desired DX station.

LAN-LINK uses the :QMH: command to request such a list. When LAN-

LINK receives a :QMH: command, it drops back to the TNC Command Mode

and issues a "MH" command to the TNC (except to a TNC1 which does

not support it). It stores the list received in a temporary buffer,

and then returns to the Converse Mode and transmits the list back to

the station it is linked to.

By judicious use of :QMH: you can determine paths to other

stations. Note however, that just because one station can hear

another station, it does not mean that it can work it. For example,

the station you are connected to may be using a power level of 1

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 143

watt or so, while the station 200 miles away that it heard was using

100 watts. Test the path yourself, or/and leave a message asking how

reliable the connect path between those two stations is.

If you and a small group use a non standard packet channel for

inter-group communications, you may also use the :QMH: function to

find out who the station you are connected with, has heard recently,

when you first sign on the LAN channel.

5.0 Binary File Transfers

In the PC world most binary file transfers seem to be taking place

using the YAPP protocol built into the YAPP user terminal program

and the BBS software written by Jeff Jacobsen, WA7MBL. As Jeff

stated in the documentation, YAPP was written to provide a terminal

program for use in testing the WA7MBL BBS code. However for many

radio amateurs, YAPP was their first terminal program.

The YAPP binary transfer protocol is similar to the Xmodem protocol

used on telephone line BBSs. In order to transmit a file, the

sending station breaks the file down into blocks of characters. Each

block is transmitted in sequence. After a block has been

transmitted, the sending station waits for the receiving station to

acknowledge that block and verify that it was received without

errors.

When two stations are alone on a frequency, they can transfer data

quite quickly. So back in 1986 when YAPP was written, it provided

the few hams using packet radio with a binary file transfer

capability. Today, however, the situation is different. In most

urban areas, each packet radio channel is occupied by many stations,

and the requirements for binary file transfers are somewhat

different.

6.0 Requirements for Transfer Protocols

The AX.25 protocol provides for a basic verification that packets

have been received error free. If the binary file transfer protocol

could establish the connection, start sending blocks, and then keep

sending data until one of three things happen; the link drops out,

the receiving station signals that some of the data were received

with errors, or the whole file is transmitted correctly.

If the whole file is transmitted, then the sending station just has

to wait for an acknowledgment from the receiving station that the

file was received. If the circuit is busy with several stations on

the channel, or the path goes through one or more Nodes, there will

be varying delays in the delivery time of the packets. By not having

to acknowledge each block of data, the file can get through much

faster than it would have, in the case in which each received block

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 144

has to be individually acknowledged.

If the link drops out, it would be nice if the computers, could

determine how much of the file was received, then pick up the

transmission at that point next time they link up.

If the receiving station signals that some data were received in

error, the sending station should then begin to retransmit from the

start of the error.

7.0 The Zmodem Protocol

This situation is not unique to the amateur radio packet network.

It is also present in the commercial packet switching environment,

and the Zmodem protocol was developed to provide the capabilities

described above. Zmodem thus seems to be an almost ideal binary file

transfer protocol for use on radio amateur packet channels, and is

incorporated in LAN-LINK 2.00.

LAN-LINK uses the :QZD: and :QZU: commands to provide the

capability to remotely download and upload binary files. :QZD:

allows you to download a file from another station, while :QZU:

allows you to upload a binary file. :QZD: is configured so that if

the link drops outs during the transfer, when the connection is once

again established, a subsequent :QZD: for the same filename, will

cause the transfer to pick up where it left off when the link

dropped out. This feature is not allowed in the upload :QZU: command

to inhibit people from injuring files that already exist on the

other computer. Each command may be initiated from either end of the

link, but to gain the most from the recovery capability, files

should be downloaded by the receiving station.

8.0 The PCZ Zmodem Package

LAN-LINK implements the Zmodem protocols using batch files in DOS

that call up external programs. This approach allows stations to try

different versions of Zmodem and experiment to find the optimal one

for their area. LAN-LINK comes with the (PCZ) Public Domain

implementation of the Zmodem, Ymodem, Xmodem, Xmodem-1K and Sealink

protocols by Drue Kennon and Gary Smith.

Since any LAN-LINK station can either receive or originate a :QZD:

and a :QZU: command, LAN-LINK comes with four default batch files to

control Zmodem transfers. These files are as follows.

QZU-RX.BAT Batch file implemented after receipt of :QZU:

QZU-TX.BAT Batch file implemented after transmission of :QZU:

QZD-RX.BAT Batch file implemented after receipt of :QZD:

QZD-TX.BAT Batch file implemented after transmission of :QZD:

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 145

Each pair of users can experiment with other protocols and Zmodem

implementations using the :QZD: and :QZU: commands after reading the

PCZ documentation. The :QBU: command can also be used if the

corresponding DOS batch files (QBU-RX.BAT and QBU-TX.BAT) are

created.

LAN-LINK contains an output buffer to store data being output to

the TNC. The TNC contains a buffer which stores data about to be

sent on the radio link. In the normal course of events, the two

buffers fill up, so that the Zmodem package used by LAN-LINK may

think that all the data has been transmitted, when in fact there are

still characters left in the buffers, or the last packet has not yet

made it to the destination station. In this case, you may see a

message on your screen that the system has sent all the data and is

waiting for the final acknowledgment, while at the same time you can

see from the LEDs on the TNC that you still have outgoing data in

your system. Have patience and wait a while to let the two systems

synchronize.

In the event that the link is bad and errors occur, this buffering

may cause problems which require some of the Zmodem timing

parameters to be changed. Should this situation arise, read the

Zmodem package documentation.

The flexibility offered by the Zmodem protocol has many advantages

when used in a packet radio network. LAN-LINK 2.00 implements Zmodem

by the use of an external public domain protocol package which

provides for the maximum degree of user customization. This

capability is provided at the cost of having the individual user

customize four batch files using the text editor built into LAN-

LINK.

9.0 BASIC NC/L DICTIONARY

Messages can be transferred into any other LAN-LINK computer you

can connect with using elements of the Q code adapted for the

purpose. NC/L command words are summarized in the following

paragraphs. There is no command word to request your message. To

receive your message, you do nothing. You will receive your mail

automatically when connecting/linking with a LAN-LINK station. You

cannot normally read messages addressed to another person.

9.1 :QBM:

To download an ASCII file, send ":QBM: file name.type". The

filename.type is the file you want. i.e. ":QBM: DIR".

9.2 :QDB:

To upload ASCII files to another LAN-LINK system send ":QDB:

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 146

filename.type". Files will be uploaded into the same sub-directory

as LAN-LINK, and will have a message header inserted at the top of

the file to allow the operator to know who uploaded the file. If the

operator then wants to make the file available for downloading, he

or she can move it to the \LAN- LINK\FILES sub-directory. The ".TYP"

following the filename is optional. If you attempt to upload a file

which has the same name as one that already exists on the remote

system, you will receive an "error" message. This command is the

reverse of :QBM:.

9.3 :QMH:

To request a call monitored list ("MH") from the LAN-LINK station

that you are connected with, send ":QMH:".

9.4 :QSM:

To request a repeat of a message from a station using LAN- LINK,

send ":QSM:". This command will be valid as long as the host station

has not deleted the YOURCALL.OLD file on his disk. If the file does

not exist, you will receive a :QNO: "error" response.

9.5 :QSP:

To leave a message, send ":QSP: callsign". The callsign is that of

the station for whom you wish to leave a message.

9.6 :QRA:

When disconnected, to see what stations (using LAN-LINK 1.53+) are

on the LAN, or which LAN-LINK stations have messages pending, send

":QRA:". All LAN-LINK stations will reply with a mail beacon within

a few seconds. The Alt-W key implements this feature.

9.7 :QRT:

To shut down a LAN-LINK AMTOR/Packet mode Mailbox or beacon station

which is causing QRM, link or connect to the station and send

":QRT:".

9.8 :QRU:

To upload messages from one LAN-LINK/PK232COM (1.38+) system to

your computer, send ":QRU:".

You may only use the QRU function with stations designated as Store

and Forward mailboxes. Put a list of stations that you can connect

to reliably in your CALLSIGN.MBX file and send it to other stations

in your LAN also using LAN-LINK. They will then be able to dump

their mail messages on you and you on them. If they do the same and

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 147

send you their file, then you will be able to send messages via them

for stations that they can work.

While QRU gives you the capability to bulk upload messages to

another station in your local area, when you take your machine off

line, it may also be used to transfer messages between two LANs via

well sighted gateway digipeaters.

9.9 :QNO:

:QNO: and error message text is a response to a request.

9.10 :QJG:

:QJG: is a response to :QRU:. It confirms that the QRU sequence is

complete. There are no more messages pending.

9.11 :QRV:

:QRV: callsign is a response to :QSP: callsign. The computer is

ready to send the message. End the message with a control Z (^Z)

character, or the sequence :EOF:

9.12 :QSL:

:QSL: is a response to a command. It confirms receipt of message to

that callsign or that a file has been successfully uploaded and

stored to disk. It is also used to acknowledge receipt of a ":QRT:"

command.

9.13 :QTA:

If LAN-LINK receives :QTA: and a message exists for the connecting

station, LAN-LINK will change the filetype from "OUT" to "OLD" and

respond with a "QSL". If a message does not exist, it will respond

with a "QNO".

9.14 :QTC:

If LAN-LINK receives :QTC: it will respond with the beacon text

showing the mail queue. :QTC: precedes a list of callsigns for whom

messages are stored up on a computer. It is used in Packet Beacon

transmissions or AMTOR Beacon mode CQ calls.

9.15 :QBU:

This feature is supplied to allow experimentation with external

protocols.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 148

9.16 :QZU:

:QZU: is used to implement an upload of a binary file from another

station using an external Zmodem protocol via the QZ*.BAT files.

9.17 :QZD:

:QZD: is used to implement a remote download of a binary file from

another station using an external Zmodem protocol via the QZ*.BAT

files. In the event that the link drops out during the transmission,

when the link is remade, and the :QZD: retried, the transfer will

pick up from the point at which it was interrupted, if the batch

files provided are used.

9.18 :QIC:

Use :QIC: to page the operator at any LAN-LINK 2.10+ station. The

sequence is:- you send :QIC: to another LAN-LINK 2.10 station. Use

the LAN-LINK Menu or type it by hand. Upon receipt, LAN-LINK begins

to page the operator if the sound flag is on. At that time it sends

you a :QRX: followed by a number, i.e. ':QRX: 30'. The number is the

number of seconds it will page the operator. At the end of the

paging sequence it will send the prompt i.e. 'G3ZCZ>'. If the sound

is inhibited it will send ':QRX: QRT' to signal that it is in a

silent configuration.

9.19 :QTR:

The :QTR: command allows you to get the date-time-text string from

the other LAN-LINK 2.10+ station. Don't confuse it with :QRT:.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 149

Appendix 5 Using Different TNCs

The Packet Mode capabilities can be used on all TNCs. There are

minor differences in the command dialogue between different TNCs.

The dialogue differences affecting the operation of LAN-LINK are

built into the program and set when the TNC type is read from the

LAN-LINK.SYS file at the program load time. The non packet modes

will obviously result in error messages if attempted on a packet

mode only TNC.

The following notes apply to different TNCs or digital

communications controllers.

1.0 TNC2 (MFJ 1270)

The following functions do not work because the TNC does not

support them.

All NON PACKET modes.

Software controlled HF/VHF switching.

Software controlled baudrate switching.

CQ/BEACON Packet terminal mode.

TNC Transmit Buffer flush.

N2GNJ noticed that he needed "LC ON" when using a Tiny 2 EPROM

Version 2.0 in the MFJ 1270B to make the automatic snatch features

work.

2.0 TNC1 (HK-4040)

The Packet Communications Mode header colors are not supported in

LAN-LINK for this TNC. The following functions do not work because

the TNC does not support them.

All NON PACKET modes.

Day/Time functions.

MH and remote :QMH: functions.

MAIL Packet Terminal Mode.

CQ/BEACON Packet Terminal Mode.

TNC Transmit Buffer flush.

Conference and multiconnect modes.

3.0 Kantronics KPC-2

The Packet Communications Mode header colors are not supported in

LAN-LINK for this TNC. The following functions do not work because

the TNC does not support them.

All NON PACKET modes.

MAIL Packet Terminal Mode.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 150

CQ/BEACON Packet Terminal Mode.

TNC Transmit Buffer flush.

N0QIX suggests adding the following lines to the LAN-LINK.SYS

file.

Parity 0 4 (means 0)

DWAIT 20 2

FRACK 6 4

The new values then become the defaults.

4.0 Kantronics KAM

LAN-LINK switches colors when it sees the /V or /H following the

callsign in the packet header, or in the calls monitored ("MH")

list. If you are monitoring both HF and VHF simultaneously, the

colors will show you which radio heard the signal. If the Mail-

Snatch or the QTC-Snatch are triggered, LAN-LINK will change modes

and attempt the connect on the correct radio. Turn the DCD flag OFF

for these automatic features to work.

The following functions do not work because the TNC does not

support them.

TNC Transmit Buffer flush.

The SIGNAL Communications Mode.

The Alt-D key in the MORSE Communications Mode.

5.0 PK-232

Turn BBS OFF, if you want the Alert Call, packet communications

mode header colors, and Digipeat Detect features to work, otherwise

turn BBS ON.

6.0 MFJ 1278

LAN-LINK assumes that the Radio 1 port is connected to the VHF/UHF

packet transceiver, and the Radio 2 port is connected to the HF

transceiver with the multi mode communications capability.

Early versions of the MFJ1278 suffers from one major deficiency as

far as LAN-LINK is concerned. The Echo-As-Sent only works in the

AMTOR Communications Mode. That means that you will not see the

contents of a file echo in the incoming window as the file is being

transmitted in the RTTY mode. Later versions have changes that

either change the defaults or work differently.

The following functions do not work because either the TNC does not

support them or does not do so in a consistent manner.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 151

TNC Transmit Buffer flush.

The SIGNAL Communications Mode.

CW Identification.

Pactor Break in.

Type ahead buffer.

7.0 Heath HK-21

K3WGF noticed that his Heath HK-21 TNC didn't seem to have the

proper circuitry to support true DCD on pin 8 of the RS232

connector. Even though the pin was labeled DCD it didn't signal

properly. Set the DCD function in LAN-LINK to OFF to make the

Capture-to-Disk close out normally rather than continue to remain

engaged.

8.0 PK-88

This TNC works like a PK-232 except that it only supports packet.

9.0 PK-900 and DSP-2232

These TNCs provide access to both ports simultaneously. LAN-LINK

supports dual windows, and some of the communications modes.

Otherwise, support is as if they were PK-232s. Sometimes, LAN-LINK

may take a few characters to synchronize windows.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 152

Appendix 6 Anomalies and Bugs

This section discusses various anomalies and bugs.

1.0 Command/Converse Mode Anomalies

Since the Program has no way to check the command/converse mode

status of the TNC, you may see various "Cmd:" and other "error

messages" from the TNC when using the menus or function key

commands. Ignore them for now. If you get an error message, try

using the function key again.

2.0 Buffer Messages in AMTOR Beacon CQ Mode

In the AMTOR Beacon CQ Mode, if the other station drops the link

while you are recording the text, BEFORE the callsign has been

recognized as a valid callsign, LAN-LINK will time out before

logging the QSO and restart the CQ sequence. If someone else calls

in before the time-out has occurred, the text from both stations

should be captured to disk, but the log entry may not show both

stations. You should compare the callsigns in the logbook with the

"RUN" file for QSO purposes.

LAN-LINK in the Beacon/Mailbox Mode, normally logs the contact at

the end of the line in which it recognized the callsign as a valid

callsign.

3.0 FEC +? Responses in AMTOR Beacon CQ Mode

In the AMTOR Beacon CQ or Mailbox mode, if another station sends a

+? in the FEC Mode, the beacon will attempt to send the LAN-LINK.TXT

file. The PK-232 however is not in the Converse Mode at that time

and will interpret the text of the file as commands. If by some

chance, the text corresponds to a valid command the PK-232 will

execute it and put the PK-232 in an unknown state and you may not

notice that it did. As such I recommend that the first character of

each line in the file LAN-LINK.TXT be a non valid command character

such as a number or a period (.) or another punctuation sign.

4.0 Using a Monochrome Monitor

If you are using a monochrome monitor, the text you type into the

Outwindow may be fainter than the incoming text. This is because of

the default colors used. Change the colors using the Parameter Menu.

5.0 The Packet Conference Mode

The Conference Mode has been tested on a PK-232, KAM, a SANYO MBC-

675 which has a clock speed of 4.7MHz and other PC clones with

faster clock speeds. If you have problems with intermittent

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 153

selection of I/O channels try slowing down your CPU clock. There is

a software delay loop in the program that is used to allow the TNC

time to switch I/O streams after receipt of the stream change

command. This delay is there because LAN-LINK tests for a connect on

each channel before outputting any text, so that it does not send

un-addressed packets on non connected I/O channels.

6.0 Packet Conference Mode Disconnects

Any Packet conference Mode disconnect returns the TNC to the

Command Mode, even if you remain connected to other stations.

7.0 Strange Events

If the TNC does not use an interrupt driven serial port you may

experience problems when using baud rates greater than 1200. The PK-

232 DOES NOT LIKE LAN-LINK to use RS-232 baud rates faster than

2400, and sometimes not even that. Use 1200 until YOU have tested

and ARE SURE that a faster one will work.

If you have any strange problems and are using an RS-232 terminal

baud rate between the PC and the TNC which is greater than 1200, set

the Terminal Baud Rate to 1200 and see if the problem goes away.

Make sure that your configuration is set the way you think it is.

Verify it using the Parameter Menu.

You must have at least one common call or scan word in your SYS

file, or you will get errors when you update the SYS file.

8.0 *** DISCONNECTED

In the Packet Communications Mode, when you are connected to

someone and the TNC sends "*** DISCONNECTED" to the computer, LAN-

LINK thinks you have been disconnected and resets. This normally

only happens when a true disconnect occurs. However, if the text

shows up in the middle of a line being sent to you, it will have the

same effect. LAN-LINK will think that you have disconnected if the

DCD Flag is not set in the SYS file, and the TNC does not use the

DCD signal on pin 8 of the RS-232 interface.

9.0 *** CONNECTED

In the Packet Communications Mode, if you are monitoring traffic on

the LAN and someone sends a packet containing "*** CONNECTED", LAN-

LINK thinks you have been connected, if the DCD-Flag is not set, and

the TNC does not use the DCD signal on pin 8 of the RS-232

interface.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 154

10.0 Greek Characters and Other Garbage in the Incoming Screen

If you see Greek characters in the Incoming screen you probably

have the wrong parity set in the TNC. First make sure that the RS-

232 baud rate is correct. If it is, take the batteries out of it and

reset it. If that fails, play with the parity. LAN-LINK sets the

serial port as configured in the TNC.SYS file for each TNC. For

example, for the PK-232 it is 8 bits, no parity. For the TNC2, it is

7 bits. If you still can't get LAN-LINK to talk to the TNC, set the

interface to 7 bits in the configuration file.

11.0 Dumb TNC

If the TNC seems to respond to commands but is not sending anything

back, you may have one of two problems. You may have a bad RS-232

cable with a wire disconnected. If the computer can operate the TNC

in another communications program, try to reset the TNC, take the

batteries out, etc. You may also have a not-so-compatible PC.

12.0 Intermittent Lockup

Watch out for flow control problems on the cable between the TNC

and the PC and between the PC and the Display.

Versions of LAN.LINK up to and including 1.55 used software

handshaking. Anytime the program received a Control-S (^S) character

from the TNC it waited until it subsequently received a Control-Q

(^Q). These characters can appear on the LAN in a binary packet,

either when binary file transfers are in progress, or in a Inter

NET/ROM message. To avoid this lockup, a line containing "MFILTER

19" was added in the LAN-LINK.SYS file for the PK-232 to make the

TNC filter the ^S character out and not pass it to LAN.LINK. LAN-

LINK 1.56 and onward use hardware handshaking so this problem should

not appear. However if the FLOW and XFLOW parameters are not set to

OFF in your TNC, LAN- LINK can appear to lockup. Make sure that they

are both set OFF in your TNC.

13.0 Automatic Functions Don't Work

This usually happens with the KAM. Set the DCD Flag off, and the

automatic functions should work. Note that in this case the multi-

connect functions will not work.

14.0 BBS Zap Failure

If the BBS Zap hangs up in the middle of sending a message, the

probability is that the LAN-LINK is waiting for the PBBS to send it

a word it recognizes. You have probably not configured the "PBBS

Read Subject" and "PBBS Message" Strings (lines 43 and 44) in the

LAN-LINK.SYS File (See Appendix 2, Section 18.18). If you are using

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 155

a KAM, read the previous section.

15.0 LAN-LINK Forgets the Band/Power Information

LAN-LINK picks up the band and power information from the logbook.

You can set them via the "0" option in the Parameters Menu. If the

logbook is empty, LAN-LINK will not remember the settings when it is

switched off. Put at least one entry in each logbook.

16.0 Zmodem Binary Transfer Problems

Some stations perform binary transfers with no problems, others

seem to have problems. If you are having problems with the binary

file transfers make sure:

You have enough memory left over after loading LAN-LINK to run

the external communications protocol.

The TNC is set for 8 bit transfers.

Both stations are using the same data rate between the PC and

the TNC.

The TNC does not filter any characters out of the data stream,

i.e. MFILTER or the equivalent parameter in your TNC is not

set.

You try the binary transfers via the LAN-LINK menu.

You read the PCZ Documentation file and set the Environmental

variables correctly.

If all else fails, try a different external protocol.

17.0 LAN-LINK 2.32 Snatch problem

In LAN-LINK 2.32 the QTC and BBS snatches only seem to work if

there is a SSID on the end of the call. For example, you have a

message for G3ZCZ on your system in the file G3ZCZ.OUT. If G3ZCZ

connects, he does not get the message. However, if G3ZCZ-4 connects,

he does. This bug should now be fixed.

18.0 Double colon after QTC

These are created by the program and show the user is not

registered (i.e. "LAN-LINK 2.32 :QTC::"). Registered versions

transmit single colon characters (i.e. "LAN-LINK 2.34 :QTC:").

19.0 Logbook 601 errors

These (and similar) errors in the log package mean the log is not

indexed. Bring up the logbook, then BEFORE you do anything else Re-

index the log. Exit from the logbook, change to hf mode and bring up

the logbook. Again re-index the hf log before you do anything else.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 156

20.0 KAM HF Packet Problem

Change line 53 in the LAN-LINK.SYS file from a | to a ~. Read

about the "STReamsw" command in the Kantronics manual. STREAMSW

should be $7E/$7C (~/|).

21.0 ASCII ANSI color transfers

LAN-LINK sends these files (packet and pactor) as text files, with

a maximum number of characters on a line of 255. check the TNC line

length parameter. They default to 80. If that fails to cure the

problem, well several popular programs which create ANSI color files

seem to generate longer lines. Sorry for the incomparability.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 157

Appendix 7 Updates, Revision History

If you are updating from a previous version of LAN-LINK, the

following changes were made in recent versions of LAN-LINK.

2.00 ELMER, Alt-W, word wrap, G option in Edit menu, Strswitch chars

& error_st in sys file, added. KAM cw/cmd bug fixed, AMTOR sets

narrow shift, connect timeout configurable, KAM RX/TX inv added,

bell filtered out of connectee/log call, KAM cw and other parameter

changes, log power/band picked up from log, custom word[20] in

prompt, Elmer added, event timer 2/6 reset change, MH fire delay for

KAM, ^T filtered out in all tncs, #T in contest text, NAVTEX and

navy MARS enabled for KAM/MFJ1278, flag_XMITOK added, base count

added to SYS file, CTEXT saved.

2.10 PACTOR (KAM, PK-232) added. Converted to TP6, Bug fixes Init:

7/8bits, log package changed, Elmer.QSO loaded at start-up, Elmer

RUN command, BBS custom commands, Flag_Inhibit_Logging, Logging

package, FLAG_Printer_Perm, TNC_parameter, mode at start up, QIC,

QMH in menu, QTR, LAN-LINK.OUT, ANSI Colors, PacketCluster Log

Search mode added. ELMER resets on disconnect, MFJ1278 radio port

customizable, Shut down flag 5 deleted, BBS upload states changed.

2.20 Added: PACTOR (MFJ1278), Scan Word and PacketCluster DX alert

colors. Converted to TP7. Bug fixes: Common call loading, KAM HF

packet color, Shut down state 4, capture-to-disk, update LAN-

LINK.SYS file. 2.20B fixed 'menu move' bug.

2.30 Support for PK-900 and DSP 2232. Better ANSI support. G-TOR in

KAM. Manual Mode, Z and B options added to BBS Menu. Expanded

PacketCluster support. Demo option added in LAN-LINK Menu. Radio

Menu added for Kenwood and Icom (untested). EDIT.EXE and UPGRADE.EXE

external programs added. Sound effects on "Busy" packet response.

2.32 QTC and BBS snatch bugs fixed.

2.34 CQ file and PacketCluster simulation added. Dual port TNC (PK-

900 and DSP-2232) manual mode shows one IN window. Auto window

select. BBS menu cleaned up. A, L and T in options added in BBS

menu. Bus fixed: AMTOR robot connect now stays in converse. Space

deleted in fromutome (fk3), shut down state of 5. Log menu, purge

and sort on date+time. Zap BBS/QTC snatch bug fixed. QSL menu added.

LAN-LINK menu 'C' window switch. Call menu 'F'.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 158

Appendix 8 ELMER: An Expert System Based on a Finite State

Machine

There are two versions of ELMER supplied with LAN-LINK. One is

built in for use in packet mode communications, when connected to

someone else. The second version is a stand alone version for use in

developing the text files and the logic. The stand alone version

lets you program and debug your own personal ELMER on your own. The

stand alone version is a separate product and requires separate

registration.

Most initial amateur radio contacts (QSO) are somewhat repetitive,

in that they send the same information each time. In digital mode

communications hams don't type exactly the same information in every

QSO, but in general the information exchanged is a subset of a

standard set. Hams have noticed this phenomenon and have each

created and used files containing standard information. This fact

was noticed years ago, and the first so called "brag tapes" were

invented to serve that need back when they were using

teletypewriters and generated paper tapes for the standard informa-

tion.

ELMER takes that concept a little further and provides expert

system. By parsing the incoming text, ELMER will recognize certain

words and transmit text files containing replies or further

questions, just as if the operator was at the keyboard.

Hams can use this feature to build a smart server they can put on-

line on the LAN. The server will prompt people and will serve an

educational purpose. ELMER can be set up to teach people about

packet radio, license upgrades, or anything else. ELMER can also be

programmed to recognize different languages and hold a conversation

in those languages. In fact, apart from the dedicated educational

and informational server applications, ELMER may be configured with

such a variety of vocabulary files so as to pass the Turing test in

most typical amateur radio digital mode conversations.

1.0 INTRODUCTION

ELMER is a program which allows you to build an expert system. It

uses the ELMER.QSO file to set up the state machine.

The ELMER contains an array of words to be matched, current states,

next states, and files to be loaded.

The ELMER starts up in an initial state. When a line of text is

received, it begins to match an array of words against the incoming

line of text. It starts matching the words in the order that they

are present in the array. It only matches words that are enabled for

the current state or for state 0. If it finds a match, it advances

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 159

the next state, and if a valid file name is present (and exists),

will output that file. The ELMER will then continue scanning the

line of text from the point that the last match was obtained. At the

end of the line, it waits for the next line.

1.1 Expert Systems

Before discussing ELMER, here's some information about expert

systems. An expert system contains knowledge about a particular

field to assist human experts or provide information to people who

do not have access to an expert in the particular field. It is an

information system that can pose and answer questions relating to

information borrowed from human experts and stored in the system's

knowledge base. Although they vary in design, most expert systems

have a user interface, a knowledge base, and an inference engine.

1.2 The User Interface

The User Interface to the expert system is the way that the user

interacts with the system to extract information from the system.

1.3 The Knowledge Base

The Knowledge Base of an expert system contains both declarative

and procedural knowledge. The facts describing the situations,

events and objects are called declarative knowledge. Procedural

knowledge is the information about courses of action and the rules

governing the actions. There are various kinds of rules that may be

employed.

1.4 The Inference Engine

The Inference Engine controls how and when the information in the

knowledge base is applied. It determines how the rules in the

knowledge base are to be applied to the problem.

1.5 Features of an Expert System

The following list of features are desirable in any expert system.

1.5.1 Useful The system should meet a specific need.

1.5.2 Usable The system should function so that even a novice

computer user finds it simple to use. It should be able to respond

to simple questions.

1.5.3 Educational The system should allow non experts to increase

their expertise. In a similar vein, the system should be able to

explain the reasoning behind its advice to allow the user to

determine the validity of the advice.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 160

1.5.4 Adaptable The system should be able to learn new knowledge.

1.6 Using an Expert System

Expert systems take the form of software packages residing on a

hardware platform (computer). The most common use of an expert

system is via a man-machine dialogue. The user types something at

the keyboard and the system replies. The user interface accepts the

input. The input is parsed in some manner. The inference and knowl-

edge engines process the user input in a predetermined manner and an

output appears. The parsing function may be a simple pattern

matching method commonly called keyword analysis, or a more

complicated function using syntactic analysis. Keyword analysis is a

logical function in which the presence of various keywords are

detected. When a keyword is found, the system responds in the manner

in which it has been programmed. For example, if the system detects

the words "the wx" in a question, it may respond with " the wx here

is sunny".

In syntactic analysis, a sentence is analyzed according rules which

allow the system to respond differently to keywords which appear in

different sequences. For example, the system may respond differently

to the question "what is the wx" and "the wx" if it recognizes those

words in a sentence.

1.7 The Semantic Network

The most general and oldest artificial intelligence scheme for

representing knowledge is the semantic network. A semantic network

is a collection of objects called nodes. The nodes are connected

together by links. Ordinarily, both the links and the nodes are

labeled. A drawing of a semantic network contains bubbles to

represent the nodes, and lines connecting the nodes to represent the

links. Both nodes and links are labeled. The drawing looks just like

a PERT or CP/M chart. It is also the drawing used to represent a

state machine.

1.8 A State Machine

A state machine is a system that exists in several states. The

system makes a transition from one state to another as a result of a

stimulus. All states and transitions are defined at the time the

system is designed. The links in the semantic network are the

transitions in the state diagram.

1.9 The User Perception

The user perception of an expert system is as follows. The system

appears to be in a rest state waiting for user input. It then

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 161

receives an input from the user and enters a transition to a state

in which it processes the data it has just received. After process-

ing the input, it generates an output and returns to what seems to

be the same rest state. In reality, the two rest states (initial and

final) are different.

The logic for the detection of words in sequence is set up by the

state numbers, i.e., when the system detects the word "what", it

advances from State 0 to State 1. In State 1 it looks for either the

word "systems" or the word "englishman" which takes it to State 2,

or to State 4. All other words are ignored.

1.10 ELMER

ELMER is an expert system for amateur radio. ELMER is based on a

finite state machine using table driven software. The declarative

knowledge is stored in the form of separate ASCII text and command

files. The procedural knowledge is stored in the state table. The

Data files in ELMER correspond to the Knowledge Engine (and the

Inference Engine). The State Tables correspond to the Inference

Engine. ELMER also contains an ASCII editor for text files, and a

tool to view the state table. These tools allow the user to "teach"

ELMER new knowledge.

1.11 Responses to an Input

ELMER has three kinds of reaction to a word match. It can do

nothing, output a test file, run a command or overlay a new state

table. ELMER's state table contains four entries:-

1.11.1 Current State This is the state that the string match is

performed in. ELMER can have up to 63 different states.

1.11.2 Next State This is the state that the ELMER will advance to

if a string match is found.

1.11.3 Repeat Flag This is a flag to allow or disallow repeats. If

it is a 0 repeats are allowed. If it is a 1, they are not.

1.11.4 Command Flag This is a flag to tell the ELMER how to treat

the file. If it is a 0, load a text file; a 2, overlay the current

state table with the defined one.

1.11.5 Keyword This is the text string to match in the syntactic

analysis of the input text. The text string can be up to 15

characters long and can include spaces. The match is case

insensitive, i.e. there is no difference between upper and lower

case.

1.11.6 Data File This is a text file to be loaded or a new

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 162

configuration file.

1.12 Syntactic Analysis

ELMER performs syntactic analysis on the input text in the manner

described below. ELMER performs a string match on the line of

received text against each keyword entry in the state table. ELMER

starts with the first entry. If a match is made, ELMER changes state

and performs the transient function defined by the Command Flag. All

words in the input text including the keyword are then deleted.

ELMER then continues the string match until either the input line of

text has been reduced to zero length, or, the line of input text has

been tested against all keywords in the state table.

1.13 Programming ELMER

Programming ELMER is simple. The knowledge base is broken out in

terms of states and transitions. States can be passive or active. A

passive state is one in which ELMER does nothing other than

accept/parse further input. An active state is one in which ELMER

loads and displays a text file containing information or shells to

DOS and runs an external program.

ELMER contains a set of tools for programming and debugging

purposes. A state table for the procedural knowledge is drawn up

using pencil and paper and a large sheet of graph paper. The built

in text editor is then used to enter information into the table. The

individual text files containing the knowledge are also created

using the editor. The state table viewing tool is used to check the

entries. The state table loading capability is used to exercise

ELMER and verify the logic. Because the procedural knowledge is

contained in the state table, all sorts of rules and linkages may be

used.

2.0 The State Table (ELMER.QSO) File

These are the contents of the ELMER.QSO file.

2.1 The Directory Path to the ELMER Text Files

This item is the directory path to the sub-directory containing the

ELMER text files. For example "c:\LAN-LINK\ELMER\" or "c:\ELMER\" or

just "c:", if you put them in the same sub-directory as ELMER.EXE.

If you do use a path you MUST have the last "\" character at the end

of the sub-directory name because the program concatenates the

directory path to the ELMER.

2.2 The Start Up File

This line is not used in LAN-LINK's ELMER. It is only used in the

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 163

stand alone version. It contains the name of the text file to output

the first time ELMER is invoked (if present);

2.3 Window Colors

These seven lines are only used in the stand alone version. They

are the numbers which correspond to the colors of the seven

different windows displayed on the screen of the stand alone

version, in the following order.

2.3.1 Status Window Color

The color of the window at the top of the screen.

2.3.2 OutWindow Color

The color of the window in which the replies are displayed.

2.3.3 InWindow Color

The color of the window in which your keystrokes are displayed.

2.3.4 BottomWindow Color

The color of the bottom line of the screen.

2.3.5 Promptwindow Color

The color of the window in which menus are displayed.

2.3.6 Option Color

The color of the highlighted characters in the menus and prompts.

2.3.7 Alarmwindow Color

The color of the window in which error and alarm messages are dis-

played.

2.4 The ELMER Initial State

This item is included to allow you to have different state

machines, each starting with a different state number.

2.5 State Machine Parameters

The next few lines contain the state machine parameters.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 164

2.5.1 Current State

This is the state the string match is performed in. LAN-LINK allows

up to 63.

2.5.2 Next State

This is the state that the ELMER will advance to if a string match

is found.

2.5.3 Repeat Flag

This is a flag to allow or disallow repeats. "0" means that repeats

are allowed. "1" means they are not.

2.5.4 Command Flag

This is a flag to tell the ELMER how to treat the file as shown

below.

FlagOperation0 Load/display a text file. 1 Execute a BAT, COM, or

EXE file.2 Overlay the current state table with the new one.

2.5.5 Keyword

This is the text string to match. It can be up to 15 characters

long and can include spaces. The match is case insensitive, i.e.

there is no difference between upper and lower case.

2.5.6 Data File

This is either a text file to be loaded or a command file to be

executed, depending on the state of the Command File Flag. An "*"

character defines a "do nothing" state in which the Data File is

ignored.

The two character sequence "#1" will cause ELMER to clear the

screen before displaying the rest of the text in the data file.

2.6 The State Machine File

An example of some state machine parameters is shown in this

typical extract from an ELMER.QSO file.

1, 2,0,0,what,*

2, 3,0,0,englishman,*

3, 4,0,0,doing,*

4, 1,0,0,country,me-usa.txt

1, 1,1,0,equipment,pk232com.txt

1, 1,0,0,wx,localwx.txt

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 165

1, 5,0,0,kenwood,kenwood.txt

5, 1,0,0,811,811.txt

1, 1,0,0,hello,hello.txt

0, 1,0,0,name,joe.txt

A "*" character defines a "do nothing" state in which the Data File

is ignored.

2.7 Example of Programming the State Table

Consider a typical question that I get asked.

"What's an englishman like you doing in this country ?"

followed by the phrase

"The equipment here is ....."

Assuming that the ELMER is in State 1, and that the first few lines

in the ELMER.QSO file are as shown above. The ELMER recognizes the

word "what" and advances to State 2. Since the file name is an "*"

character no file is sent. The ELMER then recognizes the word

"englishman" and advances to state 3. The ELMER machine then

recognizes the word "doing" and advances to State 4. The ELMER then

recognizes the word "country" advances to State 1 and transmits the

text file "me-usa.txt", because the Command Flag is a 0. It then

recognizes the word "equipment", remains in State 1 and transmits my

brag tape file called pk232com.txt. Since the Repeat Flag is a 1,

the ELMER will not retransmit pk232com.txt should it once again

recognize the word equipment while in State 1.

Since the ELMER scans sections of the line, the same effect will be

achieved if the incoming text had looked like :-

"what's an englishman like you doing in this country ?

The equipment here is ....."

You can only have 63 lines in any one state machine array. You can

define up to 64,000 different states. Each state must be an integer.

State 0 is a wild card state. This means that any word in the array

coded for state 0 will be matched against the incoming text

whichever state the machine is in. The entries in the state machine

file must end with a line that contains state 666. For example

666, end of file

The number of lines in the array and the current state at the end

of each line scanned are shown in the status window.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 166

3.0 The Stand Alone Version Status Window

The Status Window provides you with some diagnostic information

about the state machine. The three data elements are:

The current state [C].

The number of states (lines) in the state table[T].

The file name of the current state machine table.

4.0 The Stand Alone Menu

You bring up the menu by touching the "Escape" character. You are

then presented with the options shown in Figure 8-4. To activate an

option touch the highlighted key.

Figure 8-4 ELMER Menu

E Edit Any File

L Load ELMER.QSO

P Pick File to edit

S Edit ELMER.QSO

X eXit to DOS

* Show State Table

4.1 Edit Any File

This option allows you to edit text files using the LAN-LINK

compatible text editor.

4.2 Load State Table (ELMER.QSO)

This option allows you to reload the state table, or load a

different one. If a different state table is loaded, its name will

appear in the menu.

4.3 Pick File to Edit

This option allows you to pick a file with the default filetype of

"TXT", and edit it.

4.4 Edit State Table (ELMER.QSO)

This option allows you to edit the state table (configuration)

file.

4.5 eXit to DOS

This option allows you to exit to DOS.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 167

4.6 Show State Table

This option allows you to see the contents of the state machine. It

is used in debugging the logic to ensure that the contents of the

table match what you thought you put in it.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 168

Appendix 9 PCZ Binary file Transfers

LAN-LINK uses the PCZ public domain external protocol for binary

file transfers by shelling to DOS and executing the QZ*.BAT batch

files. The QZ*.BAT files are set up for you by the INSTALL program.

This appendix contains the part of the documentation (applicable to

LAN-LINK) for PCZ as supplied by the developers of PCZ, in case you

have problems making binary file transfers work in your system. The

full text is supplied on the disk.

PCZ - A Public Domain implementation Zmodem, Ymodem, Xmodem,

Xmodem-1K, Sealink for the IBM PC family.

PCZ is a public domain Zmodem implementation that will allow users

of communication software that does not implement internal Zmodem to

access it through an external program. It is a full-featured version

of Zmodem, but does not have the large volume of bells, whistles &

options provided by Omen Tech's DSZ. However, I do believe that it

will be suitable for the average demanding BBS'r. PCZ provides

support for the basic Xmodem variants: Xmodem, Xmodem-1K, Ymodem &

SEAlink. The following is a description of how to start PCZ for file

transfers.

LEGEND

Data enclosed in "[" "]" are optional.

Data enclosed in "<" ">" are required.

Send Command line:

PCZ <1-8> <300-19200> [m] [f] [d] [s] <s?> [r] [@]<filespec>

Recv Command line:

PCZ <[1-8]> <[300-19200]> [m] [f] [k] [d] [s] <r?> [r] [filespec]

Ref:

1-8 - the number 1 - 4, -(8) << PS/2 >>

300-19200 - the number 300, 1200, 2400 etc...

m - front-end menu option (see MENU)

f - enable FOSSIL comm driver routines

k - rx keep aborted received files

d - disable video memory writes (use dos)

s - request enable Super_Z transfer mode (zmodem only)

r?,s? - transfer type sz, rz, sx, rx etc...

r - resume/recover aborted transfer (zmodem only)

@ - command file transfer alert flag (SEAlink, Z,Ymodem)

filespec - file name w/extension {complete file spec allowed}

{drive:\path\filename.ext}

Flags should be placed as shown for standardization purposes. i.e.

"f" before "d"; "d" before "k"; "rz,sz" before "r" etc... Due to the

nature of the beast, no support is provided for a default port & no

default port speed is assumed. NOTE * See environment variables

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 169

below for more on default port/speed.  I personally never enter

"port" or "speed", but always enter their values. i.e. pcz 1 9600 f

sz xxxxxxx.xxx The batch files included reflect this method.

ZMODEM RECEIVE

<< SINGLE >>

PCZ 1 9600 rz myfile.ext - assume your specified file name.

<< BATCH >>

PCZ 1 2400 rz - Receive ALL sender files.

PCZ f d k rz r - Receive ALL sender files

assume PCZPORT & PCZSPEED values, use FOSSIL routines, use standard

DOS screen output, KEEP all aborted files.

RESUME any incomplete previous files.

one file mode:

PCZ 1 9600 rz INFILE.ARC - Receives one file and names INFILE.ARC

PCZ 1 9600 rz r INFILE.ARC - Recovery effort on file INFILE.ARC

NOTE * A word about command line switches might be in order here.

The switch flags " f, d, k " are global. That is they control PCZ

and not any particular protocol. The "r & s" switch is isolated

within zmodem, but is applied to both sender and receiver. If you

want PCZ's resume code enabled when sending a file then the sender

must specify "r" on the command line. Accordingly if the receiver

requests resumption of an aborted transfer, the sender must have "r"

specified or we won't resume anything. This was requested by BBS

Sysops who track download bytes, transfer points etc... Probably so

you aren't charged twice or the such like.

ZMODEM SEND

<< SINGLE >>

PCZ 1 9600 sz OUTFILE.ARC - Send one file to receiver.

<< BATCH >>

Create a TEXT command file with one complete file spec per line,

which includes drive and directory information for each file that is

not in the directory PCZ was called from.

e.g. C:\MYDIR\THISFILE.ZIP

There is NO limit to the number is allowed. Have no blank lines in

the file. Place the "@" symbol as the first character of the

filename. Pass it to PCZ on the command line as follows:

PCZ 1 2400 sz @OUTFILE.TXT

What you name the command file is your business. It simply must be

a standard ASCII file with no software or printer control codes

present.

An alternative to creating the command file for sending 2 or 3

files may be used. Simply type the file names on the dos command

line separating each one with a space.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 170

Remember the receiver must be set to accept batch uploads or you

might be spinning your wheels. Some BBS packages are just now coming

on-line with batch upload features using PCZ. They normally indicate

(BATCH) if it is supported.

XMODEM VARIANTS

To receive files using any of the Xmodem variants, the format is:

PCZ <port> <speed> rs filename - SEAlink receive

PCZ <port> <speed> rs - SEAlink Batch receive

PCZ <port> <speed> rx filename - Xmodem receive

PCZ <port> <speed> r1 filename - Xmodem-1K receive

PCZ <port> <speed> ry filename - Ymodem receive

PCZ <port> <speed> ry - Ymodem Batch receive

Remember the "k" option works for all of the receive modes. Hence

you can receive a file via SEAlink, have it abort on errors, and

then later RESUME the transfer using Zmodem.

To send files using any of the Xmodem variants, the format is:

PCZ <port> <speed> sx filename - Xmodem

PCZ <port> <speed> s1 filename - Xmodem-1K

PCZ <port> <speed> sy filename - Ymodem *

PCZ <port> <speed> ss filename - SEAlink *

* Refer to Zmodem Batch sending for batch transfers via Ymodem and

SEAlink. NOTE: The bytes and cps rate reflect the actual number of

bytes sent to, or received from the IOBuffer as a transfer

progresses. It does not necessarily reflect any specific

"BLOCKSIZE". When sending, as a transfer nears it's end, you will

see the cps decline. That's because PCZ uses buffered output vs a

simple "sector size" buffer. This will let you see actually how PCZ

is progressing in the transmission of data. In the best case "Full

Streaming Zmodem", you will see the time for actual file data; worse

case time will reflect the transmission of data headers and block

acks. It is different, but I hope everyone can appreciate a more

accurate account of what is happening inside the powerful zmodem

protocol.

** : The "Transfer Time" reflects PORT speed, (NOT MODEM) speed!

If speed conversion is being used, FORGET the transfer time.

ENVIRONMENT VARIABLES

PCZ supports several environment variables which can make setting

up a comm pkg. or BBS more "user friendly". These variables are as

follows and should be placed in the autoexec.bat file as I have them

shown here or modify the setenv.bat file included to accommodate

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 171

your system.

"DRIVE" refers to the letters "A".."Z"

"YOURDIR" refers to a complete directory specification. i.e.

\COMM\ARC\

"YOUR.FIL" refers to a filename with extension that you want PCZ to

use vs it's default name.

"DIRRX" i.e. set DIRRX=drive:\yourdir\.

This variable provides the ability for all your received files to

be placed in a single location. It is very useful for bbs purposes

as well as general comm program usage. If you specify a directory

spec on the command line, it will override DIRRX. Therefore you can

easily set the variable and then just forget about it. If DIRRX

isn't set and no spec. is given on the command line, your receive

file will be created in the current active dos directory.

"PCZLOG" i.e. set PCZLOG=DRIVE:\YOURDIR\

In the log file's case, it specifies the directory you want your

PCZ.LOG file to be kept in. If "PCZLOG" is not found in the

environment then no log file is generated.

set PCZLOG=DRIVE:\YOURDIR\YOUR.FIL

As an option to the log file name "PCZ.LOG" you may specify your

own filename in the filespec. This provides greater flexibility with

some bbs packages such as RBBS which expects it's own naming

convention to be used, and for multitasking environments etc... If

your "YOUR.FIL" exist, it overrides filename "PCZ.LOG" in all cases.

"PCZPORT" i.e. set PCZPORT=(1-8)

PCZ will use the port number (1-8) as the default port.

"PCZSPEED" i.e. set PCZSPEED=(300-19200)

PCZ will use the speed (300-19200) as the default speed.

NOTE ** If you specify the PORT or SPEED on the command line, it

will override a default set by the environment variables.

"PCZDIR" i.e. set PCZDIR=DRIVE:\YOURDIR\

If you experience any problems with PCZ not being able to access

it's overlay file, setting this variable will cure the problem.

DRIVE:\YOURDIR\ should point to the directory where you have located

PCZ.EXE

IMPORTANT NOTE: NO SPACES are allowed when setting an environment

variable!

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 172

i.e. {WRONG}

set PCZLOG = C:\COMM\ will not work.

That's not my fault. It is just DOS....

i.e. {ahhhh}

set PCZLOG=C:\COMM\

"PORT & IRQ".

It has seemingly become necessary, instead of interrupt sharing,

to reassign Comm. PORT addresses and use seldom triggered IRQ lines

e.g. "5 & 7" to allow multiple "more than two" ports on one machine.

Efforts to support this trend have been successful with PCZ.

Referring back to our PCZPORT environment variable:

set PCZPORT=1

If you have reassigned your IO card addresses etc..., you can

force PCZ to comply with your wishes.

Lets say you want to use PORT 3 @ 2E8h on irq 5. You set PCZPORT

in this manner.

set PCZPORT=3,2E8,5

| | |

| | |____ IRQ line number

| |_______ PORT address

|__________ PORT number

If you specify an invalid address or one your card isn't set for,

then PCZ will simply abort during it's port validation routine. This

method should provide the support needed for those who like to match

their wits against the cpu's.

"Super_Z" (tm)

Always in search for faster from fast. Super_Z takes zmodem to the

ragged edge! If you are one of the blessed, and enjoy relatively

noise free transmission lines, you should see an increase of 2 to 3

% efficiency. XON/XOFF MUST be disabled and the modem must pass

these characters. You may activate Super_Z via the command line

switch "s". As is usually the case, Super_Z can be called by the

receiver or by the sender. That is to say, that either can flag for

Super_Z and if the remote version supports Super_Z, we're doing it.

Regarding noise, a seldom thought of fact is that just because

your local transmission lines are great to one place, someone else's

local lines may be the pits. A good example is.... When I log on to

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 173

the "Corner" from Texas on a "596" extension I rarely ever get a

line hit. When I use the "995" extensions I am prone to hits

anytime. A reverse situation might occur in your particular area. It

simply depends on the quality of any of the many exchanges you might

encounter. I can only hope you are one of the lucky ones.

NETWORK SUPPORT

PCZ has been tested completely in a network environment. It will

allow multiple users to read the same file via use of DOS's

shareable attribute. It will also lock any files that it is

currently writing to. This has been tested using Novell Netware, LAN

Manager, and CBIS Network-OS.

MENU FRONTEND

Many fine programs exist which allow protocol engines to be called

from a communications package. In an effort to make PCZ all the more

user friendly, there is now a simple menu system built into the

program. It is controlled by the file PCZ.CFG which is included in

this zip file. Take a few moments to read over it. It may be that it

is just what you are looking for in the way of a nice user

interface. The design and layout of the system is very simple and

straight forward. You control the options and protocols you wish to

have active. PCZ.CFG comments explain the various functions you can

use. To use the menu system simply call PCZ in the following manner.

PCZ m

Make your menu selections and complete the command line with your

filespecs.

PHONE SUPPORT

Phone support can be provided when you require it, if you call

while I am home. I umpire for our local women's softball league on

Mon. & Tues. nights. I play golf on Wed. & Thurs. evenings. Fri.

night I do what Maureen tells me to do! Sat. is up for grabs and

Sun. morn & evenings I am in church. Please try me sometime other

than those mentioned ...... Seriously though, any night (late) will

be fine. Just try and make it before 10p.m. CST if possible.

Note from the author:

Over the last year PCZ has received some very good reviews. More

users rely on the program for their transfers now than I thought

would ever be interested. For this I am especially grateful and

would thank each one if possible. Gary & I will continue to grow

with the needs of the BBS community. We ask for your continued input

and support, with our assurance that your IDEAS, "cheers & jeers"

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 174

are taken with sincerity. Personal contact may be made by one of the

following methods.

Drue Kennon Gary J. Smith

Rural Route #2 Box 54 6122 Cedar Wood Drive

Center, TX. 75935 Columbia, MD 21044

Voice 409-598-3809

The Programmer's Corner,  Home of "PcConnect" & "PCZ"

Data/BBS: 301-596-1180 Data/BBS: 301-995-3744

Data/BBS: 301-621-4636

10 roll over lines in place with over 1.5 gigabytes of storage

space.

DSZ, ZCOMM, Professional-YAM are Copyrights of Omen Technology Inc,

all rights reserved. X00.SYS Copyright (c) by Raymond L. Gwinn, All

Rights Reserved. PCZ Copyright (c) by Drue Kennon, All Rights

Reserved. Super_Z(tm) is a trademark of PC#�#

The PCZ Menu Transfer System

This is the PCZ.CFG file.

;To make use of this transfer menu system PCZ should be called in

the following manner.

;PCZ m <ENTER>

;We assume in this config file that you are using SETENV.BAT to

preset your PCZ parameters for PORT and SPEED. If this isn't the

case then simply add them on the protocol lines below in their

proper place. You should be able to see from the listings the format

used here. i.e.

;SEND without the ";" begins send protocol block

;END without the ";" end the protocol block

;The protocol goes on a line followed by the ":" char followed by

PCZ's command line options for that protocol. You will be prompted

to complete the command line with your filespec(s) once the program

is started etc... pretty simple huh?

;Simple yes! but simple minded as well, and if you fail to follow

these guidelines.... who knows what type of protocol you might be

trying to use for a transfer. Just to satisfy your cats curiosity

you may uncomment the first line after SEND. Run PCZ using "m" menu

option, select upload and see what happens. No your pc won't bark

but it might barf if you select "A" as your protocol type.

;The following represent the send parameters. Using the ";" at the

beginning on a line comment out any protocol you do not wish to use

during you file transfer sessions.

SEND

;flag for beginning of send protocols

;TOSSIT: sp ;This is a curiosity test line

;XMODEM: sx

;send a file using xmodem

;X1K:s1

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 175

;send a file using xmodem 1k blocks

;YMODEM: sy

;send a file using ymodem with file header information

ZMODEM: s sz

;send a file using Super_Z zmodem

SEALINK: ss

;send a file using sealink

END

;end of send protocols

:The following represent the receive parameters.

RECEIVE

;flag for beginning of receive protocols

;XMODEM: rx

;recv a file using xmodem

;X1K:r1

;recv a file using xmodem 1k blocks

;YMODEM: ry

;recv a file using ymodem with file header information

ZMODEM: s rz

;recv a file using Super_Z zmodem

SEALINK: rs

;recv a file using sealink

END

;end of receive protocols

;You may add any of the other command line switches you wish to the

above. They are

; "f" use FOSSIL

; "d" no direct screen writes

; "k" keep all aborted downloads

;Thanks and have fun!

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 176

Appendix 10 Other PC Software by Joe Kasser, W3/G3ZCZ

1.0 PC-HAM 3.52

PC-Ham contains the following suite of programs.

1.1 LOGBOOK

Full blown logging package. With automatic check of logs for awards

such as DXCC. Allows you to recall any entry by call sign within

seconds. Indexed displays, QSL'ing, Contest mode QSL'ing (prints the

lot) and lots more. Although written in dBASE3 the package contains

a compiled version (LOGBOOK.EXE), so you don't need dBASE to run it.

The source code is ONLY given to registered users. It can convert

your exported LAN-LINK.LOG file to LAN- LINK.DBF to put this package

to work. Ideal for DX-peditions or for DX robot users to handle

QSLing and log statistics.

1.2 CONTEST

Keeps Dupes in memory, logs QSO's to disk in format which can be

processed by the LOGBOOK package. Now compiled in Turbo BASIC,

source code is supplied so that you can modify the program to meet

your requirements.

1.3 CQSS

Sweepstakes game compiled in Turbo BASIC. Work the ARRL Sweepstakes

contest on your computer. You are located just outside Washington

DC. A propagation model is built in to the program. This program is

REQUIRED training for all sweepstakes operators. Earlier version of

the program is described in detail in Software for Amateur Radio by

Joe Kasser G3ZCZ, published by TAB Books, Blue Ridge Summit, PA.

17214.

1.4 WHATSON

Predict HF Propagation for given days. Contest mode with printout

to whole world at hourly intervals. Needs BASIC.

2.0 STARTREK The Computer Program

An ideal task for the beginner to learn a language on, is a

simulation game which is written to run on a computer that the

beginner has access to. For in that case, there is complete control

of all inputs and outputs. This kind of game (in which the player

makes decisions based on the information available to him or her

available at the time), can be made sufficiently sophisticated and

complex so as to make writing it an adequate challenge for anybody.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 177

The techniques used in writing a good game are the same that

programmers use in professional activities. Writing a good game

poses a challenge that allows you to develop good habits and

techniques for programming and also allows you to learn a language

in an interesting manner. By taking an orderly approach to the game

design, complex operations may be clearly understood and converted

to computer code with the aid of a language reference manual,

irrespective of the language being used.

This product teaches the techniques for writing such a game using

the STARTREK game as an example, and the BASIC language as the

programming language in which to write the code.

Registration fees.

Single Copy         $15.00

10 - 50 Copies      $12.00 per registered copy.

50 - 100 Copies     $10.00 per registered copy.

100 + Copies        $8.00 per registered copy.

3.0 WHATS-UP 1.40

WHATS-UP is a tool for experimenting with orbital dynamics and

Telemetry Decoding and Display Program for the UoSAT-OSCAR 2, AMSAT-

OSCAR 13, Fuji-OSCAR 20, AMSAT-OSCAR 21 and the AMSAT Microsat

Spacecraft (OSCARs 16, 17, 18 and 19). It is table driven via the

configuration files to allow maximum flexibility.

WHATS-UP contains the following features:

Control Radio Memory via PC parallel port - designed for older FM

radios, you (and WHATS-UP) can step the radio memories to different

satellites if you build a simple interface circuit. Schematic

enclosed with disk: Visual and solar indication - there's a tag on

the orbit element display which indicates when a spacecraft is in

sunlight and when the spacecraft is visible for an eyeball sighting

(some, like AO-13, may need a telescope): Mission elapsed time

display - for any purpose including synchronizing to Space Shuttle

elapsed time schedule. Each spacecraft has its own indicator. Can

beacon [APRS] via, and/or automatically attempt a connect to

MIR/SAREX (better than LAN-LINK): Can automatically set Kenwood

Radios to spacecraft beacon frequency when satellite comes over your

horizon: Can automatically read the frequency from Kenwood Radios to

a disk file for Doppler frequency measurements: Can automatically

configure a PK-232 for each spacecraft data format: Reads 2 Line and

AMSAT format orbit element data files: Provides antenna azimuth and

elevation pointing data: Real time, Playback and Dumb Terminal mode:

Automatic Capture-to-disk of raw telemetry. Extracts telemetry

channel data to a database or spreadsheet readable file for further

analysis: Link quality measurement on packet telemetry: Capability

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 178

to print the raw telemetry as it is received: Up to 16 user

configurable display pages (screens): You set the position on the

page (width of engineering unit field, and number of decimal places)

that a parameter is displayed at: Wild card page (parameter shows up

on all pages): Selectable display of Engineering units or raw byte

for each display page: Display of raw packets (i.e. STATUS) Color

changes if a parameter value changed between successive frames:

Audio and visual alarms if a telemetry value exceeds, falls below or

falls outside a preset limit value(s): Dumb split screen terminal

mode (in the manner of LAN-LINK): Customizable colors, PC to TNC

baud rate, data parity and stop bits: Default spacecraft

configuration files: Time of day clock display (in HH:MM:SS format).

Real time analysis requires a receiver and demodulator. Analysis of

playback data can be performed with just the computer and WHATS-UP.

If you would like a copy of WHATS-UP and have captured any

telemetry to disk, especially daytime or southern hemisphere passes,

please put them on a disk and send it in with your request, a mailer

and return postage. Don't let the individual data files become too

long. I suggest that you rename them every day in the format

YYMMDD.SC, such as 900824.D17, 900824.F20, etc.

IF YOU ARE INTERESTED IN OSCAR AND SPACECRAFT TELEMETRY, THEN THIS

PROGRAM IS A MUST.

4.0 ELMER 1.00

ELMER is an Expert System Based on a Finite State Machine. There

are two versions of ELMER supplied with LAN-LINK. One is built in

for use in packet mode communications, when connected to someone

else. The second version is a stand alone version for use in

developing the text files and the logic. The stand alone version

lets you program and debug your own personal ELMER by yourself. The

stand alone version is a separate product and requires separate

registration.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 179

Appendix 11 How Shareware Works

The Association of Shareware Professionals (ASP) has established

standards for its members and for any organization which has "ASP

Approved" status. The ASP wants to make sure the shareware principle

works for you. If you are unable to resolve a problem with an ASP

member or organization (other than technical support), the ASP may

be able to help. Please write to

The ASP Ombudsman, 545 Grover Road,

Muskegon, MI. 49442-9427, USA.

You are encouraged to copy the floppy disk and share it freely with

others. You have the luxury of trying out the product at your own

pace and in the comfort of your own home or workplace.

After you have used the material for a reasonable evaluation period

(30 days), you should either discontinue use of the material or

register your copy. Your support is important and greatly

appreciated. With it, Shareware authors are encouraged to design and

distribute new products. Without it, a great deal of high quality,

low cost software will cease to be available.

Why pay at all?

You receive support from the author. You receive a CURRENT copy of

the program. Your input and ideas help shape future products. You

have a sense of pride and ownership in having honestly participated

in the Shareware revolution. You help to keep software prices down

by supporting a distribution method which doesn't depend on

expensive advertising campaigns.

Be aware of the following restrictions, designed to protect the

community of Shareware users and to prevent greedy people from

taking unfair advantage of the trust, hard work and good will of

Shareware authors.

1.   No price or consideration may be charged for the material.

However, a distribution cost may be charged for the cost of the

diskettes, shipping and handling, not to exceed $6.

2.   The files and programs on the disks may not be modified or

deleted.

3.   The material cannot be sold as part of some other more

inclusive package.

4.   The material cannot be "rented" or "leased" to others.

5.   The end user must be told clearly in writing on the outside of

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 180

the package and in all advertising that the diskette(s) are

"Shareware."

6.   The package must contain a written explanation that the disk is

for evaluation purposes, and that an additional "registration

fee" is expected by the author, if the material is used beyond

an initial evaluation period.

7.   In the case of distribution via any telecommunications link,

the following must be done:

An error checking protocol must be used. The individual files

must be combined into, and transferred in a library or archive

format.

8.   Shareware distribution is permitted only in the United States,

Canada, England, and Australia.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 181

Appendix 12 LAN-LINK 2.34 REGISTRATION FORM

PLEASE COPY this form and MAIL to

Software For Amateur Radio, P.O. BOX 3419, SILVER SPRING, MD 20918.

CALL ________________ TODAY'S DATE ________________

NAME ______________________________________________

ADDRESS ___________________________________________

___________________________________________________

CITY ______________________________________________

STATE _________ POSTCODE ____________ TNC TYPE_____ _________

FIRMWARE REV ____ HOME BBS ______________ DISK SIZE 5.25 ___3.5 ____

My favorite operating modes are __________________________________

Please register me as a user of LAN-LINK. I am currently using LAN-

LINK Version ______ which I obtained from _______. Please send me

the latest version of LAN-LINK or if a more recent one does not

exist at this time, QSL my registration and add my name to the list

to receive a free update when it becomes available.

I also enclose an additional amount for evaluation copies of ELMER

_, WHATS-UP _, PC-HAM _ and Startrek TCP _ ($5 for 1 program, $15.00

for the set). If I like them, I plan to register them in due course.

LAN-LINK Registration ($45.00)                         __.__

LAN-LINK manual/documentation on disk

(Word Perfect 5.1 format) ($10.00)                __.__

ELMER Registration ($45.00)                            __.__

Evaluation software (as above)                         __.__

Basic Packet Radio ($29.95)                            __.__

Total Enclosed                                         $__.__

Book shipping (overseas $10.00, US/Canada $3.75)       __.__

Md. residents please add 5% sales tax                  __.__

Please enclose check drawn on a US Bank or postal money order in US

dollars. US banks charge up to $20.00 for processing a foreign

cheque. Canadian checks in US funds count as foreign, yet Postal

Money Orders from your local post office can be exchanged at any US

Post office without charge.

You many also use VISA/Mastercard if you wish. Please charge the

amount above to my VISA ___ Mastercard ____

Card Number __________________________________ Exp date ______

Signature ____________

Many additions come into LAN-LINK as a result of user suggestions,

so here's your chance to get some input in. Comments, likes,

dislikes, wish list, etc.

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 182

Index

Accessible, 137

AFSK, 83

Alarm, 5, 32, 44, 51, 96, 99, 163

Alarmwindow, 99, 163

Alert, 2, 4, 3, 4, 5, 11, 12, 18, 19, 20, 21, 23, 32, 39, 40, 41, 42,

43, 44, 76, 82, 96, 99, 122, 126, 127, 131, 133, 134, 150, 157, 168

ALFD, 103

Allowable, 69

Alpha-numerical, 69

Alt-A, 2, 21, 66

Alt-B, 2, 21

Alt-C, 2, 8, 19, 21, 67, 111

Alt-D, 2, 22, 49, 85, 106, 117, 150

Alt-E, 2, 19, 22, 67

Alt-F, 2, 22, 33, 55, 137

Alt-G, 2, 11, 23

Alt-H, 2, 23, 67

Alt-J, 2, 23

Alt-K, 2, 23, 40, 42

Alt-L, 2, 23, 88

Alt-M, 2, 15, 23, 86

Alt-N, 2, 23, 110

Alt-O, 2, 24, 78, 88

Alt-P, 3, 24, 68

Alt-Q, 3, 24, 37, 68

Alt-R, 3, 24

Alt-S, 3, 24, 67, 87

Alt-T, 3, 29

Alt-U, 67

Alt-W, 3, 24, 60, 146, 157

Alt-X, 3, 7, 24, 68

Alt-Y, 3, 10, 25

Alt-Z, 3, 25, 40

AMTOR, 2, 3, 2, 3, 4, 5, 10, 14, 17, 18, 19, 20, 21, 22, 24, 27, 29,

32, 33, 34, 43, 44, 51, 52, 55, 58, 62, 63, 81, 82, 84, 88, 91, 96,

100, 103, 104, 105, 106, 112, 113, 131, 137, 141, 146, 147, 150, 152,

157

AMTOR-FEC, 17

Announce, 122

Announcement, 4, 39, 42

Announcements, 11

Annunciator, 2, 4, 14, 122

Anomalies, 152

ANSI, 2, 5, 12, 13, 20, 21, 55, 156, 157

AO, 177

APLINK, 21, 96

Applicable, 168

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 183

APRS, 177

ARQ, 2, 20, 21, 22, 32, 103

ASCII, 5, 2, 3, 4, 5, 9, 15, 17, 20, 27, 28, 46, 54, 57, 58, 59, 105,

115, 127, 128, 135, 136, 137, 138, 141, 142, 145, 156, 161, 169

ASP, 179

Attempt, 15, 21, 31, 43, 45, 52, 59, 78, 85, 89, 146, 150, 152, 177

Audio, 3, 5, 85, 87, 96, 178

Auto, 3, 4, 31, 36, 37, 39, 41, 43, 45, 102, 104, 117, 126, 129, 139,

157

AUTO-Upload, 136

AUTOCQ, 14, 16, 17, 103

AutoCR, 94

Autoexec, 170

Bank, 181

Battery, 7, 8, 74, 75

BAUDOT, 2, 3, 5, 10, 17, 20, 27, 28, 45, 55, 73, 84, 102, 105, 113,

126, 131, 137

BBS, 1, 4, 5, 6, 10, 12, 23, 25, 26, 29, 39, 45, 48, 49, 50, 71, 75,

102, 124, 126, 130, 136, 140, 142, 143, 150, 154, 155, 157, 169, 170,

171, 173, 174, 181

BottomWindow, 99, 163

Bragtape, 88, 103

Broadcast, 42

BTB's, 16

BTEXT, 14, 85

Bulletin, 6, 17, 75, 90, 101, 125, 141

Bureau, 62, 69, 70

Bureaus, 69

Bus, 157

Callsigns, 31, 66, 67, 79, 80, 81, 87, 91, 138, 147, 152

Canadian, 181

Capture-to-disk, 4, 5, 10, 16, 17, 28, 30, 46, 48, 49, 50, 53, 54, 55,

77, 78, 80, 81, 89, 92, 101, 115, 116, 125, 130, 134, 151, 157, 177

Cations, 86

CFROM, 94, 119

Checked, 11

Checkmark, 11

Cheque, 181

Chime, 87

Chirpcopy, 5, 33, 106

Cleaned, 157

Client-server, 14

CLOCK, 5, 9, 36, 51, 52, 73, 74, 80, 83, 88, 97, 152, 153, 178

Cluster, 4, 10, 11, 25, 39, 41, 43, 96, 102, 126, 131

CMSG, 79, 80, 81, 94

Colon, 155

Color, 2, 5, 11, 12, 13, 19, 20, 21, 26, 27, 40, 44, 53, 55, 96, 98,

99, 100, 101, 109, 126, 131, 156, 157, 163, 178

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 184

COMMENTS, 62, 63, 66, 67, 134, 136, 173, 181

Communi, 86

Compares, 133

CONFIG, 124, 135, 174

Connect, 2, 3, 5, 3, 4, 5, 8, 10, 15, 16, 19, 20, 21, 22, 23, 24, 25,

31, 32, 36, 37, 43, 44, 45, 51, 52, 54, 59, 63, 73, 74, 76, 77, 78,

80, 81, 82, 84, 85, 86, 89, 93, 94, 102, 103, 109, 110, 111, 119,

125, 128, 129, 135, 138, 140, 142, 143, 145, 146, 150, 153, 157, 177

CONOK, 80, 94, 119

Contest, 6, 4, 5, 11, 17, 24, 41, 51, 52, 55, 63, 64, 81, 87, 88, 128,

157, 176

Control-C, 46

Control-F, 137

Control-Q, 154

Control-S, 154

Control-Z, 3, 29, 58, 136, 140

Conv, 16

CONVERSE, 3, 2, 24, 33, 34, 58, 60, 74, 83, 84, 86, 111, 113, 142, 152,

157

CQ, 3, 4, 5, 4, 5, 12, 14, 17, 18, 19, 22, 31, 32, 33, 36, 37, 42, 43,

44, 45, 46, 50, 76, 78, 81, 93, 103, 108, 110, 112, 113, 115, 124,

125, 130, 139, 147, 149, 150, 152, 157

CQTEXT, 93

CR, 13, 102, 114

Ctdsk, 4, 5, 16, 46, 48, 53, 54, 81, 130

CTEXT, 5, 73, 75, 79, 80, 81, 124, 127, 139, 142, 157

Customize, 7, 11, 45, 50, 90, 94, 96, 97, 119, 139, 145

CW, 4, 9, 20, 28, 39, 41, 42, 83, 84, 96, 105, 106, 112, 113, 114, 115,

116, 117, 122, 126, 130, 131, 134, 137, 151, 157

CWID, 113

Damage, 65

Date-time-text, 29, 60, 148

Daytime, 178

DBASE, 4, 9, 62, 63, 64, 65, 67, 134, 176

DCD, 81, 88, 111, 125, 150, 151, 153, 154

DCD-Flag, 153

DCDCONN, 111

Debug, 41, 102, 158, 178

DEmo, 5, 6, 46, 50, 57, 60, 61, 139, 157

Demonstration, 139

Demonstrations, 139

Deprived, 12

DIGICALL, 12

Digipeat, 4, 5, 12, 18, 24, 78, 81, 83, 91, 109, 134, 142, 150

Digipeater, 86, 91, 94, 119

Directory, 4, 5, 4, 14, 21, 43, 44, 48, 50, 53, 54, 56, 71, 86, 92,

125, 127, 134, 142, 162, 169, 171

Directs, 131

Disc, 16

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 185

Disconnect, 2, 3, 5, 14, 19, 20, 22, 23, 27, 30, 32, 33, 74, 81, 85,

86, 89, 90, 93, 103, 112, 129, 130, 141, 153, 157

DMO, 5, 46, 50, 124, 139

Dollars, 181

Double, 155

Downlink, 76

Download, 3, 4, 10, 14, 25, 26, 27, 39, 40, 58, 59, 92, 93, 113, 125,

136, 141, 142, 144, 145, 148, 169

Dropouts, 129

DSP, 6, 2, 8, 20, 61, 83, 107, 126, 131, 132, 151, 157

DSZ, 168, 174

Dual, 4, 6, 42, 57, 61, 126, 131, 151, 157

DUPLEX, 3, 76

Duplicates, 87

DWAIT, 150

DX-alert, 12, 20, 40

DX-pedition, 5, 77, 78, 82

Dynamic, 12

Dynamically, 88

Early, 150

Echo, 30, 32, 81, 84, 85, 99, 134, 150

Echo-As-Sent, 150

Editor, 4, 9, 12, 14, 15, 30, 46, 48, 53, 71, 79, 112, 127, 135, 145,

161, 162, 166

Elapsed, 177

ELMER, 4, 18, 36, 120, 121, 124, 125, 127, 128, 139, 157, 158, 159,

161, 162, 163, 164, 165, 166, 178, 181

Ended, 83

Ensures, 83

EOF, 29, 48, 58, 136, 140, 147

Epson, 69

Equipped, 122

EVENT, 5, 4, 23, 26, 32, 36, 51, 52, 62, 74, 77, 82, 89, 91, 112, 141,

145, 148, 157

Exp, 181

Expand, 5, 53, 55, 125, 128

Eyeball, 177

Faked, 83

FEC, 3, 5, 22, 24, 32, 33, 34, 44, 113, 152

FF, 39

Figures, 1

FlagOperation, 164

Flashing, 16, 17, 18, 19, 28, 44, 78, 82, 87, 101

FRACK, 150

Fromutome, 157

FSK, 28, 83

Fuji, 126, 132

Fuji-OSCAR, 76, 177

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 186

FULLDUPLEX, 76

Funds, 181

G-TOR, 24, 107, 137, 157

Garbage, 129, 154

Gateway, 147

Grab, 2, 11, 20, 23

Greek, 7, 97, 154

Header, 4, 26, 44, 72, 77, 89, 99, 112, 141, 146, 149, 150, 175

HEADERLN, 134

Hearing, 42

Huffman, 117, 118

Icom, 132, 133, 157

Inactive, 33

Incoming, 3, 4, 5, 4, 5, 8, 11, 15, 16, 26, 27, 28, 30, 32, 33, 34,

40, 43, 46, 48, 49, 50, 55, 60, 61, 70, 71, 74, 75, 78, 80, 82, 84,

85, 97, 98, 99, 100, 102, 103, 104, 107, 111, 115, 116, 117, 128,

129, 131, 134, 138, 139, 150, 152, 154, 158, 165

Incomparability, 156

Interrogated, 122

Inwindow, 86, 98, 109, 163

Inwindows, 57

IRQ, 97, 125, 127, 172

KA-Node, 4, 22, 134, 135

KAM, 2, 8, 29, 33, 76, 83, 84, 88, 91, 95, 97, 103, 105, 107, 108, 110,

114, 116, 117, 128, 130, 134, 150, 152, 154, 155, 156, 157

KM, 102, 130

KN, 27, 88

KPC, 2, 74, 149

Labels, 69, 70

LANLINK, 88

LF, 13, 102, 114

LFADD, 84

Live, 12

LM, 25, 130, 136, 137

Log-to-disk, 80

Logcheck, 11

LogWindow, 98

Mail-Snatch, 26, 90, 150

Marks, 134

MARS, 6, 17, 102, 113, 114, 157

MBX, 8, 59, 119, 124, 135, 136, 146

MCON, 108

Memories, 177

Metabeacon, 45, 79, 80, 112

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 187

Meter, 11

MFILTER, 154, 155

MFJ, 2, 8, 33, 73, 76, 83, 84, 91, 97, 103, 105, 130, 149, 150, 157

MH, 3, 31, 60, 80, 98, 142, 146, 149, 150, 157

MHlist, 16

MHwindow, 31, 98

Milliseconds, 132

MIR, 5, 76, 77, 78, 177

Mission, 177

Modules, 62

MRPT, 44

Multi-connect, 4, 33, 74, 110, 154

Multi-mode, 2, 6, 130

Multi-stream, 23

Multi-user, 26, 45, 109

Multiconnect, 21, 100, 101, 111, 149

MYCALL, 113, 115, 116

NAVTEX, 17, 105, 107, 157

Needing, 61

NET, 4, 22, 134, 135, 137, 154

NNNN, 103, 114, 115, 116

Node, 2, 8, 10, 81, 84, 85, 125, 130

Non-packet, 45, 61, 139

Not-so-compatible, 154

Notation, 62

NTS, 45

Off-the-air, 61

Office, 181

Older, 177

One-up, 69

Orders, 181

Outwindow, 53, 98, 152, 163

Overseas, 181

PacketCluster, 2, 3, 4, 3, 5, 6, 10, 11, 12, 18, 20, 23, 25, 29, 36,

39, 40, 41, 42, 91, 96, 107, 122, 125, 126, 128, 131, 133, 157

PacketClusters, 12

Pactor, 4, 5, 10, 12, 24, 27, 45, 55, 63, 103, 105, 106, 117, 137, 151,

156, 157

Parallel, 177

Parameter, 6, 12, 13, 24, 27, 32, 36, 40, 41, 42, 44, 45, 65, 66, 76,

78, 81, 83, 84, 85, 89, 91, 94, 95, 96, 97, 99, 101, 103, 104, 106,

113, 127, 128, 129, 130, 131, 134, 152, 153, 155, 156, 157, 178

Path, 4, 8, 22, 24, 43, 44, 59, 63, 85, 86, 89, 92, 108, 125, 135, 138,

141, 142, 143, 162, 168

PC#�#, 174

Pcz, 138, 144, 145, 155, 168, 169, 170, 171, 172, 173, 174

PCZDIR, 171

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 188

PCZPORT, 169, 171, 172

PCZSPEED, 169, 171

Perfect, 181

PgDn, 47, 50, 66, 68, 71

PgUp, 3, 27, 46, 66, 68

Phrase, 165

Pictures, 12

Post, 12, 181

Postcards, 69

Prematurely, 83

Presupposes, 83

Printer, 3, 16, 20, 24, 65, 68, 69, 81, 85, 86, 126, 129, 157, 169

PromptWindow, 98, 163

PSK, 132

QBM, 5, 57, 58, 92, 125, 141, 142, 145, 146

QBU, 5, 57, 92, 93, 125, 137, 145, 147

QBU-RX, 57, 93, 124, 125, 137, 145

QBU-TX, 57, 92, 124, 125, 137, 145

QDB, 5, 57, 58, 59, 102, 142, 145

QIC, 5, 57, 60, 126, 131, 148, 157

QIX, 150

QJG, 59, 147

QMH, 5, 4, 57, 60, 142, 143, 146, 149, 157

QNO, 58, 87, 102, 141, 146, 147

QQQ, 103

QRA, 3, 5, 20, 24, 60, 146

QRM, 2, 32, 77, 82, 146

QRT, 18, 60, 77, 82, 94, 119, 146, 147, 148

QRU, 5, 57, 59, 103, 135, 146, 147

QRV, 57, 58, 102, 140, 147

QRX, 60, 148

QRZ, 3, 20, 24, 37, 79, 80

QSL, 4, 58, 62, 63, 66, 67, 68, 69, 70, 77, 79, 80, 102, 141, 147, 157,

181

QSL'd, 96

QSLd, 11

QSM, 5, 15, 57, 58, 138, 146

QSO, 3, 4, 14, 17, 19, 23, 24, 30, 33, 52, 55, 56, 80, 81, 82, 87, 88,

103, 106, 109, 113, 115, 120, 124, 139, 152, 157, 158, 162, 164, 165,

166

QSP, 5, 18, 57, 58, 60, 81, 87, 124, 136, 140, 146, 147

QSY, 3, 12, 77, 82, 122, 126, 133

QTA, 5, 57, 59, 124, 138, 141, 147

QTC, 4, 14, 15, 17, 18, 81, 82, 85, 86, 90, 102, 125, 147, 155, 157

QTC-Snatch, 15, 86, 150

QTR, 5, 57, 60, 148, 157

QZD, 5, 57, 59, 138, 144, 145, 148

QZD-RX, 138, 144

QZD-TX, 138, 144

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 189

QZU, 5, 57, 60, 138, 144, 145, 148

QZU-RX, 138, 144

QZU-TX, 138, 144

Radio, 1, 2, 3, 4, 9, 11, 12, 14, 28, 36, 41, 42, 53, 61, 75, 76, 78,

84, 103, 105, 119, 122, 123, 126, 132, 133, 138, 140, 143, 144, 145,

150, 157, 158, 161, 176, 177, 181

Random, 27, 131

Re-index, 6, 63, 65, 69, 155

Receive-Disconnect, 3

Repeated, 12

Results, 11

Retry, 3, 20, 22, 24, 76

Reverse, 13, 28, 146, 173

Robot, 2, 5, 14, 17, 18, 19, 20, 24, 51, 78, 79, 80, 81, 82, 84, 112,

113, 157, 176

Robot-Mailbox, 32

RTTY, 3, 5, 6, 17, 25, 27, 29, 32, 83, 105, 115, 126, 131, 132, 150

RY, 3, 25, 170, 175

SAREX, 5, 18, 76, 77, 78, 79, 80, 81, 101, 125, 177

Schematic, 177

Scrollback, 3, 11, 27, 28, 98

Searching, 39

Seemed, 84

SELCAL, 5, 6, 17, 19, 20, 21, 22, 24, 33, 43, 44, 91, 103, 114, 115,

116, 124

Separated, 133

Separates, 131

Sequence, 3, 8, 12, 16, 18, 20, 21, 22, 23, 24, 25, 27, 29, 31, 32,

33, 37, 42, 46, 57, 58, 59, 60, 70, 78, 81, 82, 83, 85, 89, 90, 91,

98, 102, 103, 104, 108, 113, 114, 115, 116, 125, 136, 140, 141, 143,

147, 148, 152, 161, 164

Served, 12

Sighting, 177

Simulate, 60

Simultaneous, 132

Solar, 177

SOLO, 18, 84, 94, 108, 109, 130

Sorry, 156

Speak, 122, 123

Spotted, 107

SSID, 91, 138, 155

State-sequence, 102

StatusWindow, 98

Staying, 83

Stays, 157

Step, 177

STReamsw, 156

Stuck, 69

COPYRIGHT Joe Kasser, G3ZCZ 1996.

LAN-LINK Version 2.34 Page 190

Subject, 6, 26, 48, 90, 102, 125, 154

Suggests, 150

Sunlight, 177

Supply, 122

Synchronizing, 177

Target, 4, 5, 17, 19, 43, 44, 82

Telescope, 177

TheNet, 134

There's, 177

Therein, 107

Timeout, 93, 125, 128, 157

Transparent, 21, 33, 57, 58

Tunes, 133

UNP, 12

Unproto, 78, 80, 110

Unshift, 17, 114

Untested, 157

Upgrade, 10, 157

Uplink, 76

USB, 122

Verbose, 90

Vfo, 42, 122

Visible, 177

Wise, 42

Wordperfect, 1

XFLOW, 9, 154

XMIT, 116

XMITOK, 9, 84, 106, 157

Xmodem, 4, 39, 40, 143, 144, 168, 170, 174, 175

XOFF, 172

XON, 172

YAPP, 135, 143

Ymodem, 4, 39, 40, 144, 168, 170, 175

Zap, 3, 4, 5, 17, 18, 20, 25, 26, 27, 39, 40, 43, 45, 48, 49, 51, 52,

75, 76, 77, 78, 81, 83, 89, 90, 101, 103, 104, 154, 157

Zmodem, 5, 59, 60, 138, 139, 144, 145, 148, 155, 168, 169, 170, 172,

175

COPYRIGHT Joe Kasser, G3ZCZ 1996.

#####################################################################################