TN211_15.DOC

REMOTE SYSOP PARAMETERS (17-23)

***********************************

Parameter 17, TIME-TO-LIVE INITIALIZER

--------------------------------------

Routing  paths may exist in a network that will route frames in an endless

loop.  To prevent this,  it's desirable to set a maximum limit on the number of

nodes  a frame  can be directly routed through.    The  time-to-live  parameter

limits the number of hops that network layer frames  originating from this node

will  travel  before being terminated.    The function of this parameter is  to

reduce network congestion should a connect request to a distant node get caught

in a routing loop.    Connect requests directed to destination nodes beyond the

number of hops allowed, will be quietly ignored.   The value selected should be

several  hops  longer  than  the  longest  "direct connect"  in  the system.  A

suggested value is 12 hops.

(Range: 0-255)

Parameter 18, TRANSPORT TIMEOUT

-------------------------------

Sets the time period for acknowledgement failure prior to sending a level 4

transport layer retry.    As an example, assume  a user is connected hop-by-hop

through a moderately busy system to a destination node 20 hops away.  With each

connect, level 4  transport information  frames  are exchanged between the user

and  the current destination node.   Every information frame requires a level 4

ack.  As the circuit grows longer, so does the  time for the acknowledgement to

arrive.

Should  the  transport  timeout be set too short,  there  is danger of creating

excessive information frame  retries  and  corresponding  increase  in  circuit

congestion.    Set too long,  throughput  could  suffer  while  waiting for the

retry, if a frame is abandoned somewhere along the circuit.     Most users such

as BBSes and TCPers, don't usually travel very far into the network.   Recently

however,  extensive linked  DXCluster  systems have been  observed.   Suggested

values  for this  type of service that should help prevent congestion are 180 -

240 seconds.

(Range: 5-600)

Parameter 19, TRANSPORT MAXIMUM TRIES

-------------------------------------

Establishes  the  maximum number of times the transport layer will be tried

before a circuit failure response is given.   The value selected will cause the

node  on the  sourcing side  of the failed circuit to try for a number of times

equal to the product of the values set in parameters 9 (link maximum tries) and

19  (transport maximum tries).    If the number of transport tries is set high,

link congestion results.  Suggested value is "2".

(Range: 2-127)

Parameter 20, TRANSPORT ACK DELAY

---------------------------------

Sets  the number of seconds the end-to-end level 4 transport circuit has in

sending  an  acknowledgement  to  an  information frame.  If  the  delay is too

short, the node will key-up on every ack.  If set too long, delay in throughput

could be noticeable.    Finding just the "right delay" is a function of network

type(s),  circuit length, circuit loading,  and baud rates.   These factors can

vary from user to user.    Some delay is advantageous  as it reduces the node's

key-up  cycle  by  permitting  transport  acks  to  ride  along  with  outgoing

information frames.  Suggested values are from 3 - 10 seconds.

(Range:  1-60)

Parameter 21, TRANSPORT BUSY DELAY

----------------------------------

Limits the  number of  seconds an  incoming level 4  frame shall be held if

its choke flag is set.   When the circuit isn't  cleared  within the  allocated

time,  the frame is discarded.   The purpose  of the transport busy delay is to

act as a fail-safe timer to prevent circuit lockups.  If the delay is too long,

the  circuit  will  be  slow  to  respond  to  route failures.    If too short,

unnecessary L4 information frame retries may be sent causing  congested circuit

conditions.    The delay should  be somewhat longer than the worst node-to-node

round trip time in the network.  Suggested value is 180 seconds.

(Range:  1-1000)

Parameter 22, TRANSPORT WINDOW SIZE

-----------------------------------

Transport  window size  (TRANSPORT LAYER MAXFRAME)  performs two functions.

It limits the  number of  non-acknowledged level 4  information frames  heading

toward  the destination node.    It also sets a limit to the number of incoming

out-of-sequence information  frames that will be temporarily buffered.  If this

limit  is exceeded,  a circuit choke response is sent toward the sourcing node.

A  small  window  size requires a short acknowledgement timer, parameter 20.  A

larger  window  size  would benefit from longer ack delay times by reducing the

number of overhead frames sent through the network.    Therefore,  too low of a

value  will  increase  the  node's  key-up cycle  and,  a high value may impact

circuit throughput.  A maximum buffer size of 4 is suggested.

(Range:  1-127)

Parameter 23, CONGESTION CONTROL THRESHOLD

------------------------------------------

The  congestion control threshold  also performs two functions.  It limits

the number of  level 2 link layer frames  that will  be held  before issuing  a

Receive Not Ready (RNR) response.  It also sets the maximum number of transport

layer  frames  (in sequence or not)  that  will  be  accepted  before sending a

circuit choke response toward the sourcing node.   Too low of a value can delay

LAN throughput if the backbone is moderately busy.   This  buffer should be set

to agree  with  parameter 22.  Suggested value is 4 frames.

(Range: 1-127)