AO-13 Real-Time Telemetry Program for the AEA DSP1232/DSP2232

_____________________________________________________________

(C) Copyright by ABAX BVBA, Belgium, 1994. All Rights Reserved.

This software may be used and distributed free-of-charge by radio-amateurs or

SWLs in the course of amateur radio communications. It may not be sold nor

used for any non-amateur purpose without written permission from ABAX.

This software was written by Andre Marchandise, ON1AIG.

================================================================================

__Version_Info__ * %n - V1 R03 M%v - Date %d *

* TLM13.DOC - V1 R03 M3 - Date 21-Aug-94 *

================================================================================

1. Introduction

This software was especially written for the AEA DSP-1232/DSP-2232

and G3RUH-modem.

It allows you to display AO-13 400 bps telemetry.

The program is developed using C and a commercially available

communications package.

This version has been tested with the latest EPROMS,

version 7.0 dated Dec. 1st '93.

2. Copyright

(C) Copyright by ABAX BVBA, Belgium, 1994. All Rights Reserved.

This software may be used and distributed free-of-charge by

radio-amateurs or SWLs in the course of amateur radio communications.

It may not be sold nor used for any non-amateur purpose

without written permission from ABAX.

This software was written by Andre Marchandise, ON1AIG.

3. Packing List

This package (TLM13.ZIP) consists of the following :

1. TLM13.EXE    : The AO-13 Telemetry program.

2. TLM13.DOC    : This documentation

3. TLM13.2LN    : A file with NASA 2-line Kepler data.

4. TLM13.CFG    : Configuration file, with COM-port, baud rate,

latitude, longitude, ...

4. Installation

I suggest that you create a separate sub-directory TLM13 for these

Telemetry programs.

Copy the TLM13.ZIP to this sub-directory.

First, PKUNZIP the TLM13.ZIP file.

Then edit the TLM13.CFG file and alter the statements to the

values for your station :

"port 1" : this is the com-port, to which the DSP is connected.

"baud 9600" : the baud rate at which the DSP RS-232 runs.

"dspport 1" : the DSP-port ( port 1 or port 2 ) to

which your rig is connected.

"dspport 2" will only support 400 bps telemetry.

"latitude 51.124" : this is my latitude, replace it with

your station's latitude. Forgetting to do

so will result in erroneous az/ele values.

"longitude 4.45" : replace it with your station's latitude.

"height 20.0" : This should be your station's asl.

Installation is now complete.

5. Running the program

You can run the program by typing

TLM13

at the Dos-prompt C>.

The program will look for two files :

TLM13.CFG the configuration file

TLM13.2LN the file with the kepler data.

If your AO-13 kepler data are older then 60 days,

you'll get a warning. You can update the kepler's by

editing the file or by copying a file with 2-line

elements over it.

If you run the program for the first time, check the Azimuth

and Elevation values which are displayed at the bottom of the

screen versus the values you get from your regular tracking

program. This to make sure that latitude, longitude and height

are correctly set.

The program comes up in DSPdat mode and will try to sync

between telemetry-blocks.

6. Navigating through the program.

The following keys can be used

F10 : Exit the program.

B   : Switches to DSPdat mode and starts syncing.

W   : Switches to CW mode.    |

| not available on DSPPORT 2 !

R   : Switches to RTTY mode.  |

C   : Displays communication telemetry.

P   : Displays power ( voltages and currents ) telemetry.

T   : Displays temperatures.

V   : Displays navigation telemetry ( NOT IMPLEMENTED YET )

L   : Displays L text block

M   : Displays M text block

N   : Displays N text block

7. Interpretation of the data

For an exact interpretation of the data, there is an excellent

book available from AMSAT-UK ( and perhaps other AMSAT

organisations also ).

This 52-page book is called

AMSAT-UK OSCAR 13 OPERATIONS AND TECHNICAL HANDBOOK.

It contains the following chapters

- Brief history

- Launch and positioning

- Operations

- Orbital predictions and tracking

- Satellite hardware

- Telemetry and data formats

Here is AMSAT UK's phone and fax :

Ron Broadbent

G3AAJ

AMSAT UK

E12 5EQ

England

phone 44(0)81 989 6741

fax   44(0)81 989 3430

8. I've spend numerous hours working on this program, and learned a

lot about AO-13. I suggest that you take the time to study AO-13

telemetry and with the help of the AMSAT UK handbook, you'll enhance

your knowledge about AO-13 a lot.

If you like the program, please send a donation to your AMSAT

organisation.

We NEED your HELP for the PHASE IIID project.

9. In case of problem

If you're having a problem with the program, documentation,

config files or whatever pls. feel free to contact the author :

Andre Marchandise

ON1AIG

AX.25 : @ ON7RC.BT.BEL.EU

SPACE : @ KO-23 & AO-16 (QRT UO-22)

CIS   : 100272,1644

In order to accurately trace the problem, the following

information should be provided :

- DSP model ( 1232 vs. 2232 )

- DSP software release

- TLM13 software release

- TLM13.CFG file

- CONFIG.SYS file and AUTOEXEC.BAT file

If you have access to utilities like the

NORTON_SI or MANIFEST, that info is very welcome.

- Peripherals in system

- Operating system and version

!!!!    - An ACCURATE description of the problem.

Do not expect an answer, when sending a message like :

"I have downloaded TLM13.ZIP  What should I do next ?".

==========================================================================

I hope you'll enjoy the program.

73 fm Andre - ON1AIG