LG Version 1.0d

Copyright 1995

9V1ZV Daniel Wee

Introduction

------------

To keep it very brief, there just isn't any good shareware loggers

out there which support ICOM radios properly. Most that do are really not

too well written, others are expensive. The rest all work for Kenwood. I got

tired of this so I decided to be part of the solution, and this is it. Its

got every useful feature I can think of. Though I admit I can add a little

more but I thought I'd release this first to gauge the kind of response,

whether it warrants further development. So, if you really like it, send me

comments, criticisms, ideas, suggestions, requests for features, or, best of

all, MONEY! hi hi (say about $20? Let's see how many of you really like this

program :-)

Basics

------

Recommended             486DX-33 with 4MB and EMS driver. SVGA card with 132

column modes. Have not been tested with anything else.

If EMS is present, it will be used, which is the

recommended method, otherwise, it will use conventional

memory which is fine, but you will have less QSO

storage space. This software was written with the AT-

style keyboard in mind, so it may not work correctly

with older XT-style keyboards.

Although this will work with non-TNC2 compatible TNCs,

it has been tested only with the MFJ-1270B so far, but

using the TAPR ROM, so any TNC2 compatible TNC's should

work fine. Baycom modems WILL NOT WORK here.

This program will NOT WORK correctly under Windows or any other similar

multitasking environment because it directly accesses all the hardware and

handles multitasking internally.

If the hardware does not seem to be responding, make sure you check out the

settings for the IRQs as outlined elsewhere in this doc, and if possible, try

to avoid sharing IRQs.

L.EXE                   Main executable

L2.EXE                  Same as L.EXE but uses math coprocessor for floating

point operations.

LOG2ASC.EXE             Dumps log file out in ASCII format (described below)

LOG001.DAT              Sample log file

DXCCHITS.DAT            Sample DXCC hit list. Delete if you are starting a

new DXCC tracking or if you are just starting this

program for the first time, you should delete this

after playing with the sample files

KEYMAC.DAT              Sample of some keyboard macros

LOG.CFG                 Sample configuration file

You need not edit any of the *.DAT files directly, unless you are trying to

delete a particular DXCC entry in DXCCHITS.DAT. The rest are normally done

from within the program itself. Also, if these files are not found, do not

worry, the Executable will automatically recreate them as necessary.

History

-------

Originally released as L10BP.ZIP at the oak.oakland.edu ftpsite.

L10B.ZIP     Comes with more comprehensive documentation.

L10C.ZIP     More bugs ironed out.

L10D.ZIP     Now Joystick port will accept paddle. Added Tuneup function.

L10E.ZIP     Added Contest mode and more comprehensive dupe checking.

L10F.ZIP     More enhancements to dupe checking.

Not Included (yet)

------------------

QSL label printing

Score keeping for contests

DXCC hits editor

Partial callsign match

Help screen

Auto IRQ identification

Voice keyer

Hot Key Summary

---------------

Alt-A                   About

Alt-B                   Backscroll for packet data (400 lines presently)

Alt-I                   Information

Alt-K                   CW Macro editor

Alt-L                   Logging window

Alt-O                   CW Keyboard and incoming packet spot window

Alt-V                   Config/Setup window

Alt-W                   Grayline Map

Alt-Z                   TNC terminal

(the following also applies to the QSO editor window)

Alt-C                   Jump to callsign field

Alt-S                   Jump to sent RST

Alt-R                   Jump to received RST

Alt-F                   Jump to frequency field

Alt-M                   Jump to mode field

Alt-N                   Jump to name field

Alt-Q                   Jump to QTH/Notes field

Alt-T                   Jump to time field

Alt-D                   Jump to date field

Alt-P                   Jump to power field

Ctrl-right-arrow        Increase CW WPM

Ctrl-left-arrow         Decrease CW WPM

Ctrl-backspace          Abort CW sending

Alt-F1...F10            Send CW macros

In Logging window

-----------------

F1                      Save QSO

F2                      Spot QSO

F3                      Logbook editor

Shift-F10               Reset/Abort QSO

F5                      Check DXCC info for current QSO

F6                      Revert to pre QSY frequency and mode

F12                     Tuneup, toggle TX

Enter                   Lock time/date and move to next field

Tab                     Next field

Shift-Tab               Previous field

Alt-Del                 Delete previous QSO

Alt-G                   Grab current spot, QSY rig to spot

In Logbook editor

-----------------

F2                      Spot current entry

F4                      Search for callsign/partial

F5                      Search for next match

Enter                   Edit/Modify entry

In Modify Entry

---------------

F1                      Save

Esc                     Abort modifications

Tab                     Next field

Shift Tab               Previous field

In CW Keyboard/DX Spot window

-----------------------------

Enter                   Grab current spot, QSY rig to spot

Up/Down arrow           Select spot

0-9,A-Z,etc.            Send CW

In Macro editor

---------------

Changes take effect immediately and can be used at once. Exiting the macro

editor will save current settings to disk.

In Setup window

---------------

Tab                     Next field

Shift Tab               Previous field

Space                   Toggle entry and/or move to next field

Enter                   Toggle entry for certain fields

F1                      Save

Esc                     Abort

* When a save is done, the current power field setting is saved as the default,

likewise the current CW speed.

Command line options

--------------------

/V=nnn                  Will use video mode nnn (decimal). If none, program

defaults to current video mode.

/C=xxxxx                Where xxxxx is your callsign, not exceeding 14

characters long. It will be truncated beyond 14. The

callsign will be saved with a setup save, so that if

you save the setup after using this switch, that

callsign then becomes the default callsign to be used

until you specify and save a different callsign.

CW Macros

---------

$U                      Contents of callsign field. If empty, sends nothing.

$R                      Send RST field.

$S                      Send RST field, '9' is sent as 'N'.

$N                      Contents of name field. If empty, sends "OM".

$Q                      Current QSO number (minimum 3 digits).

$M                      Your own callsign.

Misc

----

In setup boxes with [ ] options, the Space bar will move to the next option and

the Enter key will toggle it. In boxes with ( ) options, the Space bar will

toggle it AND move to the next option. Tab key moves to the next box. Selection

can be done with a mouse if one is available.

Program will automatically log onto the PacketCluster if a Cluster callsign

is defined in the Setup, and if upon startup, the TNC is not connected but

active.

If you enter a call in the callsign field, and press tab or return, dupe checks

will be performed (if option is selected) and if a dupe for the band/mode is

found, a red window will appear at the top right corner indicating so. If the

call has been worked but in either a different band or mode, a green window will

appear indicating that. The window always displays the most recent QSO with the

call regardless. Thus the color is the indicator of an actual dupe or not.

Note that the longitude is given with reference to the West, so West is positive

and East is negative in value. You should also set your PC time to your local

time. If this is the case, you need to set the environment variable TZ to

indicate your timezone. For Singapore, I set

TZ=-8            Because our local time is 8 hours ahead of UTC

TZ=GMT           If you are in Greenwich or your PC is set to UTC

TZ=8             If you are 8 hours behind UTC

This is important so that the logger clock will reflect the accurate UTC time.

If you find that TZ has already been set on your system for use by other

software, chances are you need to change nothing as TZ is quite commonly used

by different programs for the same purpose.

TNC2 compatibility tells the program to expect and send TNC2 compatible

commands. AutoDate set will set the date on a TNC2 compatible TNC upon

starting up the logger. If you find that the Terminal screen keeps saying NOT

CONNECTED even when you are, you should select the NoDCD option. Alternatively

you may want to use a more complete RS-232 cable if you're not already doing

so as some cables do not provide the necessary CD lines.

KISS mode not supported as yet. If NoDCD mode is selected, the Connect state

is ignored and spotting is allowed even if not connected. The Cluster connect

wil simply transmit the cluster call string without preceding "C". This may be

useful for TNCs/cables which do not provide true DCD.

Contest mode will change the behavior of navigation keys in the logger window.

Tabbing or hitting return from the call field will send cursor into the Name

field now, for the purpose of entering contest numbers etc. The reverse is also

true with shift-Tab.

The terminal supports ANSI so you may do a SET/ANSI on your PacketCluster,

or better still, try SET/ANSI/ALT for some really slick high-lighting.

To allow the logger to key your rig, you need to build a CT compatible keyer

interface, consisting on one NPN transistor (2N3904) and a 1k resistor. The

ICOM interface can be any CT-17 compliant interface, and one excellent example

of such appears in one of the past QST issues. The interface can be built for

about US$3 or less. The exact article is in QST, July 1992, Page 37-38 and is

extremely easy and cheap to build.

The keyer circuit looks like this:-

For LPT (Parallel) Ports:-

Transistor is any general purpose (this is NOT for grid blocks)

NPN such as the 2N2222 or 2N3904 or BC108 or the like. Not critical.

The 1k resistor is a regular 1/4 watt carbon film resistor.

c

1k       b |/--------------------o Key

Select  17   o----/\/\/\-------|

|\

Strobe  1    o-------------------+ e

Ground  18   o---------------------------------------o Rig Ground

For 25-pin serial/COM Ports:-

c

1k       b |/--------------------o Key

DTR     20   o----/\/\/\-------|

|\ e

Ground  7    o-------------------+-------------------o Rig Ground

(work out the 9-pin one yourself :-)

If the keying does not seem to work initially, it may be due to the different

naming conventions in use here. For my program, LPT1 is the printer ports

found on Hercules Monochrome cards (3B8?) so you really should just experiment

a little to see if you got it right.

*** Caution! Some softwares which use the printer port for communications, such

as parallel port based backup equipment, parallel port based EPROM burners,

etc., may leave the port in a keyed down state. So if you are using such

programs with the particular port, take extra care to ensure this doesn't

happen inadvertently.

If you rig does not appear in the setup, please let me know and I will include

it. If you find the program useful, fun. Feel free to donate $20 (or any sum

you think this program is worth). If you find bugs or have ideas for new

features, let me know, no matter how difficult you may thing those features

are. You may just see it in the next version. If response is good I may write a

Kenwood version.

The pin connections for connecting up a paddle is as follows:-

*Printer port                        Paddles/Keyer

Pin 13(Select)    -------------  Dash/Keyer

Pin 15(Error)     -------------  Dot

Pin 18(Gnd)       -------------  GND

*Joystick port                       Paddles/Keyer

Pin 2   -----------------------  Dot

Pin 4,5 -----------------------  GND

Pin 7   -----------------------  Dash/Keyer

If you are having IRQ conflict problems, you may try assigning the same IRQ to

the rig and the TNC, as the program was written to allow such sharing. However

this has not been tested. For some reason, the mouse driver, will not share its

IRQs with anyone (MS-MOUSE). Usually:-

COM1    4

COM2    3

COM3    4 or 2

COM4    3 or 5

IRQs that are free are usually 4 and 3 (used by the COM ports), 2, 5, 7.

Remember to set your hardware settings accordingly too. In general however, IRQ

sharing is not encouraged, use it if you have no other choice.

This program MAY NOT BE SOLD commercially, or be used in any profit making

venture without express written permission from the author. Business

propositions are welcome, however :-)

The program LOG2ASC.EXE will take the log filename as a parameter and dump the

log to stdio, so you can simply redirect it into a text file and print it with

your favorite WordProcessor.

If you want the DXCC id-ing and tracking feature, you should get hold of the

file called CT.CTY from the latest CT versions. This is regularly posted on

DX/Contest reflectors. I have the file but I don't think I should release it

together with my software. Enquires are welcome though.

Proposed features for future versions:-

1.      Paddle port so you won't need a separate keyer for you paddle.

2.      on-line help (if there is demand).

3.      Split screen option for terminal window.

Credits go to 9V1YC for helping me get started on this project, to ICOM for

their great CI-V interface, to CT for being such inspiration, to EQF for giving

me some ideas where to start, and whats missing, to my XYL and harmonic for

bearing with my programming frenzy, to all hams for making this such a

wonderful hobby.

73 de 9V1ZV Daniel

daniel@pandora.lugs.po.my

snail mail:     Daniel Wee

7 Mount Sophia Rd

Singapore 0922

SINGAPORE