TN211_16.DOC

EPROM PROGRAMED (FIXED) PARAMETERS

**************************************

Parameter 24, NO-ACTIVITY TIMEOUT

---------------------------------

The no-activity timeout timer performs several missions.  The node monitors

activity on either of level 2 and level 4 circuits.  If no activity is detected

during  the  period  the timer  is set for,  the circuit is  disconnected.   In

conjunction  with  circuit activity,  the timer sets the life of the  "activity

arrow"  seen on the  ROUTES  response.    It also sets  the maximum period a CQ

command  can be observed in the USERS response.    Setting the  timeout to zero

disables  the  timer  (not recommended).    Suggested  value is 900 seconds (15

minutes)  although  DXPacketCluster  operators prefer it to be maxed to prevent

their users from becoming prematurely disconnected.

(Range: 0-65535)

Parameter 25,  P-PERSISTENCE THRESHOLD

--------------------------------------

Parameters  25  and 26 work together to set up a random  delay  determining

when  the node will key up following a DCD decision that the channel is  clear.

This  is  an anti-collision technique.  When the node is ready to  transmit,  a

number  between 0 and 255 is internally generated.  If this number is equal  or

less than the value set by Parameter 25, the node keys immediately upon sensing

a clear  channel.  If the internally generated number is greater than the value

of parameter 25, the node waits for a period of time equal to the slot time and

then  internally generates a new number, etc.  A value of 64 is 25% of 255  and

thus  sets the percentage of time the node will immediately key-up when a clear

channel is sensed.

Protected  trunking nodes (those with only one transmitter on their receive

frequency)  would  have  faster   throughput  if  random  key-up  delays   were

eliminated.   Setting parameter 25 to a value of 255 will accomplish this.

(Range:  0-255)

Parameter 26, SLOT TIME

-----------------------

The  slot time interval  is adjusted  in consonance  with  P-Persistance to

establish  a  variable   delay   node  key-up  algorithm  designed  to  prevent

collisions.   Since the variable key-up delay algorithm only impacts  WHEN  the

node  is  keyed,  it  has no  relationship  to either  node or  user TXDs.   On

single transmitter channels, slot time is  disabled  when  P-Persist is  set to

255.    In  multiple transmitter  environments and with P-Persist set to 64,  a

slot time of 10 (100 milliseconds) is suggested.

(Range: 0-127)

Parameter 27, LINK T2 TIMEOUT

-----------------------------

Link  T2  timeout  is  the node's  equivalent  to  a  TNC's  RESPONSE  TIME

parameter.   It sets the  amount  of  delay following receipt of a level 2 info

frame before an  acknowledgement is sent.     A little delay is desirable as it

gives  a  chance  for  the  ack to ride along on the next info frame.  Too much

delay will slow throughput.  Suggested value is "100" (1 second).

(Range: 0-6000)

Parameter 28, LINK T3 TIMEOUT

-----------------------------

Link  T3  timeout is the node's equivalent to a TNC's CHECK parameter.  Its

purpose is to send out a level 2 "check packet" following a specified period of

inactivity, to verify the link  layer is still connected.   If a response isn't

received   prior  to   completion  of  the  retry  sequence,   the  circuit  is

disconnected.   Should the timer value (10 ms increments) be set too short, the

check packets add unnecessarily to link congestion.  The suggested value is "0"

(timer disabled).

(Range: 0 - 65535)

Parameter 29, STATION ID BEACONS

--------------------------------

Determines if and when, the node transmits it's ID.   The following options

are available:

0 = Disabled (ID's OFF)

1 = Used (ID's every 10 minutes when the frequency is in use)

2 = Enabled (ID's broadcast every 10 minutes)

ID  beacons  contribute to network overhead.   On isolated backbone trunks they

can  be turned off.   On user/LAN nodes, ID's are useful MHeard support for TNC

and KA-node listings.  Suggested value here is "1".

(Range: 0-2)

Parameter 30, CQ BROADCASTS

---------------------------

Controls whether or not the UNPROTO text sent by a CQ user will be broadcast

from the node.   This parameter does not affect the listing seen in response to

a  USERS  command  should someone be inviting a CQ contact.    0 = Disabled (no

UNPROTO broadcasts), 1 = Enabled (UNPROTO broadcasts allowed).  Suggested value

is "1".

(Range: 0-1)

Parameter 31, FULL DUPLEX

-------------------------

Full  duplex  allows the  internal  TNC  modem to independently receive and

transmit.    Its  usage is intended for a full-duplex configuration as might be

found  on modern  systems using packet repeater LANs and full-duplex backbones.

For all other  applications the recommended value is OFF (0).  0 = OFF, 1 = ON.

(Range:  0-1)

Parameter 32, PORT DIRECTION

----------------------------

NOTE:  Applies  only to nodes equipped with telemetry daughterboard.  Ports

are unidirectional for telemetry applications.     There are two ports, labeled

Port A and Port B.  Each port has eight bits, or lines.  The ports can be SYSOP

set  as either input or output.   When set as an output port, from one to eight

on-site  optional  devices per port can be controlled.    Set as an input port,

data from one to eight optional on-site sensors can be read out.   Ports can be

designated as:  A = I (input), A = O (output),  B = I (input),  B = O (output).

Typical values would be A = I, B = I.

(Range: A=I, O and B=I, O)

Parameter 33, MULTIPLIERS

-------------------------

NOTE:  Applies  only to nodes equipped  with telemetry daughterboard.  Sets

the appropriate scale for sensor voltage.