TACLog Lite

by

OZ1FDJ

USER'S GUIDE

VERSION 1,0#

Contents

--------

What is TACLog

System requirements

TACLog rights

Disclaimer

Installing TACLog

TACLog Wizard

Getting started - operation

DXCC maintenance

TACLog bandletters

TACLog files and directories

Call and WWL look-up file

CW keying and PTT

Electronic data interchange

QSL services

Special keys

TACLog representatives

Bo Hansen, OZ1FDJ, Herlevgaardsvej 32 A st. tv, DK-2730

Herlev, (+45) 44 91 53 98

OZ1FDJ@OZ6BBS     oz1fdj@qsl.net

WHAT IS TACLog Lite

===================

TACLog  is  created by Bo Hansen, OZ1FDJ. TACLog is a  contest

program,  solely devoted to VHF and above, hence its name  The

Above Contest Log. It incorporates many useful features,  some

will be presented in detail later in this document. There have

been many other contest programs around, but most of them lack

one important feature: they are definitely NOT user friendly -

or, for that matter, "contest friendly." TACLog is intended to

put  an  end to all this, and to provide the VHF/UHF contester

with a USEFUL and RELIABLE logging program.

TACLog differs from the mainstream logging programs. The  most

important  difference  is the FREE INPUT  FORMAT.  Where  most

other  programs "dictate" the order and format  of  the  input

data during contesting, TACLog gives you freedom of operation,

both  on  the  air  and  on  your  computer.  TACLog  provides

excellent  "fringe benefits" from logging, such as  ultra-fast

dupe-checking,  fast  search & find  functions,  QSL  function

......  plus the ability to configure the program for  use  in

practically  every  type of contest on  VHF/UHF/SHF.  It  also

handles print-outs, summary sheets and logs, all in accordance

with  Region  1  rules. TACLog is also capable  of  generating

several  EDI-files for electronic interchange: REG1TEST,  ARRL

suggested standard file and RSGB standard.

Just  to  let  you know how easy TACLog is, Uffe, PA3HDD  (ex.

OZ1DOQ)  managed  to log 383 QSOs in four  hours,  working  as

single operator using TACLog real-time not losing a single bit

of data and all QSOs were valid.

During   my  work  with  TACLog  many  have  helped  me   with

development, debugging and expressing wishes - I would like to

take this opportunity to thank them all. I would also like  to

express  my deep gratitude to Lars for helping me out when  my

computer  knowledge  failed, the crew  of  OZ9EDR,  especially

Torben,  OZ1KRF, and Palle, OZ1RH, for their enthusiasm  right

from   the   first  pre-release  when  bugs  could  be   found

everywhere, Richard, SM7SCJ for taking the initiative to  this

user's  guide and the marketing, Carl, OZ1IEP for the testing,

but  most  of  all  Sren, OZ1FTU and Uffe, OZ1DOQ  for  their

tremendous amount of work in all phases of the process.

SYSTEM REQUIREMENTS

===================

TACLog  will run on any IBM PC/clone with 375 kB  or  more  of

free  memory. If your PC has expanded memory available  TACLog

uses  this.  TACLog is a DOS program and requires DOS  version

3,0  or  later.  Graphic cards supported are: CGA,  EGA,  VGA,

SVGA  and XGA either in colour or monochrome mode, and MDA  in

monochrome  mode only. On some Hercules monitors  the  program

might  crash. You can run the program from a floppy or from  a

hard disk, the latter is preferred due to read/write times. If

you  use  a  floppy some features are much to  slow  for  live

contesting. A printer is useful, any "neutral" printer will do

provided  it is capable of writing 64 lines per page or  more.

You  may  use  TACLog under Windows, Windows  95,  Windows  NT

(interfaces  does not work), OS/2 for IBM PCs and  SoftWindows

for  the Macintosh. TACLog will have faster disk access if you

have a disk cache such as SMARTDRV.EXE installed. Consult your

DOS manual for proper installation.

TACLog  can  run  on  DOS  versions lower  than  3,0.  If  so,

TACLOG.EXE  must  be executed from the TACLog main  directory,

otherwise TACLog will not run.

TACLog RIGHTS

=============

Many people have asked if TACLog is for free? YES, TACLog is a

program  you may distribute free of charge - i.e. you may  not

charge  more  than the cost of the media carrying  a  copy  of

TACLog.  Imagine what amateur radio would have been  like,  if

you  would  have  to pay for building equipment  described  in

amateur  magazines etc. However, if you like TACLog and  would

like to encourage me, a donation is appreciated.

PLEASE REGISTER, IT IS FREE OF CHARGE - it is used for my  own

amusement and to see how many users I have.

Even  though  TACLog is free of charge you may  not  alter  or

patch  in  the TACLOG.EXE and TACLOG.OVR files or  the  TACLog

name  showing up on print-outs etc. Why this? - Well,  at  the

small  cost  of maximum one diskette, allow me to have  a  BIG

ego.

TACLog,  TACMan, TACRec, TACBox and C_W are all trademarks  of

Bo Hansen, OZ1FDJ.

TACMan is a contest manager program for processing contest log

EDI-files.

TACRec  is  an  interfacing program between packet  radio  and

TACMan.

TACBox  is  a PC to radio interface box enabling  PTT  and  CW

keying.

DISCLAIMER

==========

TACLog is supplied as is. The author disclaims all warranties,

expressed  or  implied  with  respect  to  the  software   and

documentation, including but not limited to implied warranties

of fitness for a particular purpose.

Under  no  circumstances shall the author be  liable  for  any

damages  whatsoever  neither from proper  usage  nor  improper

usage,  including special, incidental, consequential, indirect

or  other  similar damages arising out of the use or inability

to  use  this product, even if the author has been advised  of

the possibility of such damages.

INSTALLING TACLog Lite

======================

IMPORTANT:  TACLog will not run from the root directory  -  on

neither hard disk nor floppy.

If  you  have  received TACLog as an unpacked version  do  the

following:

1. Copy files to the directory where you want TACLog to be.

If  you  have received a packed version, TLLIT###.ZIP, do  the

following:

1. Copy TLLIT###.ZIP to the directory where you want TACLog to

be

2. Using the ZIP archive program unzip TLLIT###.ZIP, e.g.

C:\TACLOG>PKUNZIP TLLIT###.ZIP

Run TLLITE.EXE. After reading the welcome screen, you will  be

in  the main menu. Now carefully read the help screen for this

menu. It is invoked by pressing F1. You can use the F1 -  HELP

key  anywhere  in  the  program to  get  help  on  a  specific

topic/function relative to where in the program you press  F1.

A friendly advise is to start from the top in all menus.

Select 5, Change set-up

-----------------------

It  allows  you  to configure TACLog to your  specific  needs.

There are five different set-up types: custom, hardware,  band

settings,  test  defaults  and summary  defaults.  All  set-up

features  are  stored in pure ASCII-files and  can  easily  be

edited with a standard ASCII editor if preferred.

Select 1, Change custom settings

--------------------------------

Go  through each line using the cursor keys until you have the

configuration you want. These settings affect the  way  TACLog

behaves in general. When done position the scroll bar  in  "<-

OK"  and  press ENTER. Press "Y" and your custom settings  are

now saved in the C:\TACLOG\TLLITE.INI file.

Note  that  you can configure for more than one  band  in  the

"Bands  where  active" setting. See also "TACLog  Bandletters"

later in this document.

Select 2, Change test defaults

------------------------------

The  test  defaults used are either general or  band-dependent

set-up  according to your selection under custom settings.  If

you  have answered NO to use band-dependent, you will edit the

general  defaults,  otherwise you will be prompted  for  which

band  you  wish to edit. You can only edit the bands you  have

enabled  under custom settings. Press a letter to  select  the

band  you wish to configure, e.g. enter correct bonus for each

new  WWL worked in the Nordic Activity Contest. The file  name

for a band-dependent test defaults file at 144 MHz is:

C:\TACLOG\TL_C_144\TLLITE_C.DEF

Select 3, Change summary defaults

---------------------------------

The   summary   sheet   defaults  contain  information   about

participant,  responsible operator, multi operator  calls  and

station  description for the different bands. These parameters

will  be  used  as default values for the summary  sheet  when

generating  a  .LOG (MAIN MENU 4,1,1). When generating  a  log

file  under  post contest processing, you can then accept  the

default values or easily change them if required.

The file name for a summary sheet defaults file at 144 MHz is:

C:\TACLOG\TL_C_144\TLLITE_C.SSI

When   done,  you  have  completed  the  entire  set-up.   All

information that you have entered, will be saved to disk.

TACLog WIZARD

=============

To  help  setting-up  the  right  parameters  in  the  custom,

hardware, band and test parameter menus TACLog has a build  in

Wizard, that will help you out. If you absolutely do not  have

any  idea  how  to set these parameters, press F1  instead  of

ENTER at the <-OK prompt in these menus.

In  custom,  hardware  and band set-ups the  Wizard  sets  the

parameters to the ones most frequently used. This way,  TACLog

can  be  used  quickly also by contesters with  little  or  no

previous  TACLog experience. Later, when you get more familiar

with TACLog you can play with more advanced configurations.

In  test parameter set-up, the Wizard shows you a list of test

names  from which you can select the one you want. After  this

you only have to insert callsign and WWL.

Even if the Wizard helps you in setting up the parameters, you

can always fine tune afterwards. This can also be necessary if

contest  rules have changed or that I have received incomplete

rules etc.

GETTING STARTED - OPERATION

===========================

Do your set-up (see INSTALLING TACLog). Run TACLog, and kindly

read  the information screen. When in main menu, choose "Start

contest from scratch". You will be prompted for which band (if

you have enabled more than one band in the custom set-up), for

test defaults to change (if enabled in custom), and local date

and  time.  The date and time must be set to track  with  your

local  time. TACLog takes into account the difference  between

local  time and UTC, set in custom set-up parameter Time Zone.

Make sure that the time is correct since you cannot enter QSOs

ahead of the current date and time.

Real-time - Insert time automatically

-------------------------------------

Real-time  means just what it says, the QSO time is  triggered

by  typing a SIGNAL REPORT and the ENTER key. This way, it  is

possible to "monitor" a distant station with a pile-up, typing

the call and WWL and then just having to enter the reports  to

make the QSO complete for the log. The first QSO automatically

starts the log file at the proper date. All further input will

be  strictly  CALLs, RSTs and - if chosen - QSO no.,  WWL  and

Optional Exchange. The computer keeps track of time.

Non-real-time - Insert time manually

------------------------------------

If  you just worked a contest using pen & paper, this would be

your  choice. Just remember to enter the contest date.  Format

is  YYYYMMDD.  This is done only at the first  QSO  input,  or

later  if  you worked into the next day. Time input format  is

HH.MM - there has to be a full stop to separate HH and MM.  If

QSOs are within the same hour you only need to enter .MM. Just

make  sure  that you type the time before you enter  a  SIGNAL

REPORT.

TACLog  is  fully capable of seamless switching between  real-

time  and  non-real-time operation. Upon your late arrival  at

the  club  station, you can always type in the first  10  QSOs

(that your partner had to log on paper, hi) together with  the

right  times, and then continue in real-time mode. This  is  a

major  feature for late sleepers, traffic jam victims etc.  Of

course it works the other way round, too. Let's say you have a

computer  power  failure. Operation until then was  real-time.

Just continue in non-real-time, fill in the gaps, and continue

in real-time.

Before  seeing  the  contest screen, you are  prompted  for  a

comment  line. The comment line is shown in the  file  browser

when  selecting  files. You can type e.g.  "144  MHz  Activity

Contest,  February  1994", or just press  ENTER  to  leave  it

blank.

The  contest  screen is divided into three main parts:  upper,

middle and lower part. The upper part is used for totals,  the

middle  part  for  keeping track of WWLs, Optional  Exchanges,

DXCCs,  last  QSOs and Match and the lower part is  the  INPUT

window.  Below  it, there is also a command  summary  for  the

function keys F1-10. Now browse the F1 help screens, and  then

return to the contest screen.

Type  your first QSO into the input line. If in non-real-time,

do remember the date (YYYYMMDD) and time (HH.MM).

Remember  the F1 help screen, it tells you anything you  might

need about entering QSO data.

IMPORTANT: Let's say you work DL7AKA. Maybe his call is listed

in  the .C_W file. If DL7AKA is there, TACLog will look  up  a

suggested  WWL  and  write it to screen  (reverse  video)!  To

accept  this suggested locator, just press Alt+X. The  reverse

video  disappears,  and you are prompted  "All  data  correct?

(Y/N)".  Press  Y  and  go on. (You may ALWAYS  enter  another

locator to override the suggested one, by the way).

Now  enter  five or more QSOs, just to get used  to  the  free

input  format. After entering the first 10 QSOs, you are ready

to fool around a bit with the F-keys. Try F4, F8 and Shift+F8,

read  the help screens provided, and return to contest screen.

Try to search for *7THS/7 (imagine you heard just parts of the

call)  to  see  if something like it has been worked  already.

Search  J?3*  to  get  what stations you worked  with  a  JO3*

locator. Remember, to exit from help and features, just  press

ESC.

Now you are about ready to have a go at "the real thing".

One more thing before you go: The input fields are in fact two

lines  of  fields. Example: Input DL7AKA 53S  ENTER.  Then  he

disappears,  no  RST,  no WWL. If you enter  another  callsign

instead  of  DL7AKA,  and  perhaps a new  report  (a  stronger

caller,  perhaps), the old information is shown below the  new

data, just to keep track of where you were. This is one way to

clear a half-finished QSO, the other is pressing the F3 key to

"clean the table" in the input line and fields.

Try  it  out. You'll find it easy to work with TACLog. If  you

make  a mess, the most you will loose is one QSO. TACLog saves

to disk so often that you need not worry.

Operating rules

---------------

If  you  think  it  is difficult to remember  it  all  -  just

remember the following three rules:

1. A sent RST must have an "S" at the end of the report, e.g.

52S, 599S, 55AS

2. A received RST must have an "R" at the end if T-tone is

included, e.g. 519R, 59AR

3. A received Optional Exchange must have a "." at the end,

e.g. A., 14., SLD.

That  is  all  you need to know. The rest is  visible  on  the

screen, and with context dependent help windows.

DXCC MAINTENANCE

================

TACLog  is capable of identifying countries. TACLog  uses  the

file  TLLITE.DXC  in the main directory. If the  file  is  not

present TACLog cannot run. This file is a converted file which

is  output  from  a DOS program called DXC.EXE.  This  program

takes a *.CTY file as input. The *.CTY file is also used in CT

by K1EA. If you want to update TACLOG.DXC manually then edit a

*.CTY file and convert it to TACLog format.

Syntax for DXC.EXE, e.g. using the IARU.CTY file:

DXC.EXE IARU.CTY

If  you want to check your updating you can do this by running

the  TACLog utility program called SHOWDXCC.EXE. It loads  the

TLLITE.DXC file and lets you type in callsigns and returns the

DXCC and rawcall.

When running TACLog in contest mode and TACLog cannot identify

a  country a flashing "?" is shown in the callsign column.  If

you  continue  the  DXCC will be recorded as  "?",  after  the

contest   you  may  investigate  which  country  the  callsign

actually  belongs to. If the callsign is in a new country  you

will  see an early warning, flashing "!", the same goes for  a

new Optional Exchange or a new WWL.

TACLog BANDLETTERS

==================

TACLog uses letters to identify one band from the other in  an

unambiguous way. The letters are represented as shown below:

A  -   50 MHz          I   -  3,4 GHz

B  -   70 MHz          J   -  5,7 GHz

C  -  144 MHz          K   -   10 GHz

D  -  220 MHz          L   -   24 GHz

E  -  432 MHz          M   -   47 GHz

F  -  902 MHz          N   -   76 GHz

G  -  1,3 GHz          O   -  120 GHz

H  -  2,3 GHz          P   -  144 GHz

TACLog FILES AND DIRECTORIES

============================

TACLog  has  six  types of directories: main-,  band-,  data-,

INFO,  MAPS  and  USERS directory. Each  serves  a  particular

purpose.

MAIN DIRECTORY

--------------

TLLITE.C_W     General .C_W file, optional - depends on custom

setting.

TLLITE.DXC     DXCC file, must be present.

TLLITE.EXE     Executable program file, must be present.

TLLITE.DOC     English User's Guide.

TLLITE.HLP     English texts.

TLLITE.INI     Initialising file.

TLLITE.MBF     Multiband support file.

TLLITE.OVR     Overlay file, must be present.

TLLITE.WIZ     Wizard file.

The name of the TACLog main-directory is free of choice.

BAND DIRECTORIES

----------------

There is one band-directory per band containing the following:

TLLITE_?.BND   Band parameter file for ? band.

TLLITE_?.DEF   Test  defaults  file  for ?  band,  optional  -

depends on custom setting.

TLLITE_?.SSI   Summary sheet information file for ? band.

Where  ?  is the letter representing the band, see above.  The

name  of  a  band-directory  is TL_?_fff,  where  fff  is  the

"frequency".   The  band-directories  are  located   as   sub-

directories of the main-directory.

DATA DIRECTORIES

----------------

Each  contest  has  its own data-directory  with  data  for  a

particular  contest only. The data-directories are located  as

sub-directories  of  the  band-directory.  The  name  of   the

directory  is  taken from the date of the  first  QSO  in  the

contest. Files contained are:

TLyymmdd.DAT   Data file.

TLyymmdd.DUP   Duplicate QSO file.

TLyymmdd.EDI   Electronic data interchange file.

TLyymmdd.LOG   Log file ready to print.

TLyymmdd.NOT   Note  ASCII  file  used to  store  notes  taken

during the contest.

TLyymmdd.RAW   Raw-call sorted file.

TLyymmdd.RPT   Report file.

TLyymmdd.SSI   Summary sheet file.

TLyymmdd.STA   Statistics file.

TLyymmdd.QSL   QSL label file.

Actually  there is one more file, ?_yymmdd.BAK.  This  is  the

name  for the back-up file saved to floppy. Back-up files  can

be restored in the FILE MANIPULATION MENU 1 (MAIN MENU 4,6,1).

DIRECTORY STRUCTURE

===================

Assuming that TACLog is installed in C:\TACLOG and 50 MHz, 144

MHz and 432 MHz bands are enabled, the directory structure  is

as follows:

C:\---TACLOG-+-TL_A_50--+-TLyymmdd

|          +-TLyymmdd

|

+-TL_C_144-+-TLyymmdd

|          +-TLyymmdd

|          +-TLyymmdd

|

+-TL_E_432-+-TLyymmdd

|          +-TLyymmdd

Do  not  worry if you cannot remember the directory structure,

TACLog will control the directory navigation automatically.

CALL AND WWL LOOK-UP FILE

=========================

TACLog is able to look-up a World Wide Locator based upon  the

callsign,  if it is known by TACLog when you start a  contest.

To  enable this feature you must set the "Look-up possible WWL

in  .C_W  file"  in the custom set-up. For an  explanation  of

TACLog files and directories see above.

TACLog can handle up to 2147483647 (2^31 - 1) calls and  WWLs.

Each  element takes up 38 bytes on your hard disk. If you have

all possible elements, it results in a maximum .C_W file of 76

GB.  On the other hand, if you ever work so many stations, you

are  entitled to call yourself a big gun - and big  guns  need

big hard disks.

TACLog may store two WWLs per callsign in the .C_W file.  When

a call is found TACLog displays both WWLs if they exists, else

only  the first one is displayed. The two WWLs are called WWL1

and  WWL2. You can accept WWL1 by pressing Alt+X, or  you  can

swap WWL1 and WWL2 by pressing Alt+V. This key combination  is

also used for swapping WWLs entered properly, see also SPECIAL

KEYS. In the .C_W file a date related to WWL1 is also stored.

To add new callsigns and WWLs simply use CALLS AND WWLS MENU 1

(MAIN  MENU  4,5,1). Select the data file  you  want  to  add.

TACLog will do the rest. TACLog replaces the second WWL (WWL2)

with the primary WWL (WWL1) and inserts the new WWL as primary

WWL.  The date is also replaced, with the date of the QSO data

file.  If  the new WWL is similar to WWL1 TACLog only  updates

the date, i.e. the .C_W file cannot contain two identical WWLs

for the same callsign.

The  call and WWL look-up feature uses the computer hard  disk

as  resource. It is vital that you use a hard disk if you have

enabled  the  look-up feature. It is not  recommended  to  run

TACLog from a floppy disk using this feature. TACLog has  been

tested  on a 4,77 MHz PC with a 80 ms hard disk which performs

satisfactory (full dupe-check, DXCC identification, and  look-

up  for  rawcall lasts less than 300 ms searching  about  2800

elements). However, a faster hard disk will do better. Another

thing  that  can improve the speed is setting up a disk  cache

such  as  Microsoft SMARTDRV.EXE and to defragment  your  hard

disk  regularly,  e.g.  Microsoft  DEFRAG.EXE.  You  can  also

disable rawcall look-up if you have a slow system or have many

elements in your .C_W file. The delay on my PC, 486DX2-66  VLB

with  2  MB  cache  and  12 ms IDE hard  disk,  is  not  worth

mentioning.

If  you  want to merge, sort or manipulate files which do  not

come  from  TACLog into a .C_W file, you can  use  the  TACLog

utility  program  C_W.EXE.  This program  operates  as  a  DOS

command  line program, and is a tool for DOS-jockeys only!  It

converts  a  TACLog .C_W file to a plain ASCII text  file  and

vice versa. Regarding the file formats type C_W.EXE at the DOS

prompt without any command line parameter.

CW KEYING AND PTT

=================

TACLog  has a CW and PTT keying facility. To use the CW keying

you must set-up at which parallel or serial port you have your

interface circuitry, see "Interfaces" below. The messages  can

be  altered  in  the  contest screen (Alt+F5-F10).  Playing  a

message  is  done  by activating Ctrl+F5-F10  in  the  contest

screen. A 250 ms guard delay, is inserted between PTT  on  and

message  keying begins and stops. TACLog also has  a  push  to

talk,  PTT, key on the PC keyboard. The PTT is toggled on  the

TAB key. You must enable the feature in the custom parameters.

TACLog  has  some special CW commands words which  all  begins

with a dollar sign "$". The commands are:

$MC    Own call.

$MR    RST received from calling station, confirmation use.

$MN    Own QSO number, if used.

$MO    Own optional exchange code, if used.

$MS    Own short WWL, if used.

$MW    Own WWL, if used.

$C     Call of calling station.

$R     RST to calling station.

$N     QSO  number  received from calling  station,  if  used,

confirmation use.

$O     Optional  exchange  received from calling  station,  if

used, confirmation use.

$S     Short  WWL  received  from calling  station,  if  used,

confirmation use.

$W     WWL   received   from   calling   station,   if   used,

confirmation use.

$##(#) Set CW speed, in LPM 20 - 200. Only in the beginning of

message.

$#     Set auto repeat pause, 1 - 9. Only at end of message.

A CW message can not be auto repeated more than 25 times. This

is to prevent "continuos" transmitting if the operator falls a

sleep.

INTERFACE

---------

The  interface uses Data Terminal Ready (DTR) for  keying  the

message,  and uses Request To Send (RTS) to key the  PTT.  The

circuitry  consists  of  a  dual optocoupler,  providing  100%

galvanic isolation between PC and radio. Between the RS232-pin

and  the  opto-diode  a  4,7 kOhm resistor  is  inserted.  The

collector  of  the optocoupler is the first  transistor  in  a

darlington  coupling and the combined collectors are  the  PTT

line. Apply capacitors, there is RF in the air - you know!

PC                         Radio

DTR    <--[4k7]--+

(RTS)            |     --+--> PTT

--  |/  | C  (Key)

\/  |  |/

--  |\-|

|      |\ E

PC GND <---------+       |

= Radio ground

ELECTRONIC DATA INTERCHANGE

===========================

The  Electronic Data Interchange or EDI in TACLog is a special

type  of  file  that can be sent to the contest manager  using

various  types  of  media,  e.g.  packet  radio,  diskette  or

telephone modem. The EDI-file is a standard way of setting  up

the  information  in  the file defined by the  format.  TACLog

supports the REG1TEST, ARRL Suggested Standard File,  RSGB  97

and ADIF proposed standard. By using EDI it is possible to use

any  contest program and generate the standard file, i.e.  you

do  not  have  to use a particular program. You  may  use  any

program supporting the requested format.

In TACLog you can generate an EDI-file in EDI FILES MENU (MAIN

MENU  4,2). By selecting this feature TACLog asks you for  the

data  filename  and the information used in a  normal  summary

sheet,  just like generating a log. After running through  the

summary information the EDI-file is written to your disk. Then

you  must  send the file to the contest manager.  If  you  use

packet   radio,   send  the  file,  see  "TACLog   Files   and

directories", as a personal mail to the contest  manager.  How

to operate the terminal program and BBS or E-mail depends upon

the  software used locally, not upon TACLog. Therefore consult

your  manuals  for further information. Some contest  managers

accepts  entries on floppy disk if the files are  named  in  a

specified  way. If so, rename the TLyymmdd.EDI  file  to  this

naming standard accordingly.

If  you use PAX packet terminal program and your BBS used  FBB

software  do  the  following: connect to  the  BBS,  write  SP

CONTEST_MANAGER_CALL @ MANAGER_BBS (e.g. OZ5TG @ OZ8BOX) press

ENTER, type subject and press ENTER, press F3, write path\EDI-

filename (e.g. C:\TACLOG\TL_C_144\TL931102\TL931102.EDI) press

ENTER,  write  /EX press ENTER. When done you  have  submitted

your EDI-file/log to the contest manager.

QSL SERVICES

============

TACLog offers printing and maintenance of QSL cards and labels

in  the  QSL  SERVICES MENU (MAIN MENU 4,7). Before  going  to

print labels you must select QSL label type in the custom set-

up.  You can select between three different types. Type  1  is

88,9  mm  *  23,8 mm. Type 2 is 88,9 mm * 35,7  mm  (Zweckform

3611; Conrad 97 15 88-66; 97 16 18-66; 97 16 42-66; 97 16  77-

66; 97 13 75-66; 97 14 05-66; 97 16 93-66; 97 18 12-66; 97  16

26-66; 97 15 61-66; 97 15 96-66; 97 16 50-66. Type 3 is 105 mm

* 35,7 mm (Jac Tab 10731).

When aligning the labels in your printer, you can make use  of

the test label.

SPECIAL KEYS

============

In  the  TACLog contest screen the following keys are assigned

as special keys:

F1         Activates help menu.

F2         Save QSO data to floppy disk named ?_yymmdd.BAK.

F3         Clear input line and current QSO data.

F4         Show worked QSO data.

F5         Switches   between  QSOs,  DXCCs,  WWLs,   Optional

exchanges or Match.

F6         Edit the data of an already worked QSO.

F7         Insert new QSO data, non-QSO-number mode only.

F8         Find menu, used to search in worked QSO data.

F9         Exit temporarily to a DOS shell.

F10        End contest logging.

Alt+F3     Edit CW messages.

Alt+F7     Delete  an already worked QSO, non-QSO-number  mode

only.

Alt+F8     Tune key for PTT and transmitter.

Alt+C      Swap callsign with last entered callsign.

Alt+D      Show DXCCs, same as F5.

Alt+E      Edit select, same as F6.

Alt+F      Find menu, same as F5.

Alt+I      Information files menu.

Alt+L      Show missing WWLs map, same as Shift+F2,D.

Alt+O      Show worked Optional exchanges, same as F5.

Alt+P      Show polar plot, same as Shift+F2,I.

Alt+Q      Show worked QSOs, same as F5.

Alt+S/N    Stop rotator system A/B.

Alt+T      Key PTT and key line for tuning TX.

Alt+V      Swap WWL with last entered WWL or suggested WWL2.

Alt+W      Show worked WWLs, same as F5.

Alt+X      Accept suggested WWL1 (WWL2 if swapped).

Ctrl+F1-F10Play CW message 1 to 10 if CW is enabled.

Ctrl+A-N   Quick band swap, if in multiband mode.

Shift+F1   Make note to TLyymmdd.NOT file.

Shift+F2   Show statistic information menu.

Shift+F3   Change user/operator.

Shift+F4   Show worked QSO data sorted by raw-callsigns.

Shift+F5   Print all worked QSO data, status counters etc.  at

selected LPT port.

Shift+F6   Edit menu with scroll bar select in QSOs window.

Shift+F7   Delete an already worked QSO, non-QSO-number only.

Shift+F8   Find menu, used to search in C_W file.

Shift+F9   Show system status (memory and disk availability).

Shift+F10  Menu for multiband toggle.

Up arrow   Edit menu select with scroll bar, same as F6.

Down arrow Repeat last message.

Tab key    Toggles PTT on interface port, if enabled.

TACLog REPRESENTATIVES

======================

Below is a list of TACLog representatives where you may always

find  the  latest copy of TACLog. They will be  of  assistance

with copying if you send a self addressed and stamped envelope

(IRC),  SASE,  and  a formatted disk 1,44 MB.  Do  NOT  expect

anything  to happen if you do not include SASE or at  least  a

self addressed envelope (SAE) plus sufficient return postage!

If  you did not get your copy from one of the representatives,

please send a note that you got a copy elsewhere. This is used

for  my  own  amusement only, not to claim  anything  but  the

number  of  TACLog users. Please do so - else  I  might  begin

asking for user identification and password.

Please  note  that translated TACLog documents and  files  may

only be obtained from the translators - not OZ1FDJ!

Benelux, PB0AOL

---------------

Edwin  Groener, Groningensingel 577, NL-6835 EW Arnhem. PB0AOL

@ PI8DAZ. E-mail: pb0aol@tip.nl.

Czech Republic and Slovak Republic, OK1XH

-----------------------------------------

Milan Kalina, Smeralova 3, CZ-17000 Praha 7, OK1XH @ OK0PRG, E-

mail ok1xh@merck.cz.

Denmark, OZ1FDJ

---------------

Bo  Hansen,  Herlevgaardsvej 32 A  st.  tv.,  DK-2730  Herlev,

Phone:   (+45)   44  91  53  98.  OZ1FDJ  @  OZ6BBS.   E-mail:

oz1fdj@qsl.net.

Germany, DJ3LE

--------------

Hans-Jrgen Lthje, Lindenweg, D-24894 Tolk. DJ3LE @ DJ3LE. E-

mail: dj3le@cruise.com.

Greenland and North America, OX3LX

----------------------------------

Bo   G.   Christensen,  Box  504,  DK-3920  Qaqortoq.  E-mail:

bc@tele.gl.

Mediterranean Area and Africa, I4YNO

------------------------------------

Alessandro  Della  Casa,  Viale Vittorio  Veneto  70,  I-41100

Modena.  Phone:  (+39) 059 23 62 43. I4YNO  @  I4EUM.  E-mail:

alex.della@iol.it.

Poland, SP3SUX

--------------

Irek  Lochert, al. Konstytucij 3 Maja 29/67, PL-66-400 Gorzow.

Phone: (+48) 095 203300, SP3SUX @ DB0FRO.

Sweden, SM7SCJ

--------------

Richard  Jacob,  Brnnestad 163, S-231  99  Klagstorp.  Phone:

(+46)  0410 - 26664, BBS service upon request. SM7SCJ @ SK7OA.

E-mail: info@gateway-it.se.

United Kingdom, G6IQM

---------------------

Michael J. Wooding, Ware Orchard, Barby, Nr. Rugby, CV23  8UF,

UK, Phone (+44) (0) 1788 890365, fax: (+44) (0) 1788 891883. E-

mail: vhf-comm@g6iqm.demon.co.uk.

http://www.clearlight.com/~vhfcomm                           &

http://www.g6iqm.demon.uk/vhfcomm.htm.

If  you  did not find your country write OZ1FDJ, see  Denmark.

More   representatives  wanted  -  how  to  become  a   TACLog

representative? You should be familiar with DOS and a  regular

TACLog  user and preferably QRV on packet radio, E-mail and/or

have  a telephone modem. Being a representative brings nothing

but  the satisfaction and perhaps some goodwill amongst  radio

amateurs.

TACLog on Internet

------------------

You can find the TACLog WWW Home Page at:

http://www.qsl.net/oz1fdj

The TACLog home page is also the first place where to find new

releases of TACLog.

If  you  give away a copy to a friend please include at  least

the following files:

TLLITE.HLP

TLLITE.EXE

TLLITE.WIZ

Please tell her/him to register also!