��������  �������  ������     ����   ��    ��

�۱�۱�۱  �۱��۱  �۱���   �۱���  �۱   �۱

۱��۱ ۱  �۱  ۱  �۱ �۱ �۱�  ۱  ��  �۱�

� �۱  �  �۱�  �  �۱ �۱ �۱    �   ���۱�

�۱     ���۱    ����۱� �۱         �۱�

�۱     �۱۱    �۱���  �۱        ����

�۱     �۱ �    �۱     �۱   �   �۱���

�۱     �۱      �۱      ��  �۱ �۱�  ��

����    ����     ����       ���۱� �۱   �۱

����    ����     ����       ����   ��    ��

The Firmware PC Extended

Version 2.10 (10th. November 1993)

Resident AX.25-Controller for PC

and BayCom-Modem/-USCC-Card

PA0HZP-OptoPcScc-Card, Kiss

with WA8DED-Hostmode-Interface

by Ren� Stange, DG0FT @DB0KG.DEU.EU

Free for Radio Amateurs, commercial Usage not Permitted

English Translation by Denis Godfrey, G0KIU @ GB7MAM

1

Contents

--------

1.      Forward

2.      Explanation of the Documentation

3.      Introductions since Version 2.0

4.      Quick Start

5.      Installation

6.      Calling and Configuration

6.1.    General Options

6.2.    Options for Resident Loading

6.2.1.  Port- and Baudrate-Configuration

6.2.2.  TFPC- and DRSI-Interface

6.2.3.  Other Options

6.3.    Clearing TFPCX from the RAM

6.4.    Terminal Mode

7.      Installation

7.1.    SP (DL1MEN)

7.2.    GP (DH1DAE)

7.3.    THP (DL1BHO)

7.4.    TERM (DL5FBD)

7.5.    CT (K1EA)

7.6.    DIEBOX (DF3AV)

7.7.    WINPR (DG6BI)

7.8.    TOP (DF8MT)

7.9.    FBB (F6FBB)

7.10.   MS-Windows, OS/2 and others

8.      Operation

8.1.    Multi-Port-Expansion

8.2.    Individual Commands

8.3.    KISS

8.4     DAMA

APPENDICES

1.      Overview of Commands

2.      Error Correction (Modem Operation)

2.1.    Send- and Receive Problems

2.2.    Problems with other Programs

2.3.    Hardware Problems

3.      Hardware Connections

3.1.    Serial Modems

3.2.    BayCom-USCC-Card

2

4.      Information for Software Developers

4.1.    Program-Interface

4.1.1.  TFPC-Interface

4.1.2.  DRSI-Interface

4.1.3.  Special Functions

4.2.    Format of Reports

4.3.    Extended Hostmode

4.4.    Previous Versions.

5.      Copyright and Usage

6.      Reference Advice

3

1. Forward

Unfortunately this TFPCX-Version has been awaited longer than I had

originally intended. Version 2.01 that appeared in February, in early

expectation of this latest Version from me, had not been advertised in the

Mailboxes and hardly recognised. This Documentation is updated completely

from Version 2.00. In order not to delay Publication yet further I must

explain a few alterations, about the basic revision  of the Modem Control.

New to be mentioned, is the support of the KISS-Mode for simultaneous use

of more Ports, the PA0HZP-OptoPcScc-Card and the Baycom 9K6-USCC-Card.

Additionally it gives better possibilities for Configuration, improvement

with DAMA-Operation and others. The activation of the KISS-Mode in the TNC

will be delivered with the new Program KISSINIT (see KISSINIT.DOC). The

new additions are listed in Section 3. Section 4. gives some advice to be

observed about upgrading from an earlier Version.

I am not able to guarantee that TFPCX will function free of problems on

all PCs with serial Modems; with the USCC-Card it is less critical. On

slow Computers especially, it gives partial Receive problems. XTs with

speeds under 8MHz are not usable or only with reductions. Frequently

certain resident Programs cause Problems (See Appendix 2.1.). This is not

offered as a Commercial Product and should be accepted as such. However

TFPCX places negligible demands on the compatibility of the employed COM-

PORT compared to BayCom, so it will run on Computers that are not BayCom-

suitable. With certain compromises most Users are able to arrive at a

satisfactory operating condition.

My thanks go Peter (DB2OS), Asko (DG2BRS), Denis (G0KIU), Henk (PA0HZP),

Rob (PE1CHL), to the BayCom-Team all who have made donations, and all

users who have helped me with advice and recommendations, especially

73 De Ren�, DG0FT                      Strausberg, 20th. November 1993

4

2. Documentation Advice

In this Documentation I assume that on hand there is available a suitable

Modem, USCC-Card or a KISS-TNC and a Terminal Program, together with a

knowledge of the TNC-Software, and The  Firmware  (NORD><LINK) or the

WA8DED-Firmware. A File is available to be read if a description of the

Firmware 2.4c is required. Only the differences compared to TNC-Firmware

will be considered here.

Time will have been saved if a correct arrival has been made at a working

TFPCX without a lengthy Study of this Documentation, but perhaps a few

tricks may have been overlooked. In the case where there are Questions or

Problems, then I request, that a solution is sought here. Practical

experience shows it is always the same questions that will be asked and I

have born this in mind without claiming total completeness. Of course this

Documentation is not able to be a Basic Course in MS-DOS or Packet Radio.

Moreover one is able with the use of a Text Editor search function to

find a required keyword easily.

In this Documentation certain Hard- and Software will be mentioned, which

will have been developed by other Radio Amateurs, the majority have their

Call Signs following in brackets.

Concepts and Abbreviations:

Port       A Packet Radio Interface consisting of a Port (COM, LPT or

SCC-Port), Modem and Radio, more will be said about Multi-

Port-Operation.

Channel    one of the maximum 20 simultaneous Connects, with BayCom

the meaning of Port and Channel is exactly the opposite.

Frame      Complete Data unit transmitted by Packet Radio (Packet),

consists of Address field, Control field, Data and

Checksum.

Interrupt  Break-into the running Program by a Hardware-Event (e.g. Key

Press, definite Time-Interval), Software-Interrupts will not be

by the Hardware but created through a particular Program

Command.

5

KISS       (KA9Q and others) stands for 'Keep it Simple Stupid' and

defines a simple Data Format for the transfer of Frames and

TNC-Parameters over an asynchronous Serial Port. The original

object was the removal of the Protocol operation from out of

the TNC and into the Computer, in order that Protocols that

were not supported by the TNC could be used. KISS is

implemented in many TNCs, but also enabled the direct Computer-

Linkage.

SMACK      (DL5UE and DK5SG) is an abbreviation of 'Stuttgarts Modified

Amateur radio-CRC-Kiss. Developed from an Error Free transfer

outgoing KISS in order to be able to recognise a transfer error

by means of a Checksum calculation.

0x         Prefix of Hexadecimal Number (e.g. 0x300 = 300H)

3. Introductions since 2.0

- The KISS-Mode will support SMACK for up to 4 Ports and 57600 Baud

inclusive (Option -PKISS. Faulty Frames (e.g. through Character loss or

damage) will be ignored and counted (See Section 6.2.1 and 8.3.). Also

by means of TFPCX and KISS GP can Control more TNCs in parallel.

- The PA0HZP-OptoPcScc-Card will be supported (Option -POSCC, see Section

6.2.1).

- The BayCom-9K6-USCC-Card now functions with TFPCX (Option -PUSCC, see

Section 6.2.1.). With a few BayCom-USCC-Cards meaningless Data has been

sent because of Timing-Problems. Through another directive to the SCC-

Controller this Problem will be circumvented.

- For SCC-Cards and COM-Ports (with KISS) AT-IRQs (9, 10, 11, 12, 14, 15)

can be used.

- The quantity of free Buffers (Option -BU) and the Connect-Channels

(Option -CH) and therefore the required RAM-Space now can be configured.

(Standard: 600 Buffer/10 Channels, see Section 6.2.3.)

- The Initialisation File (Option -F) can contain Empty Characters and

Commentary. ESC-Characters will be recognised automatically and no

longer required to be given as '^'. (See Section 6.2.3).

- With the DRSO-Interface (Option -DR) incompatibilities with DRSI-TNCTSR-

Driver have been removed, which previously e.g. lead to problems with

Monitor and Heard list with the operation with FBB (F6FBB). In the case

where these alterations now give Problems with other Programs (e.g. with

TOP), the Option -DX the previous (modified) DRSI-Interface can be

employed (see Section 6.2.2.1).

- The new Command @PO makes possible the selective ordering of a Port to

a Channel, which then can only be connected from this Port, which is

useful e.g. with the Program TOP (DF8MT). (see Section 7.8 and 8.2.).

6

- Through a Transparent-Mode with Receive (Command @M) and switchable

Character Echo (Command @E) the #BIN#-Receive in the Terminal-Mode is

possible (e.g. with TERM (DL5FBD)). (see Section 7.4 and 8.2)

- Internal Connects (Loopback) can be prevented by choice (Option -NL),

see Section 6.2.3.).

- TXTAIL (Command @TA) with the Serial Modem and SCC-Card will be

installed with consideration of Baud Rate and Timer-Inaccuracy.

Previously it gave partial Problems with operation at 300 Baud.

- If the Option -P is not issued, a Serial Modem on COM1 will be used, no

longer as previously, but mainly no Port steered towards it, which is

only meant for Test purposes with internal Connects. Therefore in normal

cases, -P is to be given at all times.

- The Option -D (Debug) is now standing as the last one before the Option

-P (Port). With it, the monitoring of any Port is possible also with

Multi-Port-Operation (see Section 6.2.3.).

- Disconnect in the background operation, Terminal-Mode is possible with

the Remote-Command '//Q' (Command U, see Section 8.2.)

- Through a DAMA-Alteration (as TF 2.6) the known bleating Report of

TheNetNode-Digis with Multiconnect now should be kicked out.

- The Extended Host-Mode (DG3DBI) will be supported and makes possible

faster communication with the Terminal Program.

- The Default-Values for the Parameters F, N, P, R, T, U, @A3, @I, @T2,

@T3, @T4, AND @TA have been altered.

4. Quick Start

Because of the different Configuration possibilities and Terminal-Programs

no general 'Recipes' are able to be given. It is adequate, that Section

6.2.1 and 7. (Correspondingly used Program) are read. If there is a

problem then Appendix 2. should be read. For the operation of more Ports

Sections 8.1 and 8.2 are important. In Section 8.3 important Advice for

the KISS-Operation is given

If with a previous Version no -'P'-Option has been issued and the Default-

Port COM1 employed, now absolutely required, is the Option '-PCOM1'.

In the case where with Version 2.0 more than 10 Channels have been

employed, the Option '-CHnn' must be issued with the Start of TFPCX,

whereby nn is the required number of Channels.

Previously if the Option 'DR' has been employed and with it, this Version

kicks out Problems with Monitor or Heard list, then instead of '-DR'

Option '-DX' should be issued. (e.g. with TOP)

7

Up to now if you have worked with Version 1.1x  and wish to employ only

one Modem, merely the Option '-C' is required to be given with the loading

of TFPCX, if the Send-/Receive Indicator is required, the switching on is

no longer automatic. If the Option 'NC' has been issued hitherto, this is

deleted.

With 'TFPCX  -H' all Options are allowed to be called in the short form

and 'TFPCX -U' removes TFPCX from RAM.

5. Introduction

Packet Radio has been and will be operated for all, with the so called

Terminal-Node-Controller (TNC). This undertakes the entire PR-

specification Protocol development (AX.25). Therefore TNCs are nothing

more than (not more special performers) Computers with Serial Port and

Modem. By reason of further broadening of a special for all who run TNC-

Software (WA8DED-Firmware or the compatible The Firmware of NORD><LINK) on

the TNC2, in the meanwhile, certain individual performance capable and

widely distributed Programs exist, which employs the special Software-

Interface of this Firmware (WA8DED-Hostmode).

Nowadays, now that more powerful PCs are no longer very expensive, the

possibility is afforded for them to take over the work of the TNC and so

save the investment of such an extra. Additionally Portable operation with

a Laptop or Notebook is much more elegant if one does not have to drag

around an additional 'grey box'. For this purpose the Baycom-System

(Terminal, Modem or USCC-Card) has offered a quicker, cheaper and easily

understandable solution for some time. Of course BayCom is a closed

System, which the operation of the TNC-Terminal Programs does not support.

At this point TFPCX enables the Connection between the available Hardware

(Serial Modem or USCC-Card) and the Terminal Programs for TNCs. TFPCX is a

RAM resident Program that is loaded into the Computer, it makes itself

comfortable and then constantly urged in the background to perform its

work, from the Interrupts of the PC and is able also, to send/receive Data

and to react to the Commands of the Operator. Therefore the Data- and

Command Interface to the Terminal Program (Software-Interrupt) is

very similar to that employed with TNCs and additionally (what is

important) well documented. On this account it creates scarcely any

Problems in making existing Programs operable with TFPCX and for the

important Programs this is already taking place.

When TFPCX has been loaded, the System behaves as if a TNC has been

connected, also it is able to receive Connections from external sources

and save all incoming messages. Because TFPCX runs in the background,

other Programs are able to be used simultaneously. (Removal see Appendix

2.2.). For unread Information a flashing square appears in the top right

hand corner of the Screen. Immediately the Terminal Program is started,

the received Text is displayed on the Screen. TFPCX is similar to the

Program L2 (DL8MBT) of the BayCom-System.

TFPCX does not operate with TCP/IP Software and only runs on IBM-

compatible PCs.  TCP/IP can operate with the Packet Driver AX25DRV for

BayCom-compatible Modems from Pawel Jalocha.

8

6. The Calling and Configuration of TFPCX

TFPCX will be activated by the following Command Line:

TFPCX [ -N ] [ <load options> | -T | -U ]

All Options will be commenced with a '-' and separated with an empty

character. No empty characters are permitted within an Option. Upper and

Lower Case Letters are optional. Certain Options (e.g. '-P') can have more

Parameters, which will be separated with a ':'. With no declarations,

Standard Values will be installed.

Example:

For  '-PUSCC::5' '-PUSCC:300:5:1103' will be utilised, for which the

absent values, 300 und 1103 will be installed.

It is possible to start TFPCX from a Batch-File, but it is not always

necessary to type the same Options. In the following, all the Options will

be listed in the short form, also with which they are able to be called as

Help Text with 'TFPCX -H' and then individually explained. <load options>

are only relevant with the resident Loading of TFPCX and valid until

unloaded.

<general options>

-N no messages

-T terminal mode

-U unload

<load options>

-P<port>   attach packet port  -D  debug mode

-B<baud>   baud rate           -DM use DRSI messages

-F[file]   read init file      -DR emulate DRSI driver

-C[xx]     show DCD [color]    -DX modified DRSI Interface

-Ixx       TFPCX interrupt     -NB no blinking rectangle

-BU[nnnn]  number of Buffers   -ND no disk access if DCD

-CHnn      number of channels  -NL no Loopback

<port>  COMn[:xxx] | LPTn[:xxx] | KISSn[:xxx:nn] | <scc[:xxx:nn:nnnn]

1-4^  ^addr  1-4^  ^addr   1-4^addr^ ^irq    addr^ IRQ^ ^<clock>

<scc>   OSCC | USCC

<clock> 0 = disable    2 = hardclock    4 = PA0HZP port  (1 digit/

1 = softclock  3 = DF9IC modem  5 = PA0HZP time channel)

<baud>  nnnn[:nnnn ...] (1 number/port)

[]  Issue is optional

|   alternative Issue

x   Hexadecimal Number

n   Decimal Number

9

6.1. General Options

These Options are able to be employed with each other Option.

Here, there is only one in the overview:

-N  Suppress Messages

In the Case where Messages from the Program cause a disturbance. (e.g. in

Batch-Files), they are able to be suppressed by this Command. Error

Messages continue to be shown.

6.2. Options for Resident Loading

TFPCX  will always be loaded resident in RAM, if neither the Option '-T'

or '-U' is issued and has not itself, or TFPCR- or DRSI-TNCTSR been

activated already. If it is required to load more Drivers together, then

TFPCX must be the first one to be called. In this case I cannot guarantee

that it will work Problem free.

Also, TFPCX can be loaded with LOADHIGH into the upper RAM (UMB), if there

is sufficient free RAM. Of course there are partial Problems with the

EMM386-Drivers (see Appendix 2.1.).

In order that TFPCX  is able to interface quickly with various Packet-

Hardware, Terminal Programs and User Requirements, the existing Options

are dealt with as follows.

After Loading, the Report:

�������������������������������������Ŀ

� TFPCX v2.10  (Nov 20 1993) by DG0FT �

�     TF v2.3b DAMA by NORD><LINK     �

�    Free for non-commercial usage    �

�������������������������������������Ĵ

� 5 Port(s), 10 Channels, TFPC-Int FD �

�������������������������������������Ĵ

� 0: COM1 (3F8/00),  1200 Bd, MODEM   �

� 1: SCC0 (300/07),  1200 Bd, SOFTCLK �

� 2: SCC1 (301/07),  1200 Bd, SOFTCLK �

� 3: SCC3 (303/07),  9600 Bd, DF9IC   �

� 4: COM2 (2F8/03),  9600 Bd, KISS    �

���������������������������������������

is displayed with the DOS-Prompt. TFPCX is now installed and occupies

a part of the Main RAM. In the lower box the Number, the Port, the Address

and the IRQ for each Port, the Baud Rate and Mode of the Connected Modems

is given.

6.2.1. Port- and Baud Rate-Configuration

10

-P  Output of the utilised Ports

This Option can used repeatedly and of course twice for serial

Modems, once for the BayCom-SCC-Cards and four times for KISS Ports, if

with this the upper limit of 8 Ports is not overstepped. Especially with

Serial Modems, the Loading of the Computer with more Ports is not to be

underestimated.

The allocation of Port Numbers, follows in the sequence in which the Ports

will be given in the Command Line, whereby the series of the SCC-Ports is

fixed. The above Report is displayed e.g. with the Call:

TFPCX -PCOM1 -PUSCC -PKISS2

If the Option '-P' is not issued in general, no Port will be steered

towards, which is only intended for internal Connects.

The optional possible Port addresses must lie within the range from 0x100

to 0x3F8 and be divisible by 8. With SCC and KISS the IRQs 2-5, 7, 9-12

and 14-15 are possible (With XTs only IRQs smaller than 8).

ATs have no actual IRQ 2. Instead of this therefore, IRQ 9 is employed.

Each Port must have an individual IRQ.

-PCOMn or -PLPTn  Modem on COMn. LPTn (n = 1-4)

The Base Address of the Port will be taken from the BIOS-Data Range and

must be appended here. Most BIOS-Versions forget that with COM3 and COM4.

Alsp in this case it allows the address to be set explicitly in the Form

'-P<Port>:xxx'.

Example:

TFPCX -PCOM3:338

With this Call a Modem on COM3 will be employed, and the Base Address will

be 0x338. This Address must be obtained from the RS232 Description in the

Manual. The Number of the COM-PORT (here 3) will be ignored, if an Address

is given, but must lie between 1 & 4. The IRQ of the COM PORT for TFPCX is

of no interest and will not be used.

If two Modems are to be used then the Port that is most frequently used

should be given first, as this is possibly preferable. It is self evident

that another Program may not be accessed on Ports, which will be used by

TFPCX.

-PUSCC:<Base>:<IRQ>:<Modems>  BayCom-USCC-Card employed

-POSCC:<Base>:<IRQ>:<Modems>  PA0HZP-OptoPcScc-Card employed

As a Parameter the Base Address of the SCC Card, the IRQ and an issue of a

maximum of 4 digits following,that gives Information about the Modes to

the 4 USCC-Ports connected Modems. The following outputs are possible (see

also Appendix 3.2.):

11

0  Disable      Port will not be used (Switched Off)

1  Softclock    Clock for Send and Receive will be internally

created for AFSK-Modems (no Duplex possible)

2  Hardclock    Send Clock will be delivered from the Modem,

Receive Clock will be developed internally

(e.g. G3RUH)

3  DF9IC-Modem  Clock for Send and Receive will be generated by the

Modem, NRZ-Mode

4  PA0HZP-Port  Receive Clock will be developed internally, externally

divided by 32 and the SCC-Controller as Send Clock

delivered again. (Only for OptoPcScc-Card)

5 PA0HZP-Timer  Port will not be employed, but generates a time clock for

timing purposes (Only for OptoPcScc-Card)

The Modem types 1-3 are planned especially for the USCC Card, whereas Type

4 only functions with the OptoPcScc-Card.

Number 5 has a special meaning. TFPCX requires a Clock for the different

internal Timers, which however will not be delivered from the OptoPcScc-

Card. Consequently the System Timer of the PC will be used, which makes

possible, only a very inaccurate Time measurement, which is problematical

with individual Parameters (e.g. TXDELAY and TXTAIL). TFPCX offers the

possibility of an unused SCC-Port for the generation of an accurate Clock

frequency to be used, which is also unquestionably recommended, when all

Ports are required.

Examples:

TFPCX -PUSCC:300:7:1103

Base Address  0x300,  IRQ 7, USCC-Ports 0 and 1 with  Softclock for normal

AFSK-Modems (Character 1), Port 2 is switched off (Character 0) and Port 3

with  DF9IC-Modem (Character 3). This is the Standard-Installation, when

only '-PUSCC' is issued.

TFPCX -PUSCC:300:7:31

USCC-Port 0 with DF9IC Modem, Port 1 with Softclock, Ports 2 and 3

switched off. This installation is required for the 9K6-USCC Card, which

offers only 2 SCC-Ports. If absolutely no Modem-Clock is issued '1103' is

valid (as above), however if an issue follows less than 4 Digits it is

valid for the Rest '0'.

TFPCX -POSCC:150:3:4445

OptoPcScc-Card with Base Address 0x150, IRQ 3, Port 0 to 2 will be

employed as Modem Ports with External Clock, Port 3 develops the Timing

Clock. This is the Standard Installation for '-POSCC' without further

Parameters.

12

-PKISSn:<Base>:<IRQ> KISS-Port on COMn (n = 1-4)

The Base Address will be located automatically, but by choice, can be

issued manually also.. IRQ 4 will be employed as standard for COM1 and COM3,

IRQ 3 for COM2 and COM4. If these values are not correct, the IRQ must be

issued.

Examples:

TFPCX -PKISS1

KISS-Port on COM1, Base Address and IRQ will be located automatically.

In general this declaration is sufficient for COM1 and 2.

TFPCX -PKISS3:338:5

KISS-Port on COM3, Base Address 0x338, IRQ 5

-Bnnnn[:nnnn ...]  Install Baudrate for each Port

With more Ports a ':' separator will be used for the Values, in the Row

following increased Port Numbers. Following Values are possible:

Standard

Serial Modem   300, 1200, 2400 or 4800 Baud            1200

SCC Softclock    50-38400 Baud                        1200

PA0HZP-Port  50-38400 Baud                        1200

Hardclock    50-38400 Baud                        9600

DF9IC-Modem   1-65535 Baud (without significance) 9600

KISS              2400, 4800, 9600, 19200,             9600

38400 or 57600 Baud

With a Serial Modem only the designated Values are possible, with the USCC

Intermediate Values also. With Hardclock, Send Clock must be delivered

from the Modem with the indicated agreement, with the DF9IC-Modem the

Value is meaningless, here the Clock will be generated externally, but in

spite of this the correct Value should be given, because, for example, it

will be indicated with the Command 'P'.

Example:

TFPCX -PCOM1 -PUSCC:::1003 -B300::19200

Modem on COM1 with 300 Baud, USCC-Port 0 with Softclock and 1200  Baud

(Standard, '::') and USCC-Port 3 with DF9IC-Modem and 19200 Baud.

Which Baudrate is possible on a PC, depends on its Computer Speed (see

Table in the Appendix 2.1.). With the operation of a Serial Modem at 300

Baud the System Clock loses a half minute in each Hour.

13

6.2.2. TFPC- and DRSI-Interface

TFPCX offers Communication with three different Variations of Terminal

Programs, from which a selection must always be made. For Programmers the

Call and the Parameter submission is described accurately in the Appendix

4.1.

The TFPC-Interface has been developed by Sigi (DL1MEN) for his KISS-Driver

TFPCR and meanwhile will be supported for a series of Programs (e.g. SP,

GP, THP, TERM, DIEBOX). On this account this is also the Standard-

Interface of TFPCX. The disadvantage of this variation is the reduced

Multi-Port-Capability, because the transfer of Port Numbers (e.g. of the

Port on which one has been connected) is not possible. With it Connects

and Monitor-Frames are shown, as if all are delivered on the same

Frequency. Previously this was not a Problem, because (also TFPCX) in any

case only one Port was able to be controlled. For those who only use one

Port, this Variation is also recommended in the future.

-DR  DRSI-Interface usage.

This Interface, used for the DRSI-PCPA-Adapter belongs to the TNCTSR-

Driver. Now TFPCX undertakes the Multi-Port-Operation a possibility must

be found, so as to enable a differentiation of the employed Ports via the

Terminal Program. For this purpose this Variation proffers that it already

fully supports SP and FBB.  The difference to the TFPC-Interface lies

mainly in the transfer of Port Numbers with Link-Status-Reports and

Monitor-Information. This Option causes no modest Hardware Support of the

DRSI-PCPA-Adapters.

-DX Modifies DRSI-Interface operation

This Option corresponds to the Option '-DR' of TFPCX v2.0x. There it gave

incompatibilities with the TNCTSR-Driver with employment of the DRSI-

Interface, which with this Version has been eliminated. In the case where

this alteration with other Programs (e.g. TOP) now causes Problems, the

Option '-DX' should be issued instead of '-DR'

-DM  Modifies TFPC-Interface operation

This variation is conceived for Multi-Port-Operation with Programs, which

earlier were able only to use the TFPC-Interface. It delivers the same

Reports as the DRSI-Interface (also with Port Numbers) but employs the

customary TFPC-Interrupt. The Usage of this Variation is a compromise.

With many Programs (e.g. TERM) it gives no Problems, other Terminal

Programs (e.g. GP) the Port Numbers mostly show correctly, but it gives

'adjacent effect' and certain Programs (e.g. THP) arrive with the Port

Numbers not very clear. At best, probe once with and once without Option

'-DM' and hope thereon that the developer of the corresponding Program

perhaps realises a Multi-Port-Support.

WARNING!

Please do not overwhelm the developer of the Terminal Program (or me) with

Messages, if with Usage of '-DM' something else occurs randomly. This

Option is a compromise and causes peculiar effects with certain Programs.

14

6.2.3. Other Options

-BU[nnnn] Size of the TFPCX-Buffer

TFPCX dynamically saves most of the Data in 32 Byte-Buffers. The Quantity

of the required Buffers can be very different for various uses. If TFPCX

has only a small Buffer Space, intermediately it can only save a few

Frames and under certain circumstances a 'TNC BUSY' Report is obtained,

which mostly will lead to a Disconnect via the Terminal Program. On the

other hand, too much wastes Buffer Space.

With this option the Buffer size can be configured between 400 and approx.

1500. The maximum value depends upon the employed number of Channels, and

will be installed if the Option is issued without a Parameter.

Usually the Standard Value is adequate at 600. If many Channels are

worked, Gateway-Functions employed, perhaps a Mailbox operated or large

Files sent on Digi-Paths, an increase of this Value is sensible. With SP

(except for V7.50) not more than 999 Buffers should be selected (see

Section 7.1). If there is very little RAM available, then fewer Buffers

can be worked.

-CHnn Number of Connect-Channels

Previously the quantity of TFPCX administrated Channels was installed

fixed, with what RAM Space and Computer Time presented, if this quantity

had not been used. With this Option, the number of Channels that are

required for usage can be adjusted. It should be the same Value as that

selected by the Terminal Program in use. 4 to 40 Channels are possible

(Default 10).

-C[xx]  Send-/Receive Indicator Switching

The Indicator of the Send-/Receive Status appears in the top right hand

corner of the Screen ('S' for Send, 'R' for Receive). When more Ports are

to be used, the Indicator for each Port is separated, whereby Port 0

stands at the extreme left. With KISS it obtains for itself the indication

of the Connection between PC and TNC. The indication only functions in

Text-Mode and can ensue with a definite Colour Attribute that has been

issued as hexadecimal.

Example:

TFPCX -C17

^ Foreground (here White)

^  Background (here Blue)

Numbers of the Colour Attributes:

0 Black    4 Red      8 Dark-Grey   C Light-Red     Monochrome:

1 Blue     5 Magenta  9 Light-Blue  D Light-Magenta

2 Green    6 Brown    A Light-Green E Yellow        07 Normal

3 Cyan     7 White    B Light-Cyan  F Light-White   70 Inverse

^             ^

only foreground

15

-F<File>  Files for the installation of Parameters (without <File>

uses TFPCX.INI)

With the use of this option (and only then!) the file will be read during

the Initialisation and in the Terminal-Mode sent characterwise to the

Firmware, in order to make possible a Pre-Installation of the Parameters.

Normally this Option can be omitted, because the Terminal Programs

themselves, take in hand the initialisation. The File will be sought in

the Current Directory if no Pathname is given.

The File can be created with a normal Editor and can contain Commentary

(preceded by '#' or ';') and empty Characters. An Escape-Character will be

sent automatically before every Command. The previously required '^' is no

longer needed and will be ignored so that old Files can still be used.

Tabs will be handled as Empty Characters and ignored at the beginning and

end of a Line. An Example-File is contained in the Archive.

-Ixx  Software-Interrupt for TFPCX-Interface (40-FF)

The Communication between TFPCX and the Terminal-Program is found via the

Indicated Software-Interrupt. Standard usage will be the Interrupt 0xFD.

An alteration is only important when this Vector is used by other Programs

or a Program only functions with a particular installation (e.g. '-IFF'

with CT (K1EA)and FBB).

-NB  Switch off Status Flash

When TFPCX has stored unread Information or Status Reports and is not in

Hostmode (also runs in the Background) a Square flashes in the upper right

hand corner of the Screen, which for example makes aware the arrival of a

new Connection. The Terminal Program is now able to be started and react

to the Connect. This does not always function if a DOS-Shell has been

called and TFPCX therefore, (as e.g. with SP) remains in Hostmode. The

Flashing can removed with this Option.

-ND  Disk-Access delayed with Send/Receive (Expedient)

Where there are Receive Problems with Disk-Access, (Packets will not be

accurately decoded. i.e. Disk Access during a concurrent Signal (also with

SCC, only in Hostmode)) the access can be delayed with this Option.

However sometimes this leads to a somewhat 'unacceptable' delay, because

the Computer then hangs too long, until the QRG is free again. This Option

should only be used in an Emergency. When used with Soft-DCD the Command

'@C' must have been executed already with the loading of TFPCX using the

Option '-F' from an Init-File.

-NL Internal Connects (Loopback) Switch Off

In Normal cases all sent Frames will be so handled as is they would have

been received, through which internal Connects are for Test Purposes. In

certain cases this is not required (e.g. Tests with external Loopback) and

on this account can be prevented with this Option.

With high Data Increases, in this manner it allows the loading of the

Computer to be lowered somewhat, then the sent Frames must not be doubly

handled.

16

-D  Test Mode (Debug)

This Option is to be found before the '-P'-Option on the Command Line and

activated for the Ports associated with a Test Mode, which with every

Interrupt effects an edge change at the entry of the Loudspeaker and with

it, generates a crack or sound.

The Test Mode serves for all 'Cause' Investigations with Send- and Receive

Problems using Modem Operation (see Appendix 2.1.). With each Timer-

Interrupt it creates an 'Edge' change at the Input of the Loudspeaker.

With 1200 Baud a 1800 Hz-Tone should be heard (Baudrate  *  1.5).  The

Tone should be 'halfway ' clean and without Interruption. Crackling

originates, when the Interrupt is delayed. If a single Crackle sounds or

it is interrupted, the Computer is overtaxed. Here the dividing line is

always difficult to define, a 'certain ground noise' must not damage the

Function.

Using this Option with KISS or a SCC-Card, it can be checked generally

whether Interrupts will be generated. This can be established only in

Send-/Receive case.

If '-D' stands before the first '-P', so it crackles in time with the

System Clock.

6.3. Clear TFPCX from RAM

With the Command 'TFPCX -U' it allows TFPCX to be removed from the RAM.

6.4. Terminal Mode

With  'TFPCX -T' a simple Terminal Program is started, with which it is

able to work without additional Terminal-Software. Previous to this, TFPCX

must be resident already (as described above). The Program must therefore,

be called twice with different Parameters, in order to attain Terminal

Mode.

With ALT-X an exit from the Terminal Program will obtained. Before this if

the Monitor was active, it should be switched off with ESC 'MN', otherwise

unimportant RAM Buffer will be expanded and with the eventual start of a

Hostmode-Program Problems will result.

7. Installation

This Section gives advice for the Installation of a particular Program for

Operation with TFPCX. TFPCX must be activated before the Terminal Program

is started.  Additionally, most permanent installations are achieved by

configuration Files or Menus.  On the contrary, e.g. Baud-Rate

Installations are mostly unimportant in these Files/Menus for TFPCX

operation, for this purpose the Option '-B' with the call of TFPCX is

responsible.

17

With some Programs, problems can arise with Multi-Port-Operation (Option

'-DM'). In this Case, this Option and with it the pointers to the Port

Numbers must be omitted. Nevertheless with Commands the Multi-Port-Option

is possible. One does not see, from whom one is connected or on which Port

a Monitor-Frame has been received.  With these Programs the employment of

the Command '@PO' (see Section 8.2 is beneficial.

One should be aware, that TFPCX occupies a part of the RAM other than that

in which Terminal Program resides.  On this account, under some

circumstances, certain Buffer Sizes and the Number of the Connect-Channels

must be decreased.

7.1. SP (DL1MEN)

After the Port Number 2 variations are always available:

If only one Port operation is required, the TFPCX-Interface undertakes the

support of SP from v5.00. Here SP should be installed correspondingly, SP-

Manually or with the INSTALL-Program for TFPCX (or TFPCR) operation)  The

file CONFIG.SP (previously SP.CFG) must contain the line 'CFG=PORT0:5H'.

In operation there are no differences between TFPCX and TFPCR.

For Multi-Port-Operation the DRSI-Interface should be employed. The file

CONFIG.SP for this must contain the line 'CFG=PORT0:DH' and TFPCX will be

started with the Option '-DR' Otherwise it is required to locate what is

written in the SP-Manual for Installation and for the operation with the

DRSI-PCPA-Adapter, in this case it is true for the greater part of TFPCX,

with the following special features:

- SP-Commands 'DR' and 'HB' do not function, the Parameter will be

installed with the normal TNC-Commands with the additional issue

of Port (see Section 8.1.)

- What is described in Section  8.1. Port Selection with missing Port

declaration, unfortunately does not work well with certain Commands that

SP sends to the Monitor-Channel 0 and not to the actual installed

Connect-Channel, therefore in case of doubt, always issue the Port

Number.

- Commands 'TP' and '//par' show the correct Baud Rate (no Number)

With DRSI-Operation for each one, SP manages the maximum of 8 Ports an

extra QRG, except on the Monitor-Channel. The Port switching follows for

example, by ESC 'C 1:', therefore the Unproto-Port will be selected on the

Monitor-Channel. The installed Default-Port will be completed with a

Connect-Command automatically, but can be issued direct.

TFPCX is able to be employed in parallel with TNCs in parallel. How good

the whole functions, is dependent for everyone, on the Computer in use.

18

With SP6.00 or earlier the QRG-Pointer possibly outflanks and the Connect-

Bell sounds other than with TNC-Operation. That is normal and not a basis

for worry.

If when SP v7.00 (also earlier Versions) is employed, 999 TFPCX-Buffers

(Option 'BU') should be reserved no longer, because SP apparently does not

return with a 4-digit Buffer count and as a rule the Sending of Files runs

very sluggishly. With SP v7.50 this Problem has been rectified.

7.2. GP (DH1DAE)

GP makes use of TFPCX automatically when it is loaded, on account of

which, in normal cases no extra Configuration is required. Certainly with

Multiport-Operation the following Advice should be observed.

- Without question GP from Version 1.50 must be employed. Earlier Versions

are only comfortable when operated with one Port and have Problems with

higher Transfer rates.

- TFPCX should be started with the Option '-DM'.

- In the File NAMES.GP at least one Port-Command is cogently necessary,

because otherwise Connect-Requests will abort with an Error Report.

(see GP.DOC).

Example:

PORT0 = DB0BLN,438.450

PORT1 = DB0BLO,438,300

Now with the help of TFPCX GP also can control more TNCs, which for this

purpose must be undertaken by the KISS-Mode. (see Section 8.3.).

7.3. THP (DL1BHO)

THP supports the TFPC-Interface (tested with v3.03).  For TFPCX the COM

Port Number '5' must be given in the Section 'TNC-Configuration' of the

File CONFIG.PR. In the File CMD.PR certain Commands will be called upon,

which with TFPCX will lead to an Error Report (e.g. 'A' and 'Z'). It is

recommended  that a '#' is set at the front (Commentary).

Multi-Port-Operation with the Option '-DM' gains nothing, in as much as

the Port Numbers will not be displayed correctly.

7.4. TERM (DL5FBD)

This TFPCX-Version works best together with TERM from v9.71. Earlier TERM-

Versions employed other Procedures for the Send Handshake, which will no

longer be supported by TFPCX.

If more Ports are required, TFPCX should be loaded with the Option '-DM',

otherwise without. Although TERM does not itself produce TFPCX-Reports, it

does not give Problems with the Port Numbers.

19

After the start of TERM the Parameter-Menu is accessed with ALT-P.

Here will be installed:

V24-COMx:     5

Handshake:    S

Protect Time: 0

With this Version there it is now possible to receive #BIN#-Files.

Proceed as follows (tested with TERM v9.98):

- with the Commands '@M1' and 'E0' switch on the Transparent-Mode and

switch off the Character Echo.

- start the BIN-Receive with ALT-U and answer the Requests (Read Command

last.

- after the Transfer set up the previous state again with the Commands

'E1' and '@M0'.

Further Advice is to be found in the TERM-Documentation.

7.5. CT (K1EA)

Now the Contest-Program from K1EA can operate with TFPCX (tested with CT

v6.26). Of course up to now CW-Contests only operated with KISS or an SCC-

Card, CT with the production of Morse Characters, otherwise it is not

tolerated with TFPCX.

The start of TFPCX must follow with the Options '-DR' and '-IFF'. The

Parameter installation is permissable with the: Option '-F'.

In the first Dialogue Box, at the Item 'TNC' the Value 'Local' should be

installed and must be selected for Modem Operation with 'Mode' 'SSB'. In

the Communications Setup no adjustment will be undertaken (CT may not

access on the Modem-Port). On the  QSO-Screen the Command 'DRSI' will be

given first, with ALT-T the TALK Mode is chosen and one press of ENTER.

Then it is able to Connect to the DX-Cluster and to observe the DX-Info

with ALT-A in the opened ANNOUNCE-Window. The rest must be 'self'

investigated. At best, for this purpose install Single Unlimited, as one

does as a Solo Operator to overcome limitations.

7.6. DIEBOX (DF3AV)

DIEBOX can be employed via the TFPC-Interface with TFPCX.  With a simple

Modem it will operate a Box, but the use of a SCC-Card is certainly most

desirable. For the direct Coupling of a Box to a Digipeater (e.g. RMNC)

the KISS-Mode can be used and with it a TNC be saved.

Here TFPCX must be started without '-DR', whether or not '-DM' works well,

must be self tested. It is sensible to increase the number of the Channels

(Option '-CH' and Buffer (Option '-BU') For the DIEBOX-Configuration I

cannot make any declarations, that I have not tested myself.

20

7.7. WINPR (DG6BI)

WINPR can be used from TFPCX Version 2.0, as it is supported by the TFPCX-

Interface. Although WINPR runs under Microsoft Windows it functions only

with KISS or a SCC-Card (see Section 7.10.) and only on the faster

Computers. The Multiport-Operation of WINPR will not be supported. Also

the Option '-DM' should not be used.

TFPCX should be loaded before the start of Windows, or through an entry in

the File WINSTART.BAT. Windows must run in the 386 Enhanced Mode. In the

File WINPR.INI the following entries are required:

Port = Com5

Tfpcring =253

7.8 TOP (DF8MT)

TFPCX also enters the TOP-Documentation, so that here I mention only New

items. For the Multiport-Operation TFPCX must now be started by '-DX'

instead of '-DR'

Under TOP the different TFPCX-Ports are placed as separated TNCs, whereby

one after another the Channels are ordered exactly to a Port. On this

account with the Connect-Command no Port Number must be issued. Previously

this Illusion certainly has been muddied through circumstances, that

Connects from outside always arrived on the lowest free Channel with

matching MYCALL, dependent on the selected assignment in TOP.  With the

Command '@PO' (see Section 8.2.) now it allows this to be circumvented.

Example:

TFPCX is to administer 2 Ports and 20 Channels, whereby for each Port 10

Channels are planned for each Port respectively. In addition to the TNC-

Configuration, the following INI-Command should be entered in TOPSET:

@PO 00000000001111111111

Furthermore TFPCX is called with the Option '-CH20', because the standard

stands at only 10 Channels.

7.9  FBB (F6FBB)

The F6FBB-BBS-Software supports both TFPC- (from v5.15) as also the DRSI-

Interface. Here I explain only the DRSI-Multiport-Operation, because this

is the general case. For the Modem Operation FBB from v5.15 is absolutely

necessary.

TFPCX should be loaded with the Options '-DR' and '-IFF'. In the case

where more than 10 Channel Operation is required, '-CH' is also to be

issued (see Section 6.2.3.). The Configuration File PORT.SYS for

Operation with 2 Ports must appear as follows:

21

# PORT.SYS for FBB 5.15

#

#Ports TNCs

1     2

#

#Com Interface Address (Hex0) Baud

7   4         0              4800

#

#TNC NbCh Com MultCh Pacln Maxfr NbFwd MxBloc M/P-Fwd Mode Freq

1   5    7   0      230   2     1     10     00/60   UDYW 144.650

2   5    7   1      230   2     1     10     00/60   UDYW 438.458

TFPCX simulates 2 TNCs via a virtual COM-Port (COM7). Interface 4 stands

for DRSI, Address and Baud will be ignored, but must be given. The sum of

the Ports ordered to the Channels (NbCh) may not be larger than the number

of TFPCX-Channels on hand (Option '-CH').  At any instant there will be

employed only one File INITNC1.SYS, MAINT1.SYS respectively, in which all

of the required Parameters are set individually with Port declarations

(see Section 8.1). The 'P'-Command with TFPCX is not usable for the

simultaneous installation of all Parameters of a Port, as is possible with

the DRSI-TNCKISS-Driver. Further Information is to be extracted from the

FBB-Documentation.

7.10. MS-Windows, OS/2 and others.

TFPCX with Modem Operation places higher demands on the reaction time of

the System for Interrupts (see Appendix 2.), which will not be filled by

Windows 3.x and IBM OS/2 2.0 (also when it falls too heavy to be accepted)

On these grounds TFPCX reacts with an Error Report, when a start is

attempted with Modem Operation is this System.

With a SCC-Card an Operation is possible. Under Windows, this has

functioned very well with 1200 Baud. Certainly it gives Problems with

higher Baud Rates with SCC-Cards. Under OS/2 TFPCX delivers a little

better, indeed I have only tested it minimally.

The KISS-Mode functions for me up to 38400 Baud under both Systems without

Problems, so that with the employed COM-Port of Windows and OS/2 will be

supported optimally by Driver, which unfortunately with the SCC-Cards

(understandably) is not the case.

With other Multi-Tasking-Systems similar circumstances may prevail.

8. Operation

In this Section  important differences and developments of TFPCX in

comparison with TNC-Firmware TF 2.3b. will be explained

8.1. Multi Port-Developments

The most significant development is certainly the possibility of more

Ports and with them, to control Modems and Radios. If only one Port is

required, do not bother with this but turn to the next Section.

22

Internally, TFPCX-Firmware manages 8 Ports, seldom will all be employed.

On the unused Ports nevertheless, internal Connects can be run. So one can

e.g. incidentally evaluate the CTEXT, without bothering others on the

Digi-Entry (distributed Time spread)

Each Port has a Number assigned to it (0-7), whereby the sequence

following, depends on the usage of the Option '-P' at the Start of TFPCX.

This Port Number will be displayed in the Monitor by TFPCX with Link-

Status-Reports, and with certain Commands ('C', 'L' 'S'), so that an

unequivocal association is possible. Intimation only follows if, with the

Start of TFPCX the die Option '-DR' or '-DM' is given.

Example:

1:fm DG0FT to DB0BLN ctl SABM+

1:fm DB0BLN o DG0FT ctl UA-

CONNECTED to 1:DB0BLN

The precise Format of the Reports is to be found in Appendix 4.2. The

place on the Screen where the Port Number is displayed, depends on the

type of Report and on the Terminal Program.

With certain Firmware-Commands (see Section 8.2. and Appendix 1.) the

possibility to install specific Parameters, or to Connect to a Station on

a chosen Port, is dependent upon the issue of a Port Number.

Command Format:

<Command> <Port>:[Parameter]

<Port> is a Numeral  between 0 und 7, no space may stand before the

':'.

Example:

C 1:DB0BLN

T 2:15

If a Port has not been given, the Command for this input requires either

the Port employed, on which the immediate active Channel will be connected

or the Monitor-Channel on which the Unproto-Port is installed. It provides

orderliness for each Host-Mode-Terminal-Program, for which most Commands

are not sent to the Channel they have been given and to arrange the Port;

consequently in the second case, always give the Port Number.

The Parameter installation stems mostly from the Configuration Files of

the Terminal Program and must occur individually for each Port. The File

TFPCX.INI is an example of this and can be employed once Personal Data has

been installed, also to be employed directly is the Installation (Option

'-F', Section 6.2.3.).

Incoming Connects will be independent of the Port, always placed on the

next free Channel, on which the appropriate MYCALL has been installed.

With the Command '@PO' (see Section 8.2.) a fixed Port Ordering by choice,

can be considered.

23

8.2. Characteristics of Commands

There are a sequence of Commands, that are initiated with the ESC-Key and

executed with <ENTER> (see Appendix 1.).

The Commands  'A', 'H', 'K', 'Z', '@F', and of TF  2.3b do not exist.

The Parameters 'B', 'O', 'P', 'T', 'W', 'X', '@C', '@D', '@T2',

'@T4' and '@TA' should be installed separately for each Port.

The internal Timer only works with an accuracy of +/- 20ms respectively;

60ms, if only KISS or the OptoPcScc-Card without Timer-Port is used, which

with the installation of individual Parameters can be important. With

TXTAIL (Command '@TA') this inaccuracy will be considered.

The individual Commands:

C  Connect

When Multi-Connects occur with the same Station the individual SSID will

be  raised automatically to a maximum of 15, if they are already

installed. Of course, possibly the Terminal Program remains hidden and it

shows a false SSID.

The C-Command can contain an additional Port declaration, if this is not

so, then the Link-List (see @L) will be used, or the Unproto-Port used in

the case where no appropriate entry exists. If with the Command '@PO' a

Port Ordering has been considered, then the Channel ordered Port as thr

Default-Port is valid. On the Monitor-Channel, with the Port

Identification the Unproto-Port will be selected on which the UI-Frames

will be sent (Default 0).

Examples:

C 1:DB0BLN                  DB0BLN Connect on Port 1

C 2:TEST \ Monitor-Channel  Unproto-Port 2 and Unproto-ID set

C 2:     /                  only Unproto-Port 2 set

F  Frack

The Frack-Parameter can be issued in Seconds or alternatively in 10ms-

Units.  Values smaller than 16 will be converted into the new Unit (10ms).

O  MAXFRAME

MAXFRAME  will be managed individually for each Port and for each

Connection. With a Connect, the  Value of the respective Port of the

Connection will be assigned and can be independently installed for this

Connect (from other Connects on the same Port).

The 'dynamic' MAXFRAME of TF 2.3b has been removed.

P  P-Persistence

With DAMA-Operation the non-DAMA-Value will be displayed, but P=255

employed .

24

If the indicated Parameter lies between 0 und 7 (Port Number) the P-Value

will not be set anew but certain Parameters of this Port displayed in the

Form

<Port> R P W F O N @T2 @T3 T <Baud> @D

On compatibility grounds this Parameter request must be constructed with

the DRSI-TNCTSR-Driver, as it would be employed with SP. <Baud> is the

installed Baudrate in 'Clear text', not as with TNCTSR, only a co-

ordinated numeral.

The DRSI-Driver also employs this Command to set the Parameter, but does

not function here. If additional values are given behind the Port number.

TFPCX ignores the entire Command.

QRES  Reset

QRES  simply resets TFPCX back into the Terminal Mode (as 'JHOST0')  and

does not RESET the Computer. This Command was important, so that SP

generated it itself with the DRSI-Command 'HB'.

U  Unattended Mode (CTEXT)

This Command is important for all in the Terminal-Mode. With 'U0' Link-

Status-Reports will be issued immediately, also when just one other

Channel is active. With 'U1' this Report first appears, when the

corresponding Channel is activated with the 'S'-Command and with a Connect

from outside an additional offered Text is sent.

The Standard-Installation 'U2' behaves as 'U1', but additionally it

enables a Disconnect by the Connected Station using the Remote-Command

'//Q' (or '//q). For this purpose the Command must stand at the beginning

of a Frame.

The Unattended Mode can be switched on, if no CTEXT is defined. In the

Host-Mode this Command is procured only with CTEXT.

@C  DCD-Working

TFPCX has a Soft-DCD (programmable Noise Barrier). With slow Radios it is

able to allow the Squelch of the Receiver to be fully open and TFPCX

itself, decides if a PR-Signal is being received.

The Soft-DCD is controlled by the Command '@C', to which the appropriate

value is attached. The Parameter will be given a number from 0 to 63 With

'@C0' the Soft-DCD is switched off (Default), with all other values it is

activated. The larger the Parameter is, the stronger the Soft-DCD is

created. The DCD-Indicator serves for balancing (see Option '-C'). With

too small a Value the DCD-Indicator flickers, with too large a value the

Signal will no longer be correct and recognised too slowly. For the best

results the Parameter should be raised progressively, and nearby QRG

orally heard up to the Indicator being correct. Possibly with this a

compromise must be found. A reasonable value is '@C25'.

25

With SCC-Ports a balance is not definitely required, all Values larger

than 0 activate the Soft-DCD and are equivalent for the Operation. The

constant flicker of the Indicator disturbed me and so here the possibility

to undertake a balance exists, which is obtained only on the indicator.

IMPORTANT!

The Soft-DCD only recognises PR-Signals at the same Baud Rate. On this

account when Digipeating with other Baud rates, the Soft-DCD may not be

able to be operated.

With KISS the TNC undertakes the DCD-Handling and this Command defines

only the Time (in 10ms-Units), after which the RX display expires, if no

further Byte has been received from the TNC.

@L  Link List

The Internal Link-List will be managed with this Command, which it is able

to coordinate a maximum of 8 Call Signs (with SSID) for a determined Port.

The List will be used with the Digipeating (Command 'R'), so as to

ascertain the Port on which the Digipeating Frame should be sent. If there

is no appropriate Entry on hand, the Frame will be sent out on the same

Port on which it has been received.

Syntax:

@L <Port>:<Call> <Port>:<Call> ...

Example:

@L 0:DB0BLN 1:DB0BLO

With '@L-' the Link List will be cleared and '@L' without Parameters

displays the Entries.

With Crossband-Digipeating both the Target Call and the Call of the Sender

must be brought into the Link List, so that the Connection functions only

on a route.

@M Transparent-Mode

Normally in the Terminal-Mode received Control-Characters will be

converted (e.g. Control-Z will be displayed as '^Z'). Through this the

Data will be falsified, which makes the Receiving of Binary-Files

impossible.

With the Command '@M1' a Mode will be activated, by which all Data without

alteration (transparent) will be passed to the Terminal-Program without

alteration, with this Binary-receive is possible.

On the contrary to the Original-TF 2.4 this Command does not act on the

7/8-Bit-Conversion. TFPCX always employs the 8 Bit Character Set.

26

@PO

With this Command it can be fixed for each Channel, whether it can be

connected from all, or one certain Port or from no Port. If a particular

Channel has been allotted a certain Port, this is valid also as the

Default-Port with the 'C'-Command

A Character Chain will be passed over, whereby the 1. Character belongs to

Channel 1, the 2. Character to Channel 2 etc. In normal cases with 10

Channels, the Character Chain consists of 10 Characters. If less

Characters are issued, the ordering of the remaining Channels is

unaltered, if the Chain is longer, the overflowing Characters will be

ignored. The following Characters are Possible:

'0' to '7' Connect only from one Port possible (Port Number0

'*'        Connect from all Ports possible

'-'        No Connect from outside possible

Examples:

@PO 0000011111*****-----

The Channels 1-5 can be connected from Port 0 only, the Channels 6-18 only

from Port 1, the Channels 11-15 are attainable from all Ports and the

Channels 16-20 generally cannot be connected and will be held free for

Connects to outside.

@PO **********

All Channels can be connected from all Ports. This is the Standard

Installation and is compatible with earlier TFPCX Versions.

Arriving Connects will be assigned to the lowest free Channel, on which

the appropriate MYCALL is installed and, has been ordered either from

which the Connect arrives or from all Ports that can accept Connects

('*'). If there is no appropriate Channel, the Connect will be denied

(BUSY).

@ST  Statistics

With '@ST <Port>:' certain Statistical Values will be issued for the

indicated Port. The valid Values for all Connects on this Port. Please be

aware that the number after the Value 65535 will be reset again to 0.

27

Example:

0 SCC0 TX 87 11 10 RX 547 201 201 ERR 1

^ ^       ^  ^  ^     ^   ^   ^       ^

1)2)      3) 4) 5)    6)  7)  8)      9)

1) Portnumber

2) COM Port ('NULL', if internal Port)

3) Collective number of Frames sent

4) I-Frames sent

5) Acknowledged I-Frames ((4-5) have been lost)

6) Collective number of Frames Received

7) Received I-Frames

8) Effective I-Frames ((7-8) = REJects)

9) occurring Errors (will be displayed only by SCC and KISS and then only

if it is not 0)

With these Values, simple statistical statements are possible. To the Port

designation 2) with KISS  a '+' will be appended, when SMACK is activated.

Value 9) covers a SCC-Port on the Over- and Underruns of the SCC-

Controllers, which always occur if Interrupts do not react quickly enough.

With KISS Character losses, KISS-Frame- and CRC-Errors (with SMACK) leads

to the increase of this Value. If the Error Quantity becomes quickly

larger, the Computer is to slow for the employed Baudrate or with KISS the

Connection to the TNC is not in order.

With '@ST <Port>:-' the Values will be cleared.

@T4  T2 with DAMA

This Command issues the T2-Start Value for DAMA-Operation and determines

the Time that will be awaited before a received Frame will be

Acknowledged.

@TA  TXTAIL

TFPCX can be installed in 10ms-Units, but in normal cases by correction

of Baudrate and Timer- inaccuracies is set optimally (@TA=4 with 300 Baud,

@TA=1 otherwise). With KISS the correct TXTAIL depends on the TNC, on this

account no automatic installation follows here.

@U  Unproto-Poll

With this, it will be established whether Unproto-Frames will be sent with

(Default) or without a fixed Poll-Bit.

28

8.3. KISS

Certain specialties with KISS-Operation will be given at this point.

When TFPCX is started, a usable TNC must be switched on already and be in

the KISS-Mode. TFPCX itself offers no possibility for the activation of

KISS. Here, the available Command @K of TFPCR does not exist. To switch on

the KISS-Mode the Program KISSINIT can be used.

TFPCX supports the KISS-Development SMACK (Version 1.0), which improves

the security for transfers. SMACK will be activated automatically, when

the connected apparatus allows it. With the Command @ST (see Section 8.2)

it permits checking, on which Mode is to be worked (display by SMACK e.g.

'COM1+'). SMACK is first activated when at least 1 Frame is sent and has

been received.

With KISS the Send-/Receive indicator does not show on the actual transfer

Channel, but on the Serial Port to the TNC respective coupled Computer.

For the coupling of a PC to another Computer (e.g. Digipeater and Mailbox)

a null-Modem-Cable is required. TFPCX always works Duplex (Command @D has

no meaning). The Parameters are to be installed correspondingly. The

following executions are important for all normal TNC-Operation.

In KISS-Mode TFPCX has no direct Control over the Transfer Channel,

because the TNC is switched in between. TFPCX has no possibility to

detect, whether the frequency is instantaneously free and when the Frames

to be sent have actually been transferred, which can lead to unwanted

transmissions. For this two cases are especially interesting.

- TFPCX has transferred one Frame  to the TNC and waits for the

acknowledgement from the other Station. In the case at this point, the

Channel has occupied longer time through another Station (e.g. Digi),

the TNC cannot send the Frame and TFPCX accepts after a certain Time

that the Frame has been lost and repeats it, although it has not yet

been sent. The outcome, is the unwanted double sending of the Frame.

The Time between the sending out of a Frame and the Receiving of the

Acknowledgement will be measured by TFPCX and ascertained from this, how

long on average, the Answer must be awaited. With that, it adjusts for

itself the time of the Channel attachment, and in such a manner Doubling

should very seldom occur.  The delay can be influenced by choice

through the alteration of the Parameter @A3.

- If more Frames are received one behind the other, these can be

acknowledged normally by one individual Frame. Thereby the question

arises when TFPCX can assume that no further Frame follows and the

Acknowledgement can be generated. With a Serial Modem or a SCC-Card

simply will be awaited, up until the Opposite Station has ended its

transmission and the transfer Channel is Free.

With KISS this foregoing is not possible. Here the correct installation

of the Parameter @T2 is especially important. It determines the Time,

which after the receiving of a Frame should be waited, before an

Acknowledgement Frame is created. If before the run out of this delay

29

a further Frame arrives, the time measurement begins again. The Timer

should run out first, if no more Frame follows, through which all

received Frames will be acknowledged together. If @T2 is installed too

short, unrequired additional Acknowledgements will be sent.

Baud    @T2

1200    250

2400    150 (Default)

4800    100

9600     50

These issues are valid for Frames with 256 Data-Bytes and are chosen

very generously. Anyone who wishes to experiment with the Values, should

observe in the Monitor, whether with the receipt of one after another

following Frames, each Frame is acknowledged with an individual RR-

FRame. In the case where it very seldom occurs, this is also acceptable.

IMPORTANT

The Default-Value @T2=150 is too short for 1200 Baud operation.

Unquestionably a larger delay should be installed here.

With DAMA instead of @T2, the Parameter @T4 will be employed. The

Standard Installation @T4=1 is not recommended for KISS. For DAMA

the known Standard Value is also valid.

8.4. DAMA

As soon as a Connection is established to a DAMA-Master (Digipeater),

TFPCX will send only when it receives a Frame from the Master, then of

course all contiguous Frames on all Channels. In this case the DAMA-slave

will be activated only for the Port on which the Connection to the Master

is established, the Connects that are running on the other Ports work

completely normally without DAMA.

It is not important to install special Parameters for DAMA.  alternative

Problem free Operation is possible. With the 'B'- Command it is able to

ascertain if DAMA is switched on (Value in  Brackets larger than 0). The

Frames received from the DAMA-Master (not the self sent ones) are

contained in the Monitor with the addition of '[DAMA]'.

With this Version an alteration in the DAMA-Slave follows, through which

the known bleating report from TheNetNode-Digis with Multi-Connect, can

now hardly appear, however I have not been able to test this.

30

Appendices

1. Overview of Commands

Command        Parameter       Description

------        ---------       ------------

* B (120)       0             DAMA-switch on blocked

1-600          DAMA-Timeout-Time (Seconds)

* C            Call ...       Connect-Path (Channel 0: unproto)

D                           Disconnect

E(1)          0             No Echo for issued Characters

1             Echo for issued Characters

F (250)      1-15           Start Value for T1 (Seconds)

16-65535        Start Value for SRTT (10ms)

G            [0]            Obtain Information in Hostmode

[1]            Obtain Status in Hostmode

I            Call           Individual Call Sign

JHOST (0)     0             Terminal-Mode switched on

1             Hostmode switched on

L           [0-10]          Status Indicator for the Channels

M (N)       NIUSC+-         Monitor-Operation Type

N (20)       0-127          Number of Tries (0 = unlimited)

* O (2)        1-7            Number of the Unacknowledged Packets

* P (32)       0-7            Parameter request for issued Port

8-255          P-Persistence Value for non-DAMA

QRES                        Reset into Terminal-Mode

R (1)         0             Digipeater switched off

1             Digipeater switched on

S (0)        0-10           Channel-Number (0 = unproto)

* T (25)       0-127          Wait time from PTT until Data (10ms)

U (2)        0 [Text]       Suppress Connect text

1 [Text]       Connect Text active

2 [Text]       Connect Text and Remote-//Quit active

V (2)         1             Version 1 Protocol

2             Version 2 Protocol

31

* W (10)       0-127          Time slot for P-Persistence (10ms)

* X (1)         0             PTT for Send suppressed

1             PTT for Send released

Y (10)       0-10           Maximum number of Connections

@A1 (7)      0-65535        SRTT-Smoothing, if RTT increases

(SRTT'=(A1*SRTT+RTT)/(A1+1))

@A2 (15)     0-65535        SRTT-Smoothing, if RTT falls

(SRTT'=(A2*SRTT+RTT)/(A2+1))

@A3 (3)      2-16           Factor for T1 (T1=A3*SRTT)

@B                          Shows Quantity of free Buffer

* @C (0)        0             Software-DCD off

1-63           Threshold Value for Software-DCD

* @D (0)        0             Full duplex switched off

1             Full duplex switched on

@I (80)       0             IPOLL off

1-256          maximum Length of an IPOLL-Frame

@L           Port:Call ...  Linklist issue (max. 8 Entries)

'-'            Linklist clear

@M (0)        0             Conversion of Received Control Characters

1             Transparent-Mode for Receive

@PO ('*')    cccccccccc     Port Ordering (each Channel one Character)

c = '0'-'7'    Connect only possible from Port c

'*'        Connect from all Ports possible

'-'        No Connect from outside possible

@S                          Immediate Link-Status

* @ST                         Status indicator each Port

'-'            Status number reset

* @T2 (150)    0-65535        Timer T2 (10ms)

@T3 (30000)  0-65535        Timer T3 (10ms)

* @T4 (1)      0-65535        Timer T2 bei DAMA (10ms)

* @TA (1/4)    0-65535        Time from Frame end until PTT off (10ms)

@U  (1)       0             Unproto-Frames without Poll

1             Unproto-Frames with Poll

@V  (0)       0             Call Sign Check switched off

1             Call Sign Check switched on

32

*   Parameter <Port>: possible

[]  optional Parameter

()  Standard installation

... additional Parameters possible

2. Error Remedies (Modem Operation)

With a Problem the first thing that should be done is to assess where it

could lie. Also besides TFPCX, the Terminal Program, the SCC-Card or the

HF-Technique come into question.

This Section is valid for everything concerned with Modem Operation.

TFPCX makes higher demands than a normal TNC on the Computer employed. If

the Computer does not fulfill the requirements then Problems will occur.

For comprehension I must explain how TFPCX works with Receive and Send.

With Packet Radio the Data is transferred serially and synchronously.

Normally the Serial COM-Port is able to process Data asynchronously with

Start- and Stop bits, with Packet Radio this is not the case. Thereby the

COM-Port also is not employed in the usual and suitable state.

Consequently, TFPCX itself must be concerned with every individual Bit,

the serial COM-Port will only be used as a simple Latch so as to buffer a

Bit at any given time.

Consequently TFPCX can process in the pre-determined Time Frame, of e.g.

1200 Bit/sec. required as an accurate Time Norm. With Send, 1200 cycles/s

is administered for this purpose; but for the Receive, the method used

is required to be 3600 cycles/s, consequently a constant synchronisation

is possible with the received Signal. For this purpose TFPCX employs a

programmable PLL. The Modem-RX-Data Control will be tested 3 times in each

Bit time to ascertain whether an edge in the Signal appears. In the ideal

case an edge may only appear with every 3rd test, but through deviations

in the Signal, from time to time this Frame is displaced. The direction in

which the Signal runs out of the Frame can be discovered and smoothed

through the triple Test.

I have utilised as a Timer supplier, the Timer-IC that is contained in

every PC, and is used as a timer and is also responsible for the Clock

Time. The Timer releases an Interrupt 3600 times/s (Dependent upon the

Programs running), which drives the calling of a particular Routine that

is responsible for Receive and Send. Obviously this Routine is able to

work correctly, only if it is also called permanently in the determined

Frame without excessive delays.

If TFPCX is compared with a connected TNC, using a similar Baudrate then

it yields something like a 30 fold loading. This  makes a Baudrate of

28800 between TNC and Computer with a 1200 Baud Modem Operation, many PCs

already have Problems with this. It is on this account whether a TNC or

TFPCX is used.

33

2.1. Send- and Receive Problems

In the first instance the Computer used must be in a position to supply

the required number of Interrupts. With overloading it retards the system

extremely or stops it completely. From practical experience it is possible

to give the following table (without Guarantee):

PC    XT   XT   286  386

MHz   5    8    12   20

Baud

300  *    *    *    *

1200  ?    ?    *    *

2400  -    ?    *    *

4800  -    -    ?    *

*  Operation possible

?  Operation probably possible (with Limitations)

-  Operation impossible

Again there are always Problems with Computers that are actually fast

enough. They themselves are able to cause an apparent doubtful Receive

situation that creates frequent successive REJects, leading to the

continuous repeating of Frames. The origins for it are mostly certain

resident Programs, Drivers and Hardware expansions that are able to hinder

Interrupts for a longer Time. If this occurs only once during a Frame

receive for approximately 200�s, then this Packet will be lost. In the

case where this kind of Problem occurs, TFPCX should be started with the

Option '-D'. Interruption of the audible Tones during the Operation,

indicates that it points to this known Aspect.

Sources of frequent Problems:

- Usage of the Extended (XMS) or  Expanded  Memory (EMS) as the Buffer for

the Terminal Program (e.g. SP, GP) and Disk-Caches (for all on 286)

As a remedy one must prevent that this RAM will be accessed, so long as

TFPCX is being used. With SP, SPO.EXE and the XMS-Swapping ('SWP=1' in

CONFIG.SP) should not be used. GP should be started with the Option

'/NOXMS'. Nevertheless possibly the Disk-Cache can be used if the Option

'-ND' is issued (see Section 6.2.3.).

- Driver, which enables the LOAD HIGH of Resident Programs. (e.g. EMM386)

It is not sufficient for the principle of the foregoing to be born in

mind, possibly use of the EMM386-Driver must be completely dispensed

with as long as TFPCX is in use.

- Slow Keyboard-Driver (KEYB)

If Frames are always lost when a Key is pressed then another Keyboard

Driver should be tried. (e.g. CKEYGR.COM from the SP-Disk)

34

- VGA-Cards and HD-Controller

Most VGA-Cards spread out a longer time for Interrupts for everything in

Graphic-Mode (GP). It has been reported to me that the HD-Controller

causes the same Problems, if the Controller is removed everything runs

impeccably. Here it is difficult to offer a practical solution. Perhaps

a terminal Program should be sought that does not employ Graphics. With

Disk access problems the Option '-ND' should be used, which is of course

accommodating (see Section 6.2.3.).

Mostly, certain compromises are required in order to be able to work with

TFPCX, if you are not willing to accept these, then the use of TFPCX must

be relinquished.  If TFPCX suddenly fails to work, thought should be given

as to whether or not a new Driver has been loaded or something else has

been altered. What I am not willing to accept in all cases, is that

everyone with TFPCX unnecessarily loads Digi-Entrance, because it hears

each Frame multiplied by 3.

2.2. Problems with other Programs

During the time that TFPCX is active no Program may be called, which

itself upsets the Timer used by TFPCX. If this is not able to be done, the

System will run slow, or the System Clock will become incorrect. Included

among these Programs are e.g.:

- MS-Word 5.0 and 5.5

- EDIT in MS-DOS 5.0

- MS-Windows

- many Mouse Drivers

Also Problems arise with the following Programs, whose exact cause is

unknown:

- Keyboard Driver from DR-DOS 6.0 (Keyboard hangs), other Drivers (e.g.

CKEYGR.COM, which has been driven with SP) employed.

- Microsoft Mouse Driver (MOUSE.COM), Remedy through other Drivers.

- IBM VCPI.SYS-Driver (used with Notebooks), removal of the Driver clears

the Problem

2.3. Hardware Problems

Certain PCs (for all Laptops), which do not have fully compatible Serial

COM-PORTS. Although here the demands are not so high as with BayCom, it

can give greater aberrations with TFPCX on such Computers. Frequently of

course, only the Receive fails, the SEND does not. Previously I have heard

of this with the following Computers:

- Toshiba 1000XE

- NEC Multispeed

- Olivetti M24

35

With TFPCX there is the possibility to Connect the Modem to the LPT-Port,

which is thought of as a way out of the problem.

Laptops and Notebooks mostly support different Power Saving Modes, which

TFPCX after a certain time without a Keypress extracts the required

frequency performance. With these Computers (e.g. Olivetti Quaderndo) it

is often required to deactivate the Power Management (especially the

reduction of the Processor Speed).

3. Hardware Connections

3.1. Serial Modems

BayCom-Compatible Modems can be used without alteration.  In rare Cases

Problems are caused by the instability of the Supply Voltage as is similar

with BayCom. Here the TXD-Control remains steady on approximately +12V

during which BayCom runs a Cyclic Signal on this Control. For this reason

if the supplied voltage to the Modem rises a little higher, then the

Voltage on Pin 7 of the TCM3105 diverges away from the Ideal Voltage

Value. In this case a re-adjustment of the voltage on Pin 7 is required

(see Modem-Documentation).

Additionally there is the possibility to connect a Modem (e.g. from

DigiCom) to a LPT-Port. With this 6 Data Controls will be switched

statically on approximately 5V, which possibly can be employed as the

Supply Voltage to the Modem. (Utilisation with your own responsibility)

Here are the Connections for the Modem-Ports

COM-Port

Signal   25pol.  9pol.  Meaning

DTR      20      4      Send data +/- 12V

RTS      4       7      PTT, High active, -12V=RX, +12V=TX

CTS      5       8      Receive Data

GND      7       5      Earth

TXD      2       3      +12V for BayCom-Modem

LPT-Port

Signal   25pol.         Meaning

DATA1-6  2-7            static 5V approx. for Modem

DATA7    8              Send data, TTL-Level

DATA8    9              PTT, High active, 0V=RX, 5V=TX

BUSY     11             Receive Data

GND      18-25          Earth

Also Modems with the AM7911 are able to be used. For this purpose the

TXTAIL-Parameter (Command @TA) must be increased a little. At this point I

advise, that for other Baud rates, different Modems are required. Also

perhaps only small alterations are required. (Crystal change)

36

3.2. BayCom-USCC-Card

The installation of the USCC-Card must be undertaken with the

corresponding Documentation. Here only the Numbering of the Ports and the

Standard Installation for Modem-Clock-Supply and Baudrate will be

presented:

Port  SCC  Modem-Clock    Baud  Modem

SCC0  1A   Softclock      1200  AFSK (TCM3105)

SCC1  1B   Softclock      1200  AFSK (AM7911)

SCC2  2A   Disable        9600  External

SCC3  2B   DF9IC-Modem    9600  FSK  (DF9IC)

Unconditionally the second SCC-Controller (Z8530) must not be present, if

the corresponding Channels are not to be used, nevertheless the first

Controller is however, always required.  Now with it the 9K6-USCC-Card can

be used (Option -PUSCC:<Base>:<IRQ>:31).

The next Table gives the exact Cycle Source for receive (RxC) and Send

(TxC) also the Coding Mode. The first slot contains which numeral to be

issued with the Option -PUSCC, the last one gives the equivalent Values

for the Parameters CARRIER and HENNING with BayCom. Soft-DCD and Duplex-

Operation will be switched on with the Commands @C and @D.

-P              RxC  TxC  Mode CARRIER HENNING

1 Softclock    DPLL BRG  NRZI   0/1      0

2 Hardclock    DPLL RTxC NRZI   2-4      0

3 DF9IC-Modem  TRxC RTxC NRZ    1-4      1

BRG   Baud rate generator \ contained in the

DPLL  Digital PLL         / SCC-Controller

RTxC                      \ Connections of the

TRxC                      / SCC-Controller

With certain USCC-Cards there are probable Timing-Problems with the access

to the Data-Port of the SCC-Controller. Individual sensible Data has been

sent with TFPCX v2.00 for this reason. After the lowering of the Bus-Rate

respective to the exchange of the SCC-Controller by means of Original-

ZILOG-Types allowed this effect to be eliminated partially. TFPCX no

longer issues the Send-Data via the Data-Port, as that with the Baycom

also has been done. I hope that with this, these Problems will no longer

occur.

4. Information for Software Developers

4.1. Program-Interface

Communication with TFPCX is consequent upon a Software-Interrupt. There

exists various lower functions, which are able to be selected with the

calling of the Value in Register AH. Possible Parameters will be delivered

up in AL. AX contains with return either the result or 0xFFFF if a non-

37

existent Function has been selected. All of the requested available

symbols should be read in before the next issue is made. TFPCX manages 2

different Interfaces, which themselves, only differentiate a little.

4.1.1. TFPC-Interface

Sub Functions:

AH = 1  Requests, whether a Character is available for Request

Returns :   AX = 0  No Character ready

AX = 1  Character ready for Request

AH = 2  Character Request (only called, if Function 1 has

communicated that a Character stands available)

Returns:    AL      Character Code

AH = 3  Output of a Character

Parameter:  AL      issued Character

Three Bytes after the jump into the TFPCX-Interrupt-Routine the

recognition string 'N5NX' stands available, whose Interrupt used, can  be

established.

4.1.2. DRSI-Interface

The available Implementation in TFPCX resulted from SP, I have available

neither a description nor the TNCTSR-Driver itself. For this reason I am

able only to point to the Functions' abilities in connection with SP,

deviations from the original are possible.

Sub Functions:

AH = 0  Input of a Character

Return:     AH = 0  no Character ready for input

AH = 1  Character ready for input

AL      Character Code (only if AH = 1)

AH = 1  Output of a Character

Parameter:  AL      Character to Output

The Interrupt-Routine begins with the following Bytes:

0x53 0x1E 0xBB 0x?? 0x?? 0x8E 0xDB 0x84 0xE4 0x74 0x20

The Interrupt used, can be ascertained, in which the beginning of all

Routines of the Interrupt-Vectors 0x40 to 0xFF will be compared with this

Byte sequence, the Positions 0x?? are able to accept optional Values. This

procedure also functions with the DRSI-TNCTSR-Driver and will be employed

likewise with SP.

38

4.1.3. Special Functions

The following Lower Functions are Developments that only exist with TFPCX.

They are available with both Interface-Variations.

AH = 0xFB  Request from Port- and Channel Number (from v2.00)

Returns:   AL       Number of the employed Ports (0 to 8)

AH       Number of the available Channels (4 to40)

The Channel Quantity is installed with the Option '-CH'

AH = 0xFC  Request for the Send-/Receive Status (from v2.00)

Returns:   AL       Receive Status (Bit-No. = Port)

AH       Send Status

With this Function  a Send-/Receive notification can be

realised through the Terminal Program, the Option  -C  only

functions in the Text Mode and is also not optimal for the

Screen Construction. Each Port is always related to a Bit from

AL and AH. Bit 0 (LSB) belongs to Port 0, Bit 1 to Port 1 etc..

If the respective Bit is set, on this Port will be sent true

receive (DCD) respectively. With Duplex-Operation both Bits are

able to be set simultaneously. The Function 0xFB returns the

number of Ports, which should be indicated.

AH = 0xFD  Request of TFPCX-Busy-Status (from v1.11b)

Returns:   AX = 0   Busy (free Buffer < 176)

AX = 1   not Busy

With this, a Send Handshake is possible in the Terminal

Mode.

AH = 0xFE  Request for the TFPCX-Version (from v1.01)

Returns:   AH = 2   Main Version Number

AL = 0   Lower Version Number (no BCD)

This Function makes it possible to distinguish between TFPCX

and TFPCR (delivers AX = 0xFFFF) and DRSI-TNCTSR (delivers with

a Test AX = 0). It should be tested, whether 1<=AH<=20 is valid

and only in this case close TFPCX. In addition with this

function it can be ascertained whether an old Version is being

used, in which the required Function is not yet undertaken.

4.2. Format of Reports

In the following, Messages from TFPCX will be presented that contain a

Port Number and with it, deviate from the TNC-Firmware. <Port> is a

Numeral between 0 and 7. The Indicator of <Port>: follows only if TFPCX is

started with the Options '-DR', '-DX' or -DM. The Monitor-Reports with '-

DR' have been altered with this Version, in order to eliminate

unintentional differentiation from the DRSI-TNCTSR Driver.

39

- Link-Status

BUSY fm <Port>:<Call> via <Digis>

CONNECTED to <Port>:<Call> via <Digis>

LINK RESET fm <Port>:<Call> via <Digis>

LINK RESET to <Port>:<Call> via <Digis>

DISCONNECTED fm <Port>:<Call> via <Digis>

LINK FAILURE with <Port>:<Call> via <Digis>

FRAME REJECT fm <Port>:<Call> via <Digis> (x y z)

FRAME REJECT to <Port>:<Call> via <Digis> (x y z)

- Monitor

<Port>:CONNECT REQUEST fm <Call> via <Digis>

<Port>:fm <Call> to <Call> via <Digis> ctl <Name> pid <Hex>

^

empty character

Restoration of the Command 'C' without Parameter

<Port>:<Call> via <Digis>

4.3. Extended Host-Mode

The Extended Host-Mode (DG3DBI) is a compatible development of the 'G'-

Command, which with a global Poll-Command it enables, the obtaining of

information about all Channels of the Firmware, on which Data is on hand

and is called next. The usage of the Extended Host-Mode effects an

improvement of the Data throughput between TFPCX and the Terminal Program

especially with the employment of many Channels with very different Data

Arrivals.

The Global Request follows through a 'G'-Command and the virtual Channel

255 (Byte sequence: 0xFF 0x01 0x00 'G'), whereby possible Parameters will

be ignored. TFPCX consequently answers with a 0 terminated List of all

Channels which have buffered Data (Byte sequence: 0xFF 0x01 Channel+1 ...

0x00). Channel+1 ... is the series with respect to 1 increased Channel

Number).

Example:

The Channels 0 (Monitor Channel), 1 and 5 have Data on hand. TFPCX answers

with:

0xFF 0x01 0x01 0x02 0x86 0x00

On the strength of this, the Terminal Program directly requests the given

Channels until no more Data is on hand. If all Channels are Free, only

0xFF 0x01 0x00 will be delivered. With the first Global Poll must be

checked, whether possibly the Error Report "INVALID CHANNEL NUMBER"

arrives, then that, that always occurs, if the Extended Host-Mode of the

Firmware is not yet supported.

40

4.4. Previous Versions

Since v1.00 the following Alterations have been implemented.

v1.01

- Advice regarding unreadable Information through the flashing

Square if not in Hostmode (Switchable by -NB)

- Base address of the Modem-Ports allowed to be set explicitly

- TxD-Control on the Modem-Port for Supply Voltage staticaly switched

to +12V, Operating Voltage of the Modem switched off

with loading - Standard-TFPCX-Interrupt now 0xFD (previously 0xFE)

- Version Request via TFPCX-Interrupt (AH = 0xFE)

v1.10

- Transition to The Firmware v2.3b DAMA (previously TF v2.1c)

- Soft-DCD (Request with Command @C)

- Send-/Receive-Indicator in Hostmode (switched off with -NC)

- Postponement of Disk-Access by Send/Receive as required with

possible Problems (-ND)

- Automatic raising of the SSID with Connect, if the same Station

is already connected

- Internal Connects possible

- To set Frack in 1s- or 10ms-Units (Command F)

- Unattended Mode can be switched on without CTEXT,

Error Report 'NO MESSAGE AVAILABLE' removed (Command U)

- 600 Buffer (previously 400)

- Bug cleared, which with 486's makes unloading impossible

v1.11

- Command Z again available (XON/XOFF for TERM)

- LPT-Pins DATA1-6 switched to 5V for Modem-Supply Voltage

v1.11b (unofficial)

- Function for Send-Handshake in the Terminal Mode via TFPCX-

Interrupt for TERM (AH = 0xFD)

v2.00

- Management for BayCom-USCC-Card and a maximum of 2 Modems (maximum

6 Ports, internal 8 Ports)

- Options -PUSCC and -B expanded for more USCC-Cards and more Ports

- Option -NC removed, now the DCD-Indicator will be switched on by

-C[xx] (optional Screen Attribute xx)

- Now 20 Channels (previously 10)

- Emulation of the DRSI-TNCTSR-driver possible, additional Software-

Interface (-DR)

- With Option -DM DRSI-compatible Reports, but old Software-

Interface

- Output of Port Numbers in Reports

- Parameter B, O, P, T, W, X, @C, @D, @T2, @T4 and @TA will be

employed for each extra Port (Output <Port>:)

- Command QRES sets return to Terminal Mode

41

- Link List for Crossband-Digipeating (Command @L)

- Statistic-Frame-Counter (Command @ST)

- Installable TXTAIL-Parameter (Command @TA)

- Command P with Parameter 0-7 compatible with DRSI Parameter

Request (no Parameter installation)

- Command Z removed again

- 'dynamic' MAXFRAME removed (Command O)

- Error Report with Modem-Operation under Microsoft-Windows (386

Enhanced Mode)

- Functions for the Request of the Port-/Channel Number (AH = 0xFB)

and of the Send-/Receive Status (AH = 0xFC) via Software-Interrupt

- Bugs cleared, which with Buffer overflow during Background Operation

and timewise, leading to overflowing emissions.

v2.01

- USCC-Send Problems remedied, through output of the Send-Data via

Control-Port (Timing-Problems with the Data-Port of a few USCC-Cards)

- Option -BU[nnnn] for the installation of the Buffer Size (Minimum 400,

Default 600), failing which, the quantity employed will be the max.

Value

- The Number of Connect-Channels installable by Option -CHnn (4-40

Channels, Default 10)

- Command E (Echo) again on hand (Default 1, Echo on)

- Command @M for #BIN#-Receive, @M=0 Conversion of Control-Characters

(Standard), @M=1 Transport-Mode (for TERM)

- Init-File (Option -F) can contain Empty Characters and Commentary

(introduced by '#' or ';'), ESC automatically creates ('^' will be

ignored), conversion of TABs to empty Characters.

- Remote-Command '//Q' (when U=1 and JHOST=0)

- Frames will be monitored, if more than 256 Buffers are Free (previously

64 Buffers)

- Error reports with Modem-Operation under OS/2 2.0

- DRSI-Function 1 (Character Output) returns AH=0 (Compatibility with the

TNCTSR-Driver, which announces in AH, whether Characters or the issue

stands ready)

v2.10

- KISS-Mode (incl. SMACK) supports (Option -PKISS, max. 4 Ports)

- Support for PA0HZP-OptoPcScc-Card (Option -POSCC), Clock Type 4 (PA0HZP-

Port with external Clock) and 5 (PA0HZP-Timer for 75 Hz Time Normal)

- Baycom-9K6-USCC-Card support (2. SCC-Controller not activated, if not

employed)

- IRQs for SCC and KISS: 2-5/7, with AT additionally: 9-12/14-15 (2=9)

- Reports (Monitor and CONNECT REQUEST) altered with the DRSI-Interface

(Option -DR) (Incompatibilities with the TNCTSR-Driver eliminated),

Option -DX for previous modified Reports.

- Command @PO for selectable Port Ordering (Channel can be connected only

from ordered Port, Default-Port with outgoing Connects).

- Internal Connects (Loopback) switchable (Option -NL)

- TXTAIL (@TA) with Modem and SCC in consideration of Baud Rate and Timer-

Inaccuracy, optimally installed (@TA=4 with 300 Baud, @TA=1 otherwise),

max. Value for @TA is 6000 (with KISS 255)

- 0 Ports, if -P not issued (No Default-Port)

- Option -D replaces the previous permanent Option -P

42

- With DAMA waits before the Polling also on execution of T1 (as TF 2.6)

- Extended Host-Mode (DG3DBI) supported

- Remote-Command '//Q' only with U=2 (Default)

- @ST also clears the ERR-Counter

- NET/ROM-Monitor removed

- Default-Parameters F, N, P, R, T, U, @A3, @I, @T2, @T3, @T4 and @TA

altered.

5. Copyright and User Advice

TFPCX may be used in Amateur Radio and Copies given to third parties, so

long as no charge is made.  In particular the free gift of TFPCX connected

with Hard- and Software is only allowed if the respective Product is also

calculated with no fee or with my previous written agreement. It is not

permitted for the Program to be Commercially used or distributed. The

complete Archive may only be passed on to other users with all of the

Files.

A Guarantee for the correct functioning of the Program is not either

given or implied. The Author will not be held to be responsible for any

possible damage with the use of TFPCX, (Responsibility excluded)

The Author of the Program TFPCX is Ren� Stange (DG0FT). Contained in TFPCX

are Parts of the TheFirmware 2.3b from NORD><LINK (Original Version by

DC4OX, DAMA by DL8ZAW, Changes by DB2OS, DF2AU, DF7ZE, DK6PX,  DL1BHO,

DL1MEN,  DL4YBG,  DL9HCJ and others.). The development followed on from

the Program TFPCR v1.60 by DL1MEN.

6. Reference Advice

Anyone who is interested in the Program TFPCX, should send an empty

Diskette with self addressed and correctly stamped return envelope to:

(G0KIU advises UK Amateurs to send 3 IRCs)

Ren� Stange

O.-Grotewohl-Ring 34

O-1260 Strausberg

Possible Disk-Formats: 3�" 720K or 1.44M (preferred)

5�" 360K or 1.2M

Re-usable Envelopes with Address Label are also possible. PRINCIPALLY I am

not able to pay the return Postage. Please do not use 'over sized'

Envelopes, because they give problems with the Post Box at this end.

I am not able to issue Software or Hardware (e.g. Terminal Programs or

BayCom-Modem), which has not been developed by me. In these cases,

requests should be made to the corresponding originator.

Sorry but only UK Amateurs may make any query to G0KIU @ GB7MAM

43

THE FOLLOWING IS INSERTED BY G0KIU:

The decision whether a token of appreciation is given towards the

further development of TFPCX must be made by each Amateur using the

Program. Rene does need your support so please make some gesture.