Packet Terminal Driver Program For IBM-PC/Clones

================================================

Copyright Jon Welch - Ver 1.42 21/04/95

---------------------------------------

Introduction

============

The  Packet  Terminal Driver program is  a  general  purpose

communications  package  for  communicating  with  a  packet  TNC

attached  to your IBM-PC/clone. The program was written  to  work

with  the Kantronics KPC-4 TNC but should work with any TNC  that

is connected to a serial port on your computer.

The software includes many features not often found in other

TNC  driver  programs.  These include  integrated  Personal  Mail

System with FBB compatible compressed forwarding, split screen or

full  screen  display, 500 line scroll back  buffer  with  search

facilities,  pop  up menu driven YAPP, YModem, XModem  and  ASCII

file  transfer facilities, connect directory, locator  conversion

and  distance calculations, directory browser with  comprehensive

file  viewer,  file  and notepad editor,  clipboard  editor  with

associated functions, SSTV, WEFAX and RTTY support on  Kantronics

TNC's, built in ANSI driver for use with the latest BBS software,

UUencode  and UUdecode of files, 7PLUS encoding and  decoding  of

files, REQDIR and REQFIL facilities with automatic uuencoding  or

7PLUS  encoding  of  binary REQFIL requests, simple  to  use  yet

sophisticated  script  language for  automatic  operation,  timed

execution of script files, a screen blanker, FBB header broadcast

support plus many more.

DISCLAIMER OF WARRANTY

THIS  SOFTWARE  AND  MANUAL  ARE PROVIDED  "AS  IS"  AND  WITHOUT

WARRANTIES  AS  TO PERFORMANCE OF MERCHANTABILITY  OR  ANY  OTHER

WARRANTIES WHETHER EXPRESSED OR IMPLIED.  BECAUSE OF THE  VARIOUS

HARDWARE AND SOFTWARE ENVIRONMENTS INTO WHICH THIS PROGRAM MAY BE

PUT, NO WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE IS OFFERED.

TNC Driver V 1.42               1                        21/04/95

Distribution Restrictions

=========================

The Packet Terminal Driver program is NOT a public domain or

free package. The software is distributed as an 'Evaluation Copy'

package  where  all the features of the software  are  accessible

except  the on-air PMS, WEFAX capture and SSTV capture. The  user

can evaluate the product to determine it's suitability for  their

purpose  and can subsequently register to gain access to  all  of

the features if they decide to use the package.

Registration

============

Individual  copies  of  the program  may  be  registered  by

filling  in  the accompanying order form and  sending  a  minimum        ________________________________________

donation of 10 U.K. pounds to me at :

50 Quarrydale Road,

Sutton In Ashfield,

Notts.

NG17 4DR.

Great Britain

If  you are registering outside the U.K., please  send  your

registration fee in U.K. currency if possible (either as cash  or

cheques/money  order).  If you cannot arrange this,  please  send

your fee in cash currency.

I  WILL  NOT ACCEPT CHEQUES MADE OUT IN NON  U.K.  CURRENCY.

These   will   be  returned  to   sender   without   registration

information. It costs me 3.00 U.K. pounds to bank foreign cheques

and  1.50  U.K. pounds to exchange foreign  currency.  Since  the

registration fee is so low, these bank charges take a hefty  lump

out of the fee which I could well do without. International reply

coupons  can only be exchanged for postage stamps and not  cashed

so these are also out I am afraid.

Upon  receipt of your registration information, you will  be

sent your registration number enabling you to gain full access to

the program. Registration entitles you to future upgrades of this

software FREE OF CHARGE so please register and give your  support

to the program.

If  you  are  ordering from outside  Great  Britain,  please

remember to include your country in your address so I know  where

to send the information to !

TNC Driver V 1.42               2                        21/04/95

C O N T E N T S

===============

Chapter                                           Page

-------                                           ----

1 System Requirements                             5

2 Installing The Software                         6

2.1 System Files and Usage                      7

3 Configuring The Software                        9

3.1 Format of TNC.CFG                           9

3.2 Format of TNC.TIM                          16

3.3 Format of TNC.PRN                          17

4 How to use the Software                        18

4.1 General Guidelines                         18

5 Miscellaneous Commands                         19

6 Specialised Function Keys                      21

7 Specialised Alt Function Keys                  23

8 Specialised Alt Keys                           24

9 Directory Browser                              29

10 7PLUS File Encoding                            32

11 WEFAX Mode                                     34

11.1 Capture                                   34

11.2 Review                                    37

11.3 Print                                     37

12 RTTY Mode                                      38

13 SSTV Mode                                      40

13.1 Capture                                   40

13.2 Review                                    41

13.3 Print                                     41

14 Script Files                                   42

14.1 Sample Script Files                       52

14.2 Automatic Script Files                    56

TNC Driver V 1.42               3                        21/04/95

15 Personal Mail System                           57

15.1 Configuring the PMS                       57

15.2 Setting up the PMS                        61

15.3 Logging Onto The PMS                      63

15.4 Using the PMS                             64

15.5 Commands In The PMS                       65

15.6 Maintaining The PMS                       77

15.7 Gateways                                  78

15.7.1 Configuring Simple Gateway         78

Programs

15.7.2 Writing Custom Gateway             79

Programs

15.8 BBS access to PMS                         82

15.9 REQDIR and REQFIL                         84

16 Online Cross Reference Help Facility           86

16.1 Creating a Help File                      86

16.2 Using the Help System                     87

17 FBB Header Broadcast Facility                  88

18 Trouble Shooting                               90

Appendices

----------

A - Summary of Function Keys                     91

B - Summary of Editor Keys                       92

C - Summary of Script Language                   93

D - Summary of PMS Commands                      96

E - My TNC Parameters                            99

F - Closing Remarks                             102

TNC Driver V 1.42               4                        21/04/95

1 System Requirements

=====================

The software will work on any 80x86 computer running  MS-DOS

3.00  or above. 640K of base memory is required to fully use  all

the  features  of the program  but all options except  WEFAX  and

SSTV  should work with less. The video display can be  colour  or

mono  for normal operation whilst the WEFAX capture can  work  in

Hercules, CGA, EGA or VGA graphics modes. The SSTV capture  works

in  CGA or Hercules mode for 8 and 16 second frames and only  EGA

or Hercules for 32 second frames. An Epson compatible printer  is

required  for the WEFAX and SSTV picture dumps or access to  your

printer manual for editing the printer configuration file.

The  software  also works on the Atari computer running  the

PC-DITTO PC emulator program (although a little slowly !).

The software allows your TNC to be connected to any COM port

of  your  computer.  The com port, i/o  address,  interrupt  (2-7

only),  baud rate, parity etc.  are configurable using the  steps

given below. Also, the software uses hardware handshaking on  the

CTS/RTS  lines  so  you  must make sure  your  cabling  is  wired

accordingly.  Also, the software can use the DCD line to  monitor

connects  if this is available on your TNC. If so, make sure  the

cable is again wired correctly.

The software does not configure the TNC in any way. However,

the software expects the TNC to be configured as :

AUTOLF   OFF   - No auto line feed

CMDTIME  1     - Time allowed to enter character to escape

transparent mode. MUST be set to 1 for

YAPP transfer to exit correctly.

COMMAND  $03   - Enter command mode character

DELETE   $08   - Delete character (different from version

1.27 and below)

ECHO     OFF   - Character echo performed by software not

TNC

(These  are  the  Kantronics command names but  other  TNC's

should have the equivalent commands).

Also,  turn  off  any  commands which set up  software  flow

control.  These are not required and can interfere with the  YAPP

transfers  if still enabled. These commands are typically  XFLOW,

TXFLOW and TRFLOW which should all be OFF.

If  you encounter any problems running the program, it  will

probably  be due to your TNC having different settings  to  mine.

Please  compare my settings (given in Appendix E) with yours  and

alter yours to match. Doing this will sort out most problems  you

are likely to encounter.

TNC Driver V 1.42               5                        21/04/95

2 Installing The Software

=========================

To install the software you will need to copy the  following

files  TNC.EXE, TNC.CFG, TNC.CLR and TNC.PRN to either your  hard

disc or to a blank floppy disc.

If  you  wish to use the PMS system, you will also  need  to

copy the TNCPMS.CFG file to the PMS directory and the help  files

to their correct location as shown below.

If  you wish to use the DOS-SHELL feature of  the  software,

you will also need a copy of 'COMMAND.COM' on your floppy disc.

You  will also need to create several sub-directories  under

the  TNC home directory. These will be used to store  any  script

files  you  may create, the PMS system and an area for  your  log

files and other system information. The structure you must create

is therefore :

TNC

|

|

+---------------+-----------------+

|               |                 |

|               |                 |

LOG             PMS             SCRIPT

|

|

+---------------+--------+--------+---------------+

|               |                 |               |

|               |                 |               |

HELP           FORWARD          AREAINFO          MAIL

|

|

OLDMAIL

If  you are installing the software from the  release  file,

this structure will be created for you automatically.

TNC Driver V 1.42               6                        21/04/95

2.1 System Files and Usage

--------------------------

The files comprising the system, where they are located  and

their function are as follows :

TNC

---

TNC.CFG - Configuration File

TNC.CLR - Colour configuration file

TNC.DIR - Optional connect directory

TNC.EXE - Main Program

TNC.HLP - Optional online cross reference help file

TNC.LOG - Log file of connect and disconnect times

TNC.PRN - Printer configuration file

TNC.TIM - Optional timed event configuration file

TNC\LOG (Files created by system)

-------

MAMAUTO.001 - Files created by the LOG, ENDLOG script commands

BADAUTO.001 - These can be deleted once read

GB7MAM .FBB - List of grabbed FBB message headers

GB7MAM .IDX - Index for above file

TNC\PMS

-------

TNCPMS.BAD - Callsigns not allowed on PMS  (optional)

TNCPMS.CFG - Configuration file for PMS    (mandatory)

TNCPMS.DAY - Message of the day            (optional)

TNCPMS.HLP - Text file sent for H command  (optional)

TNCPMS.IDX - Message index file            (created by program)

TNCPMS.INF - Text file sent for I command  (optional)

TNCPMS.LOG - PMS usage log file            (created by program)

TNCPMS.OK  - Callsigns only allowed on PMS (optional)

TNCPMS.SYS - PMS system file               (created by program)

TNCPMS.USR - PMS user details file         (created by program)

TNC\PMS\MAIL

------------

MSGnnnnn.TXT - Message files - editable if required

TNC\PMS\OLDMAIL (Only created if OLDMAIL directory exists)

------------

MSGnnnnn.TXT - Archive of killed messages - delete if required

TNC\PMS\FORWARD (All files optional)

---------------

G7JJF.LST - Text file containing lists of message subjects which

G1EQT.LST - the user requests being made available for forwarding

TNC Driver V 1.42               7                        21/04/95

TNC\PMS\AREAINFO (All files optional)

----------------

1.INF - File displayed before listing files in file areas and can

2.INF - be used to describe what sort of files are in area

etc.

TNC\PMS\HELP (All files optional)

------------

TNCPMS.LM - Text file sent for ? LM command

TNCPMS.W  - Text file sent for ? W command

etc.

TNC\SCRIPT (All files optional)

----------

MAMAUTO.SCR - General script file

MAMAUTO.FWD - Script file for mail forwarding

MAMAUTO.MAI - Script file for mail snatch

TNCINIT.SCR - Executed on running program

TNCEXIT.SCR - Executed on exiting program

etc.

TNC Driver V 1.42               8                        21/04/95

3 Configuring The Software

==========================

The configuration for the program is stored in four separate

files called TNC.CLR, TNC.TIM, TNC.PRN and the main one  TNC.CFG.

The configuration of the PMS will be discussed in detail below.

The  file  TNC.CFG  is a standard text file  and  should  be

edited  with the inbuilt file editor or a text editor  that  does

not  add formatting characters to the text (such as  Wordstar  in

non-document  mode). Alternatively, you can use the  online  menu

driven  configuration  option  which will be  described  in  more

detail below.

The  file  TNC.CLR is automatically updated when you  change

the screen colours from the inbuilt configuration menu.

The  file  TNC.TIM is also a standard text  file  containing

time and script file information which will be described below.

The  file  TNC.PRN is also a standard text  file  containing

printer  control  information which will be described  in  detail

below.

3.1 Format of TNC.CFG

---------------------

The format of the file is given below :

Line 1

------

This tells the computer which COM port your TNC is connected

to  and  optionally  its i/o address  and  interrupt  number.  It

contains one or three parameters each separated by a comma and no

spaces.

The first parameter can be from 1-4 and is the number of the

COM port you are using.

The  second  parameter is the i/o address of the  port.  The

program knows the i/o address for COM 1 and 2 so you do not  need

to specify the address if you are using either of these.  Typical

values are :

COM 1 - 3F8, COM2 - 2F8, COM3 - 3E8, COM4 - 2E8

The third parameter is the interrupt number for the port.  I

don't  know  how to program for interrupts 8-15 yet  so  you  are

restricted  to  interrupts 2-7. The program knows  the  interrupt

number  for COM 1 and 2 so you do not need to specify it  if  you

are using either of these. Typical values are :

COM 1 - 4, COM2 - 3, COM3 - 4, COM4 - 3

TNC Driver V 1.42               9                        21/04/95

Note  : Unless your hardware has been specially modified,  it  is

unlikely that sharing the same interrupts between COM ports  will

work.  If  using  COM  3 or COM 4, you  may  need  to  alter  the

interrupt  to  be  2,  5, 6 or 7 on  your  serial  card  to  work

correctly.

Line 2

------

This  gives the computer information about the settings  for

the  COM port specified above.  It contains six  parameters  each

separated by a comma and no spaces.

This  first parameter tells the computer at which baud  rate

you wish to communicate with the TNC. Valid values are :

110, 150, 300, 600, 1200, 2400, 4800, 9600, 19200, 38400, 56000

Baud  rates  above 9600 baud cannot be  guaranteed  to  work

since  I have no test equipment for these speeds.  However,  they

should  theoretically  work if your TNC (or  modem)  can  support

them.

The  second  parameter is the word length you wish  to  use.

This can either be 7 or 8 bits.

The third parameter is the parity setting. It can either  be

N, E or O for none, even and odd parity.

The  fourth  parameter is the number of stop  bits  to  use.

Valid values are 1 and 2.

If any of these parameters are out of range or invalid,  the

software will display an error message.

The  fifth  parameter  is required for  slow  computers,  ie

standard  PC's  using  the 8086 chip eg Amstrad  1512.  For  some

unknown  reason,  such  computers require a  slight  pause  after

sending  a  character to the TNC to give it chance to  accept  it

before sending the next one. A recommended value is 10 which will

pause for 10 1/1000 th's of a second between each character.  You

can  try  to reduce this parameter if you find this too  slow.  A

test  to  validate the value is to press the F6  key  which  will

send the sequence 'DAYT yymmddhhmmss' to your TNC.  A value of 10

should  give no errors but as you reduce the value down,  you may

get  to the point where characters start to be missed out and the

TNC  no longer recognises the command.  You have then  found  the

optimum delay value for your computer. On AT type machines, there

should  be no need for the delay so you can set this value to  0.

This  delay  problem  may be caused by the way I am  sending  the

characters to the serial port.  If anyone recognises this problem

and  knows  of a way around the delay,  can they  please  let  me

know !

TNC Driver V 1.42               10                        21/04/95

The  sixth  parameter  is of use for TINY-2 users  with  the

Siskin firmware.  It appears that in normal CONVERS mode, the TNC

does not work with full 8 bit data.  Instead,  it seems to always

default to sending data with even parity to the terminal. You can

set the TNC to 7 bits, even parity to get around this problem but

with only 7 bits,  you cannot do YAPP binary transfers.  Instead,

setting the TNC for 8 bits, no parity and this sixth parameter to

1,  the TNC program will strip the 8th bit from received data  to

display it correctly. Also, setting AWLEN to 8 and 8BITCONV to ON

will allow full 8 bit binary YAPP transfers.  If you do not  wish

the 8th bit to be stripped, leave this parameter as 0.

Line 3

------

This tells the computer how many hours to subtract from  the

system clock time for different time zones (eg. put 1 when in BST

and 0 when in GMT). For UK users, during BST, you will also  need

to  add  the following line into your AUTOEXEC.BAT  file  (either

hard disc or the floppy disc you boot the computer from).

SET TZ=BST6

It will of course need removing when we are back in GMT.

Line 4

------

This  line  determines whether DCD monitoring is to be  used

when checking for CONNECT/DISCONNECT messages.  Its value can  be

either  1  to  enable  DCD monitoring or  0  to  enable  software

monitoring.

If  DCD monitoring is enabled, the software  scans  incoming

text for the CONNECTED TO, DISCONNECTED messages and only ensures

the  match  is valid when the DCD line is  in  the  corresponding

state. This stops picking up the text when monitoring other users

packet traffic.

If  software  monitoring  is  enabled,   any  match  of  the

CONNECTED TO, DISCONNECTED messages will be taken to refer to the

local user.  This can be incorrect in some cases when the text is

monitored from other users packet traffic.

Line 5

------

This will be your name when you register the software. After

receiving  your registration number,  you may edit this line  and

the subsequent line to the correct text.  If you try and cheat by

guessing a name and registration number, the software will detect

this and revert back to the evaluation copy mode.

TNC Driver V 1.42               11                        21/04/95

Line 6

------

This  will  be  your personal registration number  when  you

register the software. See above for info on editing this line.

Lines 7-16 (for function keys 1 to 10)

----------

These contain the text attached to the function keys.  There

are  four  fields per line. The text is each field  is  currently

limited  to 64 characters for the TNC text and 14 characters  for

the help text.

Field 1 : The  text sent to the TNC when pressing shift  and

the  function key.  You can add control characters into the text,

eg  ^M  is return,  ^[ is the escape character,  ^G is  the  bell

character, ^C to return to command mode etc. You can also  create

a  delay of 0.5 seconds by the string '^.'. Thus to  escape  from

transparent mode to command mode, you could define a function key

as '^C^.^C^.^C'.

You  can  also include key press emulation as  used  in  the

script  REPLY command. For example, if you are a RTTY  user,  you

can  store your CQ text in a file rather than on a  function  key

and program a function key as :

'{F2}Fcq.txt^M'

When  you press the function key, it will  emulate  pressing

the  F2 key to upload an ASCII file called 'cq.txt'. This  allows

you  to send more information by one key press that can  normally

be fitted into the 64 characters allowed for the function key.

See  the SCRIPT section below for more information on  using

emulated key presses.

Field 2 : The  text to describe the key which will appear on

the help screen.

Field 3 : The text sent to the TNC when pressing control and

the  function  key.  See above for  information  about  including

control characters in the string.

Field 4 : The text to describe the key which will appear  on

the help screen.

TNC Driver V 1.42               12                        21/04/95

Line 17

-------

This  line is only used by users of Kantronics TNC's but  it

still needs to be in the configuration file. It is used to  store

custom  values  for the DAYTWEAK parameter when using  the  WEFAX

routines. Users have found that their pictures have been  skewing

using  the default value of 0 which was pre-programmed  into  the

software.  By experimenting with different DAYTWEAK  values,  you

should  arrive  at a value which produces perfect  pictures.  The

first  parameter is your custom value for WEFAX capture, and  the

second parameter is your normal default DAYTWEAK setting which is

sent to the TNC after exiting the WEFAX capture routines.

Line 18

-------

This  line is only used by users of Kantronics TNC's but  it

still needs to be in the configuration file. It is used to  store

values for the MAXUSERS command before going into WEFAX mode  and

coming  out  as  well.  It  is  different  for  most  models   of

Kantronic's TNC because of the number of ports.

One the KAM or KPC/4, the first parameter must be 0/1

On the single port models, it must be 1

The  second  parameter is your normal  default  MAXUSERS  setting

which  is  sent  to  the TNC  after  exiting  the  WEFAX  capture

routines.

Line 19

-------

This line contains a single Y or N character and informs the

software  whether you wish it to generate the TNC.LOG  file.  For

people using only floppy discs instead of hard discs, disc  space

may  be short so you may want to disable the log file  generation

by putting a N here.

Line 20

-------

This line is possibly only used by users of Kantronics  Dual

Port TNC's but it still needs to be in the configuration file. It

is  used to store the stream indicator values which are  sent  by

the TNC when a connection is made on the non-default radio  port.

This  is  needed so the program can automatically switch  to  the

other  port when someone connects into the PMS for  example.  The

line  contains  upto  five  fields of which  only  the  first  is

mandatory.

Field 1 : This is a Y or N flag to tell the software whether

to  check  for port switching or not. If your TNC does  not  have

more than one port, you can miss out the remainder of the  fields

on this line.

TNC Driver V 1.42               13                        21/04/95

Field 2 : This  field contains the stream  switch  indicator

which is sent by the TNC when it receives data on port 1 when  it

is  not  the  default port. The indicator is  made  up  of  three

characters,  the  first  being the  stream  switch  character  as

defined  by  the TNC's STREAMSW command. The second is  a  stream

letter starting at A and going up possibly to Z depending on  the

MAXUSERS  setting with the third character being a ':'.  Whenever

the   program  detects  this  combination  of  characters  on   a

*** CONNECTED to line, it will automatically resend the first two

monitored  stream switch indicator characters to change  port  so

the user can potentially access the pms should he so wish.

Because the second character of the indicator may be A-Z  or

even  0-9 on other TNC's, in the configuration file, you need  to

replace  this  with the wildcard character of 'A'  to  match  any

alphabetic character or 'N' to match any numeric character.

Field 3-5 : These  fields  contain the same  information  as

field  2 but for ports 2-4 of the TNC. If your TNC does not  have

up to four radio ports, you may miss out any you don't require.

eg. For a two port KPC-4 with STREAMSW set to $7B/$7D,  this

line would be :

Y,{A:,}A:

Line 21

-------

This line contains three fields and tells the program if you

wish  to  use extended 43/50 line EGA/VGA screen mode,  how  many

lines  you  require  for the TNC incoming data  window  in  split

screen mode and the screen blanker delay.

Field 1 : This  is a single Y or N character and  tells  the

program  if you wish to used the extended screen mode or not.  If

you have an EGA display, it will increase the screen size from 25

to  43  lines  and if you have a VGA display, it will  go  to  50

lines. Do not set this field to Y if your display cannot  support

EGA or VGA mode.

Field 2 : This  is the number of lines you require  for  the

TNC  incoming  data window in split screen mode. It  must  be  at

least 2 but not more than 'number of screen lines - 11' (to leave

at  least  two  lines in the input window at the  bottom  of  the

screen).

eg. the default setting for a 25 five line CGA display (with

no screen blanker) would be :

N,11,0

TNC Driver V 1.42               14                        21/04/95

Field 3 : This  is the seconds delay of keyboard  inactivity

before  blanking the computer screen. A value of 0  will  disable

the  screen  blanker, any other value will  enable  the  blanker.

After  the specified number of seconds of inactivity, the  screen

will  go  blank. For the technically minded, to  accommodate  all

video  adaptors, the blanker routine simply selects video page  1

of  the  current screen mode and clears the  screen.  All  bioses

should support this feature but if the screen doesn't blank,  you

will have to disable the blanker and let me know so I can come up

with  another method of blanking the screen. Nothing  else  fancy

happens  whilst the screen is blanked but space is available  for

rent at reasonable rates ! To unblank the screen, simply press  a

key. The screen will also automatically unblank when an  incoming

connect is detected.

Line 22

-------

This line contains the amount of memory you wish to  reserve

for the inbuilt editor to edit a file in. It can be between 1 and

63 which corresponds to 1K to 63K of memory. Do not use a  figure

which  is larger than you need and a value of 16 is  recommended.

If  you  try  and  edit a file  larger  then  the  editor  memory

available,  an  error  will be generated and  you  will  need  to

increase this figure to suit.

Line 23

-------

This line contains the maximum number of lines a file  being

edited  may contain. It can be between 1 and 5000. Do not  use  a

figure  which  is  larger than you need and a value  of  1000  is

recommended. If you try and edit a file with more lines than this

figure, an error will be generated and you will need to  increase

this figure to suit.

TNC Driver V 1.42               15                        21/04/95

3.2 Format of TNC.TIM

---------------------

The format of each line in the file is given below :

HH:MM <script filename>

where  HH:MM is the time (in 24 hour format) when  you  wish

the  script  file <script filename> (stored in  the  SCRIPT  sub-

directory) to be automatically executed. The times must be stored

in chronological order.

For example, your TNC.TIM file could contain the following :

02:00 GB7MAM_2.SCR

04:00 GB7BAD_4.SCR     etc.

and use the timed execution facility to automatically log on

to  BBS's overnight at set times to download the latest  messages

for you.

When the software is first run, it interrogates the  TNC.TIM

file  (if one is present) to find the next entry to  be  executed

skipping over the times which have already gone. If there are  no

more entries for the current day, it goes back to the top of  the

file and waits for the time to arrive.

When  it is time for one of the script files to  execute,  a

prompt  pops up to ask for confirmation that it is ok to  execute

the script. If you are in the middle of doing something, you  can

then say 'No' to cancel the script. Alternatively, say 'Yes'  and

the script will execute.

There is a time out value of 1 minute on the prompt so  when

it  pops  up  in the middle of the night, it will  time  out  and

automatically default to 'Yes' to execute the script file.

After  executing the script (or cancelling the  script),  it

will wait for the next timed event, looping back to the start  of

the file if necessary. The time of the next event is shown on the

Alt I information screen. See below for details on script files.

TNC Driver V 1.42               16                        21/04/95

3.3 Format of TNC.PRN

---------------------

The format of the file is given below :

Line 1

------

This  is  the control string to alter your printer  linefeed

pitch  to  24/216" (or 8/72").  For an EPSON compatible  printer,

this would typically be :

27,'3',24

Note that you can enter values either by their decimal value

or by their ASCII equivalent character.  If using ASCII, you must

enclose the character in single quote marks.

Line 2

------

This  is  the control string to alter your printer  linefeed

pitch  to  36/216" (or 12/72").  For an EPSON compatible  printer,

this would typically be :

27,'3',36

Line 3

------

This is the control string to put your printer into graphics

mode  capable  of  printing at  least  80  dots/inch.  For  those

printers  supporting  it,  CRT graphics printing 80 dots/inch  is

ideal since a screen width would exactly by a printer page width.

A typical control string for this would be :

27,'*',4

(Note  you  do not need to specify the number of  data  bytes  to

follow - this is done automatically).

If  your  printer does not support a CRT graphic  mode,  any

mode supporting at least 80 dots/inch is required.  Most printers

should  support  a 960 double density  mode  (120  dots/inch).  A

typical control string for this would be :

27,'L'

Note  - the dump routines issues both a CR and a LF to perform  a

line  feed  and so your printer should not be set  for  automatic

linefeeds.

If  you  cannot accommodate your printer  settings  in  this

file,  you (or a friendly C programmer) can modify  the  enclosed

FAXDUMP.C file for use as a standalone FAX printer dump program.

TNC Driver V 1.42               17                        21/04/95

4 How to use the Software

=========================

To  load  the software,  install the  package  as  described

above. If you have a hard disc, you may wish to add the directory

where the TNC files are stored in your PATH statement. To run the

software type 'TNC' (and press RETURN of course !)

You will see a screen split into two main windows. The upper

screen  is where the text received from the TNC is displayed  and

the lower screen is where you will type in your commands/text.

At  the  top left of the screen is the current time and  the

top  right of the screen shows either the length of  the  current

connection or the key to access the help screen (Alt F1).

Below  this is a window which displays who you are connected

to.  (This  is  picked  up by the text 'CONNECTED  TO'  which  is

received from the TNC.  If you are not using DCD monitoring,  the

text  from another station connecting with someone else  will  be

picked  up and you will be shown as connected to that station  so

do  not  worry  if  this happens.  This text can  be  cleared  by

pressing CTRL-Left cursor key).

Between  the  two  main text windows  is  a  further  window

containing status information.  From left to right, this displays

the  registered  user of the software, the music  symbol  if  the

connect  bell is on, a P if the PMS is enabled (it will flash  if

you  have mail waiting), the uploading/downloading  files  status

together  with  a bytes to go/bytes downloaded  counter  and  the

printer dump on/off status.

4.1 General Guidelines

----------------------

When  entering information into an input box,  full  command

line  editing is available. You can use the cursor keys  to  move

around  the  line, delete backwards with the  backspace  key  and

delete forwards with the Del key. The Insert key toggles  between

insert and overwrite modes with an indicator at the bottom  right

hand side of the box showing the current mode.

When  entering  a  filename at any point, you  may  enter  a

wildcard  filename.  If  you do, you  will  enter  the  directory

browser  where  you  can select a  file  matching  your  wildcard

filename.  If  you press RETURN in an empty input box,  you  will

enter  the  directory browser with a default filename  of  '*.*'.

Pressing ESCAPE will cancel the input prompt.

To  select  from  the  light  bar  menus,  use  the   cursor

left/right  keys  to  move the bar and press  RETURN  to  select.

Alternatively, press the key corresponding to the initial  letter

of  the  required  choice. ESCAPE will cancel  the  menu  without

making a choice.

TNC Driver V 1.42               18                        21/04/95

5 Miscellaneous Commands

========================

Cursor Up - Clear the TNC receive window.

Cursor Down - Empty the TNC's output buffer.

Ctrl Cursor Left - Clears 'Connected To' window.

TAB  (or Page Up) - This will enter the scroll  back  buffer

mode.  Here you will be able to scroll back through the  previous

500  lines of text received from your TNC. Use the normal  cursor

keypad  to scroll the text. You can find text by pressing  F  and

entering the search text. The search will only occur on text  off

the  top  of the screen. If the text is found,  the  buffer  will

scroll  to  display it at the top of the  screen.  Once  finished

browsing the scroll back buffer, press any key to exit. (See  ALT

F2-8 and ALT F10 below for more scroll back buffer functions  and

ALT-N for the notepad function).

Shift Tab - This only works in split screen mode and  allows

you to access the last 16 commands you have entered. Press  shift

tab  to  scroll  back  through the list.  To  choose  a  previous

command,  press  RETURN. This will send the command  to  the  TNC

again  but  leaves the cursor at the end so you  may  modify  the

command if you wish. To cancel this option press the space bar.

END - This  key will send a BREAK signal through the  serial

port  to  the TNC. The major use of this is to  exit  transparent

mode back to command mode but other uses may be found.

HOME - This key will access the Online Cross Reference  Help

Facility which is described in more detail below.

PgDn - This key will access the list of grabbed FBB  message

headers which is described in more detail below.

@ - This is used to automatically enter remote SYSOP mode on

WORLI  and  NNA  type BBS's, this PMS and also  to  initiate  the

gateway password sequence on the PMS.

On  WORLI BBS's and when using this programs PMS,  when  you

normally  enter  a single @ (followed by RETURN),  you  get  back

three  lines of numbers from which you must choose one  line  and

use it to decode your password.

If  you enter your personal password as the text  for  Shift

Function  key  10,  now when you press @<CR>,  the  program  will

automatically  pick one of the three lines and use it  to  decode

the  password which it automatically sends to the remote end.  It

will  first confirm this action by popping up a Yes/No menu  just

to make sure you did not accidentally press the wrong key.

TNC Driver V 1.42               19                        21/04/95

The  PMS  password system is the same format  as  the  WORLI

password  system but using a password issued to you by the  SYSOP

of the PMS you are using.

On  NNA BBS's, when you normally enter a single @  (followed

by RETURN), you get back three, two digit pairs of numbers  which

index into an 8 by 8 matrix of characters.

If  you  enter your personal password as the text  for  Ctrl

Function key 10 (as a single 64 character string made by  joining

the  eight rows together), now when you press @<CR>, the  program

will automatically detect the three pairs of digits and use  them

to decode the password which it automatically sends to the remote

end.  It  will first confirm this action by popping up  a  Yes/No

menu  just to make sure you did not accidentally press the  wrong

key.

The program detects which type of BBS/PMS you are  connected

to and which password routine to enter by counting the number  of

characters  of  the first line received from  the  remote  system

after sending the @<CR>. If it receives 8 characters, it  assumes

it   is  the  NNA's  three,  two  digit  pairs  and   sends   the

corresponding  password  back.  If more  than  8  characters  are

received,  it captures the next two lines as well and enters  the

WORLI/PMS password routine.

If  you accidentally press @<CR> and say Yes to the  pop  up

menu prompt when you really did not mean to do it, you can  abort

the automatic password sequence by pressing any key.

The  program  also operates a mail snatch facility.  If  the

program monitors a 'Mail For' beacon containing your callsign (as

stored in the TNCPMS.CFG file) it will pop up a window asking  if

you  want the mail collected. The question waits for  one  minute

before timing out and defaulting to yes. This is so that mail can

be automatically collected overnight if required. If you want the

mail  collected,  the  program  executes  a  script  file  called

'<callsign>.MAI'  where  <callsign> is the call  of  the  station

holding your mail.

eg. if the beacon

GB7MAM>BEACON/1:

Mail For : G7JJF ALL

was  monitored  and your callsign appeared in the  Mail  For

line,  the script file 'GB7MAM.MAI' would be executed  upon  your

request.  The  script  file  must be  stored  in  the  TNC\SCRIPT

directory.

TNC Driver V 1.42               20                        21/04/95

6 Specialised Function Keys

===========================

F1 - ASCII  Download.  You will be asked for the name of the

file to contain the downloaded information. If the file cannot be

created  or  you do not enter a filename,  the  command  will  be

cancelled.  If the file you specify already exists,  you will  be

given  the  opportunity to overwrite the existing file or  cancel

the download.

F2 - ASCII  Upload. You will be asked if you want to  upload

from  a  file or the clipboard. If you select file, you  will  be

asked  for the name of the file you wish to upload. If  the  file

cannot be found or you do not enter a filename, the command  will

be cancelled. Selecting clipboard will send data currently stored

in the clipboard (if it is not empty !)

During  the  upload process, the program  will  periodically

wait for the TNC to empty its buffer before it can send any  more

data.  During this wait period, you may press the RETURN  key  to

abort the upload process and regain control of the keyboard.

F3 - PAUSE DOWNLOAD. This will temporarily suspend output of

characters to the capture file. Press F3 again to resume capture.

F4 - YAPP  Download.  You will be asked for the name of  the

file to contain the downloaded information. If the file cannot be

created  or  you  do not enter a filename,  the command  will  be

cancelled.  If the file you specify already exists,  you will  be

given  the  opportunity to overwrite the existing file or  cancel

the download.  You can abort the download at any time by pressing

the  ESCAPE  key  (and again if you cannot wait  for  the  remote

station  to  acknowledge your intention to abort  the  transfer).

Please refer to the System Requirements section above  concerning

the CMDTIME and flow control settings.

F5 - YAPP Upload. You will be asked for the name of the file

you  wish to upload.  If the file cannot be found or you  do  not

enter  a filename,  the command will be cancelled.  You can abort

the  upload at any time by pressing the ESCAPE key (and again  if

you  cannot  wait  for  the remote station  to  acknowledge  your

intention  to  abort the transfer).  Please refer to  the  System

Requirements  section  above  concerning  the  CMDTIME  and  flow

control settings.

F6 - This  sends the sequence DAYT YYMMDDHHMMSS to your  TNC

(if  supported by your TNC - if not, do not use). Useful  in  the

TNCINIT.SCR file to set the TNC's clock from the computer.

TNC Driver V 1.42               21                        21/04/95

F7 - Set foreground text colour.  This is used to enter ANSI

colour  sequences  into  your text.  With this,  you  can  create

colourful BBS messages, beacon texts, etc. After pressing F7, you

choose the foreground colour using the F2,  F3 keys.  To turn all

ANSI colour attributes off,  press the F6 key.  You MUST remember

to  reset the ANSI attributes with F6 preferably before  pressing

RETURN  at the end of each line or definitely before ending  your

message.  Press  RETURN to select the current colour or ESCAPE to

default to the previous colour.

eg. to set a colourful beacon text,

BT {F7 set f/g colour}{F8 set b/g colour}TEXT{F7 F6} {CR}

You can set the foreground/background colours  independently

of  each  other and you do not need to specify both at  the  same

time.

F8 - Set background text colour.  This is used to enter ANSI

colour  sequences  into  your text. With  this,  you  can  create

colourful BBS messages, beacon texts, etc. After pressing F8, you

choose  the background colour using the F4, F5 keys. To turn  all

ANSI colour attributes off, press the F6 key (from either f/g  or

b/g  selection  menus).  You  MUST remember  to  reset  the  ANSI

attributes  with F6 preferably before pressing RETURN at the  end

of  each  line or definitely before ending  your  message.  Press

RETURN  to select the current colour or ESCAPE to default to  the

previous  colour.  You can make the text flash  by  choosing  the

corresponding  bold  background colour. See above  for  a  simple

example of selecting colours.

F9 - This  will  temporarily exit back to  the  DOS  command

prompt  (assuming  you have enough spare memory)  where  you  can

enter any DOS command. To return to the TNC program, type  'EXIT'

(and RETURN).

F10 - This  key will dump the current TNC text  (either  from

the main screen, the previous screen or a saved screen) to either

your  printer  or a disc file. It will first ask you  to  confirm

your desire to dump the screen (just in case you caught the wrong

key).  It will then ask where you want to send the  dump  (either

printer  or  disc)  and  if you choose disc it  will  ask  for  a

filename.

TNC Driver V 1.42               22                        21/04/95

7 Specialised Alt Function Keys

===============================

Alt F1 - This will display the main help screen.

Alt F2 - Set  Start Marker.  This is used to set  the  start

marker  for  a  block of text in the scroll  back  buffer.  After

pressing the Alt F2 key, use the cursor up/down keys to  position

the  highlight bar at the start of the required block then  press

RETURN.

Alt F3 - Set End Marker.  This is used to set the end marker

for a block of text in the scroll back buffer. After pressing the

Alt F3 key, use the cursor up/down keys to position the highlight

bar at the end of the required block then press RETURN.

Alt F4 - Clear Markers.  This will erase any markers set  by

Alt F2 or Alt F3.

Alt F5-Alt F8 - These  keys access four screens which can be

used  to save any text received from your TNC.  This is  done  by

accessing the scroll back buffer using the TAB key, scroll to the

required  screen of text then press ALT together with F5 to F8 to

store  the  text in which ever screen you  wish.  To  recall  the

screen at a later time, press the same key again when on the main

program screen.

Alt F9 - Toggle  PMS. This will toggle the active status  of

your  PMS. When the PMS is active, a P will appear on the  status

bar (flashing if you have mail waiting) and users will be allowed

to  access the PMS. If you toggle the PMS off, when users try  to

connect  to  your  PMS,  they will be informed  that  it  is  not

available and they will be disconnected.

Alt F10 - Dump Block. This key will dump the current  marked

text  in  the scroll back buffer to either your printer,  a  disc

file  or  the clipboard. It will first ask you  to  confirm  your

desire to dump the text (just in case you caught the wrong  key).

It will then ask where you want to send the dump (either printer,

disc  or  clipboard)  and if you choose disc it will  ask  for  a

filename.

TNC Driver V 1.42               23                        21/04/95

8 Specialised Alt Keys

======================

Alt A - Toggle  local  echo.  This is only of use  to  those

users  who use the TNC driver program to communicate with a modem

as well as a TNC (like me !).  Most computer BBS's echo the users

input  back  to their screen as well as the  TNC  driver  program

echoing  the  user  input  to the screen.  This  results  in  two

keystrokes  appearing  on the display.  Using Alt A  will  toggle

local echo on and off to stop this duplication.  Remember to turn

local  echo  back on when going back to  communicating  with  the

TNC otherwise you will not see what you are typing !

Alt B - Toggle Connect Bell. When you make a connection, the

bell will normally warble. This key will toggle this action so it

can  be  turned  off if you having a  late  night  session.  This

command  will not stop the TNC from sounding the BELL  character.

You must use the TNC's BELL OFF command to disable this.

Alt C - Connect Directory. This option allows you to make  a

connection  to either a callsign in the TNC's MH or NDH  list  or

one from a personal directory of callsigns.

Choosing  the 'Station Heard' or 'Node Heard'  options  from

the  menu  will send either a MH or NDH command to the  TNC.  The

resulting  list produced will be displayed in a window.  You  can

then use the cursor up/down keys to select a callsign to  connect

to.  You can of course press the ESCAPE key to leave  the  window

without making a connection.

If you TNC doesn't support either of these two commands, the

program also maintains it's own MH list in memory. If you  choose

the 'Monitor Heard' option, the program will displays it's own MH

listing  in  a  window.  This list will show  a  callsign  and  a

date/time  stamp of when it was last heard. You can then use  the

cursor  up/down keys to select a callsign to connect to. You  can

of course press the ESCAPE key to leave the window without making

a connection.

Alternatively, you can create a directory of call signs  and

corresponding  script files and use this key to pop up  a  window

for selecting someone to connect to. The directory must be stored

in  a  file  called TNC.DIR which must be  located  in  the  same

directory as the TNC.EXE program . The format of each line of the

file is shown below :

<name>,<script file>

eg.       Jon,g7jjf.scr

When  selecting Jon from the connect directory  window,  the

program  will execute the script file 'g7jjf.scr' which  MUST  be

located  in  a subdirectory SCRIPT below the main  TNC  directory

(see below for more information on script files).

TNC Driver V 1.42               24                        21/04/95

Alt D - Directory Browser. See below for more information.

Alt E - Edit File. Select whether you want to edit a file or

the  current contents of the clipboard. If you select  file,  you

will  be  asked for the name of a file you wish to edit.  If  you

enter  a filename which does not exist on disc, the  editor  will

create a new blank file for you otherwise the editor will load in

the  existing  file. Selecting clipboard will  edit  the  current

contents  of  the clipboard which will initially  be  empty  when

first  running the program. The clipboard can be thought of as  a

temporary file which resides in memory rather than on disc.  This

makes it quicker to access than a disc file.

The  editor  operates  in  full screen  mode,  the  same  as

described below in the directory browser section. This option  is

useful  for creating messages offline ready to ASCII upload  when

online.

Alt F - Enter WEFAX mode. See below for more information.

Alt G - Perform Script File. See below for more information.

Alt H - Help  Screen.  This will show a help screen for  the

Alt keys.

Alt I - Info  Screen.  This will redisplay  the  registration

screen showing your registered name and number. It will also show

the time when the next script file will be automatically executed

(if  active)  and  also the current  script  file  command  being

executed (if active).

Alt J - Monitor   Connects.   This  will  bring  up  a  menu

containing up to three options.

The  first option will list the last 10 stations  that  were

monitored  connected to each other.   This is picked up from  the

text  string  'G0ABC>G7XYZ:'  displayed  before  each   monitored

packet.

The  second  option  allows you to monitor both sides  of  a

connection fullscreen. This is done by stringing together all the

monitored  packets  and displaying them in a  continuous  stream.

Duplicate  packets  that  are sent as retries  are  detected  and

stripped  out  to avoid corrupting the display.  All  other  data

received  by the TNC is still processed and can be viewed on  the

scroll back buffer. To select which connection to monitor, simply

select  it from the list after choosing the monitor  option.  The

connection  list  is updated continuously and  you  can  reselect

another connection at any time.

The  third  option only appears after entering  the  monitor

mode.  This  option  cancels the current  monitor  operation  and

reverts back to displaying all monitored data.

TNC Driver V 1.42               25                        21/04/95

Alt K - Exit  Kiss  Mode.  This will send the sequence  192,

255, 192 to the TNC. On a KPC-4, this will exit KISS mode.

Alt L - Locator  Calculations.  This  will bring up  a  menu

allowing     you    to    convert    between     locators     and

longitudes/latitudes,  and  to  calculate the  distances  between

locators or between two different longitudes/latitudes.

Alt M - Monitor  Data.  This command will cycle  the  output

from the TNC between ASCII, binary and hex format. If it of  most

use when tuning for RTTY reception as described below.

Alt N - Enter notepad.  This will bring up a quarter  screen

notepad in the top right hand corner of the screen.   It actually

enters  the  file  editor on a file called NOTEPAD.TXT  which  is

created in the home TNC directory.  The notepad is active on  the

main screen and on the scroll back buffer.

Alt O - Enter PMS locally. This will allow you to connect to

your PMS. See the section below for more information on using the

PMS.

Alt P - Toggle  Printer.  This  will toggle  output  to  the

printer.  When on, all characters received from the TNC  will  be

dumped to the printer.

Alt Q - TNC  I/P  from file. This allows you to  replay  TNC

captured  text  through the program as though it  had  just  been

received  again from the TNC. This will allow you  to  re-process

the  data,  for  example, to use the Alt  J  command  to  monitor

further  conversations. Upon selecting this command, you will  be

asked  for  the file to replay. All text from this file  will  be

used  as  subsequent  TNC input until the  file  has  been  fully

processed. The program will then revert back to taking input from

the TNC.

Alt R - Enter RTTY mode. See below for more information.

Alt S - Change  Setup.  This  is  the  online  configuration

option.  Here you can change the com port configuration, the baud

rate,  the DCD setting, the video colours, the time settings  and

the  function key definitions, etc plus enter  your  registration

information when you have registered the software. After altering

any  of the settings, the configuration file TNC.CFG and  TNC.CLR

are  updated  to match the new values. If you alter  any  of  the

screen  colours,  it is advisable to rerun the  software  so  the

software can repaint the display in the correct colours.

Alt T - Send Date/Time.  This will send the sequence :

DD-MON-YY  HH:MM:SS

to   the  TNC  - useful  for  date/time  stamping  messages  when

connected to another user.

TNC Driver V 1.42               26                        21/04/95

Alt U - Encode/decode  File. This option is used  to  encode

and  decode files conforming to the UUcode protocol or the  7PLUS

protocol.  Selecting this option will show a menu where  you  can

choose to use the UUCode or 7PLUS file encoder.  The encoders can

split  the source file into any number of smaller  file  sections

and the decoders can subsequently merge any number of consecutive

file  sections into the original source file. See below for  more

information on 7PLUS coding.

Selecting the UUCode option then the encode option will  ask

you for the source filename. You can of course use the browser to

select a file by entering a wildcard file spec. After entering  a

valid  source  filename, you will be asked  for  the  destination

file.  You  must not enter an extension since the  protocol  will

automatically add an extension starting at .U01 and  incrementing

by  one  for each section of the encoded file. You will  also  be

asked for the size of each encoded section. This will default  to

4K  (4096  bytes) but you can alter this to any  size  you  wish.

However,  it is advisable to keep the value low to avoid  sending

large amounts of data over the packet airwaves.

After  entering  all the data, the encoded file(s)  will  be

generated.  They will be stored in the current directory, ie  the

one  from where the program was run or the last directory  chosen

from the browser if appropriate.

Selecting the decode option will ask you for the filename of

the  first  file  section.  You must specify  the  full  filename

including  the extension.  The initial file is assumed to have an

extension  containing  either a 01 or 1,  eg .U01  or  .UU1.  The

program  will  proceed  to  generate the  original  file  merging

together all the necessary file sections which must be present in

the  same directory.  If any file sections are missing or any  of

the  files  are invalid an error will be produced.  If  all  goes

well,  the decoded file will be written to the current directory,

ie  the one from where the program was run or the last  directory

chosen from the browser if appropriate.

Alt V - Enter SSTV mode. See below for more information.

Alt W - Toggle  Connect  Window/Status Line. When  making  a

connection,  the  callsign (and date/time of connection)  of  the

person you are connected to is shown in the connect window. (This

is  picked up by the text 'CONNECTED TO' which is  received  from

the  TNC.  If  you are not using DCD monitoring,  the  text  from

another  station connecting with someone else will be  picked  up

and  you  will be shown as connected to that station  so  do  not

worry if this happens. This text can be cleared by pressing CTRL-

Left  cursor key). If you do not wish this to be shown,  you  can

toggle  the window off thereby giving you two more lines for  the

TNC  text window. NOTE : You cannot toggle the window off  whilst

there is text in the box. Clear it first with CTRL-Cursor Left.

TNC Driver V 1.42               27                        21/04/95

On  full  screen  mode, this command will  also  toggle  the

status  line found on split screen mode and put it at the  bottom

of the screen (reducing screen height to 23 lines).

Alt X - Exit. This will exit the program. You will be  given

the chance to confirm your actions before the program exits.

Alt Y - File  Transfer. This option is only of use to  modem

users.  It provides file transfer using ASCII, XModem  or  YModem

file  transfer  protocol. The ASCII transfer is the same  as  the

normal ASCII upload/download commands described above.

There  are three different variants of the  XModem  protocol

provided. The basic one is XModem which all BBS's should support.

This transfers files in 128 byte block with only simple  checksum

error  checking.  An  enhancement to  this  is  XModem/CRC  which

improves on the error detection by using a 16 bit CRC system. The

final enhancement is XModem-1K which transfers files in 1024 byte

blocks thereby speeding up the process. This method also defaults

to 16 bit CRC as well but if the remote system doesn't support 16

bit  CRC,  it will retain the 1024 byte blocks but only  use  the

simple checksum error detection method.

You  can either select to transmit or receive a  file  using

XModem. Either option asks you for the file to  transmit/receive.

After  entering a valid filename, the transfer will  commence.  A

window will appearing showing the current status of the transfer.

This  shows  the file being transmitted/received  and  the  total

bytes sent/received. Also the start time and the current time  of

the  transfer  is  shown  together with a  CPS  rate.  After  the

transfer  is  complete,  the window will disappear  and  you  can

continue using the program as normal. You can abort a transfer at

any time by pressing ESCAPE.

You  can either select to transmit a single file or  receive

multiple  files  using YModem.  You only enter  a  filename  when

sending  a file.  The receive routine automatically picks up  the

files  you  are  receiving  as part of  the  protocol.  When  the

transfer commences,  a window will appearing showing the  current

status   of   the   transfer.   This   shows   the   file   being

transmitted/received and the total bytes sent/received.  Also the

start time and the current time of the transfer is shown together

with a CPS rate. After the transfer is complete, the window  will

disappear  and you can continue using the program as normal.  You

can abort a transfer at any time by pressing ESCAPE.

Alt Z - Toggle  Full  Screen.  This  will make  the  program

display  all input/output to a single full screen  window  rather

than two separate windows for user input and tnc output.  This is

of  most  use  to  users  who  use  the  TNC  driver  program  to

communicate  with modems as well as TNC's (like me !) to get  the

full effect when logging onto BBS's supporting ANSI graphics.  It

is also useful when you are monitoring a connection with the  Alt

J command.

TNC Driver V 1.42               28                        21/04/95

9 Directory Browser

====================

Choosing this option will bring up a powerful directory file

maintenance  window.  Here you can browse through directories  on

your  discs,  copy files,  rename files,  view files (in HEX  and

ASCII), delete files, edit files and print files etc.

The following keys operate when using the directory  browser

(note that the ^ symbol means hold down CTRL whilst pressing  the

following key, eg ^C means press CTRL-C) :

Cursor key pad - Use to move about the current display.  The

home,  end, page up and page down keys also operate to  move  the

cursor in larger steps.

Return - Use  to  select the current  highlighted  filename.

When  over  a  directory,  it will change  the  display  to  that

particular directory. When on a file, if the browser was  entered

from  a filename input box, this will select the current file  as

though it was entered in the input box.

TAB - Use  to  alter the display format.  Pressing TAB  will

toggle between a four column short display showing only filenames

and  tag  markers  and  a long display  showing  two  columns  of

filenames and corresponding file sizes.

ESC - Use  to exit from the directory browser and return  to

the previous screen.

C - Use  to  copy  the  current file (not  directory  !)  to

another  area of your disc,  optionally changing its name  during

the copy process.  You will be asked for a wildcard filename  you

wish to copy the file as.  If you are copying the file to another

directory  or disc,  you can simply default on '*.*' to keep  the

same  name.  Alternatively,  you  could  for example  change  the

wildcard  filename  to  *.BAK  to create a  backup  of  the  file

elsewhere.  You  will  next be asked where you wish to  copy  the

file. This must be the full pathname of the destination directory

which  MUST already exist.  You can specify another drive in  the

pathname if you wish. (When entering the pathname, you MUST enter

the  final '\' character unless you just press RETURN on a  blank

path to copy it to the current directory).  Finally,  you will be

asked if you wish to automatically replace existing files. If you

say YES,  any file with the same name you wish to create will  be

overwritten. If you say NO, you will be prompted to overwrite any

file which already exists of the same name.

^C - Use  to  copy  all  the tagged files  from  the  current

directory to another area of your disc.  (You must of course  tag

the required files first before using this command).  After being

asked  the  same  questions as above,  each tagged file  will  be

copied to the new pathname.

TNC Driver V 1.42               29                        21/04/95

D - Use to delete the current file or directory.  If pressed

on  a  filename,  you will be asked to confirm that you  wish  to

delete it.  If pressed on a directory name, only if the specified

directory  is  empty  will it be deleted.  You are not  asked  to

confirm the deletion of a directory.

^D - Use to delete all the tagged files.  (You must of course

tag the required files first before using this command). You will

be  asked if you wish to automatically delete all files.  If  you

answer  YES,  all  tagged files will be deleted otherwise if  you

answer  NO,  you  will be asked to confirm the deletion  of  each

tagged file.

E - Use to edit the current file.  The file editor is a full

screen editor based on the Wordstar command structure which seems

to be the most common in existence. A summary of the keys used in

the  editor  is  given in Appendix B.

F - Use to change the current drive,  directory and wildcard

filespec.  Upon  using this command,  you will be allowed to edit

the current path as shown at the top of the window.

L - Use this to quickly log onto another drive without going

through the F option described above.  You will be presented with

a  menu of all available drives including those defined with  the

SUBST  command.  Simply  select a drive to log onto  its  current

directory showing all files in it.

M - Use to make a new directory under the current directory.

You will be asked to enter the new directory name.

P - Use  to print the current file to your  printer.  Whilst

the file is being printed, you may abort the printout by pressing

a  key.  After  the printout is complete,  a page throw  will  be

performed on the printer.

^P - Use to print all tagged files to your printer. (You must

of  course  tag  the  required  files  first  before  using  this

command).  Each  file will be printed as described above for  the

single file print option.

R - Use  to rename the current file.  You will be  asked  to

enter  the wildcard filename you wish to rename the current  file

as.

^R - Use to rename all the tagged files. (You must of  course

tag the required files first before using this command). You will

be  asked  to enter the wildcard filename that each  tagged  file

will be renamed as.

T - Use  to  tag  the current file.  When a  file  has  been

tagged,  a  diamond will appear after the filename on the screen.

After a file has been tagged, it can be used with all the control

key functions described in this section.

TNC Driver V 1.42               30                        21/04/95

^T - Use to tag ALL files in the current directory. See above

for information about tagged files.

U - Use  to  untag the current file.  When a file  has  been

untagged,  the  diamond  will  disappear after  the  filename  on

screen.   Also,  the file will no longer be available for use  by

the control key functions described in this section.

^U - Use  to untag ALL files in the  current  directory.  See

above for information about untagged files.

V - Use to view the current file in ASCII/HEX mode. The file

viewer  is similar to the one provided in XTREE.  Again,  use the

cursor  keys to scroll around the file, press S to set a  marker,

press  G  to go to a marker, press H to  toggle  HEX/ASCII  mode,

press T to set the tab spacing  and press RETURN to exit back to

the browser.

TNC Driver V 1.42               31                        21/04/95

10 7PLUS File Encoding

======================

The  program supports an option to encode and  decode  files

conforming to the 7PLUS protocol which was designed by Axel Bauda

DG1BBQ  of Germany to whom I am grateful for putting  the  source

code  into  the public domain which makes it possible for  me  to

incorporate it into my software.

The  encoder  can split the source file into any  number  of

smaller file sections and the decoder can subsequently merge  any

number  of  consecutive file sections into  the  original  source

file.

The  advantages  of  7PLUS  encoding/decoding  is  that  the

protocol  can detect and automatically correct  single  character

errors  in  transmission  and also  generate  'error  files'  and

'correction  files' which allow you to only re-send the parts  of

file  that were corrupted in transmission rather than  the  whole

file again.

To  encode a file, use the Alt U option, choose 7PLUS,  then

select  Encode  File.  You  will then be  asked  for  the  source

filename, the file section size and the file end string. You  can

of  course use the browser to select a source file by entering  a

wildcard  file spec. If the source file is larger than  the  file

section size, the encoded file will be split into portions,  each

approximately the file section size in length. This will  default

to about 4K but you can alter this to any size you wish. However,

it  is  advisable to keep the value low to  avoid  sending  large

amounts  of  data over the packet airwaves. The file  end  string

will  be appended to each file section and will default to  '/ex'

which  is useful when uploading the files to a BBS. However,  you

can delete this out if required.

After  entering  all the data, the encoded file(s)  will  be

generated.  They will be stored in the current directory, ie  the

one  from where the program was run or the last directory  chosen

from  the  browser if appropriate. If the encoding  process  only

resulted  in  a single file, it will have an extension  of  .7PL.

However,  if multiple files were generated, the  extensions  will

start at .P01 and increment in hex upto .PFF.

To  decode  a file, use the Alt U option,  then  7PLUS   and

select  Decode  File.  You  will then be  asked  for  the  source

filename  and  the  file end string. You can of  course  use  the

browser to select a source file by entering a wildcard file spec.

Select  the  .7PL file if there is only one section or  the  .P01

file if there are multiple sections. The program will proceed  to

generate  the  original file merging together all  the  necessary

file sections which must be present in the same directory. If any

file sections are missing an error will be produced. If all  goes

well, the decoded file will be written to the current directory.

TNC Driver V 1.42               32                        21/04/95

If  any  errors were detected in the decoding  process,  the

program  will  either  automatically correct them if  it  can  or

generate an 'error file' called '<source_filename>.err'. You  can

then  send this file to the originator of the file to obtain  the

corresponding 'correction file'.

After   receiving  the  'correction  file'  back  from   the

originator,  to  correct your files, you must select  the  Decode

File  option  again and put <source_filename>.cor as  the  source

filename.  This will correct your files ready for  decoding  once

more. If the original corruption was bad, further error files may

be  produced and you will have to repeat the above process  until

all  errors  are removed. You can combine the  above  correction,

decoding  procedure  be specify the source  filename  without  an

extension.  If  the file with the .cor extension  exists  in  the

current  directory,  the program will automatically  perform  the

correction and decoding in one step.

If after uploading a 7PLUS encoded message to a BBS, a  user

sends  you  an 'error file' due to message corruption,  you  will

need  to  generate  a correction file for him. This  is  done  by

choosing the 'Generate Correction File' off the Alt U 7PLUS  sub-

menu.  You  will  be asked for the  source  filename,  the  error

filename  and the file end string. This source filename  must  be

the  same as the original file you uploaded. The  error  filename

will be the one sent from the user and you can of course use  the

directory  browser  to select it. The file end  string  is  again

optional  and will be appended to the resulting correction  file.

After  entering  all  the data, the  program  will  generate  the

correction  file called <source_filename>.cor which you can  then

send to the user to correct his file corruption.

If  several users send error reports, you can  combine  them

together  and  produce a single correction file for  both  users.

This is done by choosing the 'Join Error Reports' option from the

Alt  U 7PLUS sub-menu. You will be asked for the source  filename

and the second filename. Specifying both filenames will merge the

second  file  into  the  source file. If  you  leave  the  second

filename  blank, the program will look for  <source_filename>.e01

and    join   it   to   the   source   file,   then   look    for

<source_filename>.e02 and so on.

Finally,  to  aid  extracting separate 7PLUS  files  from  a

capture log, you can choose the final option from the Alt U 7PLUS

sub-menu  which is called 'Extract File'. You will be  asked  for

the  log  filename and a search string. If you leave  the  search

string  blank, all 7PLUS from the capture log will  be  extracted

and  placed  in the current directory. If you  specify  a  search

string however, only those files containing the search string  in

their filename will be extracted.

TNC Driver V 1.42               33                        21/04/95

11 WEFAX Mode

=============

Note  - This function will only work on TNC's  that  support

the  MAXUSERS,  DAYTWEAK and WEFAX commands.  Typical TNC's which

satisfy  these  conditions  are  the  Kantronics  KAM  and  KPC-4

although other TNC's may also support these commands.

You must make sure the receiver is connected to the  correct

port  on your TNC before using the WEFAX mode.  This is port 2 on

the KPC-4 and the VHF port on the KAM.

The  WEFAX  mode is suitable for  capturing,  reviewing  and

printing  either WEFAX weather pictures or FAX pictures  such  as

press  photographs.  Options  are  available  for  inverting  and

reversing  the image to obtain the best possible  picture.  Also,

automatic  alignment  is  provided to synch  the  picture  before

reception starts.

Upon  pressing  Alt  F you will be  presented  with  a  menu

containing 3 options.

11.1 Capture

------------

This  is  for displaying the received image as it  is  being

broadcast  and  optionally  capturing it  to  disc.  The  program

requires  120k of free memory to use as a receive  buffer  during

capture mode. If insufficient memory is available, a system error

will be reported and the capture routine will be aborted. If  you

encounter  this problem, you could try reducing the size  of  the

edit  buffer  or removing any TSR program you may  have  in  your

CONFIG.SYS or AUTOEXEC.BAT files before re-running the  software.

As  an  aid to seeing how much memory you have  available  whilst

running  the software, if you run the program by typing 'TNC  -d'

this  will then show on the Alt I info screen the amount of  free

memory.

After  selecting  the capture option, you will  get  another

menu asking for the resolution of the image to be decoded.  There

are options for 120 lines per minute, 240 lines per minute and  a

further  option to enter a custom sample rate. If you select  the

custom option, you need to enter the value which your TNC's WEFAX

command requires to decode the image. Most WEFAX images are  sent

at 120 lines per minute so the first option is the one you  would

normally choose.

TNC Driver V 1.42               34                        21/04/95

After selecting the image resolution, the screen will  enter

CGA mode (or Hercules if available) with a menu bar at the bottom

of the screen. As data is received, it will appear at the  bottom

of  the screen with the screen scrolling up as it is filled.  You

will of course have to tune your receiver to get the best picture

quality.  If you find that the picture is skewing to the left  or

right,  you may need to alter the DAYTWEAK value using the Alt  S

setup option. Experiment with different values until the  picture

does not skew.

The options available on the menu bar are :

F1 - Capture. This will save all subsequent received data in

an internal buffer ready for saving to disc later.  The legend on

the  F1  key will then change to PAUSE.  Pressing F1  again  will

toggle between pause mode and capture mode.  In pause mode,  data

will  not  be  added to the buffer but any data  already  in  the

buffer will remain there.

The  capture buffer is 1500 lines long - sufficient for  the

reception  of all fax transmissions.  When capture mode is turned

on, a counter will appear on the status line at the bottom of the

screen  showing  how full the buffer is.  If the  buffer  becomes

full, capture mode is automatically turned off.

F2 - Reset.  This  will clear any data stored in the  buffer

and turn capture mode off.

F3 - Save.  This will save the data in the buffer to a  file

on disc. The filename is automatically chosen by the software and

will  be in the form FAX.xxx.  xxx is the first unused number  in

the  range  001  - 999 of any FAX files already stored  on  disc.

After saving,  the buffer will be cleared and capture mode turned

off.

F4 - Clear. This will clear the screen of any image received

so far.

F5 - Invert.  This  will  invert the screen  image  swapping

black  pixels for white and white for black etc.  This is  useful

for  press photos which are sometimes transmitted  as  negatives.

When invert mode is turned on, an 'I' is added to the status line

at the bottom of the screen. Pressing F5 again will re-invert the

screen.

F6 - Reverse. This will reverse the screen image flipping is

vertically about its central axis. This is again useful for press

photos which are sometimes transmitted in reverse.  When  reverse

mode  is  turned on,  an 'R' is added to the status line  at  the

bottom  of  the  screen.  Pressing F6 again will  re-reverse  the

screen.

TNC Driver V 1.42               35                        21/04/95

F7 - Align.   This   is   for  automatically  aligning   the

transmission  ready  for  capture.   At  the  start  of  any  FAX

transmission,   a  black  bar  (appearing  white  on  screen)  is

transmitted with a small white border at each end.  This is  used

the align the reception software so this bar appears centrally on

screen.  Pressing F7 will look for this bar and adjust the screen

image accordingly. Your receiver must be tuned so the bar appears

solid  white  on  screen  for this function  to  detect  the  bar

correctly.  If  you  press  F7 by mistake and the  software  goes

hunting  for the bar,  you can press a key to abort to  hunt  and

return to normal WEFAX data reception.

The following keys also operate in WEFAX mode.

0 - 9. Pressing a digit key 0 to 9 will tell the software to

skip  that many lines when displaying data (all lines are  stored

in  the  buffer if capture mode is turned on though).  This  will

compress  the  image,  perhaps making the whole  picture  fit  on

screen,  but  making the resolution lower.  Press 0 to resume  to

full data display mode.

If  you  are  receiving an image which  is  not  central  on

screen,  pressing  the following four keys will move the image in

the designated manner.

Cursor Left - One column left

Cursor Right - One column right

Control Cursor Left - Five columns left

Control Cursor Right - Five columns Right

The  following keys will not operate if your computer has  a

Hercules display.

Alt C - Toggle the current image to CGA mode.  If already in

CGA mode, this key will do nothing.

Alt E - Toggle the current image to EGA mode.  If already in

EGA mode, this key will do nothing.

Alt V - Toggle the current image to VGA mode.  If already in

VGA mode, this key will do nothing.

Only  use these command if your monitor and computer support

the higher resolution screen modes.  If you cannot support EGA or

VGA  and  you accidentally catch ALT E or Alt V, pressing  ALT  C

should revert back to CGA but no promises can be made here.

Pressing any other key will abort WEFAX reception and return

you to the main program screen.

TNC Driver V 1.42               36                        21/04/95

11.2 Review

-----------

This  option is for reviewing fax images which have  already

been captured to disc.  Upon selecting this option, will be asked

to  enter a file name.  The software defaults to FAX.* so you can

just  press  RETURN,  if  required,  to select a  file  from  the

directory browser.  If you know the name of the file you wish  to

review, simply delete the * and enter the correct file extension.

The  program will then change to the last mode a review  was

performed  in and start to display the image in a similar  manner

to the capture option described above.

The  following keys are valid in review mode which have  all

been described above :

F4, F5, F6, 0-9, Alt C, Alt E, Alt V

After the review has taken place, the program will return to

its main screen.

11.3 Print

----------

This   option is printing fax images which have already been

captured to disc.  Upon selecting this option,  will be asked  to

enter a file name. The software defaults to FAX.* so you can just

press  RETURN,  if required,  to select a file from the directory

browser.  If  you  know the name of the file you wish  to  print,

simply delete the * and enter the correct file extension.

You  will be asked if you wish to invert the image and  also

reverse  the image.  For normal WEFAX maps,  you will reply NO to

both  question,  but  for press photos,  you may need  to  select

either or both depending how the image was transmitted.

The  image will start to print out and you will be  informed

that you can press any key to abort the printout.

After the printing has finished or you abort it, the program

will return to its main screen.

As  mentioned in the section above entitled 'Format TNC.PRN'

file,  you  may  not  be able to use  the  inbuilt  printer  dump

routine  if  the  TNC.PRN file cannot  accommodate  your  printer

control strings. If this is the case, I have included the inbuilt

printer  dump as a standalone Turbo C Ver 2 source  file  (called

FAXDUMP.C)  which you or a competent programmer friend should  be

able  to  modify  to suit your printer. I have  also  included  a

standalone printer dump program especially for the HP Deskjet 500

and laserjet compatible printers which can be found in the  UTILS

directory.

TNC Driver V 1.42               37                        21/04/95

12 RTTY Mode

============

Note  - This function will only work on TNC's  that  support

the  MAXUSERS,  DAYTWEAK and WEFAX commands.  Typical TNC's which

satisfy  these  conditions  are  the  Kantronics  KAM  and  KPC-4

although other TNC's may also support these commands.

You must make sure the receiver is connected to the  correct

port  on your TNC before using the RTTY mode.  This is port 2  on

the KAM/KPC-4.

The  RTTY  mode is suitable for receiving RTTY data  at  any

baud rate with optional tone inversion, verbose mode or true data

mode.  Please  note that RTTY mode only operates in receive mode,

it CANNOT transmit RTTY.

Alt R acts as a toggle. If you are already in RTTY mode, you

will  be returned to normal operating mode.  If you are  entering

RTTY  for  the  first time,  you will be presented  with  a  menu

containing 5 options.

These  are  for choosing the baud rate at which you wish  to

receive. The popular rates have their own separate entry, but you

can  choose any baud rate between 10 and 2000  (arbitrary  limits

with no reason for choice) with the last menu option.

RTTY  mode will then be entered.  All functions available in

normal  operation  are also available in RTTY mode  except  WEFAX

reception. You must exit RTTY mode before enter WEFAX mode.

Once in RTTY mode, the software will try and synch up to the

data  being received before displaying anything on screen.  An  S

should appear on the title bar to show you that the data is being

synched  up.  At this point you must tune your receiver until the

tones are correct for the data to be decoded properly. It is best

to  try  this on a strong RTTY signal until you get used  to  the

sound  of  the correct tones.  You can also use the  inbuilt  bit

monitoring  option accessed by pressing Alt M.  This will display

the incoming RTTY as binary 1's and 0's.  When you are getting  a

good  mixture of 1's and 0's you can be sure you are nearly tuned

to  the correct frequency.  Press Alt M to cycle  between  ASCII,

binary and hex monitored data.

Once synched up,  the S will disappear and the received data

will  be displayed. If you cannot synch the data, press a key  to

abort the synch routine. A letter N will appear on the title line

to  say that the data has not yet been synched. You  can try  and

re-synch  the data using Alt S described below or press ALT R  to

exit  RTTY  mode. As data is being received,  the  software  will

automatically try to re-synch if errors occur.

TNC Driver V 1.42               38                        21/04/95

The following keys operate in RTTY mode.

Alt I - Invert  tones.  Some RTTY transmissions are sent  as

inverted tones or normal tones.  This key will toggle between the

two  states.  '-NORMAL-' will be displayed when selecting  normal

mode  and '-INVERTED-' will be displayed when selecting  inverted

mode. The software defaults to inverted when entering RTTY mode.

Alt L - Level shift.   If you start receiving data but it is

obvious that data is using the wrong FIGURES/LETTERS level shift,

this key will toggle between the two levels.

Alt S - Synch.  If errors appear in the received data  which

are not detected,  you can force the software to resynch the data

with this function.

Alt V - Verbose mode.  This will display all data regardless

of whether the reception is in synch or not.  A V will appear  on

the title line when verbose mode is on.

TNC Driver V 1.42               39                        21/04/95

13 SSTV Mode

============

Note  - This function will only work on TNC's  that  support

the  MAXUSERS,  DAYTWEAK and WEFAX commands.  Typical TNC's which

satisfy  these  conditions  are  the  Kantronics  KAM  and  KPC-4

although  other TNC's may also support these commands.  The  baud

rate  of your TNC must be set to at least 9600 for the SSTV  mode

to work.

You must make sure the receiver is connected to the  correct

port  on your TNC before using the SSTV mode.  This is port 2  on

the KPC-4 and the VHF port on the KAM.

The  SSTV  mode  is suitable for  capturing,  reviewing  and

printing 8 second, 16 second and 32 second frame SSTV picture. 32

second  frames  can only be captured and displayed on an  EGA  or

Hercules monitor at present but future versions may overcome this

limitation.  Automatic  frame synching is provided to  synch  the

picture before reception starts.

Upon  pressing  Alt  V  you will be presented  with  a  menu

asking  for the frame speed.  At the moment,  only 8  second,  16

second  and  32 second frames are supported but other may  become

available.  After choosing which speed you require, a second menu

will appear containing 3 options.

13.1 Capture

------------

This  is  for displaying the received image as it  is  being

broadcast  and optionally capturing it to disc. The SSTV  capture

works  in Hercules or CGA mode for 8 and 16 second frames and  in

Hercules  or  EGA for 32 second frames. As data is  received,  it

will appear at the top of the screen with the screen  overwriting

each  frame  as it is received. You will of course have  to  tune

your receiver to get the best picture quality.

The options available on the menu bar are :

F1   - Synch.   This  is  for  automatically  aligning   the

transmission  ready  for  capture.  At  the  start  of  any  SSTV

transmission,  a  synch pulse is transmitted which signifies  the

start of the current frame.  Pressing F1 will look for this synch

pulse  to align the received pictures.  You must make  sure  your

receiver  is  tuned  accurately so the synch  pulse  is  detected

correctly.  If you press F1 and no picture start to appears after

a  short pause,  try tuning the receiver until a picture appears.

If  nothing  at all appears,  press a key to abort the  hunt  and

return to normal SSTV data reception.

TNC Driver V 1.42               40                        21/04/95

F2 - Save. This will save the current screen image to a file

on disc. The filename is automatically chosen by the software and

will  be in the form SSTV.xxx.  xxx is the first unused number in

the  range  001 - 999 of any SSTV files already stored  on  disc.

You  can  press  the F2 at any point while the current  frame  is

being  built  up as the save to disc only works when the  current

frame has been fully displayed.

F3 - Clear. This will clear the screen of any image received

so far.

F4 - Invert.  This key acts as a toggle and will invert  the

current screen image.

Pressing any other key will abort SSTV reception and return

you to the main program screen.

13.2 Review

-----------

This  option is for reviewing SSTV images which have already

been captured to disc.  Upon selecting this option, will be asked

to enter a file name.  The software defaults to SSTV.* so you can

just  press  RETURN,  if  required,  to select a  file  from  the

directory browser.  If you know the name of the file you wish  to

review, simply delete the * and enter the correct file extension.

You will also be asked whether you wish the image to be  inverted

or not.  Some picture are sent out as negatives which look better

when inverted.

The program will then display the saved SSTV picture and ask

you to press a key to return to the main program screen.

13.3 Print

----------

This  option is printing SSTV images which have already been

captured  to disc.  Upon selecting this option,  will be asked to

enter  a  file name.  The software defaults to SSTV.* so you  can

just  press  RETURN,  if  required,  to select a  file  from  the

directory browser.  If you know the name of the file you wish  to

print, simply delete the * and enter the correct file extension.

You will also be asked whether you wish the image to be  inverted

or not.  Some picture are sent out as negatives which look better

when inverted.

The  image will start to print out and you will be  informed

that  you  can  press any key to abort the  printout.  After  the

printing has finished or you abort it, the program will return to

its main screen.

TNC Driver V 1.42               41                        21/04/95

14 Script Files

===============

Script   files  are  very  useful  for  creating   automatic

procedures  which  can  be used for anything  from  automatically

logging  onto a BBS,  spooling all your messages to a  disc  file

then  disconnecting  to automatically forwarding mail to  another

user. The uses are only limited by your imagination.

The  basic  idea  of a script file is  for  the  program  to

monitor  the incoming text for a certain string or any one  of  a

selection  of strings then automatically replying with  something

else  or  performing a certain action. To do this,  you  need  to

create a text file containing lists of strings to monitor for and

strings to reply or actions to perform. The string you reply with

can  also  contain function or Alt key presses to  perform  their

normal action.

The format of the file consists of lines beginning with  the

following  keywords :

WHEN                     REPLY

DO                       UNTIL

GOTO                     :<label>

DISPLAY                  PRINT

READ                     IF

SCAN                     ENDSCAN

STOP                     ;

LOG                      ENDLOG

WRITELOG                 CLS

COLLECT                  FORWARD

REVERSE                  RUN

FBBCOLLECT               FBBREAD

You must make sure they are spelt correctly with exactly one

space after the keyword if it uses a parameter.

The syntax of each of the keywords is given below :

WHEN <text>

-----------

This  keyword halts execution of the script file  until  the

<text> is monitored in the incoming TNC text. If the text you are

monitoring for contains a variable number, eg a port number in  a

BBS  prompt, you can use the # wildcard character in  the  <text>

field  which  will  match any number in the  incoming  text.  For

example, if your BBS prompt is :

(2) GB7MAM>

you could use the following command to monitor for it :

WHEN (#) GB7MAM>

TNC Driver V 1.42               42                        21/04/95

REPLY <text>

------------

This keyword sends the <text> to the TNC. See below for  the

allowed characters in <text>.

DO

--

This  keyword  has  no parameters and forms a  loop  in  the

script  file  in  conjunction  with  the  UNTIL  <text>   keyword

described below.

UNTIL <text>

------------

This  keyword  signifies the end of the DO ..  UNTIL  <text>

loop.  The  statements between DO and UNTIL  are  executed  until

<text>  is monitored in the incoming TNC text. You can  nest  the

DO..UNTIL  loop up to 10 levels. See below for an  example  using

the  DO..LOOP  keywords. The <text> can contain  the  #  wildcard

character as described in the WHEN command above.

GOTO <label>

------------

This keyword causes execution of the script file to continue

at the line defined with <label>. The search for the label starts

at the top of the file and continues until the label is found  or

the  end of the script file is reached. An error is generated  if

the specified label cannot be found.

:<label>

--------

This  keyword is used in conjunction with the  GOTO  keyword

and identifies a place in the script file.

eg.

:TOP

DISPLAY Hello World !^M

GOTO TOP

will send 'Hello World !' to the display forever !

DISPLAY <text>

--------------

This keyword will display <text> in the user window  without

sending it to the TNC. This is useful for displaying any  message

to  the user about the outcome of script file operation as  shown

above !

TNC Driver V 1.42               43                        21/04/95

PRINT <text>

------------

This  keyword  will  print <text>  to  the  printer  without

sending it to the TNC. This is useful for displaying any  message

to  the user about the outcome of script file operation as  shown

above !

READ <prompt>,<subject1>,<subject2>,...

---------------------------------------

This  is  a very powerful command and is used  when  listing

mail  on  a BBS. With this command, you can get  the  program  to

automatically  issue  'read message' commands for  every  message

that contains subject information that interests you.

eg.

REPLY L^M

READ GB7BSX,IBM,BBC,DANPAC,MODS,G3ZHI

This would issue a message list command to the BBS and  then

monitor  all  the  incoming  text until  the  GB7BSX  prompt  was

monitored  AT THE START OF A LINE. For every line that  contained

any  of  the subjects listed in the READ command, a  'R  <num>^M'

command  would  be  issued where <num>  was  the  message  number

displayed at the start of each message header.

For  example,  if the following text was received  from  the

initial L^M command,

Msg# Stat Size  To    From   @ BBS  Date/Time Title

5613 B     322 ALL    GW4AZW GBR    1028/1151 Have 2 813's new,swap.

5612 B     995 BBC    G3UDA  GBR    1028/1149 MUSIC 5000 AMPLE Users

5611 B     509 ALL    G6FUM  GBR    1028/1148 AR-2001 RX MODS HELP.

5609 BI    238 ALL    G3ZHI  GBR    1028/1113 LEICESTER SHOW PARK

5607 B     490 ALL    G8VHB  GBR    1028/0935 anyone using standard C58

GB7BSX MBX (B,C,D,H,?,I,J,K,L,N,R,S,T,U,V,W) [2598] 2>

the program would send :

R 5612^M

R 5611^M

R 5609^M

to  the  BBS to read the specified messages. The  next  read

command  is only sent after the last message had been  completely

read  (ie  when  the BBS prompt as defined in  the  initial  READ

command is monitored again.) This stops some BBS systems aborting

the  current message if you issue another read command before  it

has finished sending it.

TNC Driver V 1.42               44                        21/04/95

If  your BBS prompt changes each time it is  displayed,  for

example  if it contains the connection port number, you  can  use

the # wildcard character in the prompt field which will match any

number in the BBS prompt. For example, if your BBS prompt is :

(1) GB7BSX MBX >

the above READ command would be :

READ (#) GB7BSX,IBM,BBC,DANPAC,MODS,G3ZHI

IF <text>

---------

This  keyword is used as part of the SCAN..ENDSCAN loop  and

will  be described below. The <text> can contain the  #  wildcard

character as described in the WHEN command above.

SCAN

----

This  keyword  has  no parameters and forms a  loop  in  the

script file in conjunction with the IF <text>, ENDSCAN  keywords.

Using  this  loop,  you can scan for up to  10  different  <text>

strings  at a time, and depending which one is found,  execute  a

separate  series  of script keywords. This is  very  useful  when

logging  onto  a BBS through a node for example,  where  you  can

either  monitor  for the BBS prompt or a busy  reply  and  either

continue with an automatic mail download or logoff from the  node

if the BBS is busy.

An example will best illustrate this construct.

REPLY X GB7DAD^M

SCAN

IF busy

REPLY B^M

DISPLAY GB7DAD is busy !!^M

IF GB7DAD>

REPLY LM^M

WHEN GB7DAD>

REPLY B^M

ENDSCAN

(The  indentation  can be used to add clarity  to  the  loop

constructs in the script files as the initial spaces are  ignored

by the program).

TNC Driver V 1.42               45                        21/04/95

This  script file will issue the connect request  to  GB7DAD

and enter the SCAN, ENDSCAN loop. The incoming text will then  be

monitored until one of the text strings after the IF keywords  is

encountered. Once one is found, the script keywords following the

IF  statement  are  then executed until  another  IF  or  ENDSCAN

statement  is  found.  Execution then  continues  with  the  next

statement after the ENDSCAN keyword.

Thus,  if  the  text  'BBS is busy,  please  try  later'  is

received,  the  first  IF takes control, logs off  the  node  and

display  the message to the console. It then encounters an IF  so

jumps  out  of the SCAN..ENDSCAN loop and stop execution  of  the

script file because there are no more commands to execute.

However, if the connection is made and the normal BBS prompt

is  received, the second IF takes control and continues  to  list

all  your  messages before logging off. It  then  encounters  the

ENDSCAN  statement  so jumps out of the  SCAN..ENDSCAN  loop  and

stops  execution  of the script file because there  are  no  more

commands to execute.

The  SCAN..ENDSCAN  loops can be nested up to  any  required

depth for very complicated conditional script file execution.

ENDSCAN

-------

This  keyword  has  no parameters and forms a  loop  in  the

script  file in conjunction with the IF <text> and SCAN  keywords

described  above.

STOP

----

This  keyword has no parameters and stops execution  of  the

script file without completing it.

;

-

This  keyword tells the program to ignore the  remainder  of

the characters on the line and so is used to add comments to  the

script file.

TNC Driver V 1.42               46                        21/04/95

LOG <file>

---

This command is used in conjunction with the ENDLOG command.

It  is  similar to the 'REPLY {F1}capture.log^M' command  but  is

more useful. Using the REPLY command above, the filename is fixed

all  the time so you cannot use the same script  file  containing

this  command  more  than  once, say  overnight,  to  log  on  at

different times to download your mail. Using the LOG command, you

specify  the filename, and the program automatically  creates  an

extension  for  you, starting at .000 (or if some  files  already

exist,  with  the next unused extension number)  which  increases

sequentially  every time the LOG command is issued. The log  file

is  stored in the LOG directory and all subsequent text  received

will  be  stored  in it as is normal.  Thus  the  command  'REPLY

{F1}capture.log^M' could be replaced by 'LOG capture'.

ENDLOG

------

This  keyword has no parameters and is used  in  conjunction

with the LOG command described above. It is equivalent to  'REPLY

{F1}' and closes the LOG capture file.

WRITELOG <text>

---------------

This command is used in conjunction with the LOG command. It

appends  <text>  to  the current log file  opened  with  the  LOG

keyword. If a log file is not active, this command is ignored.

As well as containing normal text, the <text> parameter  can

also  contain optional macros which when expanded can be used  to

write the current date or time to the log file as well, eg

WRITELOG Today is $D and the time is $T

will  write 'Today is 21/04/95 and the time is 19:19' to the  log

file.

CLS

---

This  command simply clears the TNC output window and  moves

the  cursor to the top of the screen.

TNC Driver V 1.42               47                        21/04/95

COLLECT <label>,<whoto1>,<whoto2>,...

-------

This keyword is used in conjunction with the FORWARD keyword

and  is used to automatically collect together mail from the  PMS

mail  files  which need forwarding to another user  or  BBS.  You

specify  which  mail  you  wish  to  forward  with  the   <whoto>

parameters.   These  are  matched  with  all  un-forwarded   mail

addressed  <whoto> @ <whoto> (matches if <whoto> is contained  in

either  callsign or home BBS). If there is mail to  forward,  the

script file continues to execute, else a GOTO <label> command  is

performed to skip the forwarding part of the script file. You may

use  the GOTO to skip to the next forward routine in  the  script

file or to exit the file.

FORWARD (BBS) (FBB)

-------

This keyword is used in conjunction with the COLLECT keyword

and performs the actual forwarding part of the procedure. If  you

specify the optional BBS or FBB parameter, either FBB  compressed

forwarding  or normal BBS forwarding will take  place,  otherwise

the  messages will be sent as though typed live at the  keyboard.

Upon  executing  the FORWARD command, the program will  send  the

messages, kill them and mark them as being forwarded.

eg.

If your PMS had the following mail,

Msg# ST  Size    To    @ BBS    From  Date/Time Title

3      295   G0HMZ  GB7FLG   G7JJF 0201/1912 Forward test

2      154   G6YAL           G7JJF 0201/1912 Hello

1     1049   G1EQT  GB7FLG   G7JJF 0201/1912 Test

the script file :

COLLECT NONE,GB7

REPLY C G6YAL-8^M

WHEN ?

REPLY X GB7FLG^M

WHEN GB7FLG>

FORWARD

REPLY B^M

:NONE

DISPLAY Finished !!

would log onto GB7MAM and forward messages 1 and 3.

You would normally create a large script file containing the

forwarding routines for all likely users and add an entry to  the

TNC.TIM  file to forward all your mail overnight to the  relevant

people.

TNC Driver V 1.42               48                        21/04/95

NOTE : Unless  you consider yourself to be an expert packet  user

who  really know's what's what, you will probably need to  ignore

the next section.

If  you  are friendly with your local BBS sysop,  you  could

arrange  for him to make you a BBS on his board. Also ask him  to

edit  White Pages and the local user database so your homebbs  is

shown   as   your   full  hierarchical  address,   eg   mine   is

G7JJF.GB7MAM.#23.GBR.EU.  This would then allow you to do  proper

forwarding  and reverse forwarding between his BBS and your  PMS.

If  this is the case, you would then need to alter to  syntax  of

the FORWARD command above to be :

FORWARD BBS

or

FORWARD FBB

if the remote BBS performs FBB compressed forwarding.

You  would  also need to alter the script file slightly  to  send

different prompt's when you are a 'BBS' user.

The  following  script file will reverse/forward mail  using  BBS

forwarding :

REPLY C GB7MAM^M       } Normal connect

WHEN GB7MAM BBS >      } bit at top

; Start of reverse/forward bit

REPLY [TNC-1.42-HM$]^M

WHEN >

REVERSE BYE            } See below

COLLECT NONE,GB7MAM,G1EQT,EU

FORWARD BBS

:NONE

REPLY B^M

:BYE

STOP

and  following  script file will reverse/forward mail  using  FBB

compressed forwarding (much simpler !) :

REPLY C GB7MAM^M       } Normal connect

WHEN GB7MAM BBS >      } bit at top

; Start of reverse/forward bit

REPLY [TNC-1.42-BFHM$]^M

COLLECT NONE,GB7MAM,G1EQT,EU

:NONE

FORWARD FBB

Upon executing either script file, the program will  receive

any waiting mail and send any local messages (either as bulletins

or  as  personal  messages), kill them and  mark  them  as  being

forwarded.

TNC Driver V 1.42               49                        21/04/95

Note  that the FORWARD FBB command  automatically  downloads

mail waiting for you so there is no need for the REVERSE command.

Also,  you  are  automatically disconnected  from  the  BBS  when

forwarding is finished so the commands to disconnect you as shown

in  the  first script file are not required. This  makes  writing

script  files to perform FBB compressed forwarding  much  simpler

and straight forward.

REVERSE <label>

-------

This  command is again only for expert users and is used  to

request  messages  or bulletins from BBS's to  be  collected  and

stored in the local PMS. You will need to arrange with your local

BBS sysop for him to create a forwarding file for you.

Upon executing this command, the program will issue requests

for messages and stores them in your PMS until there are no  more

to   be  received.  I  have  noticed  that  I  occasionally   get

disconnected when downloading messages this way due to slow links

so  I  have added a label parameter. If you do get cut  off,  the

program  will automatically issue a GOTO <label> to skip  to  the

next bit of the script file.

If you are performing FBB compressed forwarding, you do  not

need to use the REVERSE command to request mail since the FORWARD

FBB  command  described above automatically  downloads  all  mail

waiting for you.

RUN <command>

---

This   keyword   will  execute  <command>  to   run   either

a  .EXE, .COM, .BAT file or an internal DOS command whilst  still

remaining  in the TNC driver program. If you do not  have  enough

free  memory when using this keyword, the <command> will  not  be

executed.

eg.       RUN DEL MYFILE.DAT

FBBCOLLECT <label>,<subject1>,<subject2>,...

----------

This keyword is used in conjunction with the FBBREAD keyword

and is used to automatically scan the list of grabbed FBB  Header

broadcasts  for  mail  which interests you (see  below  for  more

information on header broadcasts). You specify which messages you

wish to read with the <subject> parameters which are matched with

all unscanned message headers. If there are messages which  match

any  of  the  parameters in your subject list,  the  script  file

continues  to  execute,  otherwise  a  GOTO  <label>  command  is

performed to skip the next part of the script file.

TNC Driver V 1.42               50                        21/04/95

FBBREAD <label>

-------

This  keyword  is used in conjunction  with  the  FBBCOLLECT

keyword  and  is used to download the matched messages  from  the

local BBS. If the download process gets disconnected due to  slow

links,  the  program will automatically issue a GOTO  <label>  to

skip  to  the next bit of the script file.

The following script file shows an example of the FBBCOLLECT

and FBBREAD commands in operation :

FBBCOLLECT NONE,GB7MAM,JJF,MUBAY,BPQ,INTERNET,CDROM,CD-ROM

REPLY C GB7MAM^M

WHEN >

REPLY [TNC-1.42-$]^M

WHEN >

FBBREAD NONE

REPLY B^M

:NONE

The  scanned message pointers are automatically  updated  as

each  message is downloaded so next time the script file is  run,

only new message headers will be scanned.

TNC Driver V 1.42               51                        21/04/95

14.1 Sample Script Files

------------------------

A  simple  example may illustrate the use  of  script  files

better.

REPLY {F1}capture.log^M       (or LOG capture)

REPLY C GB7DAD^M

WHEN GB7DAD>

REPLY LM^M

WHEN GB7DAD>

REPLY B^M

WHEN cmd:

REPLY {F1}                    (or ENDLOG)

The first line shows how to include function keys in  script

files.  You can include cursor,  function,  shift function,  ctrl

function,  alt  function  or alt keys in this way.  Each  key  is

enclosed in { } characters.

A shift function key is defined by {SF1} to {SF10}

A ctrl function key  is defined by {CF1} to {CF10}

An alt function key  is defined by {AF1} to {AF10}

An alt key           is defined by {AA} to {AZ}

Cursor up            is defined by {CU}

Cursor down          is defined by {CD}

Cursor left          is defined by {CL}

Cursor right         is defined by {CR}

Cursor home          is defined by {CH}

Cursor end           is defined by {CE}

Page up              is defined by {PU}

Page down            is defined by {PD}

You  can also use other keys which are not included  in  the

above list by specify the high byte of its extended key code as a

number enclosed in { } 's,  eg.  the extended key code for shift-

Tab (to recall the history list) is 0x0f00.  You would enter {15}

to simulate pressing the shift-Tab key.

The  first line of the example script file above thus  says,

simulate F1 to start downloading a file.  The filename is entered

as capture.log followed by return.  Note the use of control  keys

in  the text strings.  You can use ^A to ^Z,  ^[ is escape and ^.

pauses for 0.5 seconds (excellent for creating rolling demos).

The  second  line  then issues a connect  to  GB7DAD,  again

followed  by  return. You must remember to put ^M at the  end  of

each line where you would normally press return.

The  third  line then monitors the incoming text  until  the

GB7DAD>  prompt is detected. A LM command is then issued to  list

all  my messages. The prompt is then monitored for, to know  when

all  the  text has been displayed. The B (bye)  command  is  then

issued.  After  the cmd: prompt has been received,  the  download

file is closed by simulating the F1 key again.

TNC Driver V 1.42               52                        21/04/95

You  may  find it useful to build up a collection of  script

files for connecting to users through nodes which requires  cross

connects or port specifiers.

You could thus have a script file such as

REPLY C G6YAL-8^M        (Issue connect)

WHEN Help ?              (Wait for prompt line)

REPLY X GB7DAD^M         (Issue cross connect)

A more complicated script file is :

REPLY MH^M

DO

SCAN

IF G1EQT

DISPLAY Ian is about !^M

IF G6YAL

DISPLAY Phil is about !^M

IF cmd:

;     Do nothing - but exits SCAN..ENDSCAN loop

ENDSCAN

UNTIL cmd:

DISPLAY End of MH^M

This  script file will issue an MH command and  monitor  the

list. If it sees anyones callsign it recognises, it will  display

an appropriate message. The text is scanned until the cmd: prompt

appears at the end of the listing. The IF cmd: is needed to  exit

the  inner SCAN..ENDSCAN loop and then the same text is  used  to

exit the outer DO..UNTIL loop.

TNC Driver V 1.42               53                        21/04/95

You  can get very complicated when using script  files,  for

example,  checking for busy nodes or no BBS ports available  when

connecting.  I  will  leave you to work out  what  the  following

script file does !

REPLY C G6YAL-8^M

SCAN

IF ?

;   Connected

IF busy

STOP

ENDSCAN

REPLY X GB7DAD^M

SCAN

IF GB7DAD>

;   Connected

IF RETRIED

REPLY B^M

STOP

IF ###LINK BUSY AT NODE G6YAL-8

REPLY X DANPAC^M

SCAN

IF DANPAC   #

REPLY BBS^M

SCAN

IF Please try later

REPLY B^M

STOP

IF GB7DAD>

;           Continue

ENDSCAN

IF busy

REPLY B^M^CD^M

STOP

ENDSCAN

ENDSCAN

LOG DADAUTO

REPLY L^M

READ GB7DAD,G7JJF,DANPAC,SOF,IBM,BBC,MOD,NOD,NET,SYSOP

ENDLOG

REPLY B^M

DISPLAY Finished !!^M

STOP

TNC Driver V 1.42               54                        21/04/95

I have had numerous request from people asking for copies of

script files which I use to perform my mail forwarding and  other

bits.  The following script files are ones I use on a day to  day

basis.

BYE.SCR

-------

REPLY ^MJon.                                 Message Sent {AT}^M

REPLY /=====================================\^M

REPLY | G7JJF @ GB7MAM.#23.GBR.EU      AX25 |^M

REPLY | jon@g7jjf.demon.co.uk      Internet |^M

REPLY +-------------------------------------+^M

REPLY |      Providing Support For :        |^M

REPLY |TNCV142, MUTNC205, MUBAY102, BPQAX25 |^M

REPLY \=====================================/^M

REPLY /EX^M

I  use this script file when signing off my messages  and  I

call it by pressing Ctrl-F1, setup with the following :

"{AG}C:\TNC\SCRIPT\BYE.SCR^M","Exit MSG"

GB7MAM.FWD

----------

REPLY C GB7MAM^M

WHEN >

REPLY [TNC-1.42-BFHM$]^M

COLLECT NONE1,G1EQT,GB7,.EU,DANPAC,.USA,GBR,.CAN,NZL,.

:NONE1

FORWARD FBB

This   script   file   connects  to   my   local   BBS   and

reverse/forwards all mail using FBB compressed forwarding.

GB7MAM.FBB

----------

FBBCOLLECT NONE,GB7MAM,JJF,MUBAY,BPQ,INTERNET,CDROM,CD-ROM

REPLY C GB7MAM^M

WHEN >

REPLY [TNC-1.42-$]^M

WHEN >

FBBREAD NONE

REPLY B^M

:NONE

I  sometimes  use this script file to scan the list  of  FBB

header  broadcasts and automatically connect to my local  BBS  to

grab  them.  However, I prefer to use  the  interactive  facility

described below.

TNC Driver V 1.42               55                        21/04/95

14.2 Automatic Script Files

---------------------------

You  can get the program to automatically perform  a  script

file when started by running the program as :

TNC -S<filename>

where <filename> is the name of a script file, eg.

TNC -Sstartup.scr

Alternatively,   if   the  program  finds  a   file   called

TNCINIT.SCR  in  a  directory called SCRIPT under  the  TNC  home

directory, this will also be executed automatically at start  up.

This  could be used, for example, to set the TNC's date and  time

by  issuing an F6 command from the script file. Also, the  script

file TNCEXIT.SCR will be automatically executed when the  program

exits if one exists in the SCRIPT directory.

My TNCINIT.SCR

--------------

REPLY ^M

WHEN cmd:

REPLY {F6}

WHEN cmd:

REPLY CMSG ON/ON^M

WHEN cmd:

REPLY BEACON EVERY 15/EVERY 15^M

WHEN cmd:

REPLY CTEXT [TNC-1.42-BFHM$]^V^MIf I don't reply soon, please

type PMS to enter my PMS^V^M>^M

WHEN cmd:

REPLY MON ON^M

WHEN cmd:

REPLY {AZ}{AW}^M

My TNCEXIT.SCR

--------------

REPLY ^M

WHEN cmd:

REPLY CMSG PBBS/PBBS^M

WHEN cmd:

REPLY BEACON EVERY 255/EVERY 255^M

WHEN cmd:

REPLY BTEXT G7JJF (Jon) Sutton In Ashfield, Notts.^M

WHEN cmd:

REPLY CTEXT Hello, Jon is not around.....Transferring you to his PBBS.^M

WHEN cmd:

REPLY MON OFF^M

WHEN cmd:

TNC Driver V 1.42               56                        21/04/95

15 Personal Mail System

=======================

Included in the TNC driver program is an integrated Personal

Mail  System.  It  includes all the  normal  BBS  facilities  for

message  sending,  YAPP/ASCII  file upload  and  downloads,  mail

forwarding, 'Mail For' beacon, chat mode, REQDIR, REQFIL etc plus

others  such as running external programs via DOS gateways  which

are not normally found on BBS systems.

Due  to the current licensing conditions in the UK, it  also

disallows  third party mail by only accepting incoming  mail  for

the  sysop.  However, the sysop can send mail  to  anyone.  Other

countries  may  allow  third  party  mail  so  this  facility  is

configurable.

The following sections will describe how to configure, setup

and run the PMS.

15.1 Configuring the PMS

------------------------

The  configuration  for the PMS is stored in a  file  called

TNCPMS.CFG. It is a standard text file and should be edited  with

the  inbuilt  file  editor or a text editor  that  does  not  add

formatting  characters  to  the text (such as  Wordstar  in  non-

document mode).

The  first  line is your callsign and is  used  to  identify

yourself when using the Alt O command to log onto the PMS locally

and also when using the mail snatch facility.

eg.                 G7JJF

The  second line is the name you wish to give to  your  PMS.

This  is displayed on the welcome screen when a user connects  to

your PMS.

eg. Jon's Personal Mail System

The  following lines identify the file upload  and  download

areas  you  wish to set up. Each line comprises of  five  fields,

each separated by a comma.

Field  1  - File Area Designator. This is a  four  character

string  which uniquely identifies the area. The order you  define

the  file  areas is the order they are display to  the  user.  In

keeping  with  the standard for identifying file areas,  I  would

recommend you use numbers from 1-9999.

Field 2 - File Area Directory. This is the complete pathname

(including  drive if necessary) of the directory  containing  the

files for this area.

TNC Driver V 1.42               57                        21/04/95

Field 3 - File Area Description. This text string is used on

the  W command to tell the user what type of files are  contained

in the file area.

Field  4  - Download Flag. This can be either a Y or  N  and

tells the system if file downloads are allowed from this area.

Field 5 - Upload Flag. This can be either a Y or N and tells

the system if file uploads are allowed to this area.

eg.  1,C:\TNC\PMS\DOWNLOAD,Download Area,Y,N

You must terminate the list of file areas with a single line

containing a '*'.

The  next  three  lines in the configuration  file  are  the

prompt lines sent out for each of the three user levels : normal,

expert  and sysop. They can contain any text you wish as well  as

certain  pieces  of  system information. The  following  list  of

macros will include the corresponding piece of system information

at the relevant point.

$A - Number of active PMS messages

$C - Callsign of user

$D - System date

$H - Home BBS of user

$M - Next PMS message number

$N - Name of user

$T - System time

$U - Number of PMS users

eg.       Cmd:B,C,D,E,G,H,I,K#,KM,L,LM,R#,RM,S,U,V,W,X >

Cmd: >

Ok $N, wot now ?

The first line is for normal users and displays all possible

commands.

The  second  line  is  for  expert  users  and  displays  an

abbreviated prompt line.

The last line is for Sysops and display a more user friendly

prompt.

The  next  line  in the configuration file  is  your  normal

beacon  text setting. This is needed so that a 'Mail For'  beacon

can be appended to it and used as your current beacon text.

TNC Driver V 1.42               58                        21/04/95

The  next section of the configuration file  identifies  any

DOS  gateway programs you wish to set up. Each line comprises  of

four fields, each separated by a comma.

Field 1 - Gateway  Area Designator. This is a single  letter

from  A  to Z which uniquely identifies the area.  You  need  not

specify the areas in alphabetical order.

Field 2 - Gateway  Command.  This is the  command  which  is

executed  when  the  gateway is called. It may  be  the  name  of

a .EXE, .COM or .BAT file. You must specify the complete pathname

(including  drive if necessary) of the directory  containing  the

command for this gateway.

Field 3 - Gateway Area Description. This text string is used

on  the G command to tell the user which gateways  are  available

for use.

Field 4 - Password  Flag.  This can be either a Y or  N  and

tells the system if a password is required before giving the user

access  to  this gateway. This is only really used  when  calling

sensitive programs such as a DOS shell.

eg. T,D:\TNC\GATEWAYS\TIME\SHOWTIME.EXE,Wot is the Time ?,N

You  must terminate the list of gateway areas with a  single

line containing a '*'.

See  the  section  below  for more  information  on  how  to

configure programs for use as gateway programs.

The  next  line  in  the configuration file  is  a  Y  or  N

character  which decides if the PMS accepts third party  mail  or

not. For normal UK operation this MUST be set to N.

The  next  line  in  the  configuration  file  is  a  string

identifying  your QTH. It is used in the routing  information  on

forwarded  mail and can be anything you like really but limit  it

in length to about 20 characters. You must make sure though  that

the total length of the routing information is not longer than 80

characters  which  can be checked by monitoring  your  forwarding

sessions.

eg.       [Sutton In Ashfield]

The next line contains a single Y or N character and informs

the software whether you wish it to generate the TNCPMS.LOG file.

For  people using only floppy discs instead of hard  discs,  disc

space  may  be  short so you may want to  disable  the  log  file

generation by putting a N here.

TNC Driver V 1.42               59                        21/04/95

The  next  line is a number telling the  software  the  size

(in K)  of  the  largest file which a user may  REQFIL.  I  would

recommend a value of 40 here but it can be anything you want.

The final line is a number telling the software the  maximum

size (in K) of a message which can be sent to a user in  response

to  a REQFIL request. If the requested file is larger  than  this

figure, the file will be split over several messages, all of this

size.  The  actual message size will be only  approximately  this

figure due to the extra information the uucode and 7plus encoding

routines add to the message. I would recommend a value of 4  here

but it can be anything you want.

A sample TNCPMS.CFG file is thus :

G7JJF

Jon's Personal Mail System

A,C:\TNC\PMS\DOWNLOAD\GENERAL,General Download Area,Y,N

C,F:\GRPHCGA,CD-ROM CGA Graphic Programs     89 files,Y,N

E,F:\GRPHEGA,CD-ROM EGA/VGA Graphic Programs 94 files,Y,N

G,F:\GRPHUTIL,CD-ROM General Graphic Programs 47 files,Y,N

H,F:\GAMEADVE,CD-ROM Adventure Games         143 files,Y,N

I,F:\GAMEARCA,CD-ROM Arcade Games            270 files,Y,N

J,F:\GIFS,CD-ROM GIF Pictures            336 files,Y,N

T,C:\TNC\PMS\DOWNLOAD\TNC,TNC Driver Support Files,Y,N

U,C:\TNC\PMS\UPLOAD,Upload Area,N,Y

V,F:\VIRUSERS,CD-ROM Virus Protection Programs,Y,N

*

Cmd:B,C,D,E,H,I,K#,KM,L,LM,R#,RM,S,U,V,W,X >

Cmd: >

Ok $N, wot now ?

G7JJF (Jon) S-In-A, Notts. ** TNC Driver Support Board **

A,D:\TNC\OBJ\PMS\GATEWAYS\ADVENT\ADVENT.BAT >NUL,Adventure Game,N

M,C:\TNC\MURFY.BAT,Murphy's Law,N

Q,D:\TNC\OBJ\PMS\GATEWAYS\QUOTE\QUOTEANS.BAT,Quote Of The Day (ANSI),N

R,D:\TNC\OBJ\PMS\GATEWAYS\QUOTE\QUOTEASC.BAT,Quote Of The Day (ASCII),N

S,D:\TNC\OBJ\PMS\GATEWAYS\SHELL\SHELL.BAT >NUL,DOS Shell,Y

T,D:\TNC\OBJ\PMS\GATEWAYS\TIME\SHOWTIME.EXE,Wot is the Time ?,N

Z,D:\TNC\OBJ\PMS\GATEWAYS\ZODIAC\ZODIAC >AUX,Daily Horrorscope,N

*

N

[Sutton In Ashfield, Notts]

Y

40

4

TNC Driver V 1.42               60                        21/04/95

15.2 Setting up the PMS

-----------------------

There  is very little to do in setting up the Personal  Mail

System.

If  you have not done so already, create the  PMS  directory

structure  as  shown  in  section  2  above  and  configure   the

TNCPMS.CFG file accordingly.

You   may  also  want  to  customise  the  main  help   file

TNCPMS.HLP,  the ? help files HELP\TNCPMS.xxx and the  info  file

TNCPMS.INF

You  can create an optional 'Message of the Day' file.  This

is a plain ASCII text file which is dumped to the user when  they

log  on. This must be called TNCPMS.DAY and is stored in the  PMS

directory.  You can alter the contents of this file as  often  as

you wish.

For  all  your file areas, you can also create  an  optional

'area information' file. Again, this is a plain ASCII text  file,

must  be called <area>.INF where <area> is the name given to  the

area in the TNCPMS.CFG file and stored in the directory AREAINFO.

When the user issues a 'W <area>' command, the <area>.INF file is

also displayed below the header line and before the list of  file

names.  This feature can be used to tell the user,  for  example,

that  all  files  in  the  area  are  .ZIP  files  and  must   be

decompressed with PKUNZIP before use.

If you want to bar anyone from your PMS, you may create  the

file  TNCPMS.BAD  which  is an ASCII  text  file  containing  one

callsign  per  line which cannot access the PMS. You  can  either

specify  a  full callsign or part of a callsign. If  you  specify

only  a part of it, any callsign trying to access the  PMS  which

starts with the callsign you specified will be denied access.

The  PMS  can also operate as a closed system  whereby  only

callsigns  you  specify  are allowed access to the  PMS.  If  you

create  an  ASCII  text  file  called  TNCPMS.OK  containing  one

callsign  per line, only those callsign may access the  PMS.  You

can either specify a full callsign or part of a callsign. If  you

specify  only a part of it, only callsigns which starts with  the

callsign you specified will be allowed access.

If  you  require automatic mail forwarding,  you  must  also

create  the mail forwarding script files as discussed  under  the

script  file section above and amend the TNC.TIM file to run  the

forwarding files at the required times.

If  you want other users to be able to reverse forward  mail

you  may  need to create a forward file list for  them.  This  is

simply a file containing a list of subject matters (one per line)

which  interest the user. These are then called '<call>.lst'  and

are stored in the TNC\PMS\FORWARD directory.

TNC Driver V 1.42               61                        21/04/95

eg.  if G7JJF wanted to be able to reverse forward  messages

relating  to  DANPAC, TCPIP, IBM, you would then  create  a  file

called G7JJF.LST which contained :

DANPAC

TCPIP

IBM

Any  message that was addressed either to or at any  of  the

above  subjects  would then be considered for forwarding  if  the

particular message had not been read or forwarded already.

If  no forward list file exists for a particular  user  when

they try to reverse forward mail, they will only receive messages

addressed to their callsign.

The final operation to perform is to log on locally for  the

first time and identify yourself as the sysop of the PMS.

TNC Driver V 1.42               62                        21/04/95

15.3 Logging Onto The PMS

-------------------------

The PMS can be accessed in one of two ways.

The  first is by using the Alt O function to log on  locally

as the sysop.

The second is for other users to log on. This is done by the

remote  user connected to your callsign as though wanting a  chat

as opposed to wanting to use your node or TNC's PBBS. If you  are

not  about, the user can then type 'PMS' (or 'BBS') to enter  the

mail system. You may wish to alter your CTEXT parameter to inform

the user of this, eg.

CTEXT If I do not reply within 30 seconds, please type PMS

to enter my Personal Mail System.

(Also  make sure CMSG is set to ON as well or the  user  may

either be disconnected or dumped to your TNC's PBBS).

Because  you may want a small initial chat to tell the  user

about  your new PMS, the PMS will be accessed if the remote  user

types  PMS  on a blank line within the first five lines  that  he

types.

There  are of course a few restrictions in when the PMS  can

be accessed.

Firstly,  the PMS is only active when you are  running  this

program.  You  may  therefore wish to  use  the  TNCINIT.SCR  and

TNCEXIT.SCR files to setup the above CTEXT and CMSG when  running

the program and reset them when leaving the program so users  are

aware when the PMS is running and not.

Secondly, this program is not multitasking, ie it cannot  be

used as a normal TNC driver and a PMS AT THE SAME TIME, ie if you

are  using  the  program to connect to a  BBS  or  another  user,

someone  cannot  connect to your PMS. Also, you  cannot  use  the

program  to connect to your own PMS over the air. (Well  you  can

but  the text the PMS sends out will be received back  as  though

typed by you and it will get totally confused !)

Finally,  after  the user (either locally or  remotely)  has

entered  the PMS, the normal TNC driver part of the program  will

be disabled and you cannot use it as such until the user logs  of

the PMS.

TNC Driver V 1.42               63                        21/04/95

15.4 Using The PMS

------------------

After  the user logs on to the PMS, one of five  things  may

happen.

1. If the sysop has decided not to install the PMS, the user

will  by told that the PMS is unavailable and he will  be  logged

off.

2. If the sysop has toggled the PMS offline with the Alt  F9

key,  the  user will be told that the PMS is offline and  to  try

later.

3. If the users callsign is one contained in the  TNCPMS.BAD

file,  he will be told that he cannot access the PMS and will  be

logged  off.  If the TNCPMS.OK file is being used and  the  users

callsign  is not contained in it, he will be told that he  cannot

access the PMS and will be logged off.

4.  If the user has been excluded from the PMS with  the  EU

<call>  E  command described below, he will be told that  he  has

been excluded from the PMS and will be logged off.

5. The user will be welcomed to the PMS !

If it is the first time the user has called the PMS, he will

be asked to enter his name (it must be at least three  characters

long).

The  user will then be shown some systems statistics  before

being given the command prompt.

All  the  time the remote user is logged onto the  PMS,  the

sysop  still  has control over the keyboard as well.  This  means

that the sysop can enter commands as though they were coming from

the remote user. This can be used to guide new users through  the

PMS by showing them what commands do what, or the remote user can

be logged off if he mis-uses the PMS etc.

Whilst  the  user  is in the PMS, you  will  see  on  screen

exactly  what  the remote user is seeing. If you have  the  split

screen  on, the users commands will appear in the  bottom  window

with the PMS output in the top window.

A user can be given one of three different levels.

A  normal  user  and  expert  user  have  the  same  command

privileges  but  the  expert user can have  a  different  command

prompt as configured by the sysop. The expert/normal user  status

can be altered by the user.

The third level is the sysop level. The sysop has access  to

more  commands used for maintaining the PMS and also a  different

command prompt (if configured).

TNC Driver V 1.42               64                        21/04/95

15.5 Commands In The PMS

------------------------

@ - Enter sysop mode. (sysop only command)

If a user has been configured as a sysop, he may enter sysop

mode with this command. Please note that a sysop logging into the

system  remotely  (including you !) does not gain access  to  the

sysop  facilities  until he uses the @ command. This is  to  stop

people  borrowing  your own callsign and wrecking havoc  on  your

system.

After  entering the @ command, the user will get back  three

lines of numbers from which he must choose one line and use it to

decode  the password. The password string he uses is the one  you

generated for him using the EU <call> P command described  below.

To  enter  the  password, for each of the eight  numbers  in  the

chosen  line,  the  users  counts up to  that  character  in  the

password  string  (starting at 0 being the first  character)  and

enters it. After typing all 8 characters and pressing return,  if

the  user got the password correct he will enter sysop  mode.  If

the user got the password wrong, he will stay as a normal user on

the PMS.

*DOS Commands (sysop only commands)

-----------------------------------

*DIR (directory)(filespec) - Display file directory

This  command  will  display the files  in  the  current  or

specified directory which match the optional filespec (defaulting

to *.*)

*TIME (HH:MM:SS) - Set or display the computers time

With  no  parameter, this command will display  the  current

system time of the computer.

Specifying  a  time  will set the  computers  time  to  that

required.

*DATE (DD-MM-YY) - Set or display the computers date

With  no  parameter, this command will display  the  current

system date of the computer.

Specifying  a  date  will set the  computers  date  to  that

required.

The format of the displayed date and the format in which you

must  enter a new date to change it will depend on  your  country

and should default to your normal method of entering dates.

TNC Driver V 1.42               65                        21/04/95

*CD (directory) - Set or change the current drive/directory

With  no  parameter, this command will display  the  current

logged on drive and directory of the computer.

Specifying  a  directory (with optional drive  specifier  as

well) will set the computers current directory to that required.

*MD <directory> - Create a new directory

This  command  will create the specified  directory  on  the

computer if it can.

*RD <directory> - Remove a directory

This  command  will delete the specified  directory  on  the

computer if it can.

*TYPE <filename> - Display a file on-screen

This  command  will display the contents  of  the  specified

ASCII  file  on  screen. Remember that you can still  use  the  A

(abort listing) command described below to stop the screen output

if you wish.

*DEL <filename> - Delete a file

This command will delete the specified file on the  computer

if it can. Please note that you cannot use wildcard here and only

single files can be deleted at a time.

*REN <oldfile> <newfile> - Rename a file

This command will rename the specified file on the  computer

to the new filename given. With this command you can also  rename

the file to a different directory on the same drive but not  onto

another drive. Please note that you cannot use wildcard here  and

only single files can be renamed at a time.

*COPY <oldfile> <newfile> - Copy a file

This command will copy the specified file on the computer to

the new filename given. Please note that you cannot use  wildcard

here and only single files can be copied at a time.

*HELP - Display DOS command help summary.

This will display a summary of the above commands on  screen

and give you the syntax for each of them for reference.

A - Abort.

The  current  listing being received from the  PMS  will  be

aborted and the user will be returned to the command prompt.

TNC Driver V 1.42               66                        21/04/95

B - Log off the PMS.

Disconnecting has the same effect.

The program will then revert back to being used as a  normal

TNC driver program. The 'Mail For' beacon will be updated and re-

sent to the TNC.

C - Chat to the sysop if he is available.

(Command not available in local mode)

The  sysop will be paged by the bell ringing once  a  second

for  thirty seconds. If the sysop is around, he can start  typing

at the keyboard to chat to the remote user. If the sysop does not

reply within the thirty seconds, the remote user will be told the

sysop  is  unavailable and he will be given  the  opportunity  of

sending  him a message instead. Either the user or the sysop  can

press Ctrl Z when finished with the chat to return to the PMS. If

the  user  disconnects during the chat, you MUST press  Alt X  to

exit the PMS back to the TNC driver program.

CM <message num> <callsign> - Copy message.

This  is  used  to copy a message to  another  user  without

retyping it in. Sysops can copy a message to anyone but users can

only copy messages to the sysop. The user will be asked for a new

subject but the content of the message will remain unaltered.

eg. CM 5 G7JJF

DA <area> <file> - Download file.

This  is used to download a file using ASCII  transfer.  The

area can be up to a four character string as defined by the sysop

in the TNCPMS.CFG file.

eg. DA 1 HELP.TXT

DS - Display PMS stats.

This will display several stats about the PMS, eg number  of

users and active messages etc.

DU - Display all user stats.

This will display stats about all the users of the PMS,  eg.

callsign, name, homebbs, number of times called etc.

DU <callsign> - Display user stats.

This will display stats about an individual PMS user in  the

same format as the DU command above.

eg. DU G7JJF

TNC Driver V 1.42               67                        21/04/95

DY <area> <file> - Download file.

(Command not available in local mode)

This  is  used to download a file using YAPP  transfer.  The

area can be up to a four character string as defined by the sysop

in the TNCPMS.CFG file.

eg. DY 1 HELP.COM

EC - Toggle colour status

This  is  used  to  toggle the  status  of  a  user  between

accepting colour from the pms or not. At the moment, the pms does

not  operate  in  colour  but  future  versions  may  add  colour

capability.

eg. EC

EH <homebbs> - Edit home bbs.

This will change the home bbs of the current PMS user.

eg. EH GB7MAM.#23.GBR.EU

EN <name> (H <homebbs>) - Change user info.

This  will change the current PMS users name and  optionally

his home bbs.

eg. EN Jon              or

EN Jon H GB7MAM.#23.GBR.EU

EM is for sysops only

---------------------

EM <num> U - Make unread

This will make the status of the message unread, ie. it will

appear on a RM command, etc.

eg. EM 1 U

EM <num> R - Make read.

This will make the status of the message read, ie. the  same

as reading the message.

eg. EM 1 R

TNC Driver V 1.42               68                        21/04/95

EM <num> F - Make forwarded.

This  will  make  the status of the  message  to  have  been

forwarded, ie. it will not be COLLECTed in the forwarding  script

file.

eg. EM 1 F

EM <num> N - Make not forwarded.

This  will make the status of the message not to  have  been

forwarded, ie. it will be COLLECTed in a forwarding script file.

eg. EM 1 N

EM <num> A - Make alive.

This will reverse the affect of the kill message command.

eg. EM 1 A

EM <num> K - Make killed.

This is the same as the kill message command.

eg. EM 1 K

EM <num> P - Toggle private/public status of message

If  the  message  was originally a private  one,  this  will

toggle to make it accessible to the public and vice versa.

eg. EM 1 P

EM <num> S - Set next message number

This  command  sets the next message number to the  one  you

specify.  You cannot set it below a message number  that  already

exists in the pms unless the pms is empty.

eg. EM 100 S

EU is for sysops only

---------------------

EU <call> N <name> (H <homebbs>) - Create/Edit user

This  is used to create a new user or edit the details of  a

current  PMS user. You can specify his name and  optionally,  his

home bbs.

eg. EU G7JJF N Jon                      or

EU G7JJF N Jon H GB7MAM.#23.GBR.EU

TNC Driver V 1.42               69                        21/04/95

EU <call> H <homebbs> - Create/edit user.

This  is used to create a new user or edit the details of  a

current PMS user. You need to specify his home bbs.

eg. EU G7JJF H GB7MAM.#23.GBR.EU

EU <call> D - Delete user.

This  is  used  to  delete a PMS  user.  You  cannot  delete

yourself (ie the first user in the PMS system)

eg. EU G7JJF D

EU <call> B - Toggle BBS status

This is used to toggle the status of a user between being  a

BBS and not being a BBS. This is used to allow a BBS to enter the

PMS  from  a  connect without typing the  PMS  string  and  going

straight  into  the mail transfer routines. See  below  for  more

information on BBS's and mail transfer.

eg. EU GB7MAM B

EU <call> S - Toggle sysop status

This  is used to toggle the status of a user  between  sysop

level  and non sysop level. Use with caution since  anything  you

can do, a remote sysop user can also do.

eg. EU G7JJF S

EU <call> X - Toggle expert status

This  is used to toggle the status of a user between  expert

user and normal user. At the moment, the only difference  between

an  expert  user  and a normal user is that  they  get  different

command prompts as defined in the TNCPMS.CFG file.

eg. EU G7JJF X

EU <call> P - Generate Password

This  is used to generate a password for a user in order  to

enter sysop mode and access password protected gateway  programs.

The  password is unique to each user and is sent to a  disc  file

called <callsign>.pwd in your PMS directory. You should then make

the  user  aware of this password then delete the  password  file

since it is no longer needed. The same password will be generated

for each user every time this command is used.

eg. EU G7JJF P

TNC Driver V 1.42               70                        21/04/95

EU <call> C - Toggle colour status

This  is  used  to toggle the status  of  the  user  between

accepting colour from the PMS or not. At the moment, the PMS does

not  operate  in  colour  but  future  versions  may  add  colour

capability.

eg. EC G7JJF C

EU <call> E - Toggle exclude status

This  is used to temporarily exclude a user from the PMS  if

he misbehaves for example. The user record is still maintained on

the  PMS and you can reinstate the user by issuing  this  command

again. You cannot exclude yourself with this command.

eg. EU G7JJF E

F(V) <num> <area> <filename> - Convert message to a file.

This is used to convert a mail message to a named file in  a

particular  file  area. The area can be up to  a  four  character

string  as  defined  by the sysop in the  TNCPMS.CFG  file.  This

command  is  a  sysop only command as well.  If  the  optional  V

parameter  is used, the message routing information is stored  in

the file as well.

eg. F 4 123 GAME.BAS

G - List gateways.

This will list the gateways as configured in the  TNCPMS.CFG

file.

G <area> - Enter gateway.

This  command  is  used to enter the  gateway  and  run  the

specified  program.  The  area must be specified as  A  to  Z  if

configured  by the sysop. If the gateway is  password  protected,

the  user  will  be asked to enter a password.  (No  password  is

required  when running the gateway in local mode). He  must  then

type  @<CR>. He will receive back three rows of numbers. He  then

chooses one line and taking each of the eight pairs of numbers in

turn,  types  that character from his password string  (with  the

password  string starting at character 0 going to character  63).

After entering all eight characters, he should then press return.

If  he  entered the password correctly, he will then  be  allowed

access to the gateway.

eg. G A

TNC Driver V 1.42               71                        21/04/95

H - Display help file.

This  command  will display the PMS\TNCPMS.HLP file  if  one

exists.

? <command> - Display help on <command>.

This command will display the PMS\HELP\TNCPMS.<command> file

if one exists.

eg. ? LM

I - Display info file.

This  command  will display the PMS\TNCPMS.INF file  if  one

exists.   Use   it  to  show  your   stations   hardware/software

configuration and anything else you may wish to tell the user.

K <num> - Kills message.

This  will kill a message if it exists. Users can only  kill

message either sent to them or sent from them.

eg. K 327  Kills message number 327.

KM - Kill my mail.

This will kill all mail addressed to you that you have read.

K 0 - kill all message base (sysops only).

This  command  will delete all the messages in  the  system.

They cannot be recovered after being deleted.

L ( <num> <num> ) - Lists messages.

This will list all messages in reverse order, ie. newest  to

oldest which you can read. If two message numbers are  optionally

specified,  only  those messages between the two limits  will  be

listed.

LL <num> - Lists last messages.

This will list the last <num> messages in reverse order, ie.

newest to oldest which you can read.

LM - List Mine.

This will list all messages addressed to you.

LK - List killed. (sysops only).

This   will  list  all  messages  that  have  been   killed.

Individual  messages  can be recovered by using the  EM  <num>  A

command.

TNC Driver V 1.42               72                        21/04/95

L< <callsign> - List messages from user.

This will list all messages that are addressed from the user

you specify which you can read.

L> <callsign> - List messages to user.

This  will list all messages that are addressed to the  user

you specify which you can read.

R <num> - Read message.

This command will read a message.

eg. R 325 reads message 325

RK <num> - Read killed message (sysops only).

This command will read a killed message.

eg. RK 325 reads killed message 325

RM - Read Mine.

This command will read all your unread messages.

RA - Read all messages.

This command will read all messages that you are allowed  to

read.

NOTE  -  In  any of the above four read commands,  if  the  R  is

replaced with a V, the equivalent read command is performed but a

verbose  listing of the message routing information is  displayed

as well.

S <call> - Same as SP <call>

SP <call> - Send personal message to station <call>.

This is used to send a personal message to another user. The

PMS  will prompt for the message title and then for  the  message

text. The text entry is ended with a ctrl-Z or /EX. Only the user

who sent the message or the recipient of the message can read  it

(as well as Sysops of course).

eg. SP G7JJF

TNC Driver V 1.42               73                        21/04/95

SB <call> - Send bulletin to station <call>.

This  is  used to send a bulletin to another user.  The  PMS

will prompt for the message title and then for the message  text.

The  text entry is ended with a ctrl-Z or /EX. Normal  users  can

only  mail  the sysop but a sysop can mail to  anyone  (including

ALL) if the third party mail option is disabled otherwise  anyone

can send mail to anyone. Anyone can read a bulletin message.

eg. SB ALL

SR - Send reply to last message you read.

This  will  send a message to the user who  wrote  the  last

message  you read. It will automatically address the  message  to

the user @ his home bbs as found in the user database and  retain

the same subject title as before. You then enter the message text

as described under the S command above.

eg. SR

SPR - Send personal reply to last message you read.

This will send a personal message to the user who wrote  the

last message you read. It will automatically address the  message

to  the  user @ his home bbs as found in the  user  database  and

retain  the  same  subject title as before. You  then  enter  the

message text as described under the S command above.

eg. SPR

SD <call> - Send directory listing to <call>. (Sysop only)

This  is  used to manually reply to a  REQDIR  request  from

another  user. The PMS will prompt for the directory  area  (with

optional  filespec)  which  the user wants  information  on.  The

resulting personal message(s), in the same format as the W <area>

(filespec)  command, will then be placed in the PMS ready  to  be

forwarded out .

eg. SD G7JJF @ GB7MAM

Area: 102 *.doc

will send a message listing all .DOC files in file area  102

to  G7JJF  @  GB7MAM. The resulting listing will  be  split  into

several messages if there are a lot of files.

If  you  just  RETURN at the Area  prompt  without  entering

anything,  the user will be sent a message listing all your  file

areas instead in the same format as the W command.

TNC Driver V 1.42               74                        21/04/95

SF <call> - Send file to <call>. (Sysop only)

This  is  used to manually reply to a  REQFIL  request  from

another  user.  The PMS will prompt for the area and  file  (with

optional  encoding  flag)  which  the  user  wants  sending.  The

resulting  personal  message(s) will then be placed  in  the  PMS

ready to be forwarded out.

eg. SF G7JJF @ GB7MAM

File: 102 ANALYSE.ZIP 7

will send message(s) containing the file ANALYSE.ZIP encoded

using  7PLUS.  The resulting message(s) will be split  into  file

sections  as configured in the TNCPMS.CFG file.

See  the  section below for more information on  REQDIR  and

REQFIL.

T - Tidy message base (sysops only).

This  will remove all killed messages from the system.  They

cannot be recovered after being deleted. If you have created  the

OLDMAIL  directory  under  the  PMS\MAIL  directory,  the  killed

message will be archived here.

UA <area> <file> - Upload file.

This is used to upload a file using ASCII transfer. The area

can  be up to a four character string as defined by the sysop  in

the TNCPMS.CFG file.

eg. UA 1 MYFILE.TXT

UY <area> <file> - Upload file. (Not available in local mode)

This is used to upload a file using YAPP transfer. The  area

can  be upto a four character string as defined by the  sysop  in

the TNCPMS.CFG file.

eg. UY A MYPROG.EXE

V - Show version.

This will show what version of the PMS is running.

W - List file areas.

This will list the file areas as configured by the sysop  in

the  TNCPMS.CFG  file. Each area will indicate  whether  you  can

download and upload to it.

TNC Driver V 1.42               75                        21/04/95

W <area> (filespec) - List files (matching filespec).

This  will list all the files matching the  optional  filespec

(defaulting to all files) contained in that particular area.  The

area can be upto a four character string as defined by the  sysop

in  the  TNCPMS.CFG file.

eg. W 1 *.ZIP

X - Toggle expert level.

This will toggle the level of the user between expert  level

and  normal  level. Expert level normally gives  shorter  command

prompts and normal level gives full command prompts.

TNC Driver V 1.42               76                        21/04/95

15.6 Maintaining the PMS

------------------------

How  you maintain your PMS is really up to you. I  can  only

give you common sense advice which may or may not seem obvious to

you.

It  is advisable to keep your message base tidy by  removing

killed  messages using the T - tidy command. Also, kill and  tidy

any messages which have been forwarded or read.

Also,  check  any  uploads made by users  for  viruses  etc,

before  putting  them into a download area  for  general  access.

Check  that there is no breach of copyright on the program  being

offered for download.

Analyse the log file TNCPMS.LOG to see what users have  been

doing in terms of uploading/downloading, running gateways etc. to

see  what  is  popular  and  what is  not.  If  you  are  feeling

adventurous,   you  may  want  to  write  programs   which   will

automatically  analyse  the log file and perhaps send  thank  you

messages  to people uploading files, and warnings to  people  who

are  hogging  the  PMS for downloads etc. You  could  also  write

programs  to  produce top ten lists of PMS  users,  most  popular

gateway programs or files etc. The log file may be deleted at any

time but you may wish to archive it off for later analysis.

If  you  wish  to  totally start the  PMS  from  scratch  by

deleting users and messages, you must delete all three files :

TNCPMS.SYS, TNCPMS.USR and TNCPMS.IDX

and everything in the MAIL directory.

None  of the above three files are user editable  so  please

don't  try and modify them as you will most probably destroy  the

system information contained in them.

Finally,  PLEASE don't reboot your computer  whilst  running

the  program or you will probably corrupt the PMS data files.  If

you  find  the PMS asking for your name again  after  logging  in

before,  the  files have been zapped. To recover from  this,  you

will  need  to  restart  the PMS  again  by  deleting  the  files

mentioned above.

TNC Driver V 1.42               77                        21/04/95

15.7 Gateways

-------------

A  gateway is a door from the PMS to the outside world.  You

can  use  it to run other DOS programs whilst still  in  the  PMS

environment.  At  the simplest level, you could  have  a  program

which  just  displays some text to the user as  though  they  had

downloaded an ASCII file. Alternatively, your program could be  a

fully  interactive game both sending and receiving  text  to/from

the PMS user.

There are two types of program which you may run. Those that

are  aware of the PMS and those that are not. The former type  of

program  must be custom written but can access  information  from

the  PMS  environment  such  as  PMS  user  details  and   system

information.  Also,  the  program  could  detect  when  the  user

disconnects  and act accordingly. Program which are not aware  of

the  PMS  (such  as a DOS shell, or  daily  quote  generator  for

example) cannot be personalised with the PMS user details.  Also,

problems  may  occur if the user disconnects whilst  using  these

types  of programs since they cannot look out  for  disconnection

messages. However, if used properly, gateway programs can be  fun

and very useful things.

Please note however, the program you are trying to run  must

use  DOS for all its console i/o. Programs which use the BIOS  or

write directly to screen RAM cannot be run as a gateway program.

15.7.1 Configuring Simple Gateway Programs

------------------------------------------

The  basic way for running gateway programs is  to  redirect

all console i/o to the serial port.

For  programs only producing text as opposed to reading  it,

you  can  simply run the program sending all output  to  the  AUX

device,

eg.  ZODIAC >AUX

However, for programs requiring both input and output, it is

best  to use the DOS CTTY command. As an example,  the  following

batch file will run a DOS shell as a gateway :

@ECHO OFF (only dos 3.30 and above)

CTTY AUX

COMMAND

CTTY CON

This can then be configured into the TNCPMS.CFG file as :

eg. S,D:\TNC\OBJ\PMS\SHELL.BAT >NUL,DOS Shell,Y

(Note  that this gateway is password protected as it can  be

rather dangerous)

TNC Driver V 1.42               78                        21/04/95

Note  the use of >NUL in the above example as well.  If  you

are  using  a  version of dos below 3.30, you need  this  so  the

initial ECHO OFF command is not displayed on the screen.

While  ever a gateway program is running, your  screen  will

always show what the remote user is typing and seeing. You  still

have  control over the keyboard so can still type for him as  you

can  on the normal PMS. Also, all gateways programs will work  in

both  local and remote mode without any modifications  to  either

the program or configuration files.

15.7.2 Writing Custom Gateway Programs

--------------------------------------

If  you are a competent programmer (or know a man  who  is),

you can create gateway programs which can interact directly  with

the  PMS  program.  This  is accomplished  by  writing  all  your

programs  console  i/o to use the INT 14 handler which  has  been

extended  to provide routines for accessing the PMS program.  You

must  not use direct serial port poking as this will  bypass  the

PMS interface.

The modified spec for the INT 14 handler which I am using is

shown below :

AH = 0 - Initialise Serial Port

-------------------------------

Entry Parms : None

Exit  Parms : BX = 'JW'

AH = Line Status Register

(Bit 0 = 1 if data available from user)

AL = Modem Status Register

This  is  used to check for the presence of the  TNC  driver

program running by looking for the returned value in BX.

AH = 1 - Send Character

-----------------------

Entry Parms : AL - Character to send

Exit  Parms : AH = Line Status Register

AL = unchanged

This is used to send a character. If a remote user is  using

the  PMS,  the  character is sent to the TNC and  echoed  to  the

screen. If the PMS is in local mode, the character is only echoed

to the screen.

TNC Driver V 1.42               79                        21/04/95

AH = 2 - Read Character

-----------------------

Entry Parms : None

Exit  Parms : AH = Bit 7 = 1 if no character available

AL = character read

This is used to read a character. If a remote user is  using

the  PMS,  a character is read from the serial port  (if  one  is

available).  If no character is available or the PMS is in  local

mode,  a character is read from the keyboard. No screen  echo  is

performed.

AH = 3 - Read Serial Port Status

--------------------------------

Entry Parms : None

Exit  Parms : AH = Line Status Register

(Bit 0 = 1 if data available from user)

AL = Modem Status Register

AH = 4 - Read Line of Text

--------------------------

Entry Parms : DS:DX - Buffer to receive text

BX - Maximum number of characters to read

Exit  Parms : CX = 1 - User disconnected

This will read a line of text from the keyboard if in  local

mode, or from the serial port and keyboard if in remote mode.  If

the PMS detects that the user has disconnected, CX returns 1.

AH = 5 - Send Line of Text

--------------------------

Entry Parms : DS:DX - Buffer containing text

BX = 0 - Remote user

1 - Local user

CX - Length of string in buffer

Exit  Parms : CX = 1 - User pressed A to abort listing

This is used to send text to the user. Remember to include a

return  character if necessary. You may use the standard  control

character format allowed in strings, ie ^M for a carriage return.

The  maximum length of string which can be displayed at one  time

is 2048 characters.

TNC Driver V 1.42               80                        21/04/95

AH = 6 - Read PMS Status

------------------------

Entry Parms : None

Exit  Parms : ES:DX - Pointer to PMS User Data Structure

BX = 0 - Remote User

1 - Local User

This is used to retrieve information from the PMS about  the

current logged on user. The format of the data structure is :

typedef struct {

char callsign[8];

char name[10];

char homebbs[31];

char status;          Bit 0 - 1 = Sysop, 0 = Not

Bit 1 - 1 = Expert, 0 = Normal

Bit 2 - 1 = BBS, 0 = Not

Bit 3 - 1 = Accepts colour, 0 = Not

char level;           Currently unused

long lastcalled;      In time() format

int times_called;

int num_upload;

long size_upload;

int num_download;

long size_download;

} USER;

A  small  example file is included with  the  release  files

showing  how this interface can be user. It is called  SHOWTIME.C

and  is  a  simple time of day program. It can be  added  to  the

gateway list by the following configuration line :

eg. T,D:\TNC\GATEWAYS\TIME\SHOWTIME.EXE,Wot is the Time ?,N

If you require more information on writing gateway programs,

please contact me for further advice.

TNC Driver V 1.42               81                        21/04/95

15.8 BBS access to PMS

----------------------

The  PMS  has  the ability to  perform  reverse  and  normal

forwarding between itself and other BBS's (including other  users

of this program).

In  order to perform the mail transfer, the other  user  (or

BBS)  has to enter the PMS. Depending on the status of the  user,

the  connection  can take place in one of two ways.  (The  status

being a BBS or not as defined with the EU <call> B command).

If  the  status  of the user is a  BBS,  after  the  initial

connect, if the user sends it's BBS id string, eg.  [NNA-1.09-$],

it will automatically enter the mail transfer routines. With some

types  of BBS's, you may also need to put the PMS ID string  into

your CTEXT as well as an initial '>' prompt so it knows you are a

BBS when logging in. See the example of my TNCINIT.SCR file above

to see how to do this.

If the characters BF appear in the BBS id string, the system

will enter FBB compressed forwarding mode and mail transfers will

progress  automatically.  The  mechanics of  FBB  forwarding  are

beyond the scope of this manual (good get-out clause that !)  and

any interested parties are advised to consult the manual for  the

FBB program for more information.

If the user is not a BBS, after the initial connect, he must

send  the normal PMS string. Once inside the PMS, the user  sends

it's BBS id string, eg. [TNC-1.42-$]. The prompt will then change

to a > and we are ready for mail transfer.

To  reverse forward mail, the user types 'F>'.  The  program

then finds the first message in the PMS that the user has not yet

received. It will send out a header line identifying the message.

The  user will then send either 'OK' if it wants to  receive  the

message or 'N-BID' if not. If the user wants it, the program will

send  the message and mark it in the PMS as being forwarded.  The

user  then keeps issuing a 'F>' until there are no more  messages

to  be  received  at which the point  the  program  replies  with

'*** DONE'.

To forward mail, the user sends the header line  identifying

the  message and the PMS replies 'OK' or 'N-BID' accordingly.  If

OK, the user then sends the message etc and repeats the above for

each message that needs to be sent.

Once all mail has been transferred, the user then types  the

'B' command to log off the PMS.

NOTE : Once inside the mail transfer routines, all other PMS

commands are disabled except for 'B'.

TNC Driver V 1.42               82                        21/04/95

This procedure for reverse/forwarding mail is  automatically

handled  by  the use of the REVERSE and FORWARD BBS  keywords  if

transferring  between  two copies of this program.  Normal  BBS's

should also have this procedure built into their software.

An  example script file to transfer mail between two  copies

of this software is thus :

REPLY C G7JJF^M          } Issue connect

WHEN my PMS              } When get CTEXT

REPLY PMS^M              } Enter PMS

WHEN now ?               } When get PMS prompt

REPLY [TNC-1.42-$]^M     } Send ID

WHEN >                   } When get prompt

REVERSE NONE             } Collect all my mail

COLLECT NONE1,G7JJF      } Any for Jon ?

FORWARD BBS              } Send

:NONE1

REPLY B^M                } Bye when finished

:NONE

DISPLAY Finished !!^M

or if you want to perform compressed mail forwarding between  two

copies of this software, you can use :

REPLY C G7JJF^M          } Issue connect

WHEN my PMS              } When get CTEXT

REPLY PMS^M              } Enter PMS

WHEN now ?               } When get PMS prompt

REPLY [TNC-1.42-BFHM$]^M } Send ID

COLLECT NONE,G7JJF       } Any for Jon ?

:NONE

FORWARD FBB              } Send and collect mail

TNC Driver V 1.42               83                        21/04/95

15.9 REQDIR and REQFIL

----------------------

The PMS supports REQDIR and REQFIL requests from users  with

optional automatic encoding of binary REQFIL requests into UUcode

or 7PLUS code format.

To  activate  the REQDIR or REQFIL requests,  the  PMS  must

receive  a  message  addressed  to  you  with  the  subject  line

detailing  the specific request. The message content is  ignored.

The  message  can enter the PMS either through a  direct  connect

using  the  SP command or from a message been forwarded  in  from

another BBS.

REQDIR - Request a directory listing

------

To  request a directory listing, the subject must be of  the

format :

REQDIR <area> (filespec) @ <users homebbs>

where  specifying <area> will list all files  (matching  optional

filespec)  in that particular area or leaving <area>  blank  will

give a list of available file areas.

eg.

REQDIR @ GB7MAM

will put a message into your PMS, addressed to the users callsign

at his homebbs of GB7MAM, listing all available file areas.

REQDIR 102 *.DOC @ GB7MAM

will put a message into your PMS, addressed to the users callsign

at  his homebbs of GB7MAM, listing all DOC files in area 102.  If

there  are a lot of files in a particular area, the user will  be

sent multiple messages, each one containing part of the  listing.

This is to keep the individual message size down to a  reasonable

level.

REQFIL - Request a file

------

To  request  a particular file, the subject must be  of  the

format :

REQFIL <area> <filename> (U)|(7) @ <users homebbs>

where  <area> is a valid area, <filename> is a file in that  area

and  U  or  7  tells the program to send  the  file  back  either

UUencoded  or  7PLUS encoded. If a U or 7 is not  specified,  the

file is sent as ASCII.

TNC Driver V 1.42               84                        21/04/95

eg.

REQFIL 102 ANALYSE.ZIP 7 @ GB7MAM

will  put  message(s)  into  your PMS,  addressed  to  the  users

callsign  at his homebbs of GB7MAM, containing the 7PLUS  encoded

file ANALYSE.ZIP stored in area 102.

REQFIL 102 INDEX.TXT @ GB7MAM

will put a message into your PMS, addressed to the users callsign

at  his  homebbs of GB7MAM, containing the ASCII  file  INDEX.TXT

stored in area 102.

You  can place a limit on the size of the files  transferred

and define the size of each encoded file section as described  in

the TNCPMS.CFG section above.

After the messages have been put in your PMS, they should be

forwarded to the user via your local BBS or collected by the user

if they can connect direct.

You can manually perform a REQDIR or REQFIL operation in the

PMS  using  the SD and SF commands. This is useful  is  the  user

sends you a message asking for a file or directory listing rather

than using the 'official' way of doing it.

TNC Driver V 1.42               85                        21/04/95

16 Online Cross Reference Help Facility

---------------------------------------

The  program  supports  an on  line  cross  referenced  help

facility  which  the  user can use for  whatever  purpose  he  so

desires.  Typical examples of use would be to create a help  file

of  TNC  commands  and  their usage  or  a  file  containing  BBS

callsigns,  sysop's details, their location, software  they  use,

etc.

The help system has been made compatible with the help files

produced for use with the PAKET4 program so if you have a copy of

that  as  well, you can plug in the existing help  files  without

resorting to creating your own.

The  idea  of  the  help system is to  create  a  text  file

containing   a  subject  line  followed  by  a  short   narrative

describing that particular subject and repeating this process for

however  much information you want to include in the  help  file.

Upon  calling  the help system, the program displays  a  list  of

subject  headers  and the user can scroll through  this  list  to

select  a subject of interest. Once found, the program will  then

display  the  narrative  associated  with  the  subject.  If  the

narrative  also contains cross referenced keywords, you can  also

automatically see the narrative of any keyword you wish to select

rather than going through the subject index.

16.1 Creating a Help File

-------------------------

The  help  file (called TNC.HLP and stored in the  main  TNC

directory)  is  a standard ASCII text file which can  be  created

with  the inbuilt file editor or a text editor that does not  add

formatting  characters  to  the text (such as  Wordstar  in  non-

document mode). Please note though that these help files can  get

very  large  and there is a 64K file size limit to  the  internal

editor.  If  you find your help files are nearing this  limit,  I

would suggest that you try an alternative editor which can handle

larger  files.  There are several such editors available  in  the

shareware  market and perhaps one of the more popular ones  is  a

program  called QEDIT which can be found on most  bulletin  board

systems.

Throughout the following discussion, it may be advisable  to

read it whilst the sample help file included with the  evaluation

copy is on screen. You can use the command 'ED TNC.HLP' whilst in

the TNC directory to view/edit the file.

The  first  two lines of the help file are  reserved  for  a

footer  and  header to the help menu box. They can  each  contain

whatever information you require and are usually used to identify

the version of the help file in use and also as a title line  for

the subject headers.

TNC Driver V 1.42               86                        21/04/95

Below  this  is  the first subject line  (ignore  the  lines

starting  with  '*'s.  These only act as comment  lines  and  are

ignored by the program). Each subject line is identified with the

first  character  being the '@' symbol. The text  following  this

character is then used as the subject line and is displayed in on

help  subject index screen. The format of the line is  free  text

but make sure any columns of information line up with the  header

line from the top of the file if required.

Below  the  subject line, you can specify up to  20  further

lines  which  are used as narrative to  describe  the  particular

subject.  Again, this is format is free but make sure  the  lines

are no longer that 76 characters.

To  include keywords in the narrative, enclose the  relevant

text in [ and ] characters. The text must match exactly with  the

start  of  a subject line in order for the keyword search  to  be

effective. See the example of the Function Keys narrative to  see

how this is achieved.

After specifying what narrative you require for the subject,

you can then repeat the sequence with another subject etc.  until

your help file contains all the information you require.

16.2 Using the Help System

--------------------------

To  access  the help system, simply press the  Home  key  to

bring up the subject index. The first time you access the system,

there will be a short delay whilst the program builds an internal

index  into  the help file. Subsequent accesses during  the  same

program run however, will be instantaneous.

You can use the cursor keys to scroll through the help index

until   you   find  a  subject  you   require   information   on.

Alternatively,  if  you  start  typing  text,  the  cursor   will

automatically jump to the subject line which starts with the text

you  specify. As you continue adding more characters, the  search

will be refined until you find the subject you require.

Once you have found a subject of interest, press the  return

key to bring up the narrative about the subject. If the narrative

contains  keywords, you can use the cursor keys to select one  of

them. You can then press the return key to automatically take you

to  the  narrative  screen for that  particular  keyword  without

having to go through the index again.

To return to the subject index screen, press the escape key.

You  can  then  select other subjects of interest  or  press  the

escape key again to return to the program.

TNC Driver V 1.42               87                        21/04/95

17 FBB Header Broadcast Facility

================================

The  FBB-BBS  software has a facility  for  sending  unproto

lists  of  messages  headers which are  broadcast  every  time  a

message is received by the BBS.

The  TNC driver software monitors for these  broadcasts  and

builds up a list of message headers in a disk file. You can  then

pull  up this list and navigate it using the cursor keys  marking

messages  of interest which the software can  then  automatically

download from the BBS and place in your PMS.

The broadcasts are in the form :

GB7MAM>FBB:

2089   B   3233 PROPAG@EU     OK1HH  950421 FORECAST ONDREJOV

2090   B   3265 C64   @EU     PA2NJC 950421 to: disk maker

etc..

Every time the software monitors a packet addressed to  FBB,

it  checks to see if the headers have already been  recorded  (by

checking  if  the message number of the last header  received  is

below that of the current header) and if not, it appends them  to

a  disc file called GB7MAM.FBB stored in the LOG directory.  Each

BBS  broadcasting  in  this way will have its  own  log  file  of

course.

The  program also maintains an associated index file  called

GB7MAM.IDX (also stored in the LOG directory) which contains  the

message number of the last header received and the message number

of  the  last  header  scanned with  the  FBBCOLLECT  command  or

downloaded  with FBBREAD/interactively, whichever occurred  last.

This  second  message number is therefore used  to  identify  new

messages  which  have  been  captured since  the  last  scan  was

performed.

Please  note that both these files are not editable. If  you

try and edit them, corruption will possibly occur. After a  time,

the  .FBB file will grow quite large after several days worth  of

headers have been stored. Simply delete both .FBB and .IDX  files

to start again.

Before  browsing  the list of message headers,  if  you  are

going  to actually select messages to download, you will need  to

manually  connect to the BBS in question first. This was done  to

avoid having to write further script files to connect to the  BBS

before accessing the list. Makes life simpler thats all !

TNC Driver V 1.42               88                        21/04/95

To pull up the list of message headers, press the Page  Down

key. You will be presented with a directory browser defaulting to

LOG\*.FBB  which will show the available files to  browse.  After

selecting  a  file  (or press ESC to  cancel),  the  screen  will

display (in reverse message number order) a list of the last  200

message  headers  received with a help bar at the bottom  of  the

screen.

Each header is displayed in the following format :

2090   B   3265 C64   @EU     PA2NJC 950421 to: disk maker

2089   B   3233 PROPAG@EU     OK1HH  950421 FORECAST ONDREJOV

etc..

You  can  scroll through the list using  the  normal  cursor

keys. To mark a message for retrieval, select it with the  cursor

keys  and  press the space bar which will show a  diamond  marker

beside  the message header. Press the space bar again  to  remove

the marker. After marking all the messages you want  downloading,

press  the  Return  key. This will  start  the  message  download

process  which  retrieves each message in turn and places  it  in

your local PMS where you can review them later. If you decide not

to  retrieve any marked messages, press Escape which will  cancel

the operation.

TNC Driver V 1.42               89                        21/04/95

18 Trouble Shooting

===================

It  is  very unlikely that you will encounter  any  problems

when running this software. However, due to the complex nature of

writing and testing software, it is virtually impossible to  test

out all combinations of everything that could possibly happen  at

the  same time and so I cannot guarantee that there are no  small

bugs  lurking  somewhere in the code. If you  do  find  something

strange  that is happening, please let me know about it so I  can

correct it if a problem exists.

There  are, however, errors than can occur due to  incorrect

configuration of the software.

When  you  run the program, you may get an 'out  of  memory'

error  message.  This  is  self explanatory  and  you  could  try

reducing the size of the edit buffer or removing any TSR  program

you may have in your CONFIG.SYS or AUTOEXEC.BAT files before  re-

running  the  software. As an aid to seeing how much  memory  you

have  available  whilst  running the software,  if  you  run  the

program by typing 'TNC -d' this will then show on the Alt I  info

screen the amount of free memory.

You may also get an occasional 'system error' box pop up  if

you  try to log onto an empty drive in the directory browser  for

example  or turn on printer output without a  printer  connected.

This  will  give  you  to opportunity  to  Retry  or  Ignore  the

operation and by pressing R or I enough times, the error box will

disappear.

Other pop up error message may also occur but these will  be

self explanatory.

TNC Driver V 1.42               90                        21/04/95

Appendix A - Summary of Function Keys

==========   ------------------------

F1  - ASCII Download           Alt F1  - Help Screen

F2  - ASCII Upload             Alt F2  - Set start block marker

F3  - Pause Download           Alt F3  - Set end block marker

F4  - YAPP Download            Alt F4  - Clear markers

F5  - YAPP Upload              Alt F5  - Save/recall screen 1

F6  - DAYT YYMMDDHHMMSS        Alt F6  - Save/recall screen 2

F7  - Set ANSI f/g text colour Alt F7  - Save/recall screen 3

F8  - Set ANSI b/g text colour Alt F8  - Save/recall screen 4

F9  - DOS Shell                Alt F9  - Toggle PMS availability

F10 - Dump screen              Alt F10 - Dump block

Alt A - Toggle Local Echo      Alt N - Enter notepad

Alt B - Toggle connect bell    Alt O - Enter PMS locally

Alt C - Connect directory      Alt P - Toggle printer output

Alt D - Directory browser      Alt Q - TNC I/P from file

Alt E - Edit file              Alt R - Toggle RTTY mode

Alt F - Enter WEFAX mode       Alt S - Online configuration

Alt G - Perform Script         Alt T - Send DD-MON-YY  HH:MM:SS

Alt H - Display ALT help       Alt U - Encode/Decode File

Alt I - Info Screen            Alt V - Enter SSTV mode

Alt J - Monitor Connects       Alt W - Toggle connect window

Alt K - Exit KISS mode         Alt X - Exit program

Alt L - Locator calculations   Alt Y - File Transfer

Alt M - Monitor data           Alt Z - Toggle Full Screen

WEFAX Mode                     RTTY Mode

----------                     ---------

F1 - Capture/Pause             Alt I - Invert tones

F2 - Reset                     Alt L - Level shift

F3 - Save                      Alt S - Synch data

F4 - Clear                     Alt V - Verbose mode

F5 - Invert

F6 - Reverse                   SSTV Mode

F7 - Align                     ---------

F1 - Synch  F2 - Save  F3 - Clear

0..9 - Skip lines

Cursor Left - Move image left one column

Cursor Right - Move image right one column

CTRL Cursor Left - Move image left five columns

CTRL Cursor Right - Move image right five columns

ALT C - Toggle to CGA mode

ALT E - Toggle to EGA mode

ALT V - Toggle to VGA mode

TNC Driver V 1.42               91                        21/04/95

Appendix B - Summary of Editor Keys

==========   ----------------------

Key Action                 Primary Key               Secondary Key

------------------------------------------------------------------

Character left             P: <CtrlS>                S: <Lft>

Character right            P: <CtrlD>                S: <Rgt>

Word left                  P: <CtrlA>                S: <CtrlLft>

Word right                 P: <CtrlF>                S: <CtrlRgt>

Line up                    P: <CtrlE>                S: <Up>

Line down                  P: <CtrlX>                S: <Dn>

Scroll up                  P: <CtrlW>                S:

Scroll down                P: <CtrlZ>                S:

Page up                    P: <CtrlR>                S: <PgUp>

Page down                  P: <CtrlC>                S: <PgDn>

------------------------------------------------------------------

Cursor to left side        P: <CtrlQ><CtrlS>         S: <Home>

Cursor to right side       P: <CtrlQ><CtrlD>         S: <End>

Top of screen              P: <CtrlQ><CtrlE>         S: <CtrlHome>

Bottom of screen           P: <CtrlQ><CtrlX>         S: <CtrlEnd>

Top of window              P: <CtrlQ><CtrlR>         S: <CtrlPgUp>

Bottom of window           P: <CtrlQ><CtrlC>         S: <CtrlPgDn>

Top of block               P: <CtrlQ><CtrlB>         S:

Bottom of block            P: <CtrlQ><CtrlK>         S:

------------------------------------------------------------------

New line                   P: <CtrlM>                S:

Insert line                P: <CtrlN>                S:

Toggle insert mode         P: <CtrlV>                S: <Ins>

Delete line                P: <CtrlY>                S:

Delete line right          P: <CtrlQ><CtrlY>         S:

Delete right word          P: <CtrlT>                S:

Delete current character   P: <CtrlG>                S: <Del>

Delete left character      P: <CtrlBks>              S: <CtrlH>

------------------------------------------------------------------

Begin block                P: <CtrlK><CtrlB>         S:

End block                  P: <CtrlK><CtrlK>         S:

Toggle block display       P: <CtrlK><CtrlH>         S:

Copy block                 P: <CtrlK><CtrlC>         S:

Move block                 P: <CtrlK><CtrlV>         S:

Delete block               P: <CtrlK><CtrlY>         S:

Import from clipboard      P: <CtrlK><CtrlI>         S:

Export block to clipbrd    P: <CtrlK><CtrlE>         S:

Read file into window      P: <CtrlK><CtrlR>         S:

Write block to file        P: <CtrlK><CtrlW>         S:

------------------------------------------------------------------

Find pattern               P: <CtrlQ><CtrlF>         S:

Find and replace           P: <CtrlQ><CtrlA>         S:

Find next                  P: <CtrlL>                S:

Tab                        P: <CtrlI>                S:

Exit editor                P: <CtrlK><CtrlD>         S:

Abandon editor             P: <CtrlK><CtrlQ>         S:

Save file and continue     P: <CtrlK><CtrlS>         S:

Insert control char        P: <CtrlP>                S:

Editor help                P: <F1>                   S:

------------------------------------------------------------------

TNC Driver V 1.42               92                        21/04/95

Appendix C - Summary of Script Language

==========   --------------------------

WHEN <text>

-----------

This  keyword halts execution of the script file  until  the

<text> is monitored in the incoming TNC text.

REPLY <text>

------------

This keyword sends the <text> to the TNC.

DO

--

This  keyword  has  no parameters and forms a  loop  in  the

script file in conjunction with the UNTIL <text> keyword.

UNTIL <text>

------------

This  keyword  signifies the end of the DO ..  UNTIL  <text>

loop.  The  statements between DO and UNTIL  are  executed  until

<text> is monitored in the incoming TNC text.

GOTO <label>

------------

This keyword causes execution of the script file to continue

at the line defined with <label>.

:<label>

--------

This  keyword is used in conjunction with the  GOTO  keyword

and identifies a place in the script file.

DISPLAY <text>

--------------

This keyword will display <text> in the user window  without

sending it to the TNC.

PRINT <text>

------------

This  keyword  will  print <text>  to  the  printer  without

sending it to the TNC.

TNC Driver V 1.42               93                        21/04/95

READ <prompt>,<subject1>,<subject2>,...

---------------------------------------

This keyword is used to automatically issue 'read  messages'

commands  from a BBS message list, monitoring each received  line

for requested subject matter until the prompt is received.

IF <text>

---------

This keyword is used as part of the SCAN..ENDSCAN loop.

SCAN

----

This  keyword  has  no parameters and forms a  loop  in  the

script file in conjunction with the IF <text>, ENDSCAN  keywords.

Using  this  loop,  you can scan for up to  10  different  <text>

strings  at a time, and depending which one is found,  execute  a

separate  series  of script keywords.

ENDSCAN

-------

This  keyword  has  no parameters and forms a  loop  in  the

script file in conjunction with the IF <text> and SCAN keywords.

STOP

----

This  keyword has no parameters and stops execution  of  the

script file without completing it.

;

-

This  keyword tells the program to ignore the  remainder  of

the characters on the line and so is used to add comments to  the

script file.

LOG <file>

---

This  command  is  used in create  a  sequentially  numbered

capture file stored in the LOG directory.

ENDLOG

------

This closes the file created by the LOG keyword.

TNC Driver V 1.42               94                        21/04/95

WRITELOG <text>

--------

This command writes <text> to the current log file.

CLS

---

This command clears the TNC output window.

COLLECT <label>,<whoto1>,<whoto2>,...

-------

This keyword collects together all mail who address contains

any of the whoto's specified in the command ready for forwarding.

If there is no mail, a GOTO <label> command is performed.

FORWARD (BBS) (FBB)

-------

This  keyword forwards mail, performing BBS type  forwarding

if  the BBS parameter is specified and FBB compressed  forwarding

if the FBB parameter is specified.

REVERSE <label>

-------

This  keyword  reverse forwards mail to BBS's.  If  you  get

disconnected during the forward, a GOTO <label> is executed.

RUN <command>

---

This   keyword   will  execute  <command>  to   run   either

a .EXE, .COM, .BAT file or an internal DOS command.

FBBCOLLECT <label>,<subject1>,<subject2>,...

----------

This  keyword  scans the list of FBB header  broadcasts  for

messages containing any of the subjects specified in the  command

ready  for  grabbing.  If there is no messages,  a  GOTO  <label>

command is performed.

FBBREAD <label>

-------

This keyword downloads messages found during the  FBBCOLLECT

scan. If you get disconnected during the download, a GOTO <label>

is executed.

TNC Driver V 1.42               95                        21/04/95

Appendix D - Summary of PMS Commands

==========   -----------------------

@ - Enter sysop mode. (sysop only command)

*DOS commands for sysops only

-----------------------------

*DIR (directory)(filespec) - Display file directory

*TIME (HH:MM:SS) - Set or display the computers time

*DATE (DD-MM-YY) - Set or display the computers date

*CD (directory) - Set or change the current drive/directory

*MD <directory> - Create a new directory

*RD <directory> - Remove a directory

*TYPE <filename> - Display a file on-screen

*DEL <filename> - Delete a file

*REN <oldfile> <newfile> - Rename a file

*COPY <oldfile> <newfile> - Copy a file

*HELP - Display DOS command help summary.

A - Abort.

B - Log off the PMS.

C - Chat to the sysop if he is available.

(Command not available in local mode)

CM <message num> <callsign> - Copy message.

DA <area> <file> - Download file using ASCII.

DS - Display PMS stats.

DU - Display all user stats.

DU <callsign> - Display user stats.

DY <area> <file> - Download file using YAPP.

(Command not available in local mode)

EC - Toggle colour status

EH <homebbs> - Edit home bbs.

EN <name> (H <homebbs>) - Change user info.

TNC Driver V 1.42               96                        21/04/95

EM is for sysops only

---------------------

EM <num> U - Make unread

EM <num> R - Make read.

EM <num> F - Make forwarded.

EM <num> N - Make not forwarded.

EM <num> A - Make alive.

EM <num> K - Make killed.

EM <num> P - Toggle private/public status.

EM <num> S - Set next message number

EU is for sysops only

---------------------

EU <call> N <name> (H <homebbs>) - Create/Edit user

EU <call> H <homebbs> - Create/edit user.

EU <call> D - Delete user.

EU <call> B - Toggle BBS status

EU <call> S - Toggle sysop status

EU <call> X - Toggle expert status

EU <call> P - Generate Password

EU <call> C - Toggle colour status

EU <call> E - Toggle exclude status

F(V) <num> <area> <filename> - Convert message to a file.

(sysops only)

G - List gateways.

G <area> - Enter gateway.

H - Display help file.

? <command> - Display help on <command>.

I - Display info file.

K <num> - Kills message.

TNC Driver V 1.42               97                        21/04/95

KM - Kill my mail.

K 0 - kill all message base (sysops only).

L ( <num> <num> ) - Lists messages (optionally between range).

LM - List Mine.

LK - List killed. (sysops only).

LL <num> - List last <num> messages.

L< <callsign> - List messages from <callsign>.

L> <callsign> - List messages to <callsign>.

R <num> - Read message.

RK <num> - Read killed message (sysops only).

RM - Read Mine.

RA - Read all mail.

NOTE - Use V instead of R for verbose read command

S <call> - Send private message to station <call>.

SP <call> - Send private message to station <call>.

SB <call> - Send bulletin message to station <call>.

SR - Send reply to last message you read.

SPR - Send personal reply to last message you read.

SD <call> - Manually reply to a REQDIR

SF <call> - Manually reply to a REQFIL

T - Tidy message base (sysops only).

UA <area> <file> - Upload file using ASCII.

UY <area> <file> - Upload file using YAPP.

(Command not available in local mode)

V - Show version.

W - List file areas.

W <area> (filespec) - List files (matching filespec).

X - Toggle expert level.

TNC Driver V 1.42               98                        21/04/95

Appendix E - My TNC Parameters

==========   -----------------

8BITCONV ON

ALIAS    ON/ON

AX25L2V2 ON/ON

ABAUD    9600

AUTOLF   OFF

AXDELAY  5/5

AXHANG   180/180

BEACON   EVERY 15/EVERY 15

BKONDEL  ON

BTEXT    G7JJF (Jon) S-In-A, Notts. ** TNC Driver Support Board **

BUDLIST  OFF/OFF

BUDCALLS NONE

A/2 Link state is: DISCONNECTED

CANLINE  $18

CANPAC   $19

CD       SOFTWARE/SOFTWARE

CHECK    0/0

CMDTIME  1

CMSG     ON/ON

COMMAND  $03

CONLIST  OFF/OFF

CONMODE  CONVERS

CONOK    ON

CPACTIME OFF

CR       ON

CRLFSUP  OFF

CSTAMP   ON

CTEXT    [TNC-1.42-BFHM$]

If I don't reply soon, please type PMS to enter my PMS

>

CWID     EVERY 30/EVERY 30

DAYTIME  16-06-91  15:20:09

DAYTWEAK 0

DAYUSA   OFF

DBLDISC  OFF

DELETE   $08

DIGIPEAT ON/ON

DWAIT    0/0

ECHO     OFF

ESCAPE   OFF

EXTMODEM OFF

FLOW     OFF

FILTER   ON

FRACK    4/4

FULLDUP  OFF

GATEWAY  OFF

HBAUD    1200/1200

HEADERLN ON

HID      ON/ON

INTFACE  TERMINAL

KNTIMER  15

LCOK     ON

TNC Driver V 1.42               99                        21/04/95

LCSTREAM ON

LFADD    OFF

LLIST    OFF

MONITOR  ON/ON

MALL     ON/ON

MAXFRAME 4/4

MAXUSERS 1/1

MBEACON  ON/ON

MCON     OFF/OFF

MCOM     OFF/OFF

MRESP    OFF/OFF

MRPT     OFF/OFF

MSTAMP   OFF

MYCALL   G7JJF/G7JJF

MYALIAS  G7JJF/G7JJF

MYNODE   G7JJF-8

MYPBBS   G7JJF-2

MYGATE   G7JJF-7

NDWILD   OFF

NEWMODE  ON

NOMODE   OFF

NTEXT    Hello Welcome to Jon's Node.

NUCR     0

NULF     0

NUMNODES 4

ONERADIO OFF

PACLEN   235/235

PACTIME  AFTER 10

PARITY   4

PASS     $16

PASSALL  OFF/OFF

PBBS     34

PBPERSON OFF

PERSIST  50/63

PID      OFF/OFF

PORT     1

PTEXT    Hello, Please feel free to leave a message for Jon.

REDISPLA $12

RELINK   ON/ON

RESPTIME 5/5

RETRY    15/15

RING     ON

RNRTIME  0

SCREENL  0

SENDPAC  $0D

SLOTTIME 30/10

START    $11

STATSHRT OFF

STOP     $13

STREAMSW $7B/$7D

STREAMCA ON

STREAMEV OFF

SUPLIST  BOTH

SUPCALLS NONE

SWP      17,17,108/17,17,108

TNC Driver V 1.42               100                        21/04/95

TRACE    OFF

TRFLOW   OFF

TRIES    0

TXDELAY  36/80

TXFLOW   OFF

UNPROTO  CQ VIA NORMAN/CQ VIA NORMAN

USERS    2/1

XFLOW    OFF

XOFF     $13

XON      $11

TNC Driver V 1.42               101                        21/04/95

Appendix F - Closing Remarks

==========   ---------------

If  you have any problems using the software, first  of  all

please  read this manual thoroughly. The answer to most  of  your

questions  will  be hidden somewhere. If all else  fails,  please

contact  me  at  any of the addresses given below  for  help  and

advice.

I  would  also appreciate any input you would care  to  give

concerning  the program.  If you have any ideas or comments  that

would make it a better program, then please let me know.

*****************************************************

**** Remember To Support Your Local Packet Group ****

*****************************************************

**** If You Have Registered - Thank You, If Not - Please Do! ****

Jon Welch,                    AX25 : G7JJF @ GB7MAM.#23.GBR.EU

50 Quarrydale Road,           inet : jon@g7jjf.demon.co.uk

Sutton In Ashfield,           CompuServe ID : 100012,2607

Notts.

NG17 4DR.                     Tel : 01623 511712 after 6pm

Great Britain

21st April, 1995

TNC Driver V 1.42               102                        21/04/95

################